# 3GPP TSG-RAN WG3 Meeting #128	R3-253246

# Malta, MT, 19th – 23th May 2025

Title: 	(TP for BL CR to 38.412, 38.300, 38.410, 38.412, 38.401) Leftover issues on AIoT

Source: 	ZTE Corporation, China Telecom

Agenda item:	16.2

Document for:	Discussion and Decision

- Introduction

Based on the related discussion paper (R3-253245), we provide the following TP and a draft reply LS.

- TP to TS38.413

-----------------Start of the Change-----------------

9.2.x.1	INVENTORY REQUEST

This message is sent by the A-IoT CN node to trigger the NG-RAN node to perform inventory over the A-IoT radio.

Direction: A-IoT CN node  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AIOTF Identifier | M |  | 9.3.3.xx2 |  | YES | reject |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  | YES | reject |
| Inventory Request Transfer | M |  | OCTET STRING | Containing the Inventory Request Transfer IE specified in subclause 9.3.x.1 | YES | reject |
| A-IoT Transaction ID (FFS) | M |  |  | 9.3.1.aa3 | YES | reject |

9.2.x.2	INVENTORY RESPONSE

This message is sent by the NG-RAN node to report the successful outcome of the request from the INVENTORY REQUEST message.

Direction: NG-RAN node  A-IoT CN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AIOTF Identifier | M |  | 9.3.3.xx2 |  | YES | reject |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  | YES | reject |
| Inventory Response Transfer | M |  | OCTET STRING | Containing the Inventory Response Transfer IE specified in subclause 9.3.x.2 | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| A-IoT Transaction ID (FFS) | M |  |  | 9.3.1.aa3 | YES | reject |

9.2.x.3	INVENTORY FAILURE

This message is sent by the NG-RAN node to report the unsuccessful outcome of the request from the INVENTORY REQUEST message.

Direction: NG-RAN node  A-IoT CN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AIOTF Identifier | M |  | 9.3.3.xx2 |  | YES | reject |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  | YES | reject |
| Inventory Failure Transfer | M |  | OCTET STRING | Containing the Inventory Failure Transfer IE specified in subclause 9.3.x.3 | YES | ignore |
| Cause | M[FFS] |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| A-IoT Transaction ID (FFS) | M |  |  | 9.3.1.aa3 | YES | reject |

9.2.x.4	INVENTORY REPORT

This message is sent by the NG-RAN node to provide the inventory report to the A-IoT CN node.

Direction: NG-RAN node  A-IoT CN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AIOTF Identifier | M |  | 9.3.3.xx2 |  | YES | reject |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  | YES | reject |
| Inventory Report Transfer | M |  | OCTET STRING | Containing the Inventory Report Transfer IE specified in subclause 9.3.x.4 | YES | reject |
| A-IoT Transaction ID (FFS) | M |  |  | 9.3.1.aa3 | YES | reject |

9.2.x.5	COMMAND REQUEST

This message is sent by the A-IoT CN node to the gNB to trigger the command transmission over the A-IoT radio.

Direction: A-IoT CN node  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AIOTF Identifier | M |  | 9.3.3.xx2 |  | YES | reject |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  | YES | reject |
| Command Request Transfer | M |  | OCTET STRING | Containing the Command Request Transfer IE specified in subclause 9.3.6.5. | YES | reject |
| A-IoT Transaction ID | M |  |  | 9.3.1.aa3 | YES | reject |

9.2.x.6	COMMAND RESPONSE

This message is sent by the NG-RAN node to provide the command response to the A-IoT CN node.

Direction: NG-RAN node  A-IoT CN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AIOTF Identifier | M |  | 9.3.3.xx2 |  | YES | reject |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  | YES | reject |
| Command Response Transfer | M |  | OCTET STRING | Containing the Command Response Transfer IE specified in subclause 9.3.6.5. | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| A-IoT Transaction ID | M |  |  | 9.3.1.aa3 | YES | reject |

9.2.x.7	COMMAND FAILURE

This message is sent by the NG-RAN node to report the unsuccessful transmission of the command to the A-IoT CN node.

Direction: NG-RAN node  A-IoT CN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AIOTF Identifier | M |  | 9.3.3.xx2 |  | YES | reject |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  | YES | reject |
| Command Failure Transfer | M |  | OCTET STRING | Containing the Command Failure Transfer IE specified in subclause 9.3.6.5. | YES | ignore |
| Cause | M[FFS] |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| A-IoT Transaction ID | M |  |  | 9.3.1.aa3 | YES | reject |

-----------------Next of the Change-----------------

9.3.3.xx3	Requested Service Area Information

This IE includes the Requested Service Area Information for the A-IoT inventory.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Requested Reader List |  | 0..1 |  |  |
| >Requested Reader Item |  | 1..<maxnoofReaders> |  |  |
| >>Global gNB ID | M |  | 9.3.1.6 |  |
| >>Reader Index | M |  | 9.3.1.aa1 |  |
| Requested A-IoT Area List |  | 0..1 |  |  |
| >Requested A-IoT Area Item |  | 1..<maxnoofAIoTAreas> |  |  |
| >>A-IoT Area Code | M |  | 9.3.1.aa2 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofReaders | Maximum no. of Readers. Value is 65535. |
| maxnoofAIoTAreas | Maximum no. of A-IoT Areas. Value is 256. |

-----------------Next of the Change-----------------

9.3.1.aa2 A-IoT Area Code

This IE is used to uniquely identify an A-IoT area Code.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| AIOT service Area Code | M |  | OCTET STRING (SIZE(3)) |  |

9.2. 3.1.aa3	A-IoT Transaction ID

The A-IoT Transaction ID IE is used to associate all the messages belonging to the same procedure. Messages belonging to the same procedure shall use the same A-IoT Transaction ID.

The A-IoT Transaction ID is determined by the initiating peer of a procedure.

The A-IoT Transaction ID shall uniquely identify a procedure among all ongoing parallel procedures using the same procedure code, and initiated by the same protocol peer.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| A-IoT Transaction ID | M |  | INTEGER (0..32767) |  |

-----------------End of the Change-----------------

- TP to TS38.300

<<<<<<<<<<<<<<<<<<<< Start Change >>>>>>>>>>>>>>>>>>>>

16.xx	Support of Ambient IoT

16.xx.x1	Introduction

16.xx.x2	Architecture

The NG-RAN overall architecture as depicted in section 4.1 is applicable for Ambient IoT along the following adaptations and additions:

-	For the sake of supporting the provision of A-IoT services, the involved gNBs are enabled to support A-IoT, specifically, the gNB supports communication with A-IoT devices by means of A-IoT radio.

-	A gNB may or may not only support communication with A-IoT devices by means of A-IoT radio.

-	A gNB serves one or more readers.

-	As specified in TS 23.369 [xx], the A-IoT CN node which the gNB connects to is either the AIOTF (in case of direct connectivity with the AIOTF) or the AMF (in case of indirect connectivity with the AIOTF).

NOTE 1:	It is not expected a deployment will deploy both direct connectivity and indirect connectivity.

-	In this version of the specification A-IoT specific information is transported between the gNB and the A-IoT CN node via the NG-C interface, the protocol stack in section 4.3.1.2 applies. In case of indirect connectivity with the AIOTF, A-IoT specific information is transferred transparently via the AMF.

-	In this version of the specification, no A-IoT specific communication takes place between gNBs.

-	In this version of the specification, split gNB architecture is not supported.

The AIOTF may request the gNB to perform an A-IoT operation within a specific area (requested service area). The requested service area may be indicated by means of a list of A-IoT Areas and/or a list of readers.

The AIOTF is aware of the area within which the gNB supports provision of A-IoT services by means of OAM configuration. This area may be represented as A-IoT Areas and/or the readers supported by the gNB.

The AIOTF may also be aware of the reader’s location by means of OAM configuration.

Upon receiving neither the supported Area nor the reader ID list in Inventory Request message, the gNB selects all the serving readers.

Upon receiving only the supported Area in Inventory Request message, the gNB selects readers within the indicated area.

Upon receiving only the reader list in Inventory Request, the gNB selects the readers indicated by the indicated reader list.

Upon receiving both the supported Area and Reader ID list, the gNB selects the reader(s) taking the indicated Reader ID list into account.

16.xx.x3	Inventory procedure

Figure 16.xx.x3-1 depicts the basic communication between the gNB and the A-IoT CN node for the Inventory procedure.

Figure 16.xx.x3-1: Inventory procedure

1.	The A-IoT CN node initiates the Inventory procedure over NG-C by sending the Inventory Request message to the gNB. The Inventory Request message includes a Correlation ID, the AIoT Transaction ID and A-IOTF Identifier to identify the inventory session. The Inventory Request message also includes an A-IoT Device Identification Requested corresponding to the A-IoT device(s) which are targeted for the inventory.

Editor’s Note 3:	content of the A-IoT Device Identification Requested is FFS.

The Inventory Request message also includes Requested Service Area Information (A-IoT area list and/or reader list) and may include Inventory Assistance Information. The Assistance Information may provide an Approximate number of Target A-IoT devices, estimate of expected D2R message size (FFS).

The Inventory Request message may also include a follow-on command indicator to indicate that at least for one of the targeted A-IoT device(s) a Command procedure will follow.

Editor’s Note 4:	Specification of further parameters of the Inventory Request message (e.g., other assistance information from 5GC) needs further work.

2.	The gNB allocates and co-ordinates the usage of A-IoT radio resources.

3.	The gNB confirms the request from the A-IoT CN node by replying with the Inventory Response message. The Inventory Response message includes the AIOTF Identifier, the Correlation ID and the AIoT Transaction ID received in the Inventory Request message.

NOTE 2:	If the gNB is not able to perform the Inventory procedure, it rejects the request and sends an Inventory Failure message to the A-IoT CN node.

4.	The gNB performs the Inventory procedure towards the A-IoT device(s) over the A-IoT radio interface.

5./6.	Upon receiving the inventory result from the A-IoT device(s), the gNB sends the Inventory Report message to the A-IoT CN node. If the Inventory procedure concerns multiple A-IoT devices, multiple Inventory Report messages including the different AIoT Transaction IDs may be sent to the A-IoT CN node.

The Inventory Report message includes the AIOTF Identifier and the Correlation ID received in the Inventory Request message and further includes the A-IoT NAS PDU(s) carrying the inventoried A-IoT device ID(s).

If a follow-on Command indicator was received at step 1, the Inventory Report message also includes a RAN A-IoT device NGAP ID for each A-IoT device which was inventoried.

NOTE 3:	If the Inventory procedure concerns multiple A-IoT devices, an Inventory Report may include reports from multiple A-IoT devices,

16.xx.x4	Command procedure

Figure 16.xx.x4-1 depicts the basic communication between the gNB and the A-IoT CN node for the Command procedure.

Figure 16.xx.x4-1: Command procedure

The Command procedure addresses only one A-IoT device.

1.	Prior to the Command procedure, the Inventory procedure is performed involving the concerned device and potentially other A-IoT devices.

NOTE 4: the step 2 may happen in parallel of the Inventory procedure involving other A-IoT devices.

2.	The A-IoT CN node initiates the Command procedure over NG-C by sending the Command Request message to the gNB. The Command Request message includes the same Correlation ID and AIOTF Identifier as the ones used in its corresponding inventory procedure. The Command Request message also includes the AIoT Transaction ID which uniquely identifies the Command Request procedure among all ongoing parallel procedures.

The Command Request also includes the RAN A-IoT device NGAP ID and the A-IoT NAS PDU targeting the A-IoT device and may include some command assistance information. This command assistance information may contain the estimate of expected D2R message size, command type (FFS).

3.	The gNB allocates and co-ordinates the usage of A-IoT radio resources and performs the command procedure over the A-IoT radio interface.

4.	The gNB replies to the Command Request with the Command Response message including the AIOTF Identifier and the Correlation ID received in the Command Request and the RAN A-IoT device NGAP ID.

NOTE 5:	If the gNB is not able to perform the Command procedure, it rejects the request and sends a Command Failure message to the A-IoT CN node.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

- TP to TS38.410

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.411: "NG-RAN; NG layer 1".

[3]	3GPP TS 38.412: "NG-RAN; NG signalling transport".

[4]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP) ".

[5]	3GPP TS 38.414: "NG-RAN; NG data transport".

[6]	3GPP TS 23.502: "Procedures for the 5G system".

[7]	3GPP TS 38.300: "NR; Overall Description; Stage 2".

[8]	3GPP TS 23.501: "System Architecture for the 5G system".

[9]	3GPP TS 38.415: "PDU Session User Plane Protocol".

[10]	3GPP TS 38.455: "NR Positioning Protocol A (NRPPa)".

[11]	3GPP TS 36.300: "E-UTRAN); Overall description; Stage 2".

[12]	3GPP TS 23.247: “Architectural enhancements for 5G multicast-broadcast services; Stage 2”.

[x]	3GPP TS 23.369: “Architecture support for Ambient power-enabled Internet of Things”.

3	Definitions and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

AIOTF: as defined in TS 23.xxx [xx].

A-IoT CN node: as defined in TS 38.300 [7].

gNB: as defined in TS 38.300 [7].

NB-IoT: as defined in TS36.300 [11].

ng-eNB: as defined in TS 38.300 [7].

NG-RAN node: as defined in TS 38.300 [7].

UPF: as defined in TS 23.501 [8].

3.2	Abbreviations

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

5GC	5G Core Network

A-IoT	Ambient IoT

AIOTF	Ambient IoT Function

AMF	Access and Mobility Management Function

CIoT	Cellular IoT

CN	Core Network

DRX	Discontinuous Reception

IoT	Internet of Things

MBS	Multicast/Broadcast Service

MT	Mobile Terminated

NB-IoT	Narrow Band Internet of Things

NG-U	NG User plane interface

PTP	Point to Point

PTM	Point to Multipoint

QMC	QoE Measurement Collection

QoE	Quality of Experience

RIM	Remote Interference Management

SMF	Session Management Function

UP	User Plane

UPF	User Plane Function

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

4.3	NG interface specification objectives

The NG interface specification facilitates the following:

-	inter-connection of NG-RAN nodes with AMFs supplied by different manufacturers;

-	inter-connection of NG-RAN nodes with A-IoT CN nodes supplied by different manufacturers;

Editor’s Note: whether this section needs A-IoT specific content depends on which term is used for “A-IoT CN node”. If the usage of “AMF” is specified in stage-2 to cover also “A-IoT CN node”, there might be no update necessary.

-	separation of NG interface Radio Network functionality and Transport Network functionality to facilitate introduction of future technology.

## 4.4	NG interface capabilities

The NG interface supports:

-	procedures to establish, maintain and release NG-RAN part of PDU sessions;

-	procedures to perform intra-RAT handover and inter-RAT handover;

-	the separation of each UE on the protocol level for user specific signalling management;

-	the transfer of NAS signalling messages between UE and AMF;

-	mechanisms for resource reservation for packet data streams;

-	procedures to establish, maintain and release NG-RAN part of MBS sessions;

-	the transfer of A-IoT NAS signalling messages between A-IoT device and AIOTF;

-	procedures to perform A-IoT service requests and responses.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

5.31	Timing Synchronisation Status Reporting function

The Timing Synchronisation Status Reporting function enables the AMF to request the NG-RAN node to report the RAN timing synchronisation status information, and for the NG-RAN node to provide the RAN timing synchronisation status information to the AMF.

5.xx	A-IoT Service Management Function

The A-IoT Service Management function is responsible for handling A-IoT services.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

6.27	Timing Synchronisation Status Reporting procedures

The following procedures are used to report the RAN timing synchronisation status information:

-	Timing Synchronization Status;

-	Timing Synchronization Status Report.

6.xx1	A-IoT procedures

The following procedures are used for A-IoT services:

-	Inventory Request;

-	Inventory Report;

-	Command Request.

<<<<<<<<<<<<<<<<<<<< Last Change >>>>>>>>>>>>>>>>>>>>

8.3	NG-RAN NG interface: NG application protocol (NGAP) (TS 38.413)

TS 38.413 [4] specifies the radio network layer signalling procedures of the control plane between the NG-RAN node and the AMF, or between the NG-RAN node and the A-IoT CN node.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

- TP to TS38.412

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<First change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

# 7	Transport layer

SCTP (IETF RFC 4960 [2]) shall be supported as the transport layer of NG-C signalling bearer. The Payload Protocol Identifier (ppid) to be used by SCTP for the application layer protocol NGAP and for DTLS over SCTP (IETF RFC 6083 [8]) is assigned by IANA in [10]. The byte order of the ppid shall be big-endian.

SCTP refers to the Stream Control Transmission Protocol developed by the Sigtran working group of the IETF for the purpose of transporting various signalling protocols over IP network.

NG-RAN node and AMF shall support a configuration with a single SCTP association per NG-RAN node/AMF pair. Configurations with multiple SCTP endpoints per NG-RAN node/AMF pair should be supported. When configurations with multiple SCTP associations are supported, the AMF may request to dynamically add/remove SCTP associations between the NG-RAN node/AMF pair. Within the set of SCTP associations established between one AMF and NG-RAN node pair, the AMF may request the NG-RAN node to restrict the usage of SCTP association for certain types of NG-C signalling. If no restriction information is provided for an SCTP association, any type of NG-C signalling is allowed via the SCTP association. Selection of the SCTP association by the NG-RAN node and the AMF is specified in TS 23.501 [3] and TS 23.502 [4]. The NG-RAN node shall establish the SCTP association. The SCTP Destination Port number value to be used by NGAP is assigned by IANA in [11]. When the AMF requests to dynamically add additional SCTP associations between the NG-RAN node/AMF pair, the SCTP Destination Port number value may be the one assigned by IANA in [11], or any dynamic port value (IETF RFC 6335 [9]). When the configuration with multiple SCTP endpoints per NG-RAN node is supported and the NG-RAN node wants to add additional SCTP endpoints, the RAN configuration update procedure shall be the first NGAP procedure triggered on an additional TNLA of an already setup NG-C interface instance after the TNL association has become operational, and the AMF shall associate the TNLA to the NG-C interface instance using the included Global RAN node ID.

Between one AMF and NG-RAN node pair:

-	A single pair of stream identifiers shall be reserved over at least one SCTP association for the sole use of NGAP elementary procedures that utilize non UE-associated signalling.

-	At least one pair of stream identifiers over one or several SCTP associations shall be reserved for the sole use of NGAP elementary procedures that utilize UE-associated signallings. However, a few pairs (i.e. more than one) should be reserved.

-	For a single UE-associated signalling, the NG-RAN node shall use one SCTP association and one SCTP stream, and the SCTP association/stream should not be changed during the communication of the UE-associated signalling until after current SCTP association is failed or removed, or TNL binding update is performed as described in TS 23.502 [3].

Transport network redundancy can be achieved by SCTP multi-homing between two end-points, of which one or both is assigned with multiple IP addresses. SCTP end-points shall support a multi-homed remote SCTP end-point. For SCTP endpoint redundancy, an SCTP endpoint (in the NG-RAN node or AMF) may send an INIT, at any time for an already established SCTP association, which the other SCTP endpoint shall handle as defined in IETF RFC 4960 [2] in subclause 5.2.

The SCTP congestion control may, using an implementation specific mechanism, initiate higher layer protocols to reduce the signalling traffic at the source and prioritise certain messages.

All principles and requirements specified for non UE-associated signalling across the NG interface apply for A-IoT related signalling. If NG-C is established between an A-IoT enabled NG-RAN node and an AIOTF, as specified in TS 23.369 [x], all principles and requirements specified for the AMF for support of non UE-associated signalling apply for the AIOTF.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Change end>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

- TP to TS38.401

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

6.2.1	Principle of handling Application Protocol Identities

For UE-associated signalling, an Application Protocol Identity (AP ID) is allocated when a new UE-associated logical connection is created in either an NG-RAN node or an AMF. An AP ID shall uniquely identify a logical connection associated to a UE or A-IoT device over the NG interface or Xn interface within a node (NG-RAN node or AMF) or over the F1 interface or over the E1 interface or over the W1 interface. Upon receipt of a message that has a new AP ID from the sending node, the receiving node shall store the AP ID of the sending node for the duration of the logical connection. The receiving node shall assign the AP ID to be used to identify the logical connection associated to the UE and include it as well as the previously received new AP ID from the sending node, in the first returned message to the sending node. In all subsequent messages to and from sending node, both AP IDs of sending node and receiving node shall be included.

For MBS-associated logical connections of the E1 interface and the F1 interface the same principles for AP IDs apply as for UE-associated logical connections.

For A-IoT command operation related signalling, per A-IoT session (corresponding to an AIOTF identifier and a Correlation ID) and per A-IoT device, a RAN A-IoT Device NGAP ID to be used via the NG-C interface are introduced, as specified below.

The definitions of AP IDs as used on NG interface or Xn interface or F1 interface or E1 interface are shown below:

RAN UE NGAP ID:

A RAN UE NGAP ID shall be allocated so as to uniquely identify the UE over the NG interface within an gNB. When an AMF receives an RAN UE NGAP ID it shall store it for the duration of the UE-associated logical NG-connection for this UE. Once known to an AMF this is included in all UE associated NGAP signalling.

The RAN UE NGAP ID shall be unique within the logical NG-RAN node.

AMF UE NGAP ID:

An AMF UE NGAP ID shall be allocated so as to uniquely identify the UE over the NG interface within the AMF. When a NG-RAN node receives an AMF UE NGAP ID it shall store it for the duration of the UE-associated logical NG-connection for this UE. Once known to a NG-RAN node this ID is included in all UE associated NGAP signalling.

The AMF UE NGAP ID shall be unique within an AMF Set as specified in TS 23.501 [3].

Old NG-RAN node UE XnAP ID:

An Old NG-RAN node UE XnAP ID shall be allocated so as to uniquely identify the UE over the Xn interface within an old NG-RAN node. When a new NG-RAN node receives an Old NG-RAN node UE XnAP ID it shall store it for the duration of the UE-associated logical Xn-connection for this UE. Once known to a new NG-RAN node this ID is included in all UE associated XnAP signalling. The Old NG-RAN node UE XnAP ID shall be unique within the logical NG-RAN node.

New NG-RAN node UE XnAP ID:

A New NG-RAN node UE XnAP ID shall be allocated so as to uniquely identify the UE over the Xn interface within a new NG-RAN node. When an old NG-RAN node receives a New NG-RAN node UE XnAP ID it shall store it for the duration of the UE-associated logical Xn-connection for this UE. Once known to an old NG-RAN node this ID is included in all UE associated XnAP signalling. The New NG-RAN node UE XnAP ID shall be unique within the logical NG-RAN node.

Source NG-RAN node UE XnAP ID:

A Source NG-RAN node UE XnAP ID shall be allocated so as to uniquely identify the UE over the Xn interface within a source NG-RAN node. When a target NG-RAN node receives a Source NG-RAN node UE XnAP ID it shall store it for the duration of the UE-associated logical Xn-connection for this UE. Once known to a target NG-RAN node this ID is included in all UE associated XnAP signalling. The Source NG-RAN node UE XnAP ID shall be unique within the logical NG-RAN node.

Target NG-RAN node UE XnAP ID:

A Target NG-RAN node UE XnAP ID shall be allocated so as to uniquely identify the UE over the Xn interface within a target NG-RAN node. When a source NG-RAN node receives a Target NG-RAN node UE XnAP ID it shall store it for the duration of the UE-associated logical Xn-connection for this UE. Once known to a source NG-RAN node this ID is included in all UE associated XnAP signalling. The Target NG-RAN node UE XnAP ID shall be unique within the logical NG-RAN node.

M-NG-RAN node UE XnAP ID:

An M-NG-RAN node UE XnAP ID shall be allocated so as to uniquely identify the UE over the Xn interface within an M-NG-RAN node for dual connectivity. When an S-NG-RAN node receives an M-NG-RAN node UE XnAP ID it shall store it for the duration of the UE-associated logical Xn-connection for this UE. Once known to an S-NG-RAN node this ID is included in all UE associated XnAP signalling. The M-NG-RAN node UE XnAP ID shall be unique within the logical NG-RAN node.

S-NG-RAN node UE XnAP ID:

A S-NG-RAN node UE XnAP ID shall be allocated so as to uniquely identify the UE over the Xn interface within an S-NG-RAN node for dual connectivity. When an M-NG-RAN node receives a S-NG-RAN node UE XnAP ID it shall store it for the duration of the UE-associated logical Xn-connection for this UE. Once known to an M-NG-RAN node this ID is included in all UE associated XnAP signalling. The S-NG-RAN node UE XnAP ID shall be unique within the logical NG-RAN node.

gNB-CU UE F1AP ID:

A gNB-CU UE F1AP ID shall be allocated so as to uniquely identify the UE over the F1 interface within a gNB-CU. When a gNB-DU receives a gNB-CU UE F1AP ID it shall store it for the duration of the UE-associated logical F1-connection for this UE. The gNB-CU UE F1AP ID shall be unique within the gNB-CU logical node.

gNB-DU UE F1AP ID:

A gNB-DU UE F1AP ID shall be allocated so as to uniquely identify the UE over the F1 interface within a gNB-DU. When a gNB-CU receives a gNB-DU UE F1AP ID it shall store it for the duration of the UE-associated logical F1-connection for this UE. The gNB-DU UE F1AP ID shall be unique within the gNB-DU logical node.

gNB-CU-CP UE E1AP ID:

A gNB-CU-CP UE E1AP ID shall be allocated so as to uniquely identify the UE over the E1 interface within a gNB-CU-CP (respectively an ng-eNB-CU-CP, or an eNB-CP as defined in TS 36.401[28]). When a gNB-CU-UP (respectively an ng-eNB-CU-UP, or an eNB-UP as defined in TS 36.401[28]) receives a gNB-CU-CP UE E1AP ID it shall store it for the duration of the UE-associated logical E1-connection for this UE. The gNB-CU-CP UE E1AP ID shall be unique within the gNB-CU-CP (respectively the ng-eNB-CU-CP, or the eNB-CP as defined in TS 36.401[28]) logical node.

gNB-CU-UP UE E1AP ID:

A gNB-CU-UP UE E1AP ID shall be allocated so as to uniquely identify the UE over the E1 interface within a gNB-CU-UP (respectively an ng-eNB-CU-UP, or an eNB-UP as defined in TS 36.401[28]). When a gNB-CU-CP (respectively an ng-eNB-CU-CP, or an eNB-CP as defined in TS 36.401[28]) receives a gNB-CU-UP UE E1AP ID it shall store it for the duration of the UE-associated logical E1-connection for this UE. The gNB-CU-UP UE E1AP ID shall be unique within the gNB-CU-UP (respectively the ng-eNB-CU-UP, or the eNB-UP as defined in TS 36.401[28]) logical node.

ng-eNB-CU UE W1AP ID:

An ng-eNB-CU UE W1AP ID shall be allocated so as to uniquely identify the UE over the W1 interface within an ng-eNB-CU. When an ng-eNB-DU receives an ng-eNB-CU UE W1AP ID it shall store it for the duration of the UE-associated logical W1-connection for this UE. The ng-eNB-CU UE W1AP ID shall be unique within the ng-eNB-CU logical node.

ng-eNB-DU UE W1AP ID:

An ng-eNB-DU UE W1AP ID shall be allocated so as to uniquely identify the UE over the W1 interface within an ng-eNB-DU. When an ng-eNB-CU receives an ng-eNB-DU UE W1AP ID it shall store it for the duration of the UE-associated logical W1-connection for this UE. The ng-eNB-DU UE W1AP ID shall be unique within the ng-eNB-DU logical node.

gNB-CU MBS F1AP ID:

A gNB-CU MBS F1AP ID shall be allocated so as to uniquely identify the MBS Session Context over the F1 interface within a gNB-CU. When a gNB-DU receives a gNB-CU MBS F1AP ID it shall store it for the duration of the MBS-associated logical F1-connection for that MBS Session. The gNB-CU MBS F1AP ID shall be unique within the gNB-CU logical node.

gNB-DU MBS F1AP ID:

A gNB-DU MBS F1AP ID shall be allocated so as to uniquely identify the MBS Session Context over the F1 interface within a gNB-DU. When a gNB-CU receives a gNB-DU MBS F1AP ID it shall store it for the duration of the MBS-associated logical F1-connection for this MBS Session. The gNB-DU MBS F1AP ID shall be unique within the gNB-DU logical node.

gNB-CU-CP MBS E1AP ID:

A gNB-CU-CP MBS E1AP ID shall be allocated so as to uniquely identify the MBS Session Context over the E1 interface within a gNB-CU-CP. When a gNB-CU-UP receives a gNB-CU-CP MBS E1AP ID it shall store it for the duration of the MBS-associated logical E1-connection for that MBS Session. The gNB-CU-CP MBS E1AP ID shall be unique within the gNB-CU-CP logical node.

gNB-CU-UP MBS E1AP ID:

A gNB-CU-UP MBS E1AP ID shall be allocated so as to uniquely identify the MBS Session Context over the E1 interface within a gNB-CU-UP. When a gNB-CU-CP receives a gNB-CU-UP MBS E1AP ID it shall store it for the duration of the MBS-associated logical E1-connection for this MBS Session. The gNB-CU-UP MBS E1AP ID shall be unique within the gNB-CU-UP logical node.

A-IoT Correlation ID:

An A-IoT Correlation ID shall be allocated so as to uniquely identify an A-IoT session context. When a gNB receives an A-IoT Correlation ID it shall store it for the duration of the A-IoT service operation. The A-IoT Correlation ID shall be unique within an AIOTF as specified in TS 23.369 [z].

RAN A-IoT Device NGAP ID

A RAN A-IoT Device NGAP ID shall be allocated so as to uniquely identify an A-IoT device context over the NG interface within a NG-RAN node for a certain A-IoT session. When an AIOTF receives an RAN A-IoT Device NGAP ID, it shall store it for the duration of the A-IoT session.

The RAN A-IoT Device NGAP ID shall be unique per A-IoT session within the logical NG-RAN node.

Editor’s Note:	Introduction of AIOTF A-IoT Device NGAP ID is FFS.

A-IoT Transaction ID:

An A-IoT Transaction ID shall be allocated so as to uniquely identify an AIoT service procedure among all ongoing parallel procedures using the same AIOTF identifier and A-IoT Correlation ID. The A-IoT Transaction ID is determined by the initiating peer of a procedure.

<<<<<<<<<<<<<<<<<<<< Last Change >>>>>>>>>>>>>>>>>>>>

- Draft LS to SA2

=======================<Start of Draft reply LS to R3-253015>=================================

# 3GPP TSG-RAN WG3 Meeting #128	R3-25xxxx

# Malta, MT, 19th – 23th May 2025

Title:	[DRAFT] Reply LS on Ambient IoT progress of RAN3

Response to:	R3-253015/ S2-2504288

Release:	Rel-19

Work Item:	NR_Mob_Ph4-Core

Source:	ZTE (to be RAN3)

To:	SA2, SA5

Cc:	-

Contact Person:	Ma zijiang

E-mail Address:	ma.zijiang@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1. Overall Description:

RAN3 thanks SA2 for the LS Reply on Ambient IoT progress of RAN3.

RAN3 would like to answer the following questions.

- SA2 would ask whether the ‘supported Area’ is Tracking Area or a new type of area.

Answer: RAN3 agreed that the ‘the supported Area’ is a new type of area.

- SA2 would like to ask whether one reader is only part of one supported Area or can be part of multiple supported Areas.

Answer: RAN3 agreed that one reader can either part of one supported Area or part of multiple supported Areas.

# 2. Actions:

To SA2, SA5

ACTION: RAN3 kindly asks SA2 and SA5 to take the above information into account.

# 3. Date of Next TSG-RAN3 Meetings:

TSG-RAN3 Meeting #129	25 - 29 August 2025	Bengaluru, IN

=======================<End of Draft reply LS>=================================

=======================<Start of Draft reply LS to R3-253016>=================================

# 3GPP TSG-RAN WG3 Meeting #128	R3-25xxxx

# Malta, MT, 19th – 23th May 2025

Title:	[DRAFT] Reply LS on the removal of service type information

Response to:	R3-253016/ S2-2504294

Release:	Rel-19

Work Item:	NR_Mob_Ph4-Core

Source:	ZTE (to be RAN3)

To:	SA2

Cc:	RAN2, RAN1

Contact Person:	Ma zijiang

E-mail Address:	ma.zijiang@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1. Overall Description:

RAN3 thanks SA2 for the LS on the removal of service type information.

Question: SA2 would like to ask RAN2 and RAN3 whether there is any concern if SA2 agrees to remove “Service Type” from “Assistance Information”

Answer: RAN3 has no concern that SA2 agrees to remove “Service Type” from “Assistance Information”.

# 2. Actions:

To SA2

ACTION: RAN3 kindly asks SA2 to take the above information into account.

# 3. Date of Next TSG-RAN3 Meetings:

TSG-RAN3 Meeting #129	25 - 29 August 2025	Bengaluru, IN

=======================<End of Draft reply LS>=================================