3GPP TSG-RAN3#127bis Meeting	R3-253254

# Malta, MT, 19-23 May, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.443 | CR |  | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of time-frequency interleaving (TFI) LTE-based 5G terrestrial broadcast |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_terr_bcast_Ph2 |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As part of the WID on LTE-based 5G Broadcast Phase 2 (RP-243290), time-frequency interleaving (TFI) which severely impairs its performance in the presence of multipath and fading channels has been agreed to introduce in Rel-19. The current M2 signalling need to enhanced to support time-frequency interleaving function. |  |  |  |  |  |  |  |  |
| Summary of change: |  | One new TFI Configuration IE is introduced for PMCH Configuration to enable MCE configure time and or frequency interleave per PMCH. One new TFI Configuration IE is introduced for MBMS SESSION START REQUEST and MBMS SESSION UPDATE REQUEST messages to enable MCE configure time and or frequency interleave per session. Another one new MCH Scheduling Period Extended 3 IE is introduced for PMCH Configuration to enable MCE configure extra period for better interleave. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for LTE-based 5G Terrestrial Broadcast enhancements of time-frequency interleaving in TS 36.443. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2, 84, 8.10.2, 9.1.2, 9.1.21 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 36.331 CRXXX TS 36.306 CRXXX TS 36.211 CRXXX TS 36.312 CRXXX |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>> Start CHANGE <<<

8.2	MBMS Session Start

8.2.1	General

The purpose of the MBMS Session Start procedure is to request the eNB to notify Ues about an upcoming MBMS Session of a given MBMS Bearer Service and to establish an MBMS E-RAB and an MBMS-service-associated logical M2-connection. The MBMS Session Start procedure is triggered by the MCE.

The procedure uses MBMS-Service-associated signalling.

8.2.2	Successful Operation

Figure 8.2.2-1. MBMS Session Start procedure. Successful operation.

The MCE initiates the procedure by sending an MBMS SESSION START REQUEST message. If the eNB accepts the MBMS session start request, the eNB responds with the MBMS SESSION START RESPONSE message. The eNB shall select and join the IP multicast (identified by the IP multicast address and the IP address of the multicast source as described by the TNL Information IE or, if present, the Alternative TNL Information IE) to enable the reception of MBMS data.

If the MBMS SESSION START REQUEST message contains the MBMS Session Identity IE, the eNB shall use it for broadcast of the MBMS Session Identity on the air interface.

If the MBMS SESSION START REQUEST message contains the SC-PTM information IE, the eNB shall use SC-PTM for the related MBMS service in the relevant cells. The eNB shall establish or modify the resources according to the values of the Allocation and Retention Priority IE (priority level and pre-emption indicators) and the resource situation as follows:

-	The eNB shall consider the priority level of the requested session, when deciding on the resource allocation.

-	The priority levels and the pre-emption indicators may (individually or in combination) be used to determine whether the session has to be started unconditionally and immediately. If the requested session is marked as “may trigger pre-emption” and the resource situation requires so, the eNB may trigger the pre-emption procedure which may then cause the forced release of a lower priority session which is marked as “pre-emptable”. Whilst the process and the extent of the pre-emption procedure is operator-dependent, the pre-emption indicators shall be treated as follows:

1.	If the Pre-emption Capability IE is set to “may trigger pre-emption”, then this allocation request is allowed to trigger a pre-emption procedure.

2.	If the Pre-emption Capability IE is set to “shall not trigger pre-emption”, then this allocation request is not allowed to trigger a pre-emption procedure.

3.	If the Pre-emption Vulnerability IE is set to “pre-emptable”, then this session shall be included in the pre-emption process.

4.	If the Pre-emption Vulnerability IE is set to “not pre-emptable”, then this session shall not be included in the pre-emption process.

5.	If the Priority Level IE is set to “no priority” the given values for the Pre-emption Capability IE and Pre-emption Vulnerability IE shall not be considered. Instead the values “shall not trigger pre-emption” and “not pre-emptable” shall prevail.

-	The E-UTRAN pre-emption process shall keep the following rule: E-UTRAN shall only pre-empt sessions with lower priority, in ascending order of priority.

The eNB shall report to the MCE, in the MBMS SESSION START RESPONSE message the result of the requested MBMS E-RAB.

If the MBMS SESSION START REQUEST message contains the TFI Configuration IE, the eNB shall use it for broadcast of the MBMS Session with time and or frequency interleaving accordingly.

>>> NEXT CHANGE <<<

8.4	MBMS Scheduling Information

8.4.1	General

The purpose of the MBMS Scheduling Information Procedure is to provide MCCH related information, and optional session suspension decision to the eNB.

The procedure uses non MBMS-Service-associated signalling.

8.4.2	Successful Operation

Figure 8.4.2-1. MBMS Scheduling Information procedure. Successful operation.

The MCE initiates the procedure by sending the MBMS SCHEDULING INFORMATION message to the eNB. The eNB shall store the MBSFN Area Configuration Item IE, apply the MCCH update from the modification period defined in the MCCH Update Time IE, and transmit the MCCH according to the MCCH configuration for the given MBSFN area indicated by the MCE. If an empty PMCH Configuration List IE is contained in this message for an MBSFN area, the eNB shall update the content of the corresponding MCCH so as not to include PMCH related information over the corresponding MCCH. If the Modulation and Coding Scheme 2 IE is included, the eNB shall ignore the Modulation and Coding Scheme IE and use the Modulation and Coding Scheme 2 IE instead. If the received PMCH Configuration IE contains the MCH Scheduling Period Extended IE, the eNB shall take its value into account instead of the value signalled in the MCH Scheduling Period IE. If the received PMCH Configuration IE contains the MCH Scheduling Period Extended 2 IE, the eNB shall take its value into account instead of the value signalled in the MCH Scheduling Period IE. The eNB shall schedule the MBMS services in the MCCH according to the order defined in the MBMS Session List per PMCH IE. If the MBMS Suspension Notification List IE is included in this message for an MBSFN area, the eNB shall broadcast the suspension decision over the air interface from the radio frame defined by the SFN IE, until the end of the Modification Period just before the "MCCH Update Time".

If the MBMS SCHEDULING INFORMATION message contains the Subframe Allocation Extended IE included in the MBSFN Subframe Configuration IE, the eNB shall, if supported, store it and take it into account.

If the MBMS SCHEDULING INFORMATION message contains the TFI Configuration IE included in the PMCH Configuration IE, the eNB shall, if supported, store it and take it into account for time and or frequency interleaving for the PMCH.

>>> NEXT CHANGE <<<

8.10	MBMS Session Update

8.10.1	General

The purpose of the MBMS Session Update procedure is to inform the eNB about changing characteristics of the MBMS session.

The procedure uses MBMS-Service-associated signalling.

8.10.2	Successful Operation

Figure 8.10.2-1. MBMS Session Update procedure. Successful operation.

The MCE initiates the procedure by sending a MBMS SESSION UPDATE REQUEST message.

If the MBMS Service Area IE is included in the MBMS SESSION UPDATE REQUEST message, the eNB shall check the involvement of its cells in the new service area, update correspondingly the MBMS context and resources, join or leave the IP multicast if needed and send the MBMS SESSION UPDATE RESPONSE message.

If the TNL Information IE is included in the MBMS SESSION UPDATE REQUEST message, the eNB shall ignore the contained information.

If the MBMS SESSION UPDATE REQUEST message contains the MBMS Session Identity IE for an MBMS service, the eNB shall ignore the contained information.

If the MBMS SESSION UPDATE REQUEST message contains the SC-PTM information IE, the eNB shall check the SC-PTM related involvement of its cells in the new list of Cell identities, update correspondingly the MBMS context and resources, join or leave the IP multicast if needed and send the MBMS SESSION UPDATE RESPONSE message. If the ARP parameter is updated, the corresponding update of resources shall follow the principles described for the MBMS Session Start procedure. The eNB shall provide the MBMS session only in those cells included in the new list of Cell identities.

If the MBMS SESSION START REQUEST message contains the TFI Configuration IE, the eNB shall use it for broadcast of the MBMS Session with time and or frequency interleaving accordingly.

>>> NEXT CHANGE <<<

9.1.2	MBMS SESSION START REQUEST

This message is sent by the MCE to establish an MBMS-service-associated logical M2-connection.

Direction: MCE  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MCE MBMS M2AP ID | M |  | 9.2.3.1 |  | YES | reject |
| TMGI | M |  | 9.2.3.3 |  | YES | reject |
| MBMS Session Identity | O |  | 9.2.3.4 |  | YES | ignore |
| MBMS Service Area | M |  | 9.2.3.6 |  | YES | reject |
| TNL Information |  | 1 |  |  | YES | reject |
| >IP Multicast Address | M |  | IP Address 9.2.2.1 |  | - |  |
| >IP Source Address | M |  | IP Address 9.2.2.1 |  | - |  |
| >GTP DL TEID | M |  | GTP-TEID 9.2.2.2 |  | - |  |
| Alternative TNL Information | O |  |  |  | YES | ignore |
| >Alternative IP Multicast Address | M |  | IP Address 9.2.2.1 |  | - |  |
| >Alternative IP Source Address | M |  | IP Address 9.2.2.1 |  | - |  |
| >GTP DL TEID | M |  | GTP-TEID 9.2.2.2 | Shall be set to same value as the GTP DL TEID IE of the TNL Information IE. | - |  |
| SC-PTM information | O |  | 9.2.1.22 |  | YES | reject |
| TFI Configuration | O |  | 9.2.1.xx | The configuration for Time frequency interleaving. | YES | reject |

>>> NEXT CHANGE <<<

9.1.21	MBMS SESSION UPDATE REQUEST

This message is sent by the MCE to the eNB in order to inform of the change of session characteristics e.g. service area of one MBMS service session.

Direction: MCE  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MCE MBMS M2AP ID | M |  | 9.2.3.1 |  | YES | reject |
| eNB MBMS M2AP ID | M |  | 9.2.3.2 |  | YES | reject |
| TMGI | M |  | 9.2.3.3 |  | YES | reject |
| MBMS Session Identity | O |  | 9.2.3.4 |  | YES | ignore |
| MBMS Service Area | O |  | 9.2.3.6 |  | YES | ignore |
| TNL Information | O |  |  |  | YES | reject |
| >IP Multicast Address | M |  | 9.2.2.1 |  | - |  |
| >IP Source Address | M |  | IP Address 9.2.2.1 |  | - |  |
| >GTP DL TEID | M |  | GTP-TEID 9.2.2.2 |  | - |  |
| SC-PTM information | O |  | 9.2.1.22 |  | YES | reject |
| TFI Configuration | O |  | 9.2.1.xx | The configuration for Time frequency interleaving. | YES | reject |

>>> NEXT CHANGE <<<

9.2.1.8	PMCH Configuration

| This information element provided PMCH configuration related content for MCCH. IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Allocated Subframes End | M |  | INTEGER (0..1535) | Encoded as the sf-AllocEnd IE in TS 36.331 [11]. | − | − |
| Modulation and Coding Scheme | M |  | INTEGER (0..28) | Encoded as the dataMCS IE in TS 36.331 [11]. | − | − |
| MCH Scheduling Period | M |  | ENUMERATED (rf8, rf16, rf32, rf64, rf128, rf256, rf512, rf1024) | Encoded as the mch-SchedulingPeriod IE in TS 36.331 [11]. | − | − |
| Modulation and Coding Scheme 2 | O |  | INTEGER (0..27) | Encoded as the t2 in the dataMCS IE in TS 36.331 [11]. If this IE is present, the value signalled in the Modulation and Coding Scheme IE is ignored. | YES | reject |
| MCH Scheduling Period Extended | O |  | ENUMERATED (rf4, …) | Encoded as the mch-SchedulingPeriod-r12 IE in TS 36.331 [11]. If this IE is present, the value signalled in the MCH Scheduling Period IE is ignored. | YES | reject |
| MCH Scheduling Period Extended 2 | O |  | ENUMERATED (rf1, rf2, …) | Encoded as the mch-SchedulingPeriod-v14x0 IE in TS 36.331 [11]. If this IE is present, the value signalled in the MCH Scheduling Period IE is ignored. | YES | reject |
| MCH Scheduling Period Extended 3 | O |  | ENUMERATED (FFS rf…) | The new value depends on RAN1’s decision. | Yes | reject |
| TFI Configuration | O |  | 9.2.1.xx | The configuration for Time frequency interleaving. | YES | reject |

>>> NEXT CHANGE <<<

9.2.1.xx	Time frequency Interleaving Parameters(FFS)

This information element provides time and or frequency Interleaving information to the eNB in line with the xx IE as defined in TS 36.331 [11].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Time domain interleaving | O |  |  |  |
| >N-dimension of consecutive MBSFN subframe | M |  | ENUMERATED (1,2,4,8,16, …) | The same encoding as the xx IE as specified in TS 36.331 [11]. N=1 indicates no interleaving. FFS on new value. |
| >M-dimension of consecutive MBSFN subframe | M |  | ENUMERATED (4,8,16,32 …) | The same encoding as the xx IE as specified in TS 36.331 [11]. FFS on new value. |
| Frequency domain interleaving indicator | O |  | ENUMERATED (support, not support …) |  |

>>> End of CHANGE <<<