3GPP TSG-RAN3 Meeting #128	R3-253269

Malta, MT, 19-23 May, 2025

Agenda Item:	11.3

Source: 	Qualcomm Incorporated

Title: 	Discussion on remaining issues in AI/ML enabled CCO

Document for:	Discussion/Decision

# Background

Not much progress happened in the last meeting on the CCO open issues due to time constraints. In this paper we provide our views on the remaining open issues in AI/ML enabled CCO.

RAN3#127bis The maximum value of the Time interval for predicted CCO issue and future CCO state is 60s. Legacy UE performance measurement metrics can be reused for CCO. For the receiving side, whether gNB-CU can also provide to gNB-DU a future CCO state as assistance information to derive a matching future CCO state? - Option1: no need - Opton2: gNB-CU directly forwards the received further CCO state of neighbor cells to gNB-DU. - Option3: gNB-CU generates a recommend further CCO state and send to gNB-DU. Whether/what additional UE performance measurement metrics is needed? Timing information for predicted CCO issue is NOT needed to exchange over Xn? Whether the predicted CCO issue and /or future CCO state can be updated over Xn?

# Discussion

## Exchange Time for Predicted CCO Issue over Xn

It has been proposed since few meetings by some companies to send Time for Predicted CCO issue to neighbouring CU over Xn and neighbouring CU can pass the same information to neighbouring DU. Currently the Predicted CCO issue is passed as Predicted Coverage Modification cause. Some companies in the previous meeting mentioned that if Predicted CCO issue time is provided to the neighbouring node, the neighbouring node can decide on a CCO state change time to align with the time associated with predicted CCO issue. Additional information on “time for predicted CCO issue” at the source may assist the neighbouring nodes in better decision on future CCO state. It is ok to exchange this information between neighbours.

Exchange Time for Predicted CCO issue (“Time for Predicted CCO issue” IE) to the neighbouring CU over Xn along with future CCO state and associated time as an additional information to assist the neighbouring node to derive its own future CCO state at an appropriate future time instance.

## Future CCO State as Assistance Information in the Source

Some companies propose that gNB-CU-CP can generate the future CCO state as assistance information and send it to gNB-DU. It is unclear on the input parameters used by gNB-CU-CP to generate future CCO state. Moreover, OAM provides the CCO state index configuration to gNB-DU. If gNB-CU-CP needs to generate the future CCO state, OAM also needs to configure gNB-CU-CP with CCO state index values which would involve changes to the configuration model and SA5 impact. This is also not a legacy behaviour. There could also be a race condition of CU suggesting a different CCO state and OAM informing DU a different CCO State. Hence, we propose, that gNB-DU generates the future CCO state on its own and future CCO state need not be provided as assistance information from gNB-CU-CP.

The input parameters used by gNB-CU-CP to generate future CCO state is unclear. OAM provides the CCO state index configuration to gNB-DU. If gNB-CU-CP needs to generate the future CCO state, OAM also needs to configure gNB-CU-CP with CCO state index values which would involve changes to the OAM configuration model and SA5 impact. Race condition at gNB-DU is also a possibility if CU and OAM suggest different CCO state to gNB-DU. This will also result in duplication of functionality across gNB-CU-CP and gNB-DU which is not desired.

There is no need for gNB-CU to generate Future CCO state as assistance information and provide it to gNB-DU.

## Future CCO State from Source gNB to Receiving gNB

In the last meeting a new issue was discussed to send the future CCO state from the source gNB-CU to the receiving gNB-DU via receiving gNB-CU over F1. The received Future CCO state from the source is proposed to be sent to the receiving gNB-DU from receiving gNB-CU. It was stated that gNB-DU could use the received future CCO state to derive its own appropriate future CCO state.

However it is unclear why this requirement is not there in legacy CCO. In the legacy CCO, the receiving node interprets the new CCO state received as CCO issue and sends it to its gNB-DU. The receiving gNB-DU derives its own CCO state and sends it to receiving gNB-CU.

The motivation to send the future CCO state from the source to the receiving gNB-DU via receiving gNB-CU which is not the legacy CCO behavior, is unclear.

## Future CCO State Feedback

It was agreed in the last meeting to use legacy UE performance for feedback of predicted CCO issue and future CCO state. The coverage or capacity change is for a cell and not for a UE. With legacy UE performance it is unclear which cell the UE belongs to (whether it is the cell where the future CCO state was applied or a normal cell). It is useful to enhance the legacy UE performance by including the Cell ID where the UE belong to along with the performance metrics as feedback, so that the source can know if the UE is an impacted UE due to Coverage or capacity change in the cell where the UE currently is.

Along with legacy UE performance, legacy SON reports like RLF report, RACH failure reports can also be used to understand if coverage change has a positive impact on UEs or not.

Agree to include current NR-CGI of the UE in the legacy UE Performance as feedback for the predicted CCO issue and future CCO state exchanged between neighbours over Xn.

Legacy SON reports like RLF, RACH failure reports can also be used as feedback for the predicted CCO issue and future CCO state exchanged between neighbours over Xn. No spec impact is expected with this.

## Assistance Information for Prediction of CCO Issue

gNB-CU predicts the CCO issue and forwards it to the gNB-DU. gNB-CU predicts the CCO issue based on L3 measurements. gNB-DU has L1 measurements and beam level load information. Hence gNB-DU has a better visibility of the beam level information for detecting CCO issue. We therefore propose that gNB-DU can send affected beams based on L1 measurements as assistance information to gNB-CU.

gNB-CU uses L3 measurement information as one of the inputs to predict CCO issue. However gNB-CU lacks insight into the beam level information which is available in the gNB-DU. gNB-DU has better visibility of beam level information based on L1 measurements for detecting CCO issue.

For CCO issue prediction, gNB-DU can send assistance information to gNB-CU on affected beams based on L1 measurements. RAN3 to discuss including “Assistance Information on Affected beams” over F1 as input data for CCO issue prediction.

# Summary

Based on the discussion above, we have following observations:

Observation 1.	The input parameters used by gNB-CU-CP to generate future CCO state is unclear. OAM provides the CCO state index configuration to gNB-DU. If gNB-CU-CP needs to generate the future CCO state, OAM also needs to configure gNB-CU-CP with CCO state index values which would involve changes to the OAM configuration model and SA5 impact. Race condition at gNB-DU is also a possibility if CU and OAM suggest different CCO state to gNB-DU. This will also result in duplication of functionality across gNB-CU-CP and gNB-DU which is not desired.

Observation 2.	gNB-CU uses L3 measurement information as one of the inputs to predict CCO issue. However gNB-CU lacks insight into the beam level information which is available in the gNB-DU. gNB-DU has better visibility of beam level information based on L1 measurements for detecting CCO issue.

Based on these observations and discussion above, we have the following proposals:

Proposal 1:	Exchange Time for Predicted CCO issue (“Time for Predicted CCO issue” IE) to the neighbouring CU over Xn along with future CCO state and associated time as an additional information to assist the neighbouring node to derive its own future CCO state at an appropriate future time instance.

Proposal 2:	There is no need for gNB-CU to generate Future CCO state as assistance information and provide it to gNB-DU.

Proposal 3:	The motivation to send the future CCO state from the source to the receiving gNB-DU via receiving gNB-CU which is not the legacy CCO behavior, is unclear.

Proposal 4:	Agree to include current NR-CGI of the UE in the legacy UE Performance as feedback for the predicted CCO issue and future CCO state exchanged between neighbours over Xn.

Proposal 5:	Legacy SON reports like RLF, RACH failure reports can also be used as feedback for the predicted CCO issue and future CCO state exchanged between neighbours over Xn. No spec impact is expected with this.

Proposal 6:	For CCO issue prediction, gNB-DU can send assistance information to gNB-CU on affected beams based on L1 measurements. RAN3 to discuss including “Assistance Information on Affected beams” over F1 as input data for CCO issue prediction.