3GPP TSG-RAN WG3 Meeting #128	R3-253282

Malta, MT, 19-23, May, 2025

Agenda item:	9.2

Source: 	Samsung, Nokia, ZTE, China Unicom, China Telecom, Lenovo

Title: 	Discussion on QMC coordination for RRC segmentation in NR-DC

Document for:	Discussion and decision

Introduction

The QMC coordination for RRC segmentation in NR-DC has been continued to be discussed last meeting according to the Reply LS from RAN2 in R3-251516, and the following agreement is achieved,

Before sending RRC reconfiguration message to the UE, the configuring node for a QoE configuration will have to be in-sync with another node on the RRC segmentation status.

To be continued...

In this contribution, we further discuss the possible corrections to this open issue.

Discussion

According to the reply LS from RAN2 in R3-251516, we can observe that there are two understandings in RAN2 on e.g whether AppLayerMeasConfig-r17 IE for MCG configuration can contain rrc-SegAllowedSRB5-r18 parameter, which is explained by RAN2 as follows,

Based on it, there are two different understandings in RAN2:

- Understanding 1: rrc-SegAllowedSRB4-r17 is only used for an MCG, and rrc-SegAllowedSRB5-r18 is only used for SCG. But there is no restriction that MN should not include rrc-SegAllowedSRB5-r18 in AppLayerMeasConfig-r17 or SN should not include rrc-SegAllowedSRB4-r17 in AppLayerMeasConfig-r17. In order words, UE can receive both rrc-SegAllowedSRB4-r17 and rrc-SegAllowedSRB5-r18 in a common AppLayerMeasConfig-r17 from any node.

- Understanding 2: rrc-SegAllowedSRB4-r17 is only configured in an MCG configuration and rrc-SegAllowedSRB5-r18 is only configured in an SCG configuration.

Based on the discussion last meeting, companies still had different understandings and could not converge. So alternatively, companies tended to come up with a network-based solution that can be compatible with both Understandings.

Observation 1: There’s a willing to propose a network-based solution that can be compatible with both Understandings.

It should be noted that, when we talk about two understandings, both network-side and UE can have two different understandings.

According to the agreement achieved last meeting, before sending RRC Reconfiguration message to the UE, both MN and SN share the same information on the RRC segmentation status of both nodes. And it is a fact that the potential network-based solution just ensures the MN and SN share the same information on the RRC segmentation status, but cannot ensure what segmentation parameter is contained in the MCG/SCG configuration sent by MN/SN.

Observation 2: The agreement achieved last meeting just ensures the MN and SN share the same information on RRC segmentation status, but cannot ensure what segmentation parameter is contained in the MCG/SCG configuration sent to UE.

To be more specific, let’s assume the following Case 1,

- Before sending RRC Reconfiguration message to the UE, both MN and SN share the same information that both SRB4 and SRB5 enables the RRC segmentation; and,

- MN is going to send MCG configuration contained in the RRC Reconfiguration message to the UE.

- If MN has Understanding 2 and UE has Understanding 1, when the MN tries to configure the UE, even though the RRC segmentation is enabled over SRB5, upon receiving the MCG configuration without rrc-SegAllowedSRB5-r18 parameter, the UE will release the configuration on rrc-SegAllowedSRB5-r18 parameter and consider that the RRC segmentation is NOT enabled over SRB5.

In addition, let’s assume the following Case 2,

- Before sending RRC Reconfiguration message to the UE, both MN and SN share the same information that SRB4 enables the RRC segmentation while SRB5 doesn’t enable the RRC segmentation; and,

- MN is going to send MCG configuration contained in the RRC Reconfiguration message to the UE.

- If MN has Understanding 1 and UE has Understanding 2, when the MN tries to configure the UE, even though the RRC segmentation is disabled over SRB5, upon receiving the MCG configuration without rrc-SegAllowedSRB5-r18 parameter, the UE will not release the configuration on rrc-SegAllowedSRB5-r18 parameter and consider that the RRC segmentation is still enabled over SRB5.

As can be observed from Case 1 and Case 2, if the QoE configuring node and UE has different Understandings, the mismatch on RRC segmentation status is possible to happen, and such mismatch shall be avoided.

As a consequence, we have no choice but have to achieve consensus on one common understanding.

Observation 3: After sending MCG/SCG configurations, the mismatch on RRC segmentation status is possible to happen if the network and the UE has different Understandings. Such mismatch shall be avoided. Therefore, it is impossible to come up with any solution that is able to let both understandings co-exist.

Proposal 1: RAN3 should make a decision on which Understanding to use among MN, SN and UE.

Apparently, there are two understandings on the table,

- If we follow Understanding 1, the RRC segmentation status needs to be coordinated between MN and SN, and it is possible to contain both segmentation parameters in one MCG/SCG configuration sent to the UE.

- If we follow Understanding 2, the RRC segmentation status does not need to be coordinated between MN and SN, but instead the Need field for rrc-SegAllowedSRB4-r17 and rrc-SegAllowedSRB5-r18 should be corrected from “Need R” to “Need S”, as proposed in R2-2500850.

As a summary, if we follow Understanding 1, RAN3 correction is needed and RAN2 correction is not needed; while if we follow Understanding 2, RAN2 correction is needed and RAN3 correction is not needed.

Observation 4: If Understanding 1 is followed, RAN3 correction is needed and RAN2 correction is not needed; while if Understanding 2 is followed, RAN2 correction is needed and RAN3 correction is not needed.

As clearly stated in Reply LS from RAN2 in R3-251516, RAN2 will not make any correction and let RAN3 make corrections.

Observation 5: RAN2 has agreed to not make any RAN2 correction, and let RAN3 to make corrections.

Based on Observation 4 & 5, we believe it has been clear enough that Understanding 1 should be followed; otherwise, we have to revert RAN2 agreement and send LS to RAN2 to request them to do the correction. We just see little need to re-discuss this topic back and forth between RAN2 and RAN3. And the most straight-forward way is to let RAN3 makes the correction directly.

Observation 6: We are able to achieve simple solution to follow either of the Understandings, but it is more straight-forward to let RAN3 make correction directly.

Proposal 2: Using Understanding 1 as the common understanding is preferred.

If we follow Understanding 1, the simplest way is that the RRC segmentation status is only needed to be coordinated between MN and SN when one node intends to update its own segmentation status.

The advantage of this solution is that such coordination is not dependent on any of other QMC functionalities (such as leg switching), and in fact we do not expect any complex solution to solve the problem of segmentation status mismatch, which means our proposed solution is simple and efficient enough.

Proposal 3: One node may report to its peer node whether it enables RRC segmentation for QoE report in QoE coordination procedure. That is, one node includes segmentation allowed information for its own SRB in QMC Coordination Request, and the peer node includes segmentation allowed information for peer node’s SRB in QMC Coordination Response.

Conclusion

In this paper, we provides further considerations for QoE on support of NR-DC. The following observations are provided,

Observation 1: There’s a willing to propose a network-based solution that can be compatible with both Understandings.

Observation 2: The agreement achieved last meeting just ensures the MN and SN share the same information on RRC segmentation status, but cannot ensure what segmentation parameter is contained in the MCG/SCG configuration sent to UE.

Observation 3: After sending MCG/SCG configurations, the mismatch on RRC segmentation status is possible to happen if the network and the UE has different Understandings. Such mismatch shall be avoided. Therefore, it is impossible to come up with any solution that is able to let both understandings co-exist.

Proposal 1: RAN3 should make a decision on which Understanding to use among MN, SN and UE.

Observation 4: If Understanding 1 is followed, RAN3 correction is needed and RAN2 correction is not needed; while if Understanding 2 is followed, RAN2 correction is needed and RAN3 correction is not needed.

Observation 5: RAN2 has agreed to not make any RAN2 correction, and let RAN3 to make corrections.

Observation 6: We are able to achieve simple solution to follow either of the Understandings, but it is more straight-forward to let RAN3 make correction directly.

Proposal 2: Using Understanding 1 as the common understanding is preferred.

Proposal 3: One node may report to its peer node whether it enables RRC segmentation for QoE report in QoE coordination procedure. That is, one node includes segmentation allowed information for its own SRB in QMC Coordination Request, and the peer node includes segmentation allowed information for peer node’s SRB in QMC Coordination Response.

References

[1] R3-251516

[2] R2-2500850