3GPP TSG-RAN WG3 Meeting #128	R3-253287

Malta, MT, 19-23, May, 2025

Agenda item:	16.2

Source: 	Samsung

Title: 	(TP to BL CR for 38.300) Discussion on inventory and command related issues for AIoT

Document for:	Discussion and decision

Introduction

Last meeting discussed the inventory and command related aspect, and the following agreements and open issues are captured,

- WA: Include the Correlation Identifier IE outside of the Inventory related Transfer IEs in all the Inventory related messages.

- Include the AIOTF Identifier IE outside of the Inventory related Transfer IEs in all the Inventory related messages.

- The Inventory Request Transfer IE, also includes the following:

  - Device Identification for Paging (to enable paging for single device, a group of devices, all devices)

  - Requested Service Area Information (A-IoT Area list and/or Reader list)

  - Inventory Assistance Information (Approximates Number of Target A-IoT Devices).

- It is FFS whether the Inventory Assistance Information includes the “Estimate of Expected D2R Message Size”

- WA: Include the Correlation Identifier IE in both inside and outside of the Command related Transfer IEs in all the Command related messages.

- Include the AIOTF Identifier IE outside of the Command related Transfer IEs in all the command related messages.

- The Command Request Transfer IE, also includes the following:

  - An A-IoT NAS PDU

  - Command Assistance Information: (Estimate of Expected D2R Message Size)

  - It is FFS whether the Command Assistance Information includes the Command Type (read, write, disable…)

FFS on the Release/Cancel/End related procedure(s)/IE(s) for the A-IoT service/inventory/command.

In this contribution, we further discuss these open issues.

Discussion

Last meeting continued to discuss the inventory and command related aspects for AIoT, and a bunch of open issues are captured.

The first open issue is the WA achieved last meeting,

WA: Include the Correlation Identifier IE outside of the Inventory related Transfer IEs in all the Inventory related messages.

In our understanding, it is possible that multiple inventory requests happen in parallel. In order to distinguish different service request by AMF in case of indirect deployment, it is beneficial to also include Correlation ID outside of the container.

Actually similar reason also applies to the WA to include Correlation ID outside of the container for all Command related messages.

Proposal 1: Turn the following WA into agreement:

- WA: Include the Correlation Identifier IE outside of the Inventory related Transfer IEs in all the Inventory related messages.

- WA: Include the Correlation Identifier IE in both inside and outside of the Command related Transfer IEs in all the Command related messages.

The next open issues are about the Inventory/Command Assistance Information,

It is FFS whether the Inventory Assistance Information includes the “Estimate of Expected D2R Message Size”

It is FFS whether the Command Assistance Information includes the Command Type (read, write, disable…)

Regarding whether to include ‘Estimate of Expected D2R Message Size’, RAN3 receives the following LSes from RAN2 in R3-253029 and R3-253031 with the following RAN2 agreements,

In R3-253029,

RAN2 had discussed the length of Paging ID in an A-IoT paging message and reached the below agreement on paging ID length at RAN2#129bis meeting:

Agreements on paging ID length

- A field indicating Paging ID length information is always included together with the paging ID field in the A-IoT paging message, except the case where no ID is included in the A-IoT paging message.

In R3-253031,

RAN2 discussed segmentation of D2R messages and made the following assumption on inventory response message size.

Agreements on segmentation

- For inventory response, RAN2 assumes that segmentation is not applied.  RAN2 assumes that the reader can avoid segmentation by reader being aware of inventory response size.

RAN2 would like to inform SA2 and RAN3 that RAN2 assumes that the inventory response size can always be provided by the core network to the reader.

Based on information provided by RAN2, it is clear that including ‘Estimate of Expected D2R Message Size’ in Inventory Assistance Information is necessary.

Proposal 2: Include ‘Estimate of Expected D2R Message Size’ in the Inventory Assistance Information.

While for whether to include Command Type in Command Assistance Information, recall that we’ve had the following agreement,

Command Assistance Information: (Estimate of Expected D2R Message Size)

Our understanding for now is that the ‘Estimate of Expected D2R Message Size’ has already provided enough information to the NG-RAN node for further operations. In addition, SA2 has not identified any special handling towards different command types at the NG-RAN node side. So there’s no need to include Command type in Command Assistance Information for now, unless SA2 identifies something special.

Proposal 3: No need to include Command type in Command Assistance Information.

In addition, in the latest version of TR 23.700-13, the following text is captured in the conclusion part,

The AIOTF may provide the AIoT aggregation assistance information to the AIoT RAN. Aggregation assistance information is used to assist in the aggregation of AIoT responses received from multiple AIoT devices. The AIOTF may determine the aggregation assistance information based on operator policy, considering information provided from the AF, which consists of:

-	Time interval: The fixed time interval for which AIoT RAN collects AIoT Device responses before AIoT RAN reports the aggregated AIoT responses to AIOTF. This may potentially happen multiple times until the AIoT RAN completes the request operation. If the AF has provided a time interval, then the AIOTF should signal a time interval to AIOT RAN that is equal or shorter than the time interval received from the AF.

NOTE 8:	Further details how to use the AIoT aggregation assistance information are subject to RAN3 decision.

We can obtain that such time interval can be included in the Inventory Assistance Information, and the time interval is used by the NG-RAN node for the time interval between Inventory Reports.

Proposal 4: Include ‘Time Interval between Reports’ in the Inventory Assistance Information.

The next open issue is as follows,

FFS on the Release/Cancel/End related procedure(s)/IE(s) for the A-IoT service/inventory/command.

We notice that SA2 has captured the following Note in the conclusion part of TR 23.700-13,

NOTE 10:	AIOTF is expected to receive an end indication from AIoT RAN, signaling the completion of reporting for a specific correlation ID.

As per SA2 requirement, the end indication is only used for inventory reporting. So the only procedure that needs termination is the Inventory Report; and there’s no need to introduce End mechanism for other inventory/command procedures.

In order to end the Inventory Report procedure, it is straightforward to add End indication both inside and outside of the container in INVENTORY REPORT message.

Proposal 5: Add End Indication both inside and outside of the container in INVENTORY REPORT message.

The last open issue is actually identified by RAN2, as per the conclusion of RAN2 email discussion in previous meeting,

Wait for SA2/RAN3 progress before further discussing on how the reader calculates transaction ID.

Recall that according to the RAN2 agreement in RAN2#129 meeting, the following agreement is captured,

- Agreements

- The “transaction ID” can be generated by reader based on CN corelation ID.

According to RAN2 progress, a new Transaction ID, which is contained at least in AIoT Paging message over AIoT radio, is introduced by RAN2. And this Transaction ID is generated from and shorter than the Correlation ID in order to save signaling overhead over AIoT radio.

In general, when the NG-RAN node, as the BS reader, receives the Correlation ID from AIOTF, the NG-RAN node needs to generate the Transaction ID based on the Correlation ID.

On how the network side calculates the Transaction ID, there are several options,

- Option 1: AIOTF calculates the Transaction ID directly, and sends the Transaction ID together with Correlation ID to the NG-RAN node in INVENTORY REQUEST message.

- Option 2: AIOTF provides some assistance information on calculating the Transaction ID to the NG-RAN node in INVENTORY REQUEST message, and the NG-RAN node calculates the Transaction ID based on Correlation ID and assistance information.

- Option 3: No enhancement in INVENTORY REQUEST message, and the NG-RAN node calculates the Transaction ID based on Correlation ID according to some principles (e.g. Use x-bit LSBs of the Correlation ID as the Transaction ID).

- Option 4: No enhancement in INVENTORY REQUEST message, and the NG-RAN node calculates the Transaction ID based on implementation in this release.

We believe this issue should be discussed as soon as possible, since further inter WG coordination is expected.

Proposal 6: RAN3 is kindly asked to discuss the following options on how to calculate AIoT radio Transaction ID by the network,

- Option 1: AIOTF calculates the Transaction ID directly, and sends the Transaction ID together with Correlation ID to the NG-RAN node in INVENTORY REQUEST message.

- Option 2: AIOTF provides some assistance information on calculating the Transaction ID to the NG-RAN node in INVENTORY REQUEST message, and the NG-RAN node calculates the Transaction ID based on Correlation ID and assistance information.

- Option 3: No enhancement in INVENTORY REQUEST message, and the NG-RAN node calculates the Transaction ID based on Correlation ID according to some principles (e.g. Use x-bit LSBs of the Correlation ID as the Transaction ID).

- Option 4: No enhancement in INVENTORY REQUEST message, and the NG-RAN node calculates the Transaction ID based on implementation in this release.

Conclusion

In this paper, we provide further discussion on inventory and command related aspects for Ambient IoT. The following proposals are provided,

Proposal 1: Turn the following WA into agreement:

- WA: Include the Correlation Identifier IE outside of the Inventory related Transfer IEs in all the Inventory related messages.

- WA: Include the Correlation Identifier IE in both inside and outside of the Command related Transfer IEs in all the Command related messages.

Proposal 2: Include ‘Estimate of Expected D2R Message Size’ in the Inventory Assistance Information.

Proposal 3: No need to include Command type in Command Assistance Information.

Proposal 4: Include ‘Time Interval between Reports’ in the Inventory Assistance Information.

Proposal 5: Add End Indication both inside and outside of the container in INVENTORY REPORT message.

Proposal 6: RAN3 is kindly asked to discuss the following options on how to calculate AIoT radio Transaction ID by the network,

- Option 1: AIOTF calculates the Transaction ID directly, and sends the Transaction ID together with Correlation ID to the NG-RAN node in INVENTORY REQUEST message.

- Option 2: AIOTF provides some assistance information on calculating the Transaction ID to the NG-RAN node in INVENTORY REQUEST message, and the NG-RAN node calculates the Transaction ID based on Correlation ID and assistance information.

- Option 3: No enhancement in INVENTORY REQUEST message, and the NG-RAN node calculates the Transaction ID based on Correlation ID according to some principles (e.g. Use x-bit LSBs of the Correlation ID as the Transaction ID).

- Option 4: No enhancement in INVENTORY REQUEST message, and the NG-RAN node calculates the Transaction ID based on implementation in this release.

TP to BL CR for TS 38.300

16.xx	Support of Ambient IoT

16.xx.x1	Introduction

16.xx.x2	Architecture

The NG-RAN overall architecture as depicted in section 4.1 is applicable for Ambient IoT along the following adaptations and additions:

-	For the sake of supporting the provision of A-IoT services, the involved gNBs are enabled to support A-IoT, specifically, the gNB supports communication with A-IoT devices by means of A-IoT radio.

-	A gNB may or may not only support communication with A-IoT devices by means of A-IoT radio.

-	A gNB serves one or more readers.

-	As specified in TS 23.369 [xx], the A-IoT CN node which the gNB connects to is either the AIOTF (in case of direct connectivity with the AIOTF) or the AMF (in case of indirect connectivity with the AIOTF).

NOTE 1:	It is not expected a deployment will deploy both direct connectivity and indirect connectivity.

-	In this version of the specification A-IoT specific information is transported between the gNB and the A-IoT CN node via the NG-C interface, the protocol stack in section 4.3.1.2 applies. In case of indirect connectivity with the AIOTF, A-IoT specific information is transferred transparently via the AMF.

-	In this version of the specification, no A-IoT specific communication takes place between gNBs.

-	In this version of the specification, split gNB architecture is not supported.

The AIOTF may request the gNB to perform an A-IoT operation within a specific area (requested service area). The requested service area may be indicated by means of a list of A-IoT Area Identities and/or a list of readers.

The AIOTF is aware of the area within which the gNB supports provision of A-IoT services by means of OAM configuration. This area may be represented as A-IoT Areas and/or the readers supported by the gNB.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

16.xx.x3	Inventory procedure

Figure 16.xx.x3-1 depicts the basic communication between the gNB and the A-IoT CN node for the Inventory procedure.

Figure 16.xx.x3-1: Inventory procedure

1.	The A-IoT CN node initiates the Inventory procedure over NG-C by sending the Inventory Request message to the gNB. The Inventory Request message includes a Correlation ID and A-IOTF Identifier to identify the inventory session. The Inventory Request message also includes an A-IoT Device Identification Requested corresponding to the A-IoT device(s) which are targeted for the inventory.

Editor’s Note 3:	content of the A-IoT Device Identification Requested is FFS.

The Inventory Request message also includes Requested Service Area Information (A-IoT area list and/or reader list) and may include Inventory Assistance Information. The Assistance Information may provide an Approximate number of Target A-IoT devices, estimate of expected D2R message size, time interval between inventory reports.

The Inventory Request message may also include a follow-on command indicator to indicate that at least for one of the targeted A-IoT device(s) a Command procedure will follow.

Editor’s Note 4:	Specification of further parameters of the Inventory Request message (e.g., other assistance information from 5GC) needs further work.

2.	The gNB allocates and co-ordinates the usage of A-IoT radio resources.

3.	The gNB confirms the request from the A-IoT CN node by replying with the Inventory Response message. The Inventory Response message includes the AIOTF Identifier and the Correlation ID received in the Inventory Request message.

NOTE 2:	If the gNB is not able to perform the Inventory procedure, it rejects the request and sends an Inventory Failure message to the A-IoT CN node.

4.	The gNB performs the Inventory procedure towards the A-IoT device(s) over the A-IoT radio interface.

5./6.	Upon receiving the inventory result from the A-IoT device(s), the gNB sends the Inventory Report message to the A-IoT CN node. If the Inventory procedure concerns multiple A-IoT devices, multiple Inventory Report messages may be sent to the A-IoT CN node.

The Inventory Report message includes the AIOTF Identifier and the Correlation ID received in the Inventory Request message and further includes the A-IoT NAS PDU(s) carrying the inventoried A-IoT device ID(s).

If a follow-on Command indicator was received at step 1, the Inventory Report message also includes a RAN A-IoT device NGAP ID for each A-IoT device which was inventoried.

NOTE 3:	If the Inventory procedure concerns multiple A-IoT devices, an Inventory Report may include reports from multiple A-IoT devices,

16.xx.x4	Command procedure

Figure 16.xx.x4-1 depicts the basic communication between the gNB and the A-IoT CN node for the Command procedure.

Figure 16.xx.x4-1: Command procedure

The Command procedure addresses only one A-IoT device.

1.	Prior to the Command procedure, the Inventory procedure is performed involving the concerned device and potentially other A-IoT devices.

NOTE 4: the step 2 may happen in parallel of the Inventory procedure involving other A-IoT devices.

2.	The A-IoT CN node initiates the Command procedure over NG-C by sending the Command Request message to the gNB. The Command Request message includes the same Correlation ID and AIOTF Identifier as the ones used in its corresponding inventory procedure.

The Command Request also includes the RAN A-IoT device NGAP ID and the A-IoT NAS PDU targeting the A-IoT device and may include some command assistance information. This command assistance information may contain the estimate of expected D2R message size.

3.	The gNB allocates and co-ordinates the usage of A-IoT radio resources and performs the command procedure over the A-IoT radio interface.

4.	The gNB replies to the Command Request with the Command Response messageincluding the AIOTF Identifier and the Correlation ID received in the Command Request and the RAN A-IoT device NGAP ID.

NOTE 5:	If the gNB is not able to perform the Command procedure, it rejects the request and sends a Command Failure message to the A-IoT CN node.