3GPP TSG-RAN WG3 Meeting #128	R3-253288

Malta, MT, 19-23, May, 2025

Agenda item:	16.2

Source: 	Samsung

Title: 	(TP to BL CR for 38.401) Discussion on device associations for AIoT

Document for:	Discussion and decision

Introduction

Last meeting discussed the device association related aspect, and the following agreements and open issues are captured,

- WA: Include the Correlation Identifier IE in both inside and outside of the Command related Transfer IEs in all the Command related messages.

- Include the AIOTF Identifier IE outside of the Command related Transfer IEs in all the command related messages.

- Introduce per-session per-device Device Association (“RAN NGAP Device ID”, and “(FFS) CN NGAP Device ID” pair) between gNB and AIOTF, in case of Command after inventory.

- In case of command after inventory, the gNB allocates and provides a “RAN NGAP Device ID” in the Inventory Report Transfer IE for each device.

- The Device Association is included in the Command Request Transfer IE and Command Response Transfer IE.

- It is FFS whether to introduce the Device Association or a transaction ID, outside of the Transfer IEs, to allow parallel Command Request procedures for different devices between gNB and AMF.

In this contribution, we further discuss these open issues.

Discussion

Last meeting primarily discussed the device association aspects for AIoT, and a bunch of open issues are captured.

It should be noted that it has been agreed to introduce per-session per-device Device Association between gNB and AIOTF, and RAN NGAP Device ID has already been introduced to identify the newly introduced NG-RAN node AIoT device context.

The remaining issue now is whether an additional CN NGAP Device ID should be introduced as device association over NGAP.

In our understanding, the answer to this open issue depends on whether the RAN NGAP Device ID alone is able to uniquely identify the device by both NG-RAN node and AIOTF.

As given by the latest BL CR to 38.401, the RAN NGAP Device ID is allocated to uniquely identify an A-IoT device context over the NG interface within a NG-RAN node for a certain A-IoT session; therefore, by definition currently there’s no ID defined to be used to uniquely identify an A-IoT device context over the NG interface within an AIOTF for a certain A-IoT session. As a result, either an AIOTF NGAP Device ID is introduced or the RAN NGAP Device ID is reused to uniquely identify an A-IoT device context over the NG interface within an AIOTF.

In our opinion, it is more proper to introduce a new AIOTF NGAP Device ID, i.e. introduce a Device ID pair over NGAP to identify the device association, by following legacy ID pair design logic over e.g. F1AP.

Proposal 1: Introduce RAN NGAP Device ID and AIOTF NGAP Device ID pair between gNB and AIOTF, in case of Command after inventory.

The next open issue is,

It is FFS whether to introduce the Device Association or a transaction ID, outside of the Transfer IEs, to allow parallel Command Request procedures for different devices between gNB and AMF.

In our understanding, it is apparent that more flexibility will be given to NG-RAN node to allow parallel Command Request procedures. So it is beneficial to introduce some information outside of the Transfer IEs.

Regarding what information is introduced outside of the Transfer IEs, we think the Device Association, i.e. RAN NGAP Device ID and AIOTF NGAP Device ID pair is the most straightforward information to identify the device over NGAP.

Proposal 2: Introduce RAN NGAP Device ID and AIOTF NGAP Device ID pair outside of the Transfer IEs in Command Request procedure.

Conclusion

In this paper, we provide further discussion on device associations for Ambient IoT. The following proposals are provided,

Proposal 1: Introduce RAN NGAP Device ID and AIOTF NGAP Device ID pair between gNB and AIOTF, in case of Command after inventory.

Proposal 2: Introduce RAN NGAP Device ID and AIOTF NGAP Device ID pair outside of the Transfer IEs in Command Request procedure.

TP to BL CR for TS 38.401

6.2.1	Principle of handling Application Protocol Identities

For UE-associated signalling, an Application Protocol Identity (AP ID) is allocated when a new UE-associated logical connection is created in either an NG-RAN node or an AMF. An AP ID shall uniquely identify a logical connection associated to a UE over the NG interface or Xn interface within a node (NG-RAN node or AMF) or over the F1 interface or over the E1 interface or over the W1 interface. Upon receipt of a message that has a new AP ID from the sending node, the receiving node shall store the AP ID of the sending node for the duration of the logical connection. The receiving node shall assign the AP ID to be used to identify the logical connection associated to the UE and include it as well as the previously received new AP ID from the sending node, in the first returned message to the sending node. In all subsequent messages to and from sending node, both AP IDs of sending node and receiving node shall be included.

For MBS-associated logical connections of the E1 interface and the F1 interface the same principles for AP IDs apply as for UE-associated logical connections.

For A-IoT command operation related signalling, per A-IoT session (corresponding to an AIOTF identifier and a Correlation ID) and per A-IoT device, a RAN A-IoT Device NGAP ID and an AIOTF A-IoT Device NGAP ID to be used via the NG-C interface are introduced.

The definitions of AP IDs as used on NG interface or Xn interface or F1 interface or E1 interface are shown below:

RAN UE NGAP ID:

A RAN UE NGAP ID shall be allocated so as to uniquely identify the UE over the NG interface within an gNB. When an AMF receives an RAN UE NGAP ID it shall store it for the duration of the UE-associated logical NG-connection for this UE. Once known to an AMF this is included in all UE associated NGAP signalling.

The RAN UE NGAP ID shall be unique within the logical NG-RAN node.

AMF UE NGAP ID:

An AMF UE NGAP ID shall be allocated so as to uniquely identify the UE over the NG interface within the AMF. When a NG-RAN node receives an AMF UE NGAP ID it shall store it for the duration of the UE-associated logical NG-connection for this UE. Once known to a NG-RAN node this ID is included in all UE associated NGAP signalling.

The AMF UE NGAP ID shall be unique within an AMF Set as specified in TS 23.501 [3].

Old NG-RAN node UE XnAP ID:

An Old NG-RAN node UE XnAP ID shall be allocated so as to uniquely identify the UE over the Xn interface within an old NG-RAN node. When a new NG-RAN node receives an Old NG-RAN node UE XnAP ID it shall store it for the duration of the UE-associated logical Xn-connection for this UE. Once known to a new NG-RAN node this ID is included in all UE associated XnAP signalling. The Old NG-RAN node UE XnAP ID shall be unique within the logical NG-RAN node.

New NG-RAN node UE XnAP ID:

A New NG-RAN node UE XnAP ID shall be allocated so as to uniquely identify the UE over the Xn interface within a new NG-RAN node. When an old NG-RAN node receives a New NG-RAN node UE XnAP ID it shall store it for the duration of the UE-associated logical Xn-connection for this UE. Once known to an old NG-RAN node this ID is included in all UE associated XnAP signalling. The New NG-RAN node UE XnAP ID shall be unique within the logical NG-RAN node.

Source NG-RAN node UE XnAP ID:

A Source NG-RAN node UE XnAP ID shall be allocated so as to uniquely identify the UE over the Xn interface within a source NG-RAN node. When a target NG-RAN node receives a Source NG-RAN node UE XnAP ID it shall store it for the duration of the UE-associated logical Xn-connection for this UE. Once known to a target NG-RAN node this ID is included in all UE associated XnAP signalling. The Source NG-RAN node UE XnAP ID shall be unique within the logical NG-RAN node.

Target NG-RAN node UE XnAP ID:

A Target NG-RAN node UE XnAP ID shall be allocated so as to uniquely identify the UE over the Xn interface within a target NG-RAN node. When a source NG-RAN node receives a Target NG-RAN node UE XnAP ID it shall store it for the duration of the UE-associated logical Xn-connection for this UE. Once known to a source NG-RAN node this ID is included in all UE associated XnAP signalling. The Target NG-RAN node UE XnAP ID shall be unique within the logical NG-RAN node.

M-NG-RAN node UE XnAP ID:

An M-NG-RAN node UE XnAP ID shall be allocated so as to uniquely identify the UE over the Xn interface within an M-NG-RAN node for dual connectivity. When an S-NG-RAN node receives an M-NG-RAN node UE XnAP ID it shall store it for the duration of the UE-associated logical Xn-connection for this UE. Once known to an S-NG-RAN node this ID is included in all UE associated XnAP signalling. The M-NG-RAN node UE XnAP ID shall be unique within the logical NG-RAN node.

S-NG-RAN node UE XnAP ID:

A S-NG-RAN node UE XnAP ID shall be allocated so as to uniquely identify the UE over the Xn interface within an S-NG-RAN node for dual connectivity. When an M-NG-RAN node receives a S-NG-RAN node UE XnAP ID it shall store it for the duration of the UE-associated logical Xn-connection for this UE. Once known to an M-NG-RAN node this ID is included in all UE associated XnAP signalling. The S-NG-RAN node UE XnAP ID shall be unique within the logical NG-RAN node.

gNB-CU UE F1AP ID:

A gNB-CU UE F1AP ID shall be allocated so as to uniquely identify the UE over the F1 interface within a gNB-CU. When a gNB-DU receives a gNB-CU UE F1AP ID it shall store it for the duration of the UE-associated logical F1-connection for this UE. The gNB-CU UE F1AP ID shall be unique within the gNB-CU logical node.

gNB-DU UE F1AP ID:

A gNB-DU UE F1AP ID shall be allocated so as to uniquely identify the UE over the F1 interface within a gNB-DU. When a gNB-CU receives a gNB-DU UE F1AP ID it shall store it for the duration of the UE-associated logical F1-connection for this UE. The gNB-DU UE F1AP ID shall be unique within the gNB-DU logical node.

gNB-CU-CP UE E1AP ID:

A gNB-CU-CP UE E1AP ID shall be allocated so as to uniquely identify the UE over the E1 interface within a gNB-CU-CP (respectively an ng-eNB-CU-CP, or an eNB-CP as defined in TS 36.401[28]). When a gNB-CU-UP (respectively an ng-eNB-CU-UP, or an eNB-UP as defined in TS 36.401[28]) receives a gNB-CU-CP UE E1AP ID it shall store it for the duration of the UE-associated logical E1-connection for this UE. The gNB-CU-CP UE E1AP ID shall be unique within the gNB-CU-CP (respectively the ng-eNB-CU-CP, or the eNB-CP as defined in TS 36.401[28]) logical node.

gNB-CU-UP UE E1AP ID:

A gNB-CU-UP UE E1AP ID shall be allocated so as to uniquely identify the UE over the E1 interface within a gNB-CU-UP (respectively an ng-eNB-CU-UP, or an eNB-UP as defined in TS 36.401[28]). When a gNB-CU-CP (respectively an ng-eNB-CU-CP, or an eNB-CP as defined in TS 36.401[28]) receives a gNB-CU-UP UE E1AP ID it shall store it for the duration of the UE-associated logical E1-connection for this UE. The gNB-CU-UP UE E1AP ID shall be unique within the gNB-CU-UP (respectively the ng-eNB-CU-UP, or the eNB-UP as defined in TS 36.401[28]) logical node.

ng-eNB-CU UE W1AP ID:

An ng-eNB-CU UE W1AP ID shall be allocated so as to uniquely identify the UE over the W1 interface within an ng-eNB-CU. When an ng-eNB-DU receives an ng-eNB-CU UE W1AP ID it shall store it for the duration of the UE-associated logical W1-connection for this UE. The ng-eNB-CU UE W1AP ID shall be unique within the ng-eNB-CU logical node.

ng-eNB-DU UE W1AP ID:

An ng-eNB-DU UE W1AP ID shall be allocated so as to uniquely identify the UE over the W1 interface within an ng-eNB-DU. When an ng-eNB-CU receives an ng-eNB-DU UE W1AP ID it shall store it for the duration of the UE-associated logical W1-connection for this UE. The ng-eNB-DU UE W1AP ID shall be unique within the ng-eNB-DU logical node.

gNB-CU MBS F1AP ID:

A gNB-CU MBS F1AP ID shall be allocated so as to uniquely identify the MBS Session Context over the F1 interface within a gNB-CU. When a gNB-DU receives a gNB-CU MBS F1AP ID it shall store it for the duration of the MBS-associated logical F1-connection for that MBS Session. The gNB-CU MBS F1AP ID shall be unique within the gNB-CU logical node.

gNB-DU MBS F1AP ID:

A gNB-DU MBS F1AP ID shall be allocated so as to uniquely identify the MBS Session Context over the F1 interface within a gNB-DU. When a gNB-CU receives a gNB-DU MBS F1AP ID it shall store it for the duration of the MBS-associated logical F1-connection for this MBS Session. The gNB-DU MBS F1AP ID shall be unique within the gNB-DU logical node.

gNB-CU-CP MBS E1AP ID:

A gNB-CU-CP MBS E1AP ID shall be allocated so as to uniquely identify the MBS Session Context over the E1 interface within a gNB-CU-CP. When a gNB-CU-UP receives a gNB-CU-CP MBS E1AP ID it shall store it for the duration of the MBS-associated logical E1-connection for that MBS Session. The gNB-CU-CP MBS E1AP ID shall be unique within the gNB-CU-CP logical node.

gNB-CU-UP MBS E1AP ID:

A gNB-CU-UP MBS E1AP ID shall be allocated so as to uniquely identify the MBS Session Context over the E1 interface within a gNB-CU-UP. When a gNB-CU-CP receives a gNB-CU-UP MBS E1AP ID it shall store it for the duration of the MBS-associated logical E1-connection for this MBS Session. The gNB-CU-UP MBS E1AP ID shall be unique within the gNB-CU-UP logical node.

A-IoT Correlation ID:

An A-IoT Correlation ID shall be allocated so as to uniquely identify an A-IoT session context. When a gNB receives an A-IoT Correlation ID it shall store it for the duration of the A-IoT service operation. The A-IoT Correlation ID shall be unique within an AIOTF as specified in TS 23.369 [z].

RAN A-IoT Device NGAP ID

A RAN A-IoT Device NGAP ID shall be allocated so as to uniquely identify an A-IoT device context over the NG interface within a NG-RAN node for a certain A-IoT session. When an AIOTF receives an RAN A-IoT Device NGAP ID, it shall store it for the duration of the A-IoT session.

The RAN A-IoT Device NGAP ID shall be unique per A-IoT session within the logical NG-RAN node.

AIOTF A-IoT Device NGAP ID

An AIOTF A-IoT Device NGAP ID shall be allocated so as to uniquely identify an A-IoT device context over the NG interface within an AIOTF for a certain A-IoT session. When a NG-RAN node receives an AIOTF A-IoT Device NGAP ID, it shall store it for the duration of the A-IoT session.

The AIOTF A-IoT Device NGAP ID shall be unique per A-IoT session within an AIOTF.

<<<<<<<<<<<<<<<<<<<< Last Change >>>>>>>>>>>>>>>>>>>>

6.X	A-IoT related contexts in an NG-RAN Node

NG-RAN node A-IoT session context:

An NG-RAN node A-IoT session context is a block of information in an NG_RAN node and encompasses CP and UP, transport and radio resources to support an A-IoT Session. An NG-RAN node A-IoT session context may be associated with one or multiple NG-RAN node A-IoT device contexts.

Editor’s Note:	Further Details FFS.

NG-RAN node A-IoT device context:

An NG-RAN node A-IoT device context is a block of information in an NG-RAN node and encompasses CP and UP, transport and radio resources to support an A-IoT service to one A-IoT device within an NG-RAN node A-IoT session context.

Editor’s Note:	Further Details FFS.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>