3GPP TSG-RAN WG3 #128                                       R3-253294

Malta, MT, 19th – 23rd May, 2025

Source:	CATT

Title:	(TP to BL CR for TS38.413 and TS38.300) A-IoT inventory procedures

Agenda Item:	16.2

Document for:	Decision

- Introduction

On support of A-IoT, a study item has been completed in the last RAN WG meetings. Corresponding solutions on the architecture, signallings, locating, etc. are captured in the TR38.769 [1]. In RAN#106, a new work item for Ambient IoT has been approved(WID in [2]), aimed to standardize RAN aspects of Ambient IoT. In the WID, we will only focus on Topology 1, the major RAN3 scope are highlighted below:

- RAN3 scope:

  - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including:

    - Inventory and command operations

    - Device location reporting at reader ID granularity

Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2.

In the last RAN3 meeting, we discussed the inventory and command procedure for AIoT Topology 1, corresponding agreements and working assumptions are listed below:

About Inventory

Introduce a “Correlation ID” in the respective NGAP inventory at least inside the transfer containers.

- WA: Include the Correlation Identifier IE outside of the Inventory related Transfer IEs in all the Inventory related messages.

- Include the AIOTF Identifier IE outside of the Inventory related Transfer IEs in all the Inventory related messages.

- The Inventory Request Transfer IE, also includes the following:

  - Device Identification for Paging (to enable paging for single device, a group of devices, all devices)

  - Requested Service Area Information (A-IoT Area list and/or Reader list)

  - Inventory Assistance Information (Approximates Number of Target A-IoT Devices).

  - It is FFS whether the Inventory Assistance Information includes the “Estimate of Expected D2R Message Size”

About Command after inventory

- Introduce a follow-on command indication in Inventory Request Transfer IE, in case of Command after inventory.

- For command related NGAP messages, the Correlation Identifier IE is the same one as in the related Inventory procedure.

- WA: NGAP: Command Request procedure is a per single device procedure, and no need to have a Command Report procedure.

- WA: Include the Correlation Identifier IE in both inside and outside of the Command related Transfer IEs in all the Command related messages.

- Include the AIOTF Identifier IE outside of the Command related Transfer IEs in all the command related messages.

- The Command Request Transfer IE, also includes the following:

  - An A-IoT NAS PDU

  - Command Assistance Information: (Estimate of Expected D2R Message Size)

  - It is FFS whether the Command Assistance Information includes the Command Type (read, write, disable…)

- Introduce per-session per-device Device Association (“RAN NGAP Device ID”, and “(FFS) CN NGAP Device ID” pair) between gNB and AIOTF, in case of Command after inventory.

- In case of command after inventory, the gNB allocates and provides a “RAN NGAP Device ID” in the Inventory Report Transfer IE for each device.

- The Device Associationis included in the Command Request Transfer IE and Command Response Transfer IE.

It is FFS whether to introduce the Device Association or a transaction ID, outside of the Transfer IEs, to allow parallel Command Request procedures for different devices between gNB and AMF..

In this contribution, we will further discuss the inventory procedure and signaling impact on support of Ambient IoT. Base on the discussion, we will provide the proposals accordingly.

- Discussion

For signalling message for A-IoT Inventory procedure, there are inventory request, inventory response and inventory report message captured in candidate procedures.

For AIoT Inventory request message, the following information was discussed in last RAN3 meeting:

Introduce a “Correlation ID” in the respective NGAP inventory at least inside the transfer containers.

WA: Include the Correlation Identifier IE outside of the Inventory related Transfer IEs in all the Inventory related messages.

From RAN3 perspective, Correlation Identifier could be used per AIOTF to correlate INVENTORY REQUEST/RESPONSE/FAILURE and INVENTORY REPORT message. For AIOTF and RAN, the Correlation Identifier is necessary, so, Correlation ID has been agreed to be included inside the transfer containers. While for AMF, it may be also needed to maintain the related procedure. Therefore, Correlation ID should also be included outside of the Inventory related Transfer IEs in all the Inventory related messages.

Proposal 1: Correlation Identifier should be also included outside of the Inventory related Transfer IEs in all the Inventory related messages.

In TS23.369[3] which is agreed in SA2, the following is captured:

5.4 Assistance information provided to NG-RAN node The AIOTF provides the following assistance information to the NG-RAN together with the service operation requests: a) AIoT service type (e.g. Inventory, Command). b) Optionally, approximate number of AIoT devices based on AF request. c) Optionally, approximateD2R message size based on AF request. d) Optionally, time interval as AIoT aggregation assistance information. Editor's note: Other assistance information may be added later if necessary. Editor's note: It is FFS where detailed description on time interval as AIoT aggregation assistance information will be captured, i.e. in this clause or in another clause.

SA2 suggest that Time interval may be provided to AIOT RAN. With Time interval IE, AIOT RAN should send response message (including inventory report and command response message) within this period of time. In case of time limit exceeded, the operation should be considered as completed. To align with SA2, Time interval should be included in INVENTORY REQUEST message, in the Inventory Request Transfer IE.

Proposal 2: It is proposed to include Time interval in the Inventory Request Transfer IE of the INVENTORY REQUEST message.

There is also similar FFS in RAN3 as below:

It is FFS whether the Inventory Assistance Information includes the “Estimate of Expected D2R Message Size”

As for inventory procedure, the Estimate of Expected D2R Message Size should be used to indicate the length ofA-IoT NAS PDU in Inventory Report Transfer IE.

As for command procedure, it is used to indicate the length of A-IoT NAS PDU in Command Response Transfer IE.

Proposal 3: It is proposed to include Estimate of Expected D2R Message Size in Inventory Request Transfer IE in INVENTORY REQUEST and Command Request Transfer IE in COMMAND REQUEST message.

In TS23.369[3], SA2 ask RAN3 to send a complete message to AIOTF as below:

12. Optionally, if the NG-RAN detects that no more AIoT Devices will respond to the inventory procedure, the NG-RAN informs the AIOTF that the procedure is complete. Editor's note: The details how interactions between NG-RAN and AIOTF to indicate no further Inventory Reports will be sent and completion of the procedure need to be aligned with RAN.

When AIOTF triggers inventory to multiple devices, whether AIoT RAN could detect the completion of inventory procedure is up to RAN1 and RAN2. If needed, we can further work on that according to the LS.

As for the completion of inventory procedure, whether there is inventory complete message from CN to AIoT RAN needs consideration. The Time interval IE in INVENTORY REQUEST message may implicitly indicate the completion from CN. So, for CN, whether there is a completion of inventory procedure and how to indicate the completion may need clarification from SA2.

Proposal 4: Whether to introduce AIoT Inventory Complete message from RAN to CN or from CN to RAN is pending to the other WGs (RAN1, RAN2 and SA2).

- Conclusions

In this contribution, we mainly discussed A-IoT inventory Procedures and Information exchange between A-IoT CN and A-IoT RAN. Based on the discussion above, the following proposals are provided:

Proposal 1: Correlation Identifier should be also included outside of the Inventory related Transfer IEs in all the Inventory related messages.

Proposal 2: It is proposed to include Time interval in the Inventory Request Transfer IE of the INVENTORY REQUEST message.

Proposal 3: It is proposed to include Estimate of Expected D2R Message Size in Inventory Request Transfer IE in INVENTORY REQUEST and Command Request Transfer IE in COMMAND REQUEST message.

Proposal 4: Whether to introduce AIoT Inventory Complete message from RAN to CN or from CN to RAN is pending to the other WGs (RAN1, RAN2 and SA2).

- Reference

[1] TR 38.769Study on solutions for ambient IoT (Internet of Things)

[2] RP-243326 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

[3] TS23.369 V0.3.0(2025-04)“Ambient power-enabled Internet of Things”

- TP to BL CR for TS38.413

<<<<<<<<<<<<<<<<<< Begin of the changes >>>>>>>>>>>>>>>>>>>>>>>

8.xx.1	Inventory Request

8.xx.1.1	General

The purpose of the Inventory Request procedure is to request the NG-RAN node to perform inventory over the A-IoT radio.This procedure applies only if the NG-RAN node is a gNB.

8.xx.1.2	Successful Operation

Figure 8.xx.1.2-1: Inventory Request, successful operation.

The A-IoT CN node triggers the procedure by sending an INVENTORY REQUEST message to the NG-RAN node. If the NG-RAN node accepts the request, the NG-RAN node responds with the INVENTORY RESPONSE message.

The NG-RAN node shall, include the same values of theAIOTF Identifier IE andA-IoT Correlation Identifier IE in the INVENTORY RESPONSE message as received from the INVENTORY REQUEST message.

If the Inventory Assistance Information IE is included inthe INVENTORY REQUEST message, the NG-RAN node may take the information into account as defined in TS 38.300 [8].

If the Time interval IE is included in the INVENTORY REQUEST message, the NG-RAN node should send response message within this period of time.

If the Expected D2R Message Size IE is included in the INVENTORY REQUEST message, it indicates the length of A-IoT NAS PDU in Inventory Report message.

Interaction with Inventory Report procedure

When starting the requested inventory, the NG-RAN node shall, include the same values of theAIOTF Identifier IE andA-IoT Correlation Identifier IEin the INVENTORY REPORT message(s) as received from the INVENTORY REQUEST message

If the Follow on CommandIndication IE set to "true" is included in the Inventory Request Transfer IE in the INVENTORY REQUEST message, the NG-RAN node shall consider that there will be command to be transmitted to the device(s), and shall allocate and include a RAN A-IoT Device NGAP ID IE in each Device Report ItemIE within the Inventory Report Transfer IE in the INVENTORY REPORT message(s).

8.xx.1.3	Unsuccessful Operation

Figure 8.xx.1.3-1: Inventory Request,unsuccessful operation.

If the NG-RAN node is not able to perform the inventory operation, it shall send the INVENTORY FAILURE messagewith an appropriate cause value.

8.xx.1.4		Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<< Next changes >>>>>>>>>>>>>>>>>>>>>>>

9.3.3.xx4	Inventory Assistance Information

This IE includes the assistance information for the A-IoT Inventory Service.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Approximate Number of Target A-IoT Devices | O |  | INTEGER (1..2048,…) |  |
| Time interval | O |  | FFS |  |
| Expected D2R Message Size | O |  | FFS |  |

<<<<<<<<<<<<<<<<<< Next changes >>>>>>>>>>>>>>>>>>>>>>>

9.4.5	Information Element Definitions

AIoT-InventoryAssistanceInformation ::= SEQUENCE {

approximateNoofTargetDevices				INTEGER (1..2048, ...)				OPTIONAL,

TimeInterval                             FFS 			OPTIONAL,

estimateofExpectedD2RMsgSize		INTEGER (1..256, ...)					OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {AIoT-InventoryAssistanceInformation-ExtIEs} } OPTIONAL,

...

}

AIoT-InventoryAssistanceInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<End of the changes >>>>>>>>>>>>>>>>>>>>>>>

- TP to BL CR for TS38.300

<<<<<<<<<<<<<<<<<< Begin of the changes >>>>>>>>>>>>>>>>>>>>>>>

16.xx.x3	Inventory procedure

Figure 16.xx.x3-1 depicts the basic communication between the gNB and the A-IoT CN node for the Inventory procedure.

Figure 16.xx.x3-1: Inventory procedure

1.	The A-IoT CN node initiates the Inventory procedure over NG-C by sending the Inventory Request message to the gNB.The Inventory Request message includes a Correlation ID and A-IOTF Identifier to identify the inventory session. The Inventory Request message also includes an A-IoT Device Identification Requested corresponding to the A-IoT device(s) which are targeted for the inventory.

Editor’s Note 3:	content of the A-IoT Device Identification Requested is FFS.

The Inventory Request message also includes Requested Service Area Information (A-IoT area list and/or reader list) and may include Inventory Assistance Information. The Assistance Information may provide an Approximate number of Target A-IoT devices, estimate of expected D2R message size, Time interval.

The Inventory Request message may also include a follow-on command indicator to indicate that at least for one of the targeted A-IoT device(s) a Command procedure will follow.

Editor’s Note4:	Specification of further parameters of the Inventory Request message (e.g., other assistance information from 5GC) needs further work.

2.	The gNB allocates and co-ordinates the usage of A-IoT radio resources.

3.	The gNB confirms the request from the A-IoT CN node by replying with the Inventory Response message.The Inventory Response message includes the AIOTF Identifier and the Correlation ID received in the Inventory Request message.

NOTE 2:	If the gNB is not able to perform the Inventory procedure, it rejects the request and sends an Inventory Failure message to the A-IoT CN node.

4.	The gNB performs the Inventory procedure towards the A-IoT device(s) over the A-IoT radio interface.

5./6.	Upon receiving the inventory result from the A-IoT device(s), the gNB sends the Inventory Report message to the A-IoT CN node. If the Inventory procedure concerns multiple A-IoT devices, multiple Inventory Reportmessages may be sent to the A-IoT CN node.

The Inventory Report message includes the AIOTF Identifier and the Correlation ID received in the Inventory Request message and further includes the A-IoT NAS PDU(s) carrying the inventoried A-IoT device ID(s).

If a follow-on Command indicator was received at step 1, the Inventory Report message also includes a RAN A-IoT device NGAP ID for each A-IoT device which was inventoried.

NOTE3:	If the Inventory procedure concerns multiple A-IoT devices, an Inventory Report may include reports from multiple A-IoT devices,

<<<<<<<<<<<<<<<<<<End of the changes >>>>>>>>>>>>>>>>>>>>>>>