3GPP TSG-RAN WG3 Meeting #128                                                                    R3-253302

Malta, MT, 19th – 23rd, May 2025

Source:	CATT

Title:	(TP for BLCR to 38.401) On support of WAB

Agenda Item:	12.2

Document for:	Approval

1 Introduction

In RAN3#127bis meeting [1], Wireless Access Backhaul (WAB) was studied but some aspects with respect to access control and additional ULI have not been concluded.

In this contribution, we will continue discussion about the open issues of the basic functions for WAB.

2 Discussion

2.1 Avoid multi-hop WAB topology

In the last meeting[1], avoiding multi-hop in case of initial access was discussed. The chairman notes are captured as below:

FFS from previous meeting: To be further discussed whether Solution 1 or Solution 3 (or both) need to be selected 1. For initial access, WAB-gNB may use dedicated frequencies and/or PCIs and potential other legacy OTA parameters (e.g. NR CGI), to ensure that the WAB-MTs of other WAB-nodes avoid (re)selecting the WAB-gNB cells. 3. For initial access, WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT, and the WAB-MT avoids (re)selection of cells broadcasting this indicator. Solution 1 (dedicated existing OTA parameters) is used for preventing multihop WAB topology. Solution 3 For multi-hop avoidance during WAB-MT initial access, RAN3 to agree that the WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT. Solution 1 is implementation specific and it is not subject to specifications changes. It is FFS whether Solution 3, requiring impacts to SIBs, can be agreed. If there is consensus in RAN3 on an agreement for solution 3, RAN2 involvement is needed.

It can be observed from above that Solution 1 (dedicated existing OTA parameters) would totally rely on implementations and hence has no impact on specifications, comparing with Solution 3 which needs RAN2 involvement on enhancements to SIB and WAB-MT handling.

Though Solution 1 is assumed as implementation specific, we’d like to analysis the feasibility:

- Dedicated frequency and/or PCI

WAB-MT does not need reading cells’ broadcasting information to know WAB cells, that the implementation of WAB-MT is simple. The operators need to reserve dedicated frequency and/or PCI for WAB deployment and it additionally requires the OAM to pre-configure the dedicated frequency/PCI to WAB-gNB so that the co-located WAB-MT can avoid selecting the cell with WAB dedicated frequency/PCI. One problem is that this solution is very difficult requirement to OAM in mobility scenario. Because frequency/PCI are precious resource, the dedicated frequency/PCI should be locally assigned to facilitate the radio resource reusing, that the dedicated frequency/PCI for WAB used in one area would not be applicable to another area where the WAB-node moves. What’s more, when WAB-MT roams to a visited PLMN/SNPN different to the PLMN/SNPN broadcasted by the co-located WAB-gNB, OAM of PLMN/SNPN broadcasted by WAB-gNB has to co-ordinate with the WAB-MT visited PLMN/SNPN to configure the WAB-gNB with dedicated frequency/PCI.

So, this solution is only suitable to stationary scenario or mobility scenario with limited scale.

- Dedicated NR CGI

It requires WAB-MT reading the broadcasting information (SIB1) to know WAB cells, so the complexity of WAB-MT’s implementation is similar to Solution 3. It requires operator to reserve some NR CGIs or a range of NR CGI for WAB deployment, also the additional OAM pre-configuration to WAB-gNB is needed. Although the number of NR CGI would be enough, the assignment is assumed to be PLMN/SNPN specific. So, the problem is similar with the dedicated frequency/PCI solution in roaming scenario, i.e., the OAM of WAB-gNB would have to co-ordinate with the WAB-MT visited PLMN/SNPN. This solution is not suitable for the roaming scenario.

- Dedicated TAC

WAB dedicated TA(s) are included to WAB-MT’s forbidden TA list, thus no additional OAM configuration or inter-PLMN OAM co-ordination is necessary. There is no enhancement to WAB-MT, as WAB-MT can handle the forbidden TA list as legacy UE. While the disadvantage still exists for occupying TA resource which is a global arrangement. Ideally, all WAB-node can share the same TAC if they move without changing the AMF, that one dedicated TAC reserved for WAB deployment is enough. However, for WAB-node mobility with UEs’ AMF changing, WAB-node should be configured with different TACs corresponding to different AMFs towards which the WAB-gNB may potentially set up NG connections, thus, different TACs should be reserved for different AMFs for WAB. Further, operator may need to reserve multiple TACs for each AMF for WAB, considering the WAB cell’s TAC would be changed for other purpose (as described by TS 38.401 BL CR) in practice.

BLCR to TS 38.401: ### X.2.3 TAC/RANAC (re-)configuration for a WAB-gNB’s cell The TAC/RANAC of WAB-gNB’s cell is configured by the OAM. During the mobility of WAB-node, it can be reconfigured by the OAM, or it can remain unchanged in a certain geographical area. The TAC/RANAC of the WAB-gNB’s cell may be the same as, or different than, the TAC/RANAC of the co-located WAB-MT’s serving cell. The TAC/RANAC broadcast by the WAB-gNB’s cell can be changed in order to reflect the WAB-node’s physical location.

To sum up, there are restrictions on using the dedicated frequency/PCI/NR CGI solution in mobility scenario and/or roaming scenario. The dedicated TAC solution is assumed feasible in stationary, mobility and roaming scenarios, but it requires reserving multiple TACs for each AMF for WAB in the globally arranged TA pool.

On the contrast, solution 3 does not require reserving dedicated resources locally/globally, hence it can make the operators to deploy WAB easily. Although Solution 3 is subject to RAN2 spec change, i.e., introducing new WAB-MT behavior and new indicator to the broadcast information, the impact to RAN2 is limited, as WAB-MT anyway has to read the broadcast information before camping on a cell and it’s a widely used solution by RAN2 to bar UE via broadcasting information.

Since solution 3 does not require dedicated resource reservation in all scenarios, it can be supported as an alternative for the operators who feel difficult on deploying WAB with dedicated resource arrangement.

WAB cells can optionally broadcast a WAB-Cell indicator, though this behavior needs no specification, but requires a new indicator added to SIB. And the optional capability can be introduced to WAB-MT on decoding the new indicator to avoid selecting WAB cells.

Proposal 1-1: Solution 3 should be supported as another alternative except solution 1, which means:

- WAB cells can broadcast a new indicator to bar WAB-MT, and WAB-MT can support the capability of decoding the new indicator to avoid selecting such cells.

RAN3 is proposed to send LS to RAN2 on introducing Solution 3 for multi-hop topology avoidance. With this LS, RAN2 can also work on the mandatory capabilities for WAB-MT, which have not been discussed at all but would be similar to the mandatory capabilities for mobile IAB-MT.

Proposal 1-2: RAN3 send LS to RAN2 to trigger the specification on supporting solution 3 and potentially other WAB-MT’s capabilities.

2.2 Additional ULI

In the last meeting[1], it was agreed that the additional ULI is introduced to the non-UE associated NG procedure:

RAN3#127bis: RAN3 to agree the WAB-gNB reports the additional ULI to the network within the NG Setup and RAN Configuration Update procedures. It is FFS whether the additional ULI shall be removed from NG UE associated procedures

Now the additional ULI has been captured in NG SETP REQUEST and RAN CONDIGURATION UPDATE message by the NGAP BLCR [2]. However, the open issue is that whether the additional ULI should be removed from UE associated NG procedures. The stage-2 BLCR has captured the functionality of WAB-gNB reporting the additional ULI which reflects the WAB-node’s geo-location to CN for the UEs served by the WAB-gNB, also, the NGAP BLCR [2] captured additional ULI IE  being included in User Location Information IE which is sent via UE associated messages in the last meeting.

The WAB-node’s physical location is only mapped to one pair of NR CGI and TAI no matter how many cells the WAB-node serves. With the additional ULI introduced to non-UE associated procedures, the NR CGI and TA reflecting the WAB-node’s physical location can be included in the additional ULI and provided to CN via the non-UE associated messages, which is done only if the NG connection is setup between the WAB-node and CN no matter the WAB-gNB is serving UE or not. The additional ULI sent via the UE associated messages also contain the NR CGI and TAI pair mapped to the WAB-node’s physical location, but the same information does not need to be indicated to CN. Thus, the additional ULI does not need to be included in NG UE associated messages.

To be specific, the additional ULI IE should be removed in User Location Information IE in the NGAP BLCR, and the corresponding section of BLCR to TS 38.401 (clause X.5) should be updated.

Proposal 2-1: Remove the additional ULI IE in NG UE associated messages from BL CR to TS 38.413.

The associated section (X.5) in stage-2 BLCR should be updated for the introduction of reporting the additional ULI of UEs via non-UE associated procedures by WAB.

Proposal 2-2: Update the BLCR to TS 38.401 for introduction of additional ULI to non-UE associated procedures as the Annex.

2.3 WAB integration

Based on TS 23.501 [4], the behaviors of authorization focus on the cases when the authorization status is changed. But SA2 does not discuss how to handle if WAB-MT or WAB-gNB is “not authorized” during the WAB-node network integration.

During the WAB-node integration procedure, the WAB-MT establishes the BH PDU session(s), the WAB-gNB is initialized and sets up the NG/Xn connection(s). That will only happen if the WAB-MT and WAB-gNB are “authorized” respectively. If the WAB-MT is not authorized, it should not establish the BH PDU session(s). It has been captured in BLCR of TS 38.401 that WAB-MT will not establish the BH PDU session(s) until its authorization status changes to “authorized”. Similarly, the WAB-gNB should not be initialized if it is not authorized by OAM. The WAB-gNB should be authorized by OAM before the OAM configuring proper parameters to the WAB-gNB.

Present BLCR to TS 38.401 should be updated to capture that.

Section X.1 in BLCR to TS 38.401: Phase 1: WAB-MT setup. The WAB-MT of a WAB-node connects to the network in the same way as a UE by performing RRC connection setup procedure with the BH-RAN-node. The WAB-MT then performs, authorization and authentication with the BH-5GC. After the WAB-MT is authorized, the WAB-MT can establish one or more PDU sessions for backhauling. Phase 2: WAB-gNB setup. This phase includes the following 3 sub-phases: Sub-phase 2-1: WAB-gNB initialization. In this phase, the WAB-gNB is configured by the OAM (e.g., with the information needed to establish NG connections towards one or more AMF(s) and the WAB-gNB is service-authorized by the SeGW or by the OAM.

A TP is provided for BLCR to TS 38.401 in the Annex to capture that OAM configuring proper parameters to the WAB-gNB after the WAB-gNB is authorized.

Proposal 3-1: In phase 2-1 of WAB-node integration procedure, WAB-gNB should be authorized by OAM before the OAM configuring proper parameters to it. And BLCR to TS 38.401 should be updated to capture that as Annex.

Another issue in phase 2-1 is that SeGW is out of RAN/3GPP scope, a note should be added to clarify that.

Proposal 3-2: A note should be added for WAB-node integration procedure in BLCR to TS 38.401 that the SeGW is out of RAN’s scope.

2.4 Dual connectivity

In the RAN3#127 meeting [3], an issue was raised on whether a WAB-gNB can support dual-connectivity for UE. The issue was not discussed in the last meeting.

It is FFS whether a WAB gNB can act as an MN or SN in Dual Connectivity.

Dual connectivity allows the UE to use radio resources from two gNBs and prevents unnecessary UE handovers. R18 mobile IAB focuses on scenarios of mobile-IAB nodes mounted on vehicles. And, in TS 38.300, mobile IAB supports the functions of gNB, and gNB supports the function of dual connectivity. In mobile IAB, UE DC is supported for in-band and out-band backhauling, applicable to FR1 and FR2. The mobile IAB-nodes can act as an MN or SN in DC for onboard UEs.

Similar to mobile IAB, WAB can provide 5G access for UEs onboard aircraft, ships, vehicles and etc. So, it makes sense that WAB-nodes also support dual connectivity for the onboard UEs.

WAB has some features which are not supported by mobile IAB, including:

-	Backhauling of NG and Xn via TN and NTN, including support of NTN <-> TN handover for backhaul.

-	Support for backhauling without RAN-sharing or roaming agreements between access PLMN(s) and backhaul PLMN(s).

In WAB, the backhaul link can be NTN, or access PLMN(s) and backhaul PLMN(s) can be different, but it should not be a showstopper for WAB-acting as an MN or SN of UE.

Thus, WAB supporting dual connectivity for UE should not be precluded. Nevertheless, as normal gNB supports DC for UE and WAB-gNB inherits the normal gNB’s functionalities, no spec work is observed for WAB supporting DC.

Proposal 4: RAN3 to confirm that WAB can support UE dual connectivity.

3 Conclusion

In the previous sections we made the following proposals:

Proposal 1-1: Solution 3 should be supported as another alternative except solution 1, which means:

- WAB cells can broadcast a new indicator to bar WAB-MT, and WAB-MT can support the capability of decoding the new indicator to avoid selecting such cells.

Proposal 1-2: RAN3 send LS to RAN2 to trigger the specification on supporting solution 3 and potentially other WAB-MT’s capabilities.

Proposal 2-1: Remove the additional ULI IE in NG UE associated messages from BL CR to TS 38.413.

Proposal 2-2: Update the BLCR to TS 38.401 for introduction of additional ULI to non-UE associated procedures as the Annex.

Proposal 3-1: In phase 2-1 of WAB-node integration procedure, WAB-gNB should be authorized by OAM before the OAM configuring proper parameters to it. And BLCR to TS 38.401 should be updated to capture that as Annex.

Proposal 3-2: A note should be added for WAB-node integration procedure in BLCR to TS 38.401 that the SeGW is out of RAN’s scope.

Proposal 4: RAN3 to confirm that WAB can support UE dual connectivity.

4 Reference

- RAN3#127bis minutes, RAN3 Chair

- R3-251551, (BL CR to 38.413 for WAB) Additional ULI for UEs served by WAB-Nodes, Huawei, Ericsson, Nokia, Nokia Shanghai Bell, China Telecom, ZTE, Qualcomm, Samsung, CATT, Jio Platforms (JPL), Lenovo, RAN3#127bis

- RAN3#127 minutes, RAN3 Chair

5 Annex (TP to BL CR for TS 38.401)

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>Start of Changes<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

# X	Wireless Access Backhaul

## X.1	WAB-node integration procedure

Figure X.1-1 WAB-node integration procedure

Phase 1: WAB-MT setup. The WAB-MT of a WAB-node connects to the network in the same way as a UE by performing RRC connection setup procedure with the BH-RAN-node. The WAB-MT then performs, authorization and authentication with the BH-5GC. After the WAB-MT is authorized, the WAB-MT can establish one or more PDU sessions for backhauling.

Phase 2: WAB-gNB setup. This phase includes the following 3 sub-phases:

Sub-phase 2-1: WAB-gNB initialization. In this phase, the WAB-gNB is service-authorized by the SeGW or by the OAM. After the WAB-gNB is authorized, the WAB-gNB is configured by the OAM (e.g., with the information of AMF(s) to serve the UE).

NOTE: The SeGW is out of RAN scope.

Sub-phase 2-2: NG connection setup. The WAB-gNB establishes NG connection(s) toward the AMF(s). This step follows legacy procedures. After the NG is set up, the WAB-gNB can start serving UE(s).

Sub-phase 2-3: Xn connection setup. If needed, the WAB-gNB can establish Xn connection(s) towards the BH-RAN-node and/or other NG-RAN node(s).

---------------------------------------Next Change -------------------------------------------

## X.5 	User Location Information for UEs served by a WAB-gNB

In addition to the User Location Information (ULI) for UEs served by a WAB-gNB, the WAB-gNB also provides the core network with additional ULI via the NG Setup or RAN Configuration Update procedures, which includes a TAI and a NR CGI pertinent to the WAB-gNB’s broadcasted PLMN/SNPN.

If the PLMN/SNPN broadcasted by a WAB-gNB is the same as the PLMN/SNPN serving the WAB-MT, and the WAB-MT connects to the BH-gNB by means of a terrestrial link, the additional ULI includes the TAI and the NR CGI of the cell serving the WAB-MT.

If the PLMN/SNPN serving the WAB-MT is different from the WAB-gNB’s broadcasted PLMN/SNPN, and the WAB-MT connects to the BH-gNB by means of a terrestrial link, the additional ULI is determined by the WAB-gNB, based on the WAB-node’s geo-location.

If the WAB-MT connects to the BH-gNB by means of a non-terrestrial link, the additional ULI is determined by the WAB-gNB, based on WAB-node’s geo-location. This applies regardless of whether the PLMN/SNPN serving the WAB-MT is the same as, or different than, the WAB-gNB’s broadcasted PLMN/SNPN.

In case additional ULI changes, e.g., due to WAB-node movement, the WAB-gNB derives the new additional ULI and reports it via the RAN Configuration Update procedures, if required by the core network.

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>End of Changes<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<