3GPP TSG-RAN WG3 Meeting #128                                                                    R3-253304

Malta, MT, 19th – 23rd, May 2025

Source:	CATT

Title:	Discussion on remaining issue of NR Femto

Agenda Item:	12.3

Document for:	Discussion and Decision

1 Introduction

In the last meeting [1], access control of NR Femto was discussed and the agreement was made. The description of NR Femto has been modified in the TP to BLCR of TS 38.300 [2].

In this contribution, we will discuss three remaining issue of NR Femto.

2 Discussion

In the last meeting, one issue was raised as follows [1]:

If the SMF sends both IP versions in the Transport Layer Address IE, the NR Femto node selects the correct IP version. It is FFS whether the GW can perform IP Version selection in addition to the NR Femto.

In LTE, HeNBs may be mixed deployed via a HeNB GW, some HeNBs support IPv4 only, and other HeNBs support IPv6. The IP versions of S1-U and S1-C on the eNB are usually the same, thus HeGW can select the IP version of S1-U for HeNB according to the IP version of S1-C, when S1-C is terminated at HeNB GW while S1-U does not. The MME can send two transport layer addresses of different versions. The HeNB GW takes the role of IP version selection of S1-U for HeNB. If S1-C of HeNB does not terminate on HeNB GW, the MME can determine the IP version of S1-U based on that of S1-C by itself. That’s the reason why the HeNB does not need to host the function of IP version selection.

However, in NR, C-plane and U-plane can be split and IP version for NG-C and NG-U on the same gNB may be different. So, NR-RAN nodes support the function of IP version selection for NG-U, see clause 8.3 in TS 38.300. The AMF can send two transport layer addresses of different versions, enabling NG-RAN nodes to select either IPv4 or IPv6.  According to the NR Femto architecture in BLCR [3], when the NR Femto GW is deployed, NG-C is terminated at NR Femto GW while NG-U cannot be terminated by NR Femto GW. But it’s not reliable for the NR Femto GW to select the IP version of NG-U for NR Femto node according to the IP version of NG-C, as the IP version for NG-C and NR-U on the NR Femto node may be different. In both cases of NR Femto GW terminating NG-C and not terminating NG-C, the NG-U IP version selection can be done by NR Femto node. So, NR Femto GW does not need to support the function of selecting IP version of NG-U for NR Femto.

Observation: It’s not reliable nor necessary for NR Femto GW to support the function of selecting IP version of NG-U for NR Femto node.

For the FFS on whether the NR Femto GW can perform IP Version selection, we understand that NR Femto GW can take the role of IP version selection for NG-U only if the NR Femto GW terminate NR-U. However, the architecture of NR Femto does not support terminating NR-U at the NR Femto GW. Thus, NR Femto GW does not need host the function of IP version selection.

Proposal 1: NR Femto GW does not need to host the function of IP version selection.

The second issue is the inconsistency of terminology.

It is FFS how to harmonise terminology for “NR Femto”, “NR Femto Node”. Terminology shall be consistent.

In the last meeting, some company indicates that the current BL CRs do not align “NR Femto” and “NR Femto Node”. In our opinion, “NR Femto” is the name of this study, while “NR Femto Node” is the name of gNB. The two terminologies should not be used interchangeably.

Thus, we propose that “NR Femto” which indicates NR Femto node shall be modified to “NR Femto Node”, which involve the BLCRs to TS 38.300 [3] and TS 38.410 [4].

Proposal 2: Terminology “NR Femto node” should be used instead of “NR Femto”. Change “NR Femto” to “NR Femto Node” in BL CRs to TS 38.300 and TS 38,410.

The last issue is about the reply LS [5] from SA3. The reply LS addresses the questions in LS [6] sent by RAN3 in RAN3#127 meeting. The answers of SA3 are copied as follow:

SA3 kindly asks RAN3 to consider following answers: SA3 has agreed that all the verifications listed in the LS need to be performed in NR Femto GW (or AMF if NR Femto GW is not deployed). SA3 agreed the attached CRs to update TS 33.545 to incorporate relevant verification requirements and mechanisms as well.

The verifications listed in the LS [6]  are copied as follow:

For the NR Femto Gateway: 1: In case of NGAP INITIAL UE MESSAGE message, whether to verify that reported CAG ID is valid for that NR Femto. 2: In case of NGAP SETUP REQUEST message, whether to verify that the identity used by the NR Femto is valid. 3: In case of NGAP PWS RESTART INDICATION message and NGAP PWS FAILURE INDICATION message, whether to verify that the indicated cell identity is valid. For the AMF: 1: In case of an NR Femto directly connected, whether to verify that the CAG ID reported in the NGAP INITIAL UE MESSAGE message is valid for the indicated cell. 2: In case of an NR Femto directly connected, whether to verify that the identity used by the NR Femto is valid when receiving the NGAP SETUP REQUEST message. 3: In case of an NR Femto directly connected, whether to verify that the indicated gNB identity of the NR Femto is valid when receiving the NGAP PWS RESTART INDICATION message and the NGAP PWS FAILURE INDICATION message.

Since all the verifications have been confirmed to be performed in NR Femto GW (or AMF if NR Femto GW is not deployed), the corresponding functions should be captured in BL CR to TS 38.300.

Proposal 3: The verification functions for NR Femto Gateway and AMF on receiving NGAP INITIAL UE MESSAGE message, NGAP SETUP REQUEST message, NGAP PWS RESTART INDICATION message and NGAP PWS FAILURE INDICATION message should be captured in BL CR to TS 38.300.

3 Conclusion

In the previous sections we made the following observations and proposals:

Observation: It’s not reliable nor necessary for NR Femto GW to support the function of selecting IP version of NG-U for NR Femto node.

Proposal 1: NR Femto GW does not need to host the function of IP version selection.

Proposal 2: Terminology “NR Femto node” should be used instead of “NR Femto”. Change “NR Femto” to “NR Femto Node” in BL CRs to TS 38.300 and TS 38,410.

Proposal 3: The verification functions for NR Femto Gateway and AMF on receiving NGAP INITIAL UE MESSAGE message, NGAP SETUP REQUEST message, NGAP PWS RESTART INDICATION message and NGAP PWS FAILURE INDICATION message should be captured in BL CR to TS 38.300.

4 Reference

- RAN3#127bis meeting report, RAN3 Chair;

- R3-252333, [TP to BL CR to 38.300 for femto] Clarification of shared NR Femto, Nokia, Verizon Wireless, TMO US, LG Electronics, Baicells, Huawei, Qualcomm Incorporated, Samsung, Lenovo, ZTE, Ericsson, RAN3#127bis;

- R3-251548, (BL CR to 38.300 for Femto) Introduction of NR Femto Architecture and Protocol Aspects, Ericsson, Nokia, TMO US, AT&T, Verizon Wireless, BT, Charter, Huawei, LG Electronics, Samsung, Lenovo, Baicells, ZTE, NEC, RAN3#127bis;

- R3-251546, (BL CR to 38.410 for Femto) Introduction of NR Femto in NGAP list of functions, ZTE Corporation, Nokia, RAN3#127bis;

- R3-253021, Reply LS on security verification related to NR Femtos, RAN3#128

- R3-250822, LS on security verification related to NR Femtos, RAN3#127