3GPP TSG-RAN WG3 Meeting #128                                                                                R3-253306

Malta, MT, 19th – 23rd, May 2025

Source:	CATT

Title:	(TP for BLCRs to 38.423, 38.473) On support OD-SSB

Agenda Item:	17.2

Document for:	Discussion and Approval

1 Introduction

For NES on-demand SSB SCell operation topic, the WID [1] content is as follows:

Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

In the last RAN1 meeting [2], some agreements are achieved on various aspects related to on-demand SSB SCell.

In this contribution, we will continue to discuss the topic of on-demand SSB SCell operation, our views will be given as well.

2 Discussion

2.1 Interaction over F1 interface

Before Rel-19, several network energy-saving techniques were introduced with respect to SSB transmission saving:

- Cell deactivation

From Rel-15, the cell can be deactivated for network energy saving. If a cell is deactivated, the cell coverage does not exist and the cell cannot act as any kind of serving cell for UE. In split gNB architecture, it is the gNB-CU to control a cell’s deactivation based on the cell’s condition and energy saving requirement, and gNB-DU just stop all the SSB transmissions based on gNB-CU’s instruction.

- SSB beam deactivation

During Rel-18, SSB beam level deactivation was introduced. This is an operation determined and performed by gNB-DU based on SSB usage condition and energy saving requirement. As SSB deactivation will impact the SSB’s coverage, a Cells Allowed to be Deactivated List IE was introduced to F1AP and used to indicate the gNB-CU’s permission to gNB-DU on such operation. However, SSB beam deactivation need not be configured to UE, as it’s totally the network’s implementation and irrelevant with UE’s support.

- SSB-less SCell

SSB-less SCell was also introduced during Rel-18. This allows the cell without any SSB transmission to still act as the SCell for the UEs which get DL synchronization to this SCell from PCell/PSCell or other cells. An SSB-less cell cannot be configured as PCell/PSCell for connected UEs or support idle/inactive UEs to camp on, so it does not provide cell coverage. The UE served by SSB-less SCell may be informed about the cell where it can get DL synchronization with this SCell. In split gNB, gNB-DU takes the change of SSB-less operation as well as relevant UE configuration. Nevertheless, we recalled that gNB-CU is not involved in SSB-less operation, that is probably because transition from normal PCell to SSB-less operation was not considered or because such cells are basically fixed and always can be pre-configured by OAM.

Back to OD-SSB, it’s a SCell operation used when SSB-less is not possible. Based on RAN1 conclusions, the OD-SSB transmissions are SSB transmissions different to always-on SSB (i.e., legacy SSB), that OD-SSB is not always transmitted but only activated as needed (e.g., used for SCell activation or L3 measurement). Due to the OD-SSB transmission, always-on SSB may not be needed, an OD-SSB cell would not transmit always-on SSB or would transmit spares always-on SSB. There are two cases of OD-SSB cell:

Case #1: No always-on SSB on the cell

Case #2: Always-on SSB is periodically transmitted on the cell, where the always-on SSB can be cell-defining SSB (CD-SSB) on synchronization raster or non-cell-defining SSB (NCD-SSB).

To facilitate UE to use OD-SSB, gNB-DU can configure the UE with OD-SSB via RRC message, activate or deactivate the OD-SSB transmission by RRC or MAC CE. OD-SSB operation including the OD-SSB transmission, always-on SSB deactivation and spares always-on SSB transmission are all triggered and controlled by gNB-DU based on cell condition and energy saving requirement.

Observation 1: The OD-SSB operation including OD-SSB transmission, always-on SSB deactivation or spares always-on SSB transmission is triggered and controlled by gNB-DU.

In the RAN3#127 meeting [3], the issue about whether CU indicating the allowed cells for OD-SSB to DU was discussed and the potential WF is provided:

RAN3#127: For the allowing On-demand SSB SCell, we discussed the two way-forward as follows: - WF1: No new List from the gNB-CU to the gNB-DU for “allowing OD SSB SCell” is needed, instead we can reuse, if needed, the Rel-18 existing “Cells Allowed to be Deactivated List” (CU to DU). Potential rewording of the procedure may be needed to make the “list” more general. - WF2: It is the gNB-DU to decide how and when to perform “deactivation” or “OD SSB”. But the above is not agreed, there are two concerns raised during the discussion: - One concern is that gNB-DU does not always need the “CU to DU” assistance in order to go to the OD-SSB SCell operation. - Another concern is how and whether to handle the transition from PCell normal operation to OD-SSB SCell only operation is unclear. To be continued at the next meeting on the above basis.

RAN3#127bis: The CU reuses the existing IE (the Cells Allowed to be Deactivated List IE) to indicate the allowed cells for on-demand SSB operation to the DU. The details of stage2/3 impact can be further discussed.

However, since the OD-SSB is only transmitted as needed, we believe that the OD-SSB transmissions should not be able to impact the cell coverage. So, gNB-CU would not care about whether OD-SSB is configured or the OD-SSB transmission status. Hence it makes no sense for gNB-DU to inform the OD-SSB transmission to gNB-CU or gNB-CU to indicate allowance on OD-SSB transmissions to gNB-DU.

Observation 2: It’s not necessary for gNB-DU to inform gNB-CU the OD-SSB transmission status, also not needed for gNB-CU to send allowance of OD-SSB transmissions to gNB-DU.

As mentioned above, the always-on SSB (i.e., legacy SSB) deactivation can impact the cell coverage. We believe it’s important to discuss the transition from normal cell serving as PCell/PSCell to OD-SSB operation, as it may lead to always-on SSB deactivation. There are three consequences after a normal cell switching to OD-SSB operation:

- Case a): No always-on SSB transmission

- Case b): With always-on SSB of only NCD-SSB

- Case c): With always-on SSB of CD-SSB on synch-raster

For the former two (without always-on SSB or with always-on SSB of only NCD-SSB), the legacy CD-SSB transmissions are deactivated, which results in the cell coverage reduction and disabling PCell/PSCell operation. While, in the last one, the cell coverage and normal operation can be kept unchanged. Considering which case to select is purely decided by gNB-DU, it’s understood that the legacy CD-SSB deactivation caused by former two cases may need gNB-CU’s permission. Some companies would assume it can rely on OAM. Of course, OAM solution always works, but we would prefer a more flexible solution than OAM as this is for energy saving. For example, gNB-CU can take charge of allowing a cell to be sent to OD-SSB disabling PCell operation for energy saving, i.e., deactivating always-on CD-SSB. The solution is quite similar as SSB beam deactivation in Rel-18.

Proposal 1-1: gNB-CU can inform gNB-DU whether a cell is allowed to disable the PCell/PSCell operation, i.e., deactivating CD-SSB transmission for OD-SSB operation.

In the previous meetings, companies thought about reusing the existing IE (the Cells Allowed to be Deactivated List IE) for allowing SSB beam deactivation in GNB-CU CONFIGURATION UPDATE message, that is because both SSB beam deactivation and disabling PCell/PSCell operation are for energy-saving purpose and impact the cell coverage, though one is spatial domain while another is time domain technique. If the list is reused, the cells outside the list are not allowed to use either SSB beam deactivation or deactivate CD-SSB for energy saving, while cells in the list are allowed to use one or both of them.

Proposal 1-2: Reuse the existing Cells Allowed to be Deactivated List IE in F1AP messages from CU to DU to indicate the allowed cells to disable PCell/PSCell operation (deactivating CD-SSB) for energy saving.

Hence, we do not need to introduce a new IE. As the usage extension of Cells Allowed to be Deactivated List IE, rewording is needed for the gNB-CU Configuration Update procedure text.

Clause 8.2.5.2 in TS 38.473: If the Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to deactivate the SSB beams within the indicated cells for network energy saving purpose.

Corresponding to SSB beam deactivation, the beam level activation was also introduced in Rel-18, it is a functionality that allows gNB-CU to request gNB-DU activating the SSB beams which were deactivated due to energy saving so that the beam coverage can be resumed, as needed. The SSBs within the cell to be Activated List IE was added to the Cells to be Activated List IE in some F1AP messages from gNB-CU to gNB-DU for this function. It can be understood the similary function would be beneficial in case CD-SSB transmission are deactivated when a normal cell switch to OD-SSB, i.e., gNB-CU can request gNB-DU to resume CD-SSB transmissions for a cell whose PCell operation was disabled, to recover the cell coverage. We consider the existing Cells to be Activated List IE in the F1AP messages from gNB-CU to gNB-DU can be reused, as this is used for recovering a normal cell in legacy. For a cell included in the list, gNB-DU can activate the cell if the cell was deactivated, or gNB-DU can turn on the CD-SSB transmissions which were deactivated when switching to OD-SSB operation. Description in F1AP procedure text handling this IE may need rewording.

Clause 8.2.5.2 in TS 38.473: If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall activate the cell indicated by NR CGI IE and reconfigure the physical cell identity for which the NR PCI IE is included, or it turn on always-on SSB of the cell indicated by NR CGI IE in OD-SSB.

Proposal 1-3: gNB-CU can request gNB-DU to resume CD-SSB transmissions (i.e., recover the cell coverage), the existing Cells to be Activated List IE in F1AP messages from gNB-CU to gNB-DU is reused for this function.

Proposal 1-4: For the cell included in the Cells to be Activated List IE, gNB-DU can turn on always-on CD-SSB transmissions which were deactivated when switching to OD-SSB operation, which means the cell coverage is recovered.

2.2 Interaction over Xn interface

As we discussed in section 2.1, the transition from normal cell serving as PCell/PSCell to OD-SSB operation may lead to CD-SSB deactivation and the cell coverage is not present.

In that case, the gNB should indicate the coverage change of the NES cell to the neighbour gNBs, although such cell is still activated for only SCell operation. Considering presence of the existing Deactivation Indication IE in Served Cells NR To Modify IE of NG-RAN Node Configuration Update message is used for informing the deactivation of cell (due to network energy saving), the gNB can reuse the IE and the message to indicate neighbour gNBs invalidity of the cell coverage when switching to OD-SSB operation. The neighbour gNB considers the cell coverage is invalid due to cell deactivation or OD-SSB operation. As the usage extension to the existing Deactivation Indication IE, rewording for description of relevant IEs and handling in NG-RAN Node configuration update procedure should be necessary, for example:

Clause 8.4.2.2 in TS 38.423: - If the Deactivation Indication IE set to "deactivated" is contained in the Served Cells NR To Modify IE, it indicates that the concerned cell was switched off to lower energy consumption or loosed coverage due to OD-SSB.

Proposal 2-1: In case CD-SSB transmissions are deactivated upon a cell switching from normal PCell/PSCell operation to OD-SSB operation, the gNB should indicate neighbour gNB the invalidity of cell coverage

Proposal 2-2: The existing Deactivation Indication IE in Served Cells NR To Modify IE of the NG-RAN Node Configuration Update message is reused for such purpose.

The neighbour gNB receiving the Deactivation Indication IE for a cell assumes the cell coverage does not exist, but it has no knowledge about whether the cell is totally deactivated or still activated for just SCell operation. Afterwards, if the neighbour gNB needs coverage of the cell, e.g., due to the increased traffic load, the neighbour gNB can just use the legacy Cell Activation request message for request resuming the cell coverage. It is left to the implementation of gNB receiving the request to activate the cell which was deactivated or resume the PCell/PSCell operation for the cell whose PCell/PSCell operations were disabled when switched to OD-SSB operation. It can be observed that the usage of Cell Activation procedure is extended, therefore, rewordings to the Cell Activation procedure text as well as the description for relevant messages are necessary.

Clause 8.4.3.2 in TS 38.423: If either the NR Cells IE or the E-UTRA Cells IE is included in the CELL ACTIVATION REQUEST message, the NG-RAN node2 should activate the cell/s indicated in the CELL ACTIVATION REQUEST message and shall indicate in the CELL ACTIVATION RESPONSE message for which cells the request was fulfilled.

Proposal 2-3: gNB receiving the Cell Activation Request message from the neighbour gNB can resume the normal PCell/PSCell operation for the indicated cells whose CD-SSB transmission were disabled at switching to OD-SSB operation, which means the cell coverage is recovered.

3 Conclusion

According to the analysis in section 2, we have following observations and proposals:

Interaction over F1 interface

Observation 1: The OD-SSB operation including OD-SSB transmission, always-on SSB deactivation or spares always-on SSB transmission is triggered and controlled by gNB-DU.

Observation 2: It’s not necessary for gNB-DU to inform gNB-CU the OD-SSB transmission status, also not needed for gNB-CU to send allowance of OD-SSB transmissions to gNB-DU.

Proposal 1-1: gNB-CU can inform gNB-DU whether a cell is allowed to disable the PCell/PSCell operation, i.e., deactivating CD-SSB transmission for OD-SSB operation.

Proposal 1-2: Reuse the existing Cells Allowed to be Deactivated List IE in F1AP messages from CU to DU to indicate the allowed cells to disable PCell/PSCell operation (deactivating CD-SSB) for energy saving.

Proposal 1-3: gNB-CU can request gNB-DU to resume CD-SSB transmissions (i.e., recover the cell coverage), the existing Cells to be Activated List IE in F1AP messages from gNB-CU to gNB-DU is reused for this function.

Proposal 1-4: For the cell included in the Cells to be Activated List IE, gNB-DU can turn on always-on CD-SSB transmissions which were deactivated when switching to OD-SSB operation, which means the cell coverage is recovered.

Interaction over Xn interface

Proposal 2-1: In case CD-SSB transmissions are deactivated upon a cell switching from normal PCell/PSCell operation to OD-SSB operation, the gNB should indicate neighbour gNB the invalidity of cell coverage

Proposal 2-2: The existing Deactivation Indication IE in Served Cells NR To Modify IE of the NG-RAN Node Configuration Update message is reused for such purpose.

Proposal 2-3: gNB receiving the Cell Activation Request message from the neighbour gNB can resume the normal PCell/PSCell operation for the indicated cells whose CD-SSB transmission were disabled at switching to OD-SSB operation, which means the cell coverage is recovered.

4 Reference

- RP-242354, Revised WID: Enhancements of network energy savings for NR, RAN#105, Ericsson

- RAN1 Chair’s Notes, RAN1#120bis

- draft_MeetingReport_RAN3#127

Annex A: TP for BLCR to TS 38.423

Start of TP

8.4.2	NG-RAN node Configuration Update

8.4.2.1	General

The purpose of the NG-RAN node Configuration Update procedure is to update application level configuration data needed for two NG-RAN nodes to interoperate correctly over the Xn-C interface.

NOTE:	Update of application level configuration data also applies between two NG-RAN nodes in case the SN (i.e. the gNB) does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [8]. How to use this information when this option is used is not explicitly specified.

The procedure uses non UE-associated signalling.

8.4.2.2	Successful Operation

Figure 8.4.2.2-1: NG-RAN node Configuration Update, successful operation

The NG-RAN node1 initiates the procedure by sending the NG-RAN NODE CONFIGURATION UPDATE message to a peer NG-RAN node2.

If Supplementary Uplink is configured at the NG-RAN node1, the NG-RAN node1 shall include in the NG-RAN NODE CONFIGURATION UPDATE message the SUL Information IE and the Supported SUL band List IE for each cell added in the Served NR Cells To Add IE and in the Served NR Cells To Modify IE.

If Supplementary Uplink is configured at the NG-RAN node2, the NG-RAN node2 shall include in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message the SUL Information IE and the Supported SUL band List IE for each cell added in the Served NR Cells IE if any.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

Update of Served Cell Information NR:

-	If Served Cells NR To Add IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, NG-RAN node2 shall add cell information according to the information in the Served Cell Information NR IE.

-	If Served Cells NR To Modify IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, NG-RAN node2 shall modify information of cell indicated by Old NR-CGI IE according to the information in the Served Cell Information NR IE.

-	When either served cell information or neighbour information of an existing served cell in NG-RAN node1 need to be updated, the whole list of neighbouring cells, if any, shall be contained in the Neighbour Information NR IE. The NG-RAN node2 shall overwrite the served cell information and the whole list of neighbour cell information for the affected served cell.

-	If the Deactivation Indication IE set to "deactivated" is contained in the Served Cells NR To Modify IE, it indicates that the coverage of concerned cell was removed to lower energy consumption.

-	If Served Cells NR To Delete IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, NG-RAN node2 shall delete information of cell indicated by Old NR-CGI IE.

-	If the Intended TDD DL-UL Configuration NR IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 should take this information into account for cross-link interference management and/or NR-DC power coordination with the NG-RAN node1. The NG-RAN node2 shall consider the received Intended TDD DL-UL Configuration NR IE content valid until reception of a new update of the IE for the same NG-RAN node2.

-	If the NR Cell PRACH Configuration IE is contained in the Served Cell Information NR IE in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node receiving the IE may use this information for RACH optimisation.

-	If the SFN Offset IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node receiving the IE shall, if supported, use this information to update the SFN0 time offset of the reported cell.

-	If the Supported MBS FSA ID List IE is contained in the Served Cell Information NR IE in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node receiving the IE may use it according to TS 38.300 [9].

-	If the RedCap Broadcast Information IE is contained in the Served Cell Information NR IE in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 may use this information to determine a suitable target in case of subsequent outgoing mobility involving RedCap UEs.

-	If the eRedCap Broadcast Information IE is contained in the Served Cell Information NR IE in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 may use this information to determine a suitable target in case of subsequent outgoing mobility involving eRedCap UEs.

-	If the Mobile IAB Cell IE is included in the Served Cell Information NR IE in the NG-RAN NODE CONFIGURATION UPDATE message or the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message, the receiving NG-RAN node may use it accordingly.

-	If the XR Broadcast Information IE is contained in the Served Cell Information NR IE in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, consider the indicated cell does not allow 2Rx XR UEs in case of subsequent outgoing mobility involving XR UEs.

-	If the Barring Exemption for Emergency Call Information IE is included in the Served Cell Information NR IE in the NG-RAN NODE CONFIGURATION UPDATE message or the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message, the receiving NG-RAN node may use this information to determine a suitable target in case of subsequent outgoing mobility during emergency call.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

Next change

8.4.3	Cell Activation

8.4.3.1	General

The purpose of the Cell Activation procedure is to enable an NG-RAN node to request a neighbouring NG-RAN node to recover the coverage of all the SSB beams or only some of the SSB beams within one or more cells, previously reported as inactive due to energy saving.

The procedure uses non UE-associated signalling.

8.4.3.2	Successful Operation

Figure 8.4.3.2-1: Cell Activation, successful operation

The NG-RAN node1 initiates the procedure by sending the CELL ACTIVATION REQUEST message to the peer NG-RAN node2.

If either the NR Cells IE or the E-UTRA Cells IE is included in the CELL ACTIVATION REQUEST message, the NG-RAN node2 should recover the coverage of the cell/s indicated in the CELL ACTIVATION REQUEST message and shall indicate in the CELL ACTIVATION RESPONSE message for which cells the request was fulfilled.

If case of network sharing with multiple cell ID broadcast with shared Xn-C signalling transport, as specified in TS 38.300 [9], the CELL ACTIVATION REQUEST message and the CELL ACTIVATION RESPONSE message shall include the Interface Instance Indication IE to identify the corresponding interface instance.

If the NR Cells and SSBs IE is included in the CELL ACTIVATION REQUEST message

-	and for an NR cell the SSBs to be Activated List IE is included, the NG-RAN node2 shall, if supported, activate only the SSB beams indicated by the SSBs to be Activated List IE. Otherwise, the NG-RAN node2 shall, if supported, activate all the SSB beams in the cell.

-	and if at least one SSB beam requested in the SSBs to be Activated List IE is activated, the NG-RAN node2 shall include the SSBs Activated List IE in the CELL ACTIVATION RESPONSE message. The NG-RAN node1 shall consider only the SSB beams indicated by the SSBs Activated List IE as activated.

Interactions with NG-RAN Configuration Update procedure:

The NG-RAN node2 shall not send the NG-RAN CONFIGURATION UPDATE message to the NG-RAN node1 just for the reason of the cell(s) or the SSB beam(s) indicated in the CELL ACTIVATION REQUEST message changing cell or SSB beam activation state, as the receipt of the CELL ACTIVATION RESPONSE message by the NG-RAN node1 is used to update the information about the activation state of NG-RAN node2 cell(s) or SSB beam(s) in the NG-RAN node1.

Next change

9.1.3.7	CELL ACTIVATION REQUEST

This message is sent by the NG-RAN node1 to the peer NG-RAN node2 to request a cell(s) or SSB beam(s) whose coverage is previously removed to be recovered.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| CHOICE Served Cells To Activate | M |  |  |  | YES | reject |
| >NR Cells |  |  |  |  |  |  |
| >>NR Cells List |  | 1 |  |  | – |  |
| >>>NR Cells item |  | 1 .. < maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>>>NR CGI | M |  | 9.2.2.7 |  | – |  |
| >E-UTRA Cells |  |  |  |  |  |  |
| >>E-UTRA Cells List |  | 1 |  |  | – |  |
| >>>E-UTRA Cells item |  | 1 .. < maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>>>E-UTRA CGI | M |  | 9.2.2.8 |  | – |  |
| >NR Cells and SSBs |  |  |  |  | YES | ignore |
| >>To Be Activated NR Cells and SSBs List |  | 1 |  |  | – |  |
| >>>To Be Activated NR Cells and SSBs item |  | 1 .. < maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>>>NR CGI | M |  | 9.2.2.7 |  | – |  |
| >>>>SSBs to be Activated List |  | 0..1 |  |  | – |  |
| >>>>>SSBs to be Activated Item |  | 1 .. < maxnoofSSBAreas > |  |  | – |  |
| >>>>>>SSB Index | M |  | INTEGER (0..63) | Identifier of the SSB beam requested to be activated. | – |  |
| Activation ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | reject |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by an NG-RAN node. Value is 16384. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |

9.1.3.8	CELL ACTIVATION RESPONSE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to indicate that one or more cell(s) whose coverage is previously removed has (have) been recovered.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| CHOICE Activated Served Cells | M |  |  |  | YES | reject |
| >NR Cells |  |  |  |  |  |  |
| >>NR Cells List |  | 1 |  |  | – |  |
| >>>NR Cells Item |  | 1 .. < maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>>>NR CGI | M |  | 9.2.2.7 |  | – |  |
| >E-UTRA Cells |  |  |  |  |  |  |
| >>E-UTRA Cells List |  | 1 |  |  | – |  |
| >>>E-UTRA Cells Item |  | 1 .. < maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>>>E-UTRA CGI | M |  | 9.2.2.8 |  | – |  |
| >NR Cells and SSBs |  |  |  |  | YES | ignore |
| >>Activated NR Cells and SSBs List |  | 1 |  |  | – |  |
| >>>Activated NR Cells and SSBs Item |  | 1 .. < maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>>>NR CGI | M |  | 9.2.2.7 |  | – |  |
| >>>>SSBs Activated List |  | 0..1 |  |  | – |  |
| >>>>>SSB Activated Item |  | 1 .. < maxnoofSSBAreas > |  |  | – |  |
| >>>>>>SSB Index | M |  | INTEGER (0..63) | Identifier of the activated SSB beam. | – |  |
| Activation ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by an NG-RAN node. Value is 16384. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |

Next change

9.2.2.15	Served Cells To Update NR

This IE contains updated configuration information for served NR cells exchanged between NG-RAN nodes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Served Cells NR To Add |  | 0 .. < maxnoofCellsinNG-RAN node> |  | List of added cells served by the NG-RAN node. | GLOBAL | reject |
| >Served Cell Information NR | M |  | 9.2.2.11 |  | – |  |
| >Neighbour Information NR | O |  | 9.2.2.13 |  | – |  |
| >Neighbour Information E-UTRA | O |  | 9.2.2.14 |  | – |  |
| >Served Cell Specific Info Request | O |  | 9.2.2.102 |  | YES | ignore |
| Served Cells To Modify NR |  | 0 .. < maxnoofCellsinNG-RAN node> |  | List of modified cells served by the NG-RAN node. | YES | reject |
| >Old NR CGI | M |  | NR CGI 9.2.2.7 |  | – |  |
| >Served Cell Information NR | M |  | 9.2.2.11 |  | – |  |
| >Neighbour Information NR | O |  | 9.2.2.13 |  | – |  |
| >Neighbour Information E-UTRA | O |  | 9.2.2.14 |  | – |  |
| >Deactivation Indication | O |  | ENUMERATED (deactivated, ...) | Indicates that coverage of the concerned cell is removed for energy saving reasons. | – |  |
| Served Cells To Delete NR |  | 0 .. < maxnooffCellsinNG-RAN node > |  | List of deleted cells served by the NG-RAN node. | YES | reject |
| >Old NR-CGI | M |  | NR CGI 9.2.2.7 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RAN node | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

End of TP

Annex B: TP for BLCR to TS 38.473

Start of TP

### 8.2.5	gNB-CU Configuration Update

#### 8.2.5.1	General

The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

#### 8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU Configuration Update procedure: Successful Operation

The gNB-CU initiates the procedure by sending a GNB-CU CONFIGURATION UPDATE message including the appropriate updated configuration data to the gNB-DU. The gNB-DU responds with a GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall interpret that the corresponding configuration data is not changed and shall continue to operate the F1-C interface with the existing related configuration data.

The updated configuration data shall be stored in the respective node and used as long as there is an operational TNL association or until any further update is performed.

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall recover the coverage of the cell indicated by NR CGI IE and reconfigure the physical cell identity for which the NR PCI IE is included.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to remove the coverage of SSB beams within the indicated cells for network energy saving purpose.

End of TP