3GPP TSG-RAN WG3 Meeting #128	R3-253311

Malta, MT, May 19 – 23, 2025

Agenda item:	 10.2

Source:		Lenovo

Title:		(TP for SON BLCR for 38.473) Discussion on MRO for LTM

Document for:		Discussion and Decision

1	Introduction

RAN3#124 meeting agreed that:

For failures due to wrong selection of candidate LTM cell, CU is in charge of root cause analysis and performs optimization.

For failures due to inappropriate cell switch triggering (e.g. wrong cell selection at cell switch, wrong cell switch timing, …) source DU performs optimization.

RAN3 to discuss enhancements related to the information of RACH-less access and RACH-based access from UE related to MRO for LTM.

RAN3#125 meeting agreed that:

Reuse the existing connection failure definition of too late handover, too early handover, handover to wrong cell for LTM failure case. Add a sentence for LTM handover.

Take the existing detection mechanism descriptions as baseline.

For failures due to inappropriate cell switch triggering, CU initiates analysis and may forwards the RLF report to the DU responsible for the failure.

CU needs to forward the RLF related information to DU using existing Access and Mobility Indication procedure.

RAN3#125bis meeting agreed that:

For TP to BL CR to TS 38.300:

- Remove editor’s note 1 and 2. No need to capture LTM recovery in the failure definitions.

- For detection mechanism, use same approach as for CHO i.e. reuse existing sub-bullets to add LTM.

- Change “successful handover" definition, as proposed in R3-245230.

For TP to BL CR to TS 38.401:

- Capture wrong LTM prepared cell selection.

Add failure type to F1AP ACCESS AND MOBILITY INDICATION message.

CU does not forward the RLF Report to DU if failure is due to wrong selection of prepared LTM cell.

For TP to BL CR to TS 38.473:

- In ACCESS AND MOBILITY INDICATION message, the presence of the C-RNTI IE is kept optional. FFS on stage2 or stage3 to define the condition to include this optional C-RNTI.

For TP to BL CR to TS 38.401:

- For failure due to inappropriate cell switch triggering, CU forwards RLF Report to the failure DU in case of too late LTM.

- For failure due to inappropriate cell switch triggering, CU forwards RLF Report to the source DU in case of too early LTM, or LTM to wrong cell.

RAN3#126 meeting agreed that:

CU can adjust the candidate Cell list for LTM based on UHI.

DU should be able to resolve LTM ping-pong based on information over F1.

Network based solution is used for LTM candidate cell list.

RAN3#127bis meeting further agreed that:

BFR shortly after successful LTM cell switch to the wrong beam

RAN3 supports network-based solution for the case of BFR shortly after a successful LTM cell switch.

The target DU identifies the BFR happened in UE shortly after successful LTM cell switch caused due to wrong beam.

In case that the source DU selects a wrong beam among candidate beam list, the source DU is responsible for MRO optimization

In case that the target DU provides a wrong candidate beam list, the target DU is responsible for MRO optimization

Target DU needs to send the recovery beam information to CU and CU forwards it to source DU.

CU does not need to send the old beam information to source DU.

FFS if/how the solution for case 1 could also be used for the case of LTM cell switch failure due to wrong beam (case 2)?

LTM failure due to outdated TA

MRO will cover the scenario that RACH-less LTM fails including outdate TA and UE performs RACH based LTM.

RAN3 will not consider the case of LTM cell switch failure due to outdated TA calculated by UE.

For PDCCH order triggered early TA acquisition, RAN3 consider the case of RACH-less LTM fails due to outdate TA and UE performs RACH-based LTM failure recovery or RRC re-establishment to the same cell.

Whether UE based solution or network based solution is used for LTM failure due to outdated TA?

Whether LTM failure due to outdated TA is defined as a new failure case or it can covered by existing failure case?

Network based or UE based solution for TA acquisition type

For inter-DU case, CU aware TA acquisition type without any signaling impact in RAN3.

FFS for intra-DU case.

UHI and ping-pong issue

For intra-CU case, CU detects the potential LTM ping-pong issue based on local information, DU decides and solves the LTM ping-pong issue.

In this paper, we would discuss left issues of MRO for LTM.

2	Discussion

BFR shortly after successful LTM cell switch to the wrong beam

For the case of BFR shortly after successful LTM cell switch to the wrong beam, last RAN3#127bis meeting agreed to adopt network-based solution, i.e. the target DU identifies the BFR happened in UE shortly after successful LTM cell switch caused due to wrong beam,

- in case that the target DU identifies it provides a wrong candidate beam list, the target DU itself performs MRO optimization;

- otherwise, target DU identifies BFR is caused due to the source DU selects a wrong beam among candidate beam list, then, target DU needs to send the recovery beam information to CU, e.g. via a non-UE associated message (e.g. DU-CU ACCESS AND MOBILITY INDICATION). In this case, target DU also needs to explicitly indicate the BFR is caused by the source DU selecting a wrong beam to CU, e.g. via a non-UE associated message (e.g. DU-CU ACCESS AND MOBILITY INDICATION), in order to trigger CU to forward the recovery beam information to source DU for MRO optimization at source DU side, e.g. via a non-UE associated message (e.g. ACCESS AND MOBILITY INDICATION), also, CU explicitly indicate the BFR is caused by source DU selecting a wrong beam to source DU.

Proposal 1: Besides recovery beam information, target DU explicitly indicate the BFR is caused by source DU selecting a wrong beam to CU, e.g. via DU-CU ACCESS AND MOBILITY INDICATION message.

Proposal 2: Besides recovery beam information, CU explicitly indicate the BFR is caused by source DU selecting a wrong beam to source DU, e.g. via ACCESS AND MOBILITY INDICATION message.

LTM cell switch failure due to wrong beam

Different from the case of BFR shortly after successful LTM cell switch, a RLF report would be stored and reported by the UE for LTM cell switch failure case. We should define LTM cell switch failure due to wrong beam as a new failure case in stage 2 specification, since it can’t be covered by the existing too early LTM or LTM to wrong cell case, in which the failure is caused mainly due to wrong beam/TCI state selection at source DU for LTM cell switch. For example, LTM cell switch failure due to wrong beam/TCI state can be defined in TS38.300 such as: a failure occurs during the LTM cell switch execution; the UE recovers or re-establishes the radio link connection in a cell where the failure occurs, and the TCI state where LTM cell switch execution failed and the TCI state where the UE succeeded to recover or re-establish the radio link connection are different but of the same cell.

Proposal 3: LTM cell switch failure due to wrong beam/TCI state should be defined as a new failure case in stage 2 specification.

For the case of LTM cell switch failure due to wrong beam/TCI state, it is FFS if/how the solution for BFR shortly after successful LTM cell switch could also be used. Since LTM cell switch failure due to wrong beam/TCI state may happen normally, if we acknowledge to consider this case, similar as BFR case, a network-based solution is better regarding RAN2’s preference and quite limited TU.

Proposal 4: RAN3 supports network-based solution for LTM cell switch failure due to wrong beam/TCI state.

However, candidate DU can’t detect the failure type for LTM cell switch failure since it can’t identify and correlate the UE during LTM cell switch and LTM failure recovery/RRC re-establishment.

At network side, it is better for CU to detect the failure type, following what we did for the case of too late LTM, too early LTM, or LTM to wrong cell:

- candidate DU sends the TCI state ID where LTM cell switch failure happened, and, the TCI state ID where the UE successfully performed LTM failure recovery or RRC re-establishment to CU, e.g. via DU-CU ACCESS AND MOBILITY INDICATION message;

- after receiving RLF report, CU detects the failure is caused due to wrong beam/TCI state selection at source DU; then, CU sends a new failure type (e.g. wrong beam/TCI state selection at source DU) to source DU, e.g. via ACCESS AND MOBILITY INDICATION message; also, CU transfers the TCI state ID where LTM cell switch failure happened, and, the TCI state ID where the UE successfully performed LTM failure recovery or RRC re-establishment to source DU;

- based on the received ACCESS AND MOBILITY INDICATION message, source DU may perform corresponding MRO optimisation, e.g. update TCI state ID in the LTM cell switch command MAC CE.

Proposal 5: To detect LTM cell switch failure due to wrong beam/TCI state:

- candidate DU sends the TCI state ID where LTM cell switch failure happened, and the TCI state ID where the UE successfully performed LTM failure recovery or RRC re-establishment to CU, e.g. via DU-CU ACCESS AND MOBILITY INDICATION message;

- CU then detects the failure is caused due to wrong TCI state selection at source DU;

- CU then sends a new failure type (e.g. wrong TCI state selection at source DU), the TCI state ID where LTM cell switch failure happened and the TCI state ID where the UE successfully performed LTM failure recovery or RRC re-establishment to source DU, e.g. via ACCESS AND MOBILITY INDICATION message.

LTM cell switch failure due to outdated TA

Last meeting agreed that for PDCCH order triggered early TA acquisition, RAN3 consider the case of RACH-less LTM fails due to outdate TA and UE performs RACH-based LTM failure recovery or RRC re-establishment to the same cell. This failure case can’t be covered by the existing one, since the failure is caused mainly due to the maintained early TA value is outdated other than the LTM cell switch command MAC CE is triggered at a early time point or a wrong target cell is included in the LTM cell switch command MAC CE. So, LTM cell switch failure due to outdated TA should be defined as a new failure case in stage 2 specification.

Proposal 6: LTM cell switch failure due to outdated TA should be defined as a new failure case in stage 2 specification.

Another FFS issue is whether to adopt UE based solution or network based solution for LTM failure due to outdated TA. Since only two meetings are left in R19 and RAN2 has quite limited TU, similar as MRO for BFR/LTM cell switch failure due to wrong beam, we should prioritize network-based solution.

At network side, to detect LTM cell switch failure due to outdated TA:

- source DU sends the configured TA value to CU, e.g. via DU-CU Cell Switch Notification message;

- candidate DU sends the TA value used for LTM failure recovery or RRC re-establishment to CU, e.g. via ACCESS SUCCESS message;

- after receiving RLF report, CU identifies the failure is caused due to outdated TA, and sends a new failure type (e.g. outdated TA) to source DU, e.g. via Access and Mobility Indication message; also, CU transfers the configured TA value, and, the TA value used for LTM failure recovery or RRC re-establishment to source DU;

- based on the received Access and Mobility Indication message, source DU may perform corresponding MRO optimisation, e.g. update the timer duration for checking the TA validity.

Proposal 7: To detect LTM cell switch failure due to outdated TA:

- source DU sends the configured TA value to CU, e.g. via DU-CU Cell Switch Notification message;

- candidate DU sends the TA value used for LTM failure recovery or RRC re-establishment to CU, e.g. via ACCESS SUCCESS message;

- CU identifies the failure is caused due to outdated TA, and then sends a new failure type (e.g. outdated TA), the configured TA value and the TA value used for LTM failure recovery or RRC re-establishment to source DU, e.g. via Access and Mobility Indication message.

UHI and ping-pong issue

Last RAN3#127bis meeting agreed that for intra-CU case, CU detects the potential LTM ping-pong issue based on local information, DU decides and solves the LTM ping-pong issue. The potential options were summarized in [1] as following:

- Option 1: CU sends the UHI to DU upon successful LTM cell switch/UHI is updated, e.g. via the CU-DU CELL SWITCH NOTIFICATION message.

- Option 2: CU sends the UHI to DU when LTM ping-pong is detected, e.g. via the ACCESS AND MOBILITY INDICATION message.

- Option 3: CU sends an indication to DU that LTM ping-pong has occurred, e.g. via the CU-DU CELL SWITCH NOTIFICATION message.

- Option 4: CU filters the LTM related UHI, and sends to DU via UE Context Setup Request, UE Context Modification Request or CU-DU CELL SWITCH NOTIFICATION.

- Option 5: No need to send UHI from CU to DU.

For intra-DU LTM, the source DU itself can store the UHI, but for subsequent LTM in inter-DU intra-CU LTM procedure, source DU may not always have the latest and whole UHI information. So, Option 5 should be excluded since it can’t enable LTM ping-pong decision and resolution at DU.

Option 1 or Option 4 would cause much signalling overhead, since UHI would be updated frequently especially for subsequent LTM. In Option 3, only an indication concerning LTM ping-pong has occurred is transferred in F1, but it is not sufficient for LTM ping-pong decision and resolution.

In general, Option 2 is preferred with less signalling overhead, since CU only needs to forward the UHI to DU when ping-pong is detected e.g. via the ACCESS AND MOBILITY INDICATION message, then DU can perform ping-pong resolution based on the indicated UHI from CU.

Proposal 8: CU can forward the UHI to DU when ping-pong is detected, e.g. via the ACCESS AND MOBILITY INDICATION message.

Stage2 or stage3 to define the condition of the optional C-RNTI IE in ACCESS AND MOBILITY INDICATION message?

Based on agreements achieved so far, we can sumarize the failure cause analysis and optimisation as well as RLF report forwarding in F1 as below:

- For failures due to wrong selection of candidate LTM cell, CU is in charge of root cause analysis and performs optimization, CU does not forward the RLF Report to DU;

- For failures due to inappropriate cell switch triggering (e.g. wrong cell selection at cell switch, wrong cell switch timing, …), CU initiates analysis and may forward the RLF report to the DU responsible for the failure (e.g. CU forwards RLF Report to the failure DU in case of too late LTM, or, CU forwards RLF Report to the source DU in case of too early LTM or LTM to wrong cell) , the DU responsible for the failure performs optimization.

For the RLF shortly after successful LTM cell switch, C-RNTI of target cell is included in RLF report, however, in case of too early LTM or LTM to wrong cell, the C-RNTI of source PSCell needs to be transferred in F1 interface together with the RLF report to enable source DU to identify UE context. Compared with adding description in TS38.401, it is more straightforward to clarify the presence of the optional C-RNTI IE in ACCESS AND MOBILITY INDICATION message in TS38.473 to improve text readability. The TP for TS38.473 is attached in the Annex.

Proposal 9: Clarify the presence of the optional C-RNTI IE in ACCESS AND MOBILITY INDICATION message in TS38.473.

Network based or UE based solution for TA acquisition type?

It is FFS on whether CU is aware of TA acquisition type without any signalling impact in RAN3 for inter-DU case or intra-DU case.

We can find that for inter-DU case, source DU may send the TA value(s) related information applicable for subsequent LTM to CU in the DU-CU CELL SWITCH NOTIFICATION message, since target cell for the triggered LTM cell switch may also be available for later subsequent LTM execution, TA value of this target cell may be sent to CU, which may implicitly indicate to CU that it is PDCCH order triggered early TA acquisition type. Therefore, CU is aware of TA acquisition type without any signalling impact for inter-DU case.

Proposal 10: CU is aware of TA acquisition type without any signalling impact for inter-DU case.

For intra-DU case, DU may also send the DU-CU CELL SWITCH NOTIFICATION message to CU. Based on current stage 2 description in TS38.401, DU sends the DU-CU CELL SWITCH NOTIFICATION message to the CU to indicate the initiation of the Cell Switch Command to the UE including the target cell ID, but after checking stage 3 details for DU-CU CELL SWITCH NOTIFICATION message, it is not restricted that the TA value(s) can only be included in this message for inter-DU case, which means that current stage 3 signalling design can also support intra-DU case without any signalling impact. If so, CU is also aware of TA acquisition type without any signalling impact for intra-DU case.

Proposal 11: Current signalling design of the DU-CU CELL SWITCH NOTIFICATION message may also support the CU to be aware of TA acquisition type without any signalling impact for intra-DU case.

If stage 2 principle should be strictly followed for intra-DU case, then, currently CU can’t know TA acquisition type for intra-DU case. One solution is that DU indicates the TA acquisition type to CU, but it would introduce signalling impacts. RAN2#127bis meeting agreed that unless RAN3 defines a NW-based solution, the UE logs and reports whether and how the UE got the TA value used for a failed LTM switch (gNB indicated or UE determined), since there is an already agreed UE-based solution, if RAN3 can’t achieve consensus on whether/which network based solution to be adopted for TA acquisition type in this meeting, we would like to maintain the agreed UE based solution, for sake of reducing more workload in RAN2/3.

Proposal 12: If RAN3 can’t achieve consensus on whether/which network based solution to be adopted for TA acquisition type, the UE based solution agreed by RAN2 should be maintained.

3	Conclusion

In this paper, MRO for LTM are further discussed. We have the following observations and proposals:

Proposal 1: Besides recovery beam information, target DU explicitly indicate the BFR is caused by source DU selecting a wrong beam to CU, e.g. via DU-CU ACCESS AND MOBILITY INDICATION message.

Proposal 2: Besides recovery beam information, CU explicitly indicate the BFR is caused by source DU selecting a wrong beam to source DU, e.g. via ACCESS AND MOBILITY INDICATION message.

Proposal 3: LTM cell switch failure due to wrong beam/TCI state should be defined as a new failure case in stage 2 specification.

Proposal 4: RAN3 supports network-based solution for LTM cell switch failure due to wrong beam/TCI state.

Proposal 5: To detect LTM cell switch failure due to wrong beam/TCI state:

- candidate DU sends the TCI state ID where LTM cell switch failure happened, and the TCI state ID where the UE successfully performed LTM failure recovery or RRC re-establishment to CU, e.g. via DU-CU ACCESS AND MOBILITY INDICATION message;

- CU then detects the failure is caused due to wrong TCI state selection at source DU;

- CU then sends a new failure type (e.g. wrong TCI state selection at source DU), the TCI state ID where LTM cell switch failure happened and the TCI state ID where the UE successfully performed LTM failure recovery or RRC re-establishment to source DU, e.g. via ACCESS AND MOBILITY INDICATION message.

Proposal 6: LTM cell switch failure due to outdated TA should be defined as a new failure case in stage 2 specification.

Proposal 7: To detect LTM cell switch failure due to outdated TA:

- source DU sends the configured TA value to CU, e.g. via DU-CU Cell Switch Notification message;

- candidate DU sends the TA value used for LTM failure recovery or RRC re-establishment to CU, e.g. via ACCESS SUCCESS message;

- CU identifies the failure is caused due to outdated TA, and then sends a new failure type (e.g. outdated TA), the configured TA value and the TA value used for LTM failure recovery or RRC re-establishment to source DU, e.g. via Access and Mobility Indication message.

Proposal 8: CU can forward the UHI to DU when ping-pong is detected, e.g. via the ACCESS AND MOBILITY INDICATION message.

Proposal 9: Clarify the presence of the optional C-RNTI IE in ACCESS AND MOBILITY INDICATION message in TS38.473.

Proposal 10: CU is aware of TA acquisition type without any signalling impact for inter-DU case.

Proposal 11: Current signalling design of the DU-CU CELL SWITCH NOTIFICATION message may also support the CU to be aware of TA acquisition type without any signalling impact for intra-DU case.

Proposal 12: If RAN3 can’t achieve consensus on whether/which network based solution to be adopted for TA acquisition type, the UE based solution agreed by RAN2 should be maintained.

Reference

[1] R3-252298, SoD of MRO for LTM, Lenovo

Annex: TP for TS38.473

<<<<<<<<<<<<<<<<<<<< Start of Change >>>>>>>>>>>>>>>>>>>>

### 9.2.2	UE Context Management messages

<skip>

#### 9.2.2.14	ACCESS SUCCESS

This message is sent by the gNB-DU to inform the gNB-CU of which cell the UE has successfully accessed during conditional handover, conditional PSCell addition, conditional PSCell change, LTM, or subsequent CPAC.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| NR CGI | M |  | 9.3.1.12 |  | YES | reject |
| TA value of RA based access | O |  | INTEGER (0..4095) | Indicates the TA value used for RA based LTM failure recovery or RRC re-establishment. | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.2.15	DU-CU CELL SWITCH NOTIFICATION

This message is sent by the gNB-DU to inform the gNB-CU about the initiation of the cell switch  command to the UE.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cell ID | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateId IE, as defined in TS 38.331 [8]. | - |  |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8]. | YES | ignore |
| TA value of LTM | O |  | INTEGER (0..4095) | Indicates the TA value included in LTM cell switch command as defined in TS 38.213 [31]. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.2.10	Self Optimisation Support Messages

#### 9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RA Report List |  | 0..1 |  |  | YES | ignore |
| >RA Report Item |  | 1 .. <maxnoofRAReports> |  |  | - |  |
| >>RA Report Container | M |  | OCTET STRING | Includes the RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | Includes the nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| >>C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the source gNB-DU in case that the connection Failure type IE in the nr-RLF-Report-r16 IE is set to “RLF” and RLF Report Failure Type IE is set to “too early LTM” or “LTM to wrong cell” | YES | ignore |
| >>RLF Report Failure Type | O |  | ENUMERATED (too late LTM, too early LTM, LTM to wrong cell, LTM with wrong TCI state, outdated TA, ...) |  | YES | ignore |
| >> TA value of LTM | O |  | INTEGER (0..4095) | Indicates the TA value included in LTM cell switch command as defined in TS 38.213 [31]. | YES | ignore |
| >> TA value of RA based access | O |  | INTEGER (0..4095) | Indicates the TA value used for RA based LTM failure recovery or RRC re-establishment. | YES | ignore |
| >> TCI State of Reconnection | O |  | OCTET STRING | Includes the TCI-State Id used for LTM failure recovery or RRC re-establishment. | YES | ignore |
| >> Failed TCI State | O |  | OCTET STRING | Indicates the TCI-StateId IE used for LTM cell switch, as defined in TS 38.331 [8]. | YES | ignore |
| Successful HO Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report Information Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | - |  |
| >>Successful HO Report Container | M |  | OCTET STRING | Includes the SuccessHO-Report IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| Successful PSCell Change Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful PSCell Change Report information Item |  | 1..<maxnoofSuccessfulPSCellChangeReports> |  |  | - |  |
| >>Successful PSCell Change Report Container | M |  | OCTET STRING | Includes the SuccessPSCell-Report IE as defined in TS 38.331 [8]. | - |  |
| Beam Failure Recovery Information | O |  |  |  | YES | ignore |
| >TCI State of Beam Failure Recovery | M |  | OCTET STRING | Includes the TCI-State Id used for Beam Failure Recovery. | - |  |
| >Wrong TCI State Indication | M |  |  | Indicates BFR occurs due to Wrong TCI-StateId IE as defined in TS 38.331 [8] | - |  |
| UE History Information List |  | 0..1 |  |  | YES | ignore |
| > UE History Information Item |  | 1..< maxnoofCellsinUEHistoryInfo> |  |  | - |  |
| >>NR CGI |  |  | 9.3.1.12 |  | - |  |
| >>Time Stay |  |  | INTEGER (0..40950) | The duration of time the UE stayed in the cell in 1/10 seconds. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRAReports | Maximum no. of RA Reports, the maximum value is 64. |
| maxnoofRLFReports | Maximum no. of RLF Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64. |
| maxnoofSuccessfulPSCellChangeReports | Maximum no. of Successful PSCell Change Reports. Value is 64. |
| maxnoofCellsinUEHistoryInfo | Maximum no. of cells in the UE history information. Value is 16. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.10.2	DU-CU ACCESS AND MOBILITY INDICATION

This message is sent by the gNB-DU to provide access and mobility information to the gNB-CU.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| DL LBT Failure Information List |  | 0..1 |  |  | YES | ignore |
| > DL LBT Failure Information Item |  | 1 .. <maxnoofLBTFailureInformation> |  |  | - |  |
| >>DL LBT Failure Information | M |  | 9.3.1.327 |  | - |  |
| Beam Failure Recovery Information | O |  |  |  | YES | ignore |
| >TCI State of Beam Failure Recovery | M |  | OCTET STRING | Includes the TCI-State Id used for Beam Failure Recovery. | - |  |
| >Wrong TCI State Indication | M |  |  | Indicates BFR occurs due to Wrong TCI-StateId IE defined in TS 38.331 [8] | - |  |
| LTM Failure Information | O |  |  |  | YES | ignore |
| >TCI State of Reconnection | M |  | OCTET STRING | Includes the TCI-State Id used for LTM failure recovery or RRC re-establishment. | - |  |
| >Failed TCI State | M |  | OCTET STRING | Indicates the TCI-StateId IE used for LTM cell switch, as defined in TS 38.331 [8]. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLBTFailureInformation | Maximum no. of UEs for which LBT Failure Information is provided, the maximum value is 64. |