3GPP TSG-RAN WG3 Meeting #128	R3-253323

Malta, MT, 19th – 23th May, 2025

Agenda Item:	12.3

Source: 	Lenovo

Title: 	(TP to BL CR 38.300) Function split for NR Femto

Document for:	Discussion and decision

# Introduction

In this paper, text is proposed to capture the aspects related to NR Femto based on conclusions in another paper [1] and TR 38.799 [2].

# Reference

- R3-253322. Discussion on remaining issues for NR Femto. Lenovo.

- TR 38.799. Study on additional topological enhancements for NR (Release 19).

# TP to 38.300

START OF CHANGE

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[x]	3GPP TS 33.545: "Security aspects of NR Femto".

NEXT CHANGE

### 4.X.2	Functional Split

#### 4.X.2.1	NR Femto

An NR Femto hosts the same functions as a gNB as described in clause 4.1, with the following additional specifics in case of connection to the NR Femto GW:

-	Discovery of a suitable Serving NR Femto GW;

NOTE: The NR Femto may be configured with the identity and/or address of an appropriate NR Femto GW.

-	An NR Femto shall only connect to a single NR Femto GW at one time:

-	The NR Femto will not simultaneously connect to another NR Femto GW, or another AMF.

NOTE:	NR Femtos may be deployed without network planning. An NR Femto may be moved from one geographical area to another and therefore it may need to connect to a different NR Femto GW depending on its location.

-  	The TAC and PLMN ID(s) used by the NR Femto shall also be supported by the NR Femto GW;

-	Selection of an AMF at UE attachment is performed by the NR Femto GW:

-	AMF selection is based on the GUAMI or requested NSSAI, if received from the UE, provided to the NR Femto GW in the INITIAL UE MESSAGE message.

-	At Xn handover:

-	Allowing the NR Femto GW to identify the serving AMF at path switch:

-	Including the GUAMI of the serving AMF in the PATH SWITCH REQUEST message.

-	Allowing the AMF to identify the termination of the source UE associated signalling connection:

-	Including the Source AMF UE NGAP ID to the NR Femto GW in the PATH SWITCH REQUEST message.

#### 4.X.2.2	NR Femto GW

The NR Femto GW hosts the following functions:

-	Relaying UE-associated NGAP messages between the AMF and the NR Femto serving the UE, applying the following additional functions:

-	Terminating the UE Context Release request procedure if an explicit GW Context Release Indication is included. In this case, the NR Femto GW releases the UE context if it determines that the UE identified by the received UE NGAP IDs is no longer served by another NR Femto attached to it.

-	At UE context establishment (Initial Context Setup or NG Handover) the NR Femto GW sends to the NR Femto the serving AMF’s GUAMI as well as the AMF UE NGAP ID assigned by the AMF and the AMF UE NGAP ID assigned by the NR Femto GW for the UE.

-	At Path Switch, the NR Femto GW sends to the NR Femto the AMF UE NGAP ID assigned by the AMF and the AMF UE NGAP ID assigned by the NR Femto GW for the UE.

-	In case of NGAP INITIAL UE MESSAGE message, verifying, as defined in TS 33.545 [x], that the reported CAG ID is valid for the NR Femto.

-	Terminating non-UE associated NGAP procedures towards the NR Femto and towards the AMF, applying the following additional functions:

- 	In case of NG PWS Restart Indication and PWS Failure Indication, replacing the gNB ID of the NR Femto with the NR Femto GW ID before sending the respective message to the AMF.

-	At Overload Start/Stop, the NR Femto GW should provide the NR Femto with the identities of the affected AMF node(s). The NR Femto uses the received information to identify the traffic and the AMF to which the overload indication applies. The NR Femto shall apply the defined rejections until reception of an OVERLOAD STOP message applicable to this traffic, or until the NR Femto receives a further OVERLOAD START message applicable to the same traffic, in which case it shall replace the ongoing overload action with the newly requested one.

-	In case of NG SETUP REQUEST message, verifying, as defined in TS 33.545 [x], that the identity used by the NR Femto is valid.

-	In case of NGAP PWS RESTART INDICATION message and NGAP PWS FAILURE INDICATION message, verifying, as defined TS 33.545 [x], that the indicated cell identity is valid.

-	Supporting TAC and PLMN ID used by the NR Femto.

-	Relaying the PATH SWITCH REQUEST message towards the AMF indicated by the GUAMI of the source AMF received from the NR Femto.

#### 4.X.2.3	AMF

In addition to functions specified in clauses 4.1 and 16.7, the AMF hosts the following functions:

-	Routing of handover messages and Downlink RAN Configuration Transfer message towards NR Femto GWs based on the Selected TAI contained in these messages.

-	In case of an NR Femto directly connected:

-	verifying as defined in TS 33.545 [x], that the CAG ID reported in the NGAP INITIAL UE MESSAGE message is valid for the indicated cell;

-	verifying as defined in TS 33.545 [x], that the identity used by the NR Femto is valid when receiving the NG SETUP REQUEST message;

-	and verifying, as defined in TS 33.545 [x], that the indicated gNB identity of the NR Femto is valid when receiving the NGAP PWS RESTART INDICATION message and the NGAP PWS FAILURE INDICATION message.

A TAI used in a NR Femto GW shall not be reused in another NR Femto GW.