3GPP TSG-RAN WG3 Meeting #128	R3-253327

Malta, MT, 19th – 23rd May, 2025

Agenda item:	16.2

Source: 	Lenovo

Title: 	Discussion on Open issues for A-IoT procedures

Document for:	Other

- 1. Introduction

The contribution discusses the open issues on A-IoT management, inventory and command related procedures.

- 2. Discussion

2.1	General

Explicit A-IoT Only Indication

In RAN3#127bis meeting, it was agreed that

Introducing an AIoT indicator in the NGAP Setup Request message. The detail of this indicator can be further discussed.

If the NG-RAN node only supports A-IoT radio, there are some mandatory IEs not applicable to A-IoT should be ignored. It would be cleaner to introduce an explicit indication e.g., Only Support A-IoT IE in the NG Setup Request message so that AMF/AIOTF can ignore the mandatory IEs that are not applicable to A-IoT.

Proposal 1: Introduce an explicit IE e.g., Only Support A-IoT IE in the NG Setup Request message so that AMF/AIOTF can ignore the mandatory IEs that are not applicable to A-IoT.

A-IOT Area

According to the discussion in Ran3#127bis meeting, it is FFS whether the new A-IoT Area is encoded as GNSS info or new A-IoT Area ID.

As asked by SA2 [1], the first question is whether the ‘supported Area’ is Tracking Area or a new type of area.

-	Option 1: Use A-IoT dedicated TAI;

-	Option 2: a new A-IoT Area.

In option 2, a new area ID should be introduced which needs to coordinate with other groups. While for option 2, it can be reused directly. We slightly prefer to reuse the TAI for identify the A-IOT area.

Proposal 2: Use A-IoT dedicated TAI for identifying the A-IOT area.

SA2 also asks whether one reader is only part of one supported Area or can be part of multiple supported Areas. From our view, for simplicity, one reader can only be part of one supported area.

Proposal 3: one reader can only be part of one A-IOT area.

In previous meeting, we agreed that AIOTF obtains the A-IoT RAN information (supported Area, served reader ID list) via OAM configuration, FFS on RAN3 signalling for the A-IoT RAN information.

We think that the supported area, the served read list and reader location can be configured in NG-RAN node only. For example, we add or remove one reader, the upgraded configuration can be configured in NG-RAN node and then the NG-RAN node can send the upgraded configuration to AIOTF by signalling. from this point of view, it makes sense to support signalling the supported rea(s), the Served Reader List and the Location of Reader from NG-RAN node to AIOTF.

Proposal 4: Support signalling the supported area(s), the Served Reader List and the Location of Reader from NG-RAN node to AIOTF.

We concur with the proposal from some companies to use the following to represent the location of the reader:

-	Civic Address (as defined in TS 29.572),

-	NG-RAN Access Point Position, and

-	NG-RAN High Accuracy Access Point Position (both as defined in TS 38.455).

Proposal 5: Use the Civic Address, NG-RAN Access Point Position and NG-RAN High Accuracy Point Position to represent the location of reader.

2.2	Inventory

Estimate of Expected D2R Message Size

As discussed in RAN2#129 meeting, RAN2 assumes that the reader should know the inventory response size by Inventory Assistance Information to avoid supporting segmentation function for msg 3:

Agreements on segmentation

- For inventory response, RAN2 assumes that segmentation is not applied.  RAN2 assumes that the reader can avoid segmentation by reader being aware of inventory response size.

As specified in TS 23.369, the AIoT Device Permanent Identifier is shown as the following figure.

Figure 5.7.2-1: AIoT Device Permanent Identifier Structure

An operator allocated AIoT Device Permanent Identifier should include the identifier of the network for the operator. The identifier of the network is present as either a PLMN Identifier, NID or both in the AIoT Device Permanent Identifier.

A third party allocated AIoT Device Permanent Identifier may include none of the following information or include any combination of at least one kind of the following information: a PLMN Identifier, NID or third party identifier.

The length of ID Type, PLMN Identifier (if present), NID (if present) and the third party identifier (if present) components is fixed. The following lengths are supported for the Identification Information in an AIoT Device Permanent Identifier: 96 bits, and 128 bits.

From these perspectives, the AIoF is able to know the length of the AIoT Device Permanent Identifier and thus is able to provide the Estimate of Expected D2R message size in the Inventory Assistance Information.

Proposal 6: The Inventory Assistance Information should include the Estimate of Expected D2R Message Size.

AIoT aggregation assistance information

As per SA2’s agreement, the AIOTF may provide the AIoT aggregation assistance information to the AIoT RAN. Aggregation assistance information is used to assist in the aggregation of AIoT responses received from multiple AIoT devices. The AIOTF may determine the aggregation assistance information based on operator policy, considering information provided from the AF, which consists of:

-	Time interval: The fixed time interval for which AIoT RAN collects AIoT Device responses before AIoT RAN reports the aggregated AIoT responses to AIOTF. This may potentially happen multiple times until the AIoT RAN completes the request operation. If the AF has provided a time interval, then the AIOTF should signal a time interval to AIOT RAN that is equal or shorter than the time interval received from the AF.

Proposal 7: The Inventory Assistance Information should include the time interval as AIoT aggregation assistance information.

AIOT Device Type

Considering that AIoT devices have limited energy storage and rely on energy harvesting from radio waves, light, or other suitable power sources, they cannot always be in an “always-on” state to receive messages from the NG-RAN node. Therefore, the NG-RAN node needs to determine the optimal inventory starting time or completion time for these devices.

As discussed in RAN1, if the antenna is shared between the RF energy harvester and the receiver/transmitter, the NG-RAN node cannot communicate directly with the AIoT device while it is performing energy harvesting. To address this, it would be beneficial for the AIOTF to provide assistance information in the Inventory Request message to the NG-RAN node. This information can help the NG-RAN node determine the inventory timing for the AIoT device.

For example, based on the antenna type (e.g., whether it is shared or separate between the RF energy harvester and receiver/transmitter) provided by the AIOTF, the NG-RAN node can determine the duration for which RF energy should be provided to the AIoT device for harvesting, or the waiting time required for the AIoT device to complete energy harvesting.

Proposal 8: The Inventory Assistance Information should include AIoT device type which will enable the NG- RAN node to more effectively determine the optimal inventory starting time or completion time.

A-IoT Device Identification Requested

In the BL CR, the definition of A-IoT Device Identification Requested IE is still FFS:

9.3.3.xx5	A-IoT Device Identification Requested

This IE represents the Requested Identity of the A-IoT device(s).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| FFS |  |  |  |  |

Editor’s Note: this IE enables identification of a single A-IoT device, a group of A-IoT devices, all A-IoT devices, detailed coding FFS.

As specified in TS 23.369, the AIoT Identification Information to be provided to NG-RAN can include Filtering Information, or a single AIoT Device Identifier. The Filtering Information is used to represent a group of AIoT devices.

The filtering information is used to identify or filter multiple AIoT Devices and is constructed by one or multiple components of the AIoT Device Permanent Identifier. The filtering information includes a list of filtering element information. The filtering element information includes:

-	Information indicating a which component of the AIoT Device Permanent Identifier that is used to match the bitstring.

-	A bitstring which is used to compare with the component.

Proposal 9: The A-IoT Device Identification Requested IE includes either the filtering information or a single AIOT Device ID.

Requested Service Area Information

In RAN3#127, it was agreed that

Upon receiving neither the area nor the reader list in Inventory Request, the gNB selects all the served readers.

However, the Requested Service Area Information IE is mandatory in the Inventory Request Transfer IE, which is not aligned with the agreement.

Proposal 10: The presence of the Requested Service Area Information IE should be optional in the Inventory Request Transfer IE.

Contents of inventory report

In the current BL CR, NG-RAN node can include a list of AIoT device IDs in the Inventory Report Transfer IE. It means that the NG-RAN node can aggregate the inventory reporting for a AIoT inventory session. It is assumed that how to aggregate different AIoT device IDs is up to NG-RAN node’s implementation.

For an inventory session, AIOTF is expected to receive an end indication from AIoT RAN, signaling the completion of reporting for a specific correlation ID.

Proposal 11: The Inventory Report Transfer IE should include an end indication to indicate the completion of reporting for the inventory session.

2.3	Command

Correlation ID outside of the transfer IEs

In the case of indirect communication, we believe that the AMF does not need to be aware of the device association which should be included in the transfer IEs. However, to enable parallel Command Request procedure between the gNB and the AMF, the correlation ID should be included outside the AIOTF Transfer IEs.

Proposal 12: The A-IoT Correlation Identifier should be included outside of the transfer IEs in Command related message.

Command Cancel Procedure

In last meeting, we agreed to include an ‘Follow on Command Indication’ in the inventory request message in case a Command message followed by inventory. If the ‘Follow on Command Indication’ is provided, the NG-RAN node assumes a Command Request will come followed by the inventory report message.

As specified by SA2, when receives A-IoT device ID in the inventory report message, the AIOTF validates the results as specified in TS 33.369 [9], and determines whether the command should be sent to an AIoT Device, e.g., by checking the Target AIoT device information. 	For each successful Inventory Report received, the AIOTF sends Command Request message to the NG-RAN node.

If the AIOTF fails to validate the AIoT device ID, it will not initiate the Command Request procedure. In this case, AIOF should inform NG-RAN node to stop waiting for the command request message. Otherwise, the NG-RAN still wait for the command request message based on the received ‘Follow on Command Indication’.

Proposal 13: A Command Cancel procedure with providing the RAN A-IoT Device NGAP ID should be defined to inform the NG-RAN node the Command procedure is cancelled by the AIOTF.

Command End Indication

There would be multiple signalling exchange between NG-RAN node and AIOTF for a Command procedure. In this case, NG-RAN node should know the command procedure is complete and then NG-RAN node can perform inventory towards next A-IoT device.

Proposal 13: A Command End Indication or Command Release should be provided to NG-RAN node from the AIOTF.

A-IoT Device NGAP ID

In last meeting, the RAN A-IoT Device NGAP ID was introduced which is allocated so as to uniquely identify an A-IoT device context over the NG interface within a NG-RAN node for a certain A-IoT session. However, introduction of the AIOTF A-IoT Device NGAP ID is FFS. We believe the AIOTF A-IoT Device NGAP ID is needed to uniquely identify an A-IoT device context over the NG interface within an AIOTF for a certain A-IoT session.

AIOTF A-IoT Device NGAP ID A AIOTF A-IoT Device NGAP ID shall be allocated so as to uniquely identify an A-IoT device context over the NG interface within a AIOTF for a certain A-IoT session. When an NG-RAN node receives an AIOTF A-IoT Device NGAP ID, it shall store it for the duration of the A-IoT session. The AIOTF A-IoT Device NGAP ID shall be unique per A-IoT session within the logical AIOTF.

Proposal 14: AIOTF A-IoT Device NGAP ID is needed to uniquely identify an A-IoT device context over the NG interface within an AIOTF for a certain A-IoT session.

Command Type (read, write, disable…)

From our point of view, command type is useful especially for resource scheduling. For example, if command type is ‘read’ and reader is visible of it, reader may include a large amount of D2R resource for the data to be read or reader may request D2R message size from device, when sending the command to the device. If the command type is ‘write’ or ‘disable’ and reader is visible of it, reader may include a small amount of D2R resource for ACK/NACK when sending the command to the device, and doesn’t need to request D2R message size from device. Hence, command type indication (e.g., read/write/disable) is preferred than just indicating ‘command’, without introducing more signalling overhead and with benefits for resource scheduling.

Proposal 15: Command type (e.g., read/write/disable) information is useful to be visible to the NG-RAN node from AIOTF.

- 3. Conclusion

Based on the discussion in this paper, we have the following proposals.

General Issues

Proposal 1: Introduce an explicit IE e.g., Only Support A-IoT IE in the NG Setup Request message so that AMF/AIOTF can ignore the mandatory IEs that are not applicable to A-IoT.

Proposal 2: Use A-IoT dedicated TAI for identifying the A-IOT area.

Proposal 3: one reader can only be part of one A-IOT area.

Proposal 4: Support signalling the supported area(s), the Served Reader List and the Location of Reader from NG-RAN node to AIOTF.

Proposal 5: Use the Civic Address, NG-RAN Access Point Position and NG-RAN High Accuracy Point Position to represent the location of reader.

Inventory

Proposal 6: The Inventory Assistance Information should include the Estimate of Expected D2R Message Size.

Proposal 7: The Inventory Assistance Information should include the time interval as AIoT aggregation assistance information.

Proposal 8: The Inventory Assistance Information should include AIoT device type which will enable the NG- RAN node to more effectively determine the optimal inventory starting time or completion time.

Proposal 9: The A-IoT Device Identification Requested IE includes either the filtering information or a single AIOT Device ID.

Proposal 10: The presence of the Requested Service Area Information IE should be optional in the Inventory Request Transfer IE.

Proposal 11: The Inventory Report Transfer IE should include an end indication to indicate the completion of reporting for the inventory session.

Command

Proposal 12: The A-IoT Correlation Identifier should be included outside of the transfer IEs in Command related message.

Proposal 13: A Command Cancel procedure with providing the RAN A-IoT Device NGAP ID should be defined to inform the NG-RAN node the Command procedure is cancelled by the AIOTF.

Proposal 14: AIOTF A-IoT Device NGAP ID is needed to uniquely identify an A-IoT device context over the NG interface within an AIOTF for a certain A-IoT session.

Proposal 15: Command type (e.g., read/write/disable) information is useful to be visible to the NG-RAN node from AIOTF.

- 4. Reference

- [1] R3-253015 LS Reply on Ambient IoT progress of RAN3	SA2(NEC)