3GPP TSG-RAN WG3 Meeting #128	R3-253352

Malta, MT, 19 – 23 May, 2025

Title:	(TP for SON BLCR for 38.473) Ping-Pong

Agenda Item:	10.2

Source:	Huawei, CMCC, Qualcomm, China Unicom, Jio Platforms

Document for:	discussion

# 1	Introduction

This is a text proposal to capture the UE history enhancements to the BL CR.

# Text proposal

# Annex	TP for LTM for 38.473

#### 9.2.2.16	CU-DU CELL SWITCH NOTIFICATION

This message is sent by the gNB-CU to inform the gNB-DU about the initiation of the cell switch command to the UE.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cell ID | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateId IE, as defined in TS 38.331 [8]. | - |  |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8]. | YES | ignore |
| LTM UE History | O |  | 9.3.1.x |  | YES | Ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |

[snip]

#### 9.3.1.x	LTM UE History

This IE includes information on previously visited cells associated with LTM mobility.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| LTM UE History List |  | 1 |  |  |
| >LTM UE History Item |  | 1..<maxnoofCellsinLTMUEHistoryInfo> |  |  |
| >>NR CGI |  |  | 9.3.1.12 |  |
| >>Time UE Stayed in Cell |  |  | INTEGER (0..40950) | The duration of time the UE stayed in the cell. |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinLTMUEHistoryInfo | Maximum no. of cells in the LTM UE history information. Value is 16. |

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

[snip]

RequestedSRSPreconfigurationCharacteristics-List,

SRSPreconfiguration-List,

Broadcast-MRBs-Transport-Request-Item,

TAInformation-List,

NonIntegerDRXCycle,

AggregatedPosSRSResourceSetList,

F1U-PathFailure,

LTMUEHistory,

[snip]

id-AggregatedPosSRSResourceSetList,

id-RANSharingAssistanceInformation,

id-F1U-PathFailure,

id-LTMUEHistory,

maxCellingNBDU,

maxnoofCandidateSpCells,

maxnoofDRBs,

maxnoofIndividualF1ConnectionsToReset,

[snip]

maxnoofTRPs,

maxnoofMRBs,

maxnoofUEIDforPaging,

maxnoofMRBsforUE,

maxnoofServingCellMOs

[snip]

-- **************************************************************

--

-- CU-DU CELL SWITCH NOTIFICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- CU-DU Cell Switch Notification

--

-- **************************************************************

CUDUCellSwitchNotification ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ CUDUCellSwitchNotificationIEs}},

...

}

CUDUCellSwitchNotificationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-NRCGI					CRITICALITY reject	TYPE NRCGI							PRESENCE mandatory	}|

{ ID id-LTMCellSwitchInformation	 CRITICALITY ignore	TYPE LTMCellSwitchInformation	PRESENCE optional	}|

{ ID id-TAInformation-List			CRITICALITY ignore	TYPE TAInformation-List			PRESENCE optional		}|

{ ID id-LTMUEHistory				CRITICALITY ignore	TYPE LTMUEHistory				PRESENCE optional	},

...

}

[snip]

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

[snip]

maxnoPreconfiguredSRS,

maxnoHopsMinusOne,

maxnoAggCombinations,

maxnoAggregatedPosSRSCombinations

[snip}

LTMUEHistory = 	::= SEQUENCE {

lTMUEHistoryList 	LTMUEHistoryList,

iE-Extensions				ProtocolExtensionContainer { { LTMUEHistory-ExtIEs} }	OPTIONAL,

...

}

LTMUEHistory-ExtIEs		F1AP-PROTOCOL-EXTENSION ::= {

...

}

LTMUEHistoryList ::= SEQUENCE (SIZE(1..maxnoofCellsinLTMUEHistoryInfo)) OF LTMUEHistoryItem

LTMUEHistoryItem ::= SEQUENCE {

nRCGI					NRCGI,

timeUEStayedinCell		INTEGER (0..40950),

iE-Extensions			ProtocolExtensionContainer { { LTMUEHistoryItem-ExtIEs } }		OPTIONAL,

...

}

LTMUEHistoryItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

[snip]

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules

[snip]

id-LocalOrigin										ProtocolIE-ID ::= 854

id-LTMResetInformation								ProtocolIE-ID ::= 855

id-SRSPosPeriodicConfigHyperSFNIndex				ProtocolIE-ID ::= 856

id-PreconfiguredSRSInformation						ProtocolIE-ID ::= 857

id-rLFReportFailureType								ProtocolIE-ID ::= 999 --to be assigned by MCC

id-LTMUEHistory										ProtocolIE-ID ::= 998 --to be assigned by MCC