3GPP TSG-RAN WG3 Meeting #128	R3-253357

Malta, MT, 19 – 23 May, 2025

Title:	(TP for SON BLCR for 37.340) MRO for others

Agenda Item:	10.2

Source:	Huawei

Document for:	other

# 1	Introduction

The following was discussed in last RAN3 meeting

MRO for CHO with Candidate SCG(s):

No additional information is needed in handover report message.

Check stage 3 TP on the Target cell CGI related in handover report message.

One SCGFailureInfomation is included in SCG Failure Indication message, no need aggregation.

Respective MN in stage2 is at least a candidate MN.

Information needed in the SCG Failure Indication message: The Xn UE AP ID in the receiving node.

Additional information reported from the UE in SCGFailureInformation: The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the SCG failure is encountered.

No support of near failure case in R19.

Whether Handover Report message is needed for case 7a?

In addition to SCGFailureInformation, information needed in the SCG Failure Indication message?

- CPC failure type (CPC failure type is needed in case of SCG Failure Indication is sent from the candidate to the source?)

- Suitable PCell ID

MRO for S-CPAC

The description on S-CPAC information forwarding is technically correct. Cleanup of the text is needed.

Whether to enhance UHI in case of S-CPAC? The motivation and benefits need to be clarified.

# 2	Discussion

## 2.1 	MRO for CHO with Candidate SCG(s):

## 2.1.1 	Handover report for case 7a

| Case 7 |  | Both CHO and CPAC execution conditions are met. 7a) CHO execution to a candidate PCell failure. 7b) CPAC to a candidate PSCell failure. 7c) Execution to SCG failure and toMCG failure. |
| --- | --- | --- |

For case 7a, both CHO and CPAC execution conditions are met so CHO execution and CPAC execution are performed by the UE. The CHO execution to a candidate PCell fails. Some company mentions that the delayed CPAC execution fulfilment causes the CHO execution failure. From our point of view, regardless whether the CPAC execution is fulfilled at a good timing or at a delayed timing, at that moment the CHO execution is fulfilled to enable the CHO execution. In this case, the CHO execution condition is set too easily to be fulfilled, leading to CHO execution failure, while the successful CPAC execution implies that the CPAC execution is set in a good manner. The node responsible for determining the CHO execution is the source MN. The candidate MN which the failed target PCell belongs to are not involved in this CHO execution failure. Therefore, there is no need to send the handover report for the candidate MN to perform the optimization.

RAN3 agrees that there is no need to send the handover report for the candidate MN to perform the optimization for case 7a.

### 2.1.2	Additional information in the SCG Failure Indication message

CPC failure type is detected based on the time stayed in the last serving PSCell, the suitable PSCell and the time stay threshold, e.g. Tstore_UE_cntxt. The source MN and candidate MN may have configuration for time stay threshold and the rule to choose a suitable PSCell. Therefore, there is no need for source MN to provided CPC failure type detected by itself to the candidate MN.

The candidate MN performs the candidate PSCell mobility parameter based on the associated PCell. As above mentioned, whether the suitable PCell to be selected by source MN or candidate MN is an issue. We would rather that the UE selects the suitable PCell instead. RAN3 has agreed that UE reports the fulfilled PCell ID before SCG failure is encountered:

Additional information reported from the UE in SCGFailureInformation: The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the SCG failure is encountered.

From our point of view, the reported fulfilled PCell ID is equivalent to “the suitable PCell”, with which the associated PSCell execution delayed the CPC execution causing the SCG failure.

RAN3 agrees that no new information is needed in the SCG Failure Indication message.

On the other hand, it seems RAN2 did not yet discuss the above agreement form RAN3. Therefore, RAN3 could send an LS to RAN2.

RAN3 should send an LS to RAN2 to let them know about the RAN3 agreement.

## 2.2	Stage2 corrections

Per the current BLCR [R3-251748], the forwarding mechanism is restructured in two aspects of initial failure and following failure. The forwarding the analysis mechanism regarding initial failure and following failure are settled and stable.  Therefore, the editor’s note to check the forwarding mechanism could be removed since the forwarding mechanism is settled and stable.

It is noteworthy that the type of PSCell addition/change could be caused by CHO with candidate SCGs other than CPA, CPC, and S-CPAC. Therefore, we would like to have additional texts to point out CHO with candidate SCGs.

RAN3 agrees to remove the editor’s note for checking the S-CPAC forwarding mechanism.

RAN3 agrees that the type of PSCell addition/change could also be related to CHO with candidate SCGs.

# 3	Conclusion

This document proposes:

Proposal 1:	RAN3 agrees that there is no need to send the handover report for the candidate MN to perform the optimization for case 7a.

Proposal 2:	RAN3 agrees that no new information is needed in the SCG Failure Indication message.

Proposal 3:	RAN3 should send an LS to RAN2 to let them know about the RAN3 agreement.

Proposal 4:	RAN3 agrees to remove the editor’s note for checking the S-CPAC forwarding mechanism.

Proposal 5:	RAN3 agrees that the type of PSCell addition/change could also be related to CHO with candidate SCGs.

# Annex	TP for 37.340

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 10.18.3	Conditional PSCell addition or change failure

One of the functions of self-optimization for CPAC is to detect CPAC failures that occur due to Too late CPC execution or Too early CPC/CPA execution, or CPC/CPA execution to wrong PSCell. These problems are defined as follows:

-	Too Late CPC Execution: UE receives CPC configuration/S-CPAC configuration/CHO with Candidate SCG(s) configuration, while a SCG failure occurs before CPC execution; a suitable PSCell different from source PSCell is found based on the measurements reported from the UE.

-	Too Early CPC/CPA Execution: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; in case of CPC, the source PSCell is still the suitable PSCell based on the measurements reported from the UE; in case of CPA, no suitable PSCell is found based on the measurements reported from the UE.

-	CPC/CPA Execution to wrong PSCell: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; a suitable PSCell different from the source PSCell or the target PSCell is found based on the measurements reported from the UE.

For Too late CPC execution and CPC/CPA execution to wrong PSCell, there are two sub-cases for wrong candidate PSCell list selection:

-	if the suitable PSCell is one of the candidate target PSCells provided by the node initiating the CPC/S-CPAC/CHO with Candidate SCG(s) or by the MN initiating the CPA, but not one of the candidate PSCells selected by the candidate or target SN, it is wrong target PSCell selection at the candidate or target SN;

-	else, it is wrong candidate PSCell list selection at the node initiating the CPC/S-CPAC/CHO with Candidate SCG(s) or at the MN initiating the CPA.

In the definition above, the "successful CPC/CPA execution" refers to the UE state, namely the successful completion of the RA procedure.

Detection mechanism

The MN performs the initial analysis when SCGFailureInformation is received from the UE. In the first step, MN verifies whether intra-SN PSCell change has been triggered in the last serving SN. In case the intra-SN PSCell change has been triggered in the last serving SN, the MN forwards the SCG Failure Information Report message to this last serving SN, which performs the final root cause analysis. In case of no intra-SN PSCell change, the MN determines the type of PSCell addition/change, e.g., whether it is CPA or CPC or CHO with Candidate SCG(s) in case of conditional mobility, if CPC whether it is MN initiated or SN initiated.

The detailed detection mechanisms for Too Late CPC Execution, Too Early CPC/CPA Execution and CPC/CPA Execution to wrong PSCell are carried out in the NG-RAN node which initiates the CPC/CPA which may also be a part of CHO with Candidate SCG(s) or S-CPAC procedure:

-	Too Late CPC Execution: if CPC/S-CPAC/CHO with Candidate SCG(s) is configured and there is no recent CPC execution for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt) , and there is a suitable PSCell different from the PSCell where the UE is located at the time of the failure occurrence.

-	Too Early CPC/CPA Execution: there is a recent CPC/CPA execution for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and in case of CPC, the source PSCell is a suitable PSCell, in case of CPA, no suitable PSCell is found.

-	CPC/CPA Execution to wrong PSCell: there is a recent CPC/CPA execution for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the suitable PSCell is not the source PSCell or the target PSCell.

The "UE reported timer" above indicates the time elapsed since the CPC/CPA execution until connection failure.

For CPA or MN initiated CPC or initial S-CPAC initiated by MN or CHO with Candidate SCG(s), if the suitable PSCell is one of the candidate PSCells provided by the MN at CPAC preparation, but not one of the candidate PSCells selected by the candidate or target SN, MN sends the SCG Failure Information Report message to the candidate or target SN, which perform the final MRO related optimisation. Otherwise, the MN performs the final MRO related optimisation.

For a following S-CPC failure during an MN initiated S-CPAC, if the suitable PSCell is not one of the candidate PSCells provided by the MN at CPAC preparation, the MN performs the final MRO related optimisation. Otherwise, MN sends the SCG Failure Information Report message to the candidate or target SN, which perform the final MRO related optimisation.

For SN initiated CPC, the MN sends the SCG Failure Information Report message to source SN, and source SN performs root cause analysis. If the suitable PSCell is one of the candidate PSCells provided by the source SN, but not one of the candidate PSCells selected by the candidate or target SN, the source SN indicates to MN that the root cause of the SCG failure may have occurred in the other nodes. MN then sends the SCG Failure Information Report message to the candidate or target SN. Otherwise, the source SN performs the final MRO related optimisation.

For SN initiated S-CPAC, the MN sends the SCG Failure Information Report message to the initiating SN, and the initiating SN performs root cause analysis.

-	For initial S-CPAC failure during a SN initiated S-CPAC, the forwarding mechanism is the same as that for SN initiated CPC. The source SN is also the initiating SN at S-CPAC preparation.

-	For a following S-CPC failure during a SN initiated S-CPAC, if the suitable PSCell is not one of the candidate PSCells provided by the initiating SN at CPAC preparation, the initiating SN performs the final MRO related optimisation. Otherwise, the initiating SN indicates to MN that the root cause of the SCG failure may have occurred in the other nodes. MN then sends the SCG Failure Information Report message to the candidate or target SN.

Editor’s note: The forwarding mechanism in the above two paragraphs needs to be further checked.

# Annex	TP for LS to RAN2

3GPP TSG-RAN WG3 Meeting #128	<TDdoc#>

Malta, MT, 19 – 23 May, 2025

Title:	[DRAFT] LS on SON and MDT enhancements for MRO

Response to:

Release:	Rel-19

Work Item:	NR_ENDC_SON_MDT_Ph4-Core

Source:	Huawei [will be RAN3]

To:	RAN2, SA5

Cc:

Contact person: 	Henrik Olofsson

henrik.olofsson@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN3 would like to inform RAN2 about the latest progress in MRO for SON/MDT.

MRO for CHO with candidate SCGs

- RAN3 agreed to the benefit of including the Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the SCG failure is encountered in SCGFailureInformation.

# 2	Actions

To RAN2,

ACTION: 	RAN3 kindly asks RAN2 to take above progress into account.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#128	2025-05-19 - 2025-05-23		Malta, MT

RAN3#129	2025-08-25 - 2025-08-29		Bangalore, IN

RAN3#129-bis	2025-10-13 - 2025-10-17		Prague, CZ

RAN3#130	2025-11-17 - 2025-11-21		Dallas, US