3GPP TSG-RAN WG3 Meeting #128	R3-253360

Malta, MT, 19 - 23 May, 2025

Title:	(TP for 38.401) Multi-hop relay

Agenda Item:	22.2

Source:	Huawei

Document for:	discussion

# 1	Introduction

In RAN3#127bis meeting, RAN3 has good progress on the control plan aspects for supporting multi-hop relay. We have the following agreements and agreed the BL CRs for TS 38.401[2], TS 38.473[3], and TS 38.470[4].

It is proposed to capture the following Editor’s Note into the BL CRs to TS 38.401 and 38.473. - Editor’s Note: The definition of terminologies related to the Multi-hop relay operation is pending to RAN2 progress. It is proposed to introduce the following terminologies to help RAN3’s further discussion, - The Parent Relay UE of an Intermediate Relay UE is the Intermediate Relay UE’s next hop Relay UE in UL. - The Child Relay UE of an Intermediate Relay UE is the Intermediate Relay UE’s next hop Relay UE in DL. Move the signaling flow for Multi-hop Relay operation into the clause 8.19.1a of TS 38.401 (i.e., Rel-17 Single-hop Relay related section). For the PC5 Relay RLC channel between the Relay UE and child/parent Relay UE, the gNB-CU may request to the gNB-DU to establish a new PC5/Uu Relay RLC channel for relaying of U2N Remote UE’s SRBs (i.e., SRB0, SRB1, and SRB2). The SRB0 is newly introduced into the PC5 Control Plane Traffic Type IE of the UE CONTEXT MODIFCATION REQUEST message. The gNB-CU provides the counterpart information of a configured PC5 Relay RLC channel to the gNB-DU. Potential proposal 1: Decide which information is provided to gNB-DU as the counterpart information of a configured PC5 Relay RLC channel for Multi-hop Relay operation: - Option 1: Reuse the Peer UE ID IE: HW, CATT, E///, Nokia, LGE - Option 2: Define a new IE : LGE, Samsung, ZTE WA: Reuse the Peer UE ID IE as counter part information. The gNB-CU determines the QoS for PC5 Relay RLC channels over each PC5 hop. The gNB-CU separately initiates the existing UE Context Modification procedure for each U2N Relay UE to configure the PC5 Relay RLC channel for relaying of U2N Remote UE’s SRB/DRB.

In this paper, we will discuss the opens issues and provided the corresponding TPs in the Annex.

# 2	Discussion

## Remote UE identification during initial access

In Rel-17 SL relay, we have the following definition for the remote UE local ID, in which the connected relay UE connects to the network and the remote UE directly. During the remote UE initial access, the gNB-DU UE F1AP ID of the relay UE and the remote UE local ID will be contained in the INITIAL UL RRC MESSAGE TRANSFER message to the gNB-CU, so that gNB-CU can related the SRB0 message to a specific remote UE which was reported in the SUI message by the relay UE.

-----------------------------------------------------------TS 38.473 V18.4.0----------------------------------------------------------

#### 9.3.1.267	Remote UE Local ID

This IE uniquely identifies a L2 U2N Remote UE within the connected Relay UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Remote UE Local ID | M |  | INTEGER (0..255, ...) | Corresponds to the sl-LocalIdentity contained in the SL-SRAP-Config IE defined in TS 38.331 [8]. |

--------------------------------------------------------------------------------------------------------------------------------------------

Similarly for multi-hop relay, the gNB-CU needs to identify the remote UE based on the INITIAL UL RRC MESSAGE TRANSFER message, but what information should be contained is still FFS, as shown by the FFSs in the BLCR [2] as below.

Editor’s Note: FFS whether the gNB-DU UE F1AP IDs of the U2N First Relay UE and the U2N Intermediate Relay UE are included in the INITIAL UL RRC MESSAGE TRANSFER message. Editor’s Note: FFS whether and how the gNB-DU becomes aware that the U2N Remote UE is connected to the gNB-DU via the U2N First Relay UE, the U2N Intermediate Relay UE and U2N Last Relay UE.

In RAN2#129bis meeting, RAN2 has made the assumption that the uniqueness of the remote UE local ID is guaranteed within the cell by implementation.

RAN2#129bis Agreements for remote UE local ID: Reuse the single-hop relay mechanism to support the Local ID allocation for multi-hop relay: • First relay UE reports the L2 ID of the remote UE to the gNB to request the local ID allocation, the uniqueness of the local ID within the cell is assumed to be guaranteed by the gNB by implementation. • The remote UE local ID is 8 bits.

The remote UE local ID size limits the number of remote UE (including those also acting as intermediate relay UEs), and 8 bits means at most 256 remote UE can be connected in the scope of uniqueness. For example, in release 17, the uniqueness is guaranteed within the connected Relay UE, so that the maximum number of remote UEs connected to one relay UE can be 256.

Based on the RAN2 assumption form last meeting, we can have the following option for the remote UE local ID allocation in multi-hop relay scenario, in which the uniqueness can be guaranteed by the gNB implementation.

- Option 1. The remote UE local ID uniquely identifies a multi-hop remote UE within the serving cell.

In option 1, except for the remote UE local ID, the gNB-DU shall indicate the serving cell information or alternatively indicate the last relay UE ID information (used to derive the serving cell information) in the INITIAL UL RRC MESSAGE TRANSFER message. The serving cell information can be the NCGI, while the last relay UE ID information can be the gNB-DU UE F1AP ID of last relay UE.

- Observation 1. If the remote UE local ID uniquely identifies a multi-hop remote UE within the serving cell, the maximum number of remote UE connected to one cell is 256, and the gNB-DU shall indicate the remote UE local ID as well as the serving cell ID or the gNB-DU UE F1AP ID of the last relay UE in the INITIAL UL RRC MESSAGE TRANSFER message.

- Option 2. The remote UE local ID uniquely identifies a multi-hop remote UE within the last relay UE.

In option 2, the gNB-DU shall indicate the remote UE local ID and the last relay UE ID information in the INITIAL UL RRC MESSAGE TRANSFER message. The last relay UE ID information can be the gNB-DU UE F1AP ID of last relay UE. This option can be supported by the current spec with the gNB-DU UE F1AP ID of Relay UE IE in the Sidelink Relay Configuration filled with the gNB-DU UE F1AP ID of the last relay UE.

- Observation 2. If the remote UE local ID uniquely identifies a multi-hop remote UE within the last relay UE, the maximum number of remote UE connected to one last relay UE is 256, the gNB-DU shall indicate the remote UE local ID and the gNB-DU UE F1AP ID of the last relay UE in the INITIAL UL RRC MESSAGE TRANSFER message. This can be supported by the current spec.

- Option 3. The remote UE local ID uniquely identifies a multi-hop remote UE within the first relay UE.

In option 3, the gNB-DU shall indicate the remote UE local ID and the gNB-DU UE F1AP ID of the U2N First Relay UE and the remote UE local ID in the INITIAL UL RRC MESSAGE TRANSFER message, then the gNB-CU can know it is a remote UE with the indicated local ID in the scope of the first relay UE. However, since the first relay UE does not connect to the gNB directly, the gNB-DU cannot know which first relay UE is related to the remote UE. Therefore, how for the gNB-DU to identify the gNB-DU UE F1AP ID of the first relay UE is a problem.

- Observation 3: if the remote UE local ID uniquely identifies a multi-hop remote UE within the first relay UE, the maximum number of remote UE connected to a first relay UE is 256, and RAN3 need to discuss how for the gNB-DU to identify the gNB-DU UE F1AP ID of the first relay UE is a problem.

For the above options, RAN3 can send a LS to RAN2 to check with the understanding, then decides the contents in the INITIAL UL RRC MESSAGE TRANSFER message.

RAN3 can send a LS to RAN2 to ask the uniqueness of the local ID before determining the contents in the INITIAL UL RRC MESSAGE TRANSFER message.

The LS to RAN2 can be found in Annex2.

## 2.2	Other FFSs

In the BLCR of TS 38.401, we have the following FFSs for step 18 and step 33.

Editor’s Note: FFS whether Step 18 can be performed earlier, e.g., via Steps 6-13. Editor’s Note: FFS whether this step can be performed earlier.

In step 18, gNB-CU and gNB-DU configures the relay RLC channels for remote UE’s SRB1 message. Technically, after receiving the SUI message from the first relay UE in Step 5, the gNB-CU can know there is one remote UE identified with the indicated L2 ID in the SUI is trying to access and prepare the necessary configurations for this remote UE, including the relaying configuration for SRB1. However, there can be some cases that the RRC Setup of the remote UE fails, e.g., the remote UE SRB0 message cannot be successfully forwarded or the access control failure. In above cases, the gNB-CU and gNB-DU has to de-configure the relaying configuration for the remote UE, which causes signalling and resource overhead.

It is gNB implementation when to configure the relay RLC channels for remote UE’s SRB1 message, including before or after the successfully forwarding of remote UE’s SRB0 message.

Similarly, in step 33, when to configure the relaying configuration for the SRB2 and DRBs can be gNB implementation. For the relaying configuration for DRB, it should be after the after the PDU Session resource setup, since the gNB-CU and gNB-DU need the QoS information of the DRBs to determine the mapping configuration and the relay RLC channel configurations.

It is gNB implementation when to configure the relay RLC channels for remote UE’s SRB2 and DRB message. The relaying configuration determination for remote UE’s DRB needs the QoS information of the DRBs, thus should be after the PDU Session resource setup procedure.

Reword these editor’s notes into notes that step 18 and step 33 can be performed earlier.

According to the above discussion and proposals, we update the BL CRs for TS 38.401 in the Annex 2.

# 3	Conclusion

This document proposes:

Observation 1. If the remote UE local ID uniquely identifies a multi-hop remote UE within the serving cell, the maximum number of remote UE connected to one cell is 256, and the gNB-DU shall indicate the remote UE local ID as well as the serving cell ID or the gNB-DU UE F1AP ID of the last relay UE in the INITIAL UL RRC MESSAGE TRANSFER message.

Observation 2. If the remote UE local ID uniquely identifies a multi-hop remote UE within the last relay UE, the maximum number of remote UE connected to one last relay UE is 256, the gNB-DU shall indicate the remote UE local ID and the gNB-DU UE F1AP ID of the last relay UE in the INITIAL UL RRC MESSAGE TRANSFER message. This can be supported by the current spec.

Observation 3: if the remote UE local ID uniquely identifies a multi-hop remote UE within the first relay UE, the maximum number of remote UE connected to a first relay UE is 256, and RAN3 need to discuss how for the gNB-DU to identify the gNB-DU UE F1AP ID of the first relay UE is a problem.

Proposal 1:	RAN3 can send a LS to RAN2 to ask the uniqueness of the local ID before determining the contents in the INITIAL UL RRC MESSAGE TRANSFER message.

Proposal 2:	It is gNB implementation when to configure the relay RLC channels for remote UE’s SRB1 message, including before or after the successfully forwarding of remote UE’s SRB0 message.

Proposal 3:	It is gNB implementation when to configure the relay RLC channels for remote UE’s SRB2 and DRB message. The relaying configuration determination for remote UE’s DRB needs the QoS information of the DRBs, thus should be after the PDU Session resource setup procedure.

Proposal 4:	Reword these editor’s notes into notes that step 18 and step 33 can be performed earlier.

# 4	Reference

[1] Chairman notes, RAN2#129bis.

[2] R3-252526, (BLCR for TS 38.401) Support of Multi-hop relay, LG Electronics Inc., ZTE, Nokia, Nokia Shanghai Bell, Ericsson, FirstNet, Huawei, 3GPP TSG-RAN WG3 Meeting #127bis.

[3] R3-252528, (BLCR for TS 38.473) Support of Multi-hop relay, Huawei, LG Electronics, Nokia, Nokia Shanghai Bell, NEC, CATT, Ericsson, Samsung, ZTE, Interdigital, FirstNet, 3GPP TSG-RAN WG3 Meeting #127bis.

[4] R3-252527, (BL CR to 38.401) Support of Multi-hop relay, LG Electronics Inc., ZTE, Nokia, Nokia Shanghai Bell, Ericsson, FirstNet, Huawei, 3GPP TSG-RAN WG3 Meeting #127bis.

# Annex1	Text Proposal for TS 38.401

The text proposal include the changes based on the CR for TS 38.401 [2].

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 8.19.xx	Remote UE initial access for Multi-hop Layer-2 UE-to-Network Relay

Editor’s Note: The definition of terminologies related to the Multi-hop relay operation is pending to RAN2 progress.

Editor’s Note: Current signaling flows are based on the agreements that the intermediate Relay UE is in RRC_CONNECTED state. Whether the intermediate Relay UE in RRC_IDLE/RRC_INACTIVE state is supported or not is pending to RAN2.

The signalling flow for Remote UE Initial access is shown in Figure 8.19.xx-1.

Figure 8.19.xx-1: Overall procedure for Remote UE’s initial access via multi-hop relay

1.	The U2N Remote UE, the U2N First Relay UE, the U2N Intermediate Relay UE, and the U2N Last Relay UE perform discovery procedure, and establish PC5 connection using NR ProSe procedure.

2.	The U2N Remote UE sends an RRCSetupRequest message to the U2N First Relay UE via PC5 Relay RLC channel.

3.	The U2N First Relay UE withholds the received RRC message. If the U2N First Relay UE is in RRC_IDLE/RRC_INACTIVE state, it should send its own RRCSetupRequest message to the U2N Intermediate Relay UE via PC5 Relay RLC channel in order to trigger the RRC establishment/resume procedure to enter RRC_CONNECTED state upon reception of the RRC message from the U2N Remote UE. If the U2N Intermediate Relay UE is in RRC_IDLE/RRC_INACTIVE state, it should trigger the RRC establishment/resume procedure in clause 8.19.1 to enter RRC_CONNECTED state upon reception of the RRC message from the U2N First Relay UE. If all Relay UEs are in RRC_CONNECTED state, this step could be skipped.

4.	The U2N First Relay UE in RRC_CONNECTED state sends the SidelinkUEInformationNR message to the gNB-DU via the Intermediate Relay UE and U2N Last Relay UE.

5.	The gNB-DU sends the UL RRC MESSAGE TRANSFER message of the U2N First Relay UE by encapsulating the SidelinkUEInformationNR message to gNB-CU, and gNB-CU allocates the local ID of U2N Remote UE.

6.	The gNB-CU sends the UE CONTEXT MODIFICATION REQUEST message of the U2N Last Relay UE to gNB-DU. Such message may request the establishment of Uu Relay RLC channel(s) and PC5 Relay RLC channel(s) for the transmission of U2N Remote UE’s SRB0.

7.	The gNB-DU sends the UE CONTEXT MODIFICATION RESPONSE message of the U2N Last Relay UE to gNB-CU.

8.	The gNB-CU sends the DL RRC MESSAGE TRANSFER message of the U2N Last Relay UE to gNB-DU by encapsulating the RRCReconfiguration message, which contains the local ID allocated to the U2N Remote UE. The RRCReconfiguration message shall also contain the Uu Relay RLC channel(s) configuration and PC5 Relay RLC channel(s) configuration if not configured and bearer mapping for relaying of U2N Remote UE’s SRB0.

9.	The gNB-DU sends the RRCReconfiguration message to the U2N Last Relay UE to configure the local ID of the U2N Remote UE, the Uu Relay RLC channel(s) configuration, PC5 Relay RLC channel(s) configuration and bearer mapping for relaying of U2N Remote UE’s SRB0.

10.	The U2N Last Relay UE sends the RRCReconfigurationComplete message to gNB-DU.

11.	The gNB-DU sends the UL RRC MESSAGE TRANSFER message of the U2N Last Relay UE by encapsulating the RRCReconfigurationComplete message to gNB-CU.

12.	The gNB-CU configures the U2N Intermediate Relay UE with the local ID allocated to the U2N Remote UE, PC5 Relay RLC channel and bearer mapping for relaying of U2N Remote UE’s SRB0. According to the configuration from gNB-CU, the U2N Intermediate Relay UE may establish a PC5 Relay RLC channel for relaying of U2N Remote UE’s SRB0 over PC5. This step follows the same signaling flow as described in steps 6-11.

13.	The gNB-CU configures the U2N First Relay UE with the local ID allocated to the U2N Remote UE, PC5 Relay RLC channel and bearer mapping for relaying of U2N Remote UE’s SRB0. According to the configuration from gNB-CU, the U2N First Relay UE may establish a PC5 Relay RLC channel for relaying of U2N Remote UE’s SRB0 over PC5. This step follows the same signaling flow as described in steps 6-11.

14.	After receiving the local ID of the U2N Remote UE and the PC5 Relay RLC channel(s) configuration and bearer mapping for relaying of U2N Remote UE’s SRB0, the U2N First Relay UE sends the RRCSetupRequest message of the U2N Remote UE to gNB-DU via the U2N Intermediate Relay UE and the U2N Last Relay UE. The local ID of the U2N Remote UE and RB ID for SRB0 are conveyed in the SRAP header.

15.	The gNB-DU allocates a C-RNTI and a gNB-DU UE F1AP ID for the U2N Remote UE and sends the INITIAL UL RRC MESSAGE TRANSFER message to gNB-CU by encapsulating the RRCSetupRequest message of the U2N Remote UE. In addition, the local ID of the U2N Remote UE, the gNB-DU UE F1AP ID of the U2N Last Relay UE and the sidelink configuration container for the PC5 Relay RLC channel configuration for relaying of U2N Remote UE’s SRB1 are included in the INITIAL UL RRC MESSAGE TRANSFER message.

Editor’s Note: FFS whether the gNB-DU UE F1AP IDs of the U2N First Relay UE and the U2N Intermediate Relay UE are included in the INITIAL UL RRC MESSAGE TRANSFER message.

Editor’s Note: FFS whether and how the gNB-DU becomes aware that the U2N Remote UE is connected to the gNB-DU via the U2N First Relay UE, the U2N Intermediate Relay UE and U2N Last Relay UE.

16.	The gNB-CU allocates a gNB-CU UE F1AP ID for the U2N Remote UE and generates a RRCSetup message towards the U2N Remote UE. The RRC message is encapsulated in the DL RRC MESSAGE TRANSFER message, and includes the configurations of PC5 Relay RLC channel and bearer mapping at least for the transmission of U2N Remote UE’s SRB1.

17.	The gNB-DU sends the RRCSetup message to the U2N Remote UE via the U2N First Relay UE, the U2N Intermediate Relay UE and U2N Last Relay UE.

18.	The gNB-CU configures the U2N Last Relay UE with PC5 Relay RLC channel, Uu Relay RLC channel and bearer mapping for relaying of U2N Remote UE’s SRB1. According to the configuration from gNB-CU, the U2N Last Relay UE establishes a PC5 Relay RLC channel for relaying of U2N Remote UE’s SRB1 over PC5 and establishes a Uu Relay RLC channel for relaying of U2N Remote UE’s SRB1 towards gNB-DU if not configured yet.

The gNB-CU configures the First Relay UE and the U2N Intermediate Relay UE with PC5 Relay RLC channel and bearer mapping for relaying of U2N Remote UE’s SRB1. According to the configuration from gNB-CU, the First Relay UE and the U2N Intermediate Relay UE establish the PC5 Relay RLC channels for relaying of U2N Remote UE’s SRB1 over PC5 if not configured yet.

Note 1: This Step can be performed earlier, e.g., via Steps 6-13.

19.	The U2N Remote UE sends the RRCSetupComplete message to the gNB-DU via the U2N First Relay UE, the U2N Intermediate Relay UE and U2N Last Relay UE.

20.	The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and sends it to the gNB-CU.

21.	Upon receiving the RRCSetupComplete message of U2N Remote UE, the gNB-CU sends the INITIAL UE MESSAGE message to the AMF.

22.	The AMF sends the INITIAL CONTEXT SETUP REQUEST message to the gNB-CU.

23.	The gNB-CU sends the UE CONTEXT SETUP REQUEST message to establish the U2N Remote UE context in the gNB-DU. Such message may request the configuration of PC5 Relay RLC channels for the transmission of U2N Remote UE’s SRB2 and DRBs, and may also encapsulate the SecurityModeCommand message.

24.	The gNB-DU sends the SecurityModeCommand message to the U2N Remote UE via the U2N First Relay UE, the U2N Intermediate Relay UE and U2N Last Relay UE.

25.	The gNB-DU sends the UE CONTEXT SETUP RESPONSE message of the U2N Remote UE to the gNB-CU, which contains the configuration of PC5 Relay RLC channels for the transmission of U2N Remote UE’s SRB2 and DRBs.

26.	The U2N Remote UE responds with the SecurityModeComplete message.

27.	The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and sends it to the gNB-CU.

28.	The gNB-CU generates the RRCReconfiguration message for U2N Remote UE and encapsulates it in the DL RRC MESSAGE TRANSFER message. The RRCReconfiguration message contains the configuration of PC5 Relay RLC channels and bearer mapping for the transmission of U2N Remote UE’s SRB2 and DRBs.

29.	The gNB-DU sends RRCReconfiguration message to the U2N Remote UE via the U2N First Relay UE, the U2N Intermediate Relay UE and U2N Last Relay UE.

30.	The U2N Remote UE sends RRCReconfigurationComplete message to the gNB-DU via the U2N First Relay UE, the U2N Intermediate Relay UE and U2N Last Relay UE.

31.	The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and send it to the gNB-CU.

32.	The gNB-CU sends the INITIAL CONTEXT SETUP RESPONSE message to the AMF.

33.	The gNB-CU configures additional Uu Relay RLC channels between the gNB-DU and the U2N Last Relay UE, and additional PC5 Relay RLC channels for the U2N First Relay UE, the U2N Intermediate Relay UE, and the U2N Last Relay UE for relaying of U2N Remote UE’s DRBs and SRBs. Also, such step may configure the bearer mapping between U2N Remote UE’s DRB/SRB and PC5/Uu Relay RLC channel at the U2N First Relay UE, the U2N Intermediate Relay UE, and the U2N Last Relay UE.

Note 2: This step can be performed earlier.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Annex2	LS to RAN2

3GPP TSG-RAN WG3 Meeting #128	<TDdoc#>

Malta, MT, 19 – 23 May, 2025

Title:	[DRAFT] LS Multi-hop Relay

Response to:

Release:	Rel-19

Work Item:	NR_SL_relay_multihop-Core

Source:	Huawei [will be RAN3]

To:	RAN2, SA5

Cc:

Contact person: 	Henrik Olofsson

henrik.olofsson@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN3 would like to inform RAN2 about the latest progress in Multi-hop Relay

From RAN3 perspective, the uniqueness of the remote UE local ID would impact the RAN3 spec and would also limit the maximum number of remote UEs connected to the network.

The available options are:

Option 1. The remote UE local ID uniquely identifies a multi-hop remote UE within the serving cell.

Option 2. The remote UE local ID uniquely identifies a multi-hop remote UE within the last relay UE.

Option 3. The remote UE local ID uniquely identifies a multi-hop remote UE within the first relay UE.

RAN3 considers option1 as too limiting and concluded that option2 is currently supported in the Rel-18 SL relay. RAN3 considers option3 feasible and would like to ask RAN2 whether there are any obstacles in supporting option3, since this would allow maximize the flexibility regarding the number of remote UEs connected to the network

# 2	Actions

To RAN2,

ACTION: 	RAN3 kindly asks RAN2 to take the above into account.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#128	2025-05-19 - 2025-05-23		Malta, MT

RAN3#129	2025-08-25 - 2025-08-29		Bangalore, IN

RAN3#129-bis	2025-10-13 - 2025-10-17		Prague, CZ

RAN3#130	2025-11-17 - 2025-11-21		Dallas, US