3GPP TSG RAN WG3#128	R3-25XXXX

Malta, MT, 19 – 23 May, 2025

Agenda item:	12.3

Source: 	China Telecom

Title: 	On remaining issues for NR Femto

Document for:	Discussion and decision

- Introduction

In RAN3#127, the agreement was made as follows:

  - If the SMF sends both IP versions in the Transport Layer Address IE, the NR Femto node selects the correct IP version.

And the TP to 38.300 i.e. R3-252333 was agreed. Two issues were left as follows:

  - It is FFS whether the GW can perform IP Version selection in addition to the NR Femto.

  - It is FFS how to harmonise terminology for “NR Femto”, “NR Femto Node”. Terminology shall be consistent.

In this paper, we discuss some remaining issues and present our proposals.

- Discussion

2.1 Whether GW can perform IP Version selection

In last meeting, how to select an IP version for NG-U was discussed, the remaining issue was noted as following:

It is FFS whether the GW can perform IP Version selection in addition to the NR Femto.

The Figure X shows that an architecture of a Femto and a gNB connecting to 5GC.

A NR Femto node typically connects to the public IP network through an access device (e.g., a cable modem, home router, or DSL modem). Currently, the public IP network is predominantly IPv4-based, which operators consider untrusted. In contrast, a gNB connects to the 5G Core (5GC) via a dedicated operator-managed network, which is typically IPv6-based and trusted.

If a Femto Gateway (GW) is deployed between the NR Femto node and the 5G Core (5GC), it may connect to the Femto node via the public IP network (typically IPv4) on one side, while connecting to the 5GC via a dedicated IP network (typically IPv6) on the other side.

In this architecture, the NR Femto GW serves as a concentrator for the C-Plane, specifically the NG-C interface. For U-Plane, the NG-U interface from the NR Femto node may be ether terminated at the NR Femto GW, or establish a direct logical connection to 5GC.

If a direct logical U-Plane connection exists between the NR Femto node and the 5GC, we have agreed that the SMF may send both IP versions in the Transport Layer Address IE, the NR Femto node selects the correct IP version. However, consensus has not been reached on whether the GW itself can perform IP version selection.

We propose that enabling the NR Femto GW to support IP version selection and translation offers significant advantages, regardless of whether the U-Plane terminates at the GW or bypasses it. Specifically:

- For Non-Terminated U-Plane: The GW can facilitate seamless interoperability between IPv4 and IPv6 networks.

- For Terminated U-Plane: The GW can:

  - Select the optimal IP version and address for the NR Femto cell.

  - Perform IP version translation (e.g., IPv4 ↔ IPv6) when bridging between the Femto node (IPv4) and the 5GC (IPv6).

  - Enhance security by hiding 5GC IP addresses and isolating the operator’s network from the public internet.

Thus, we support implementing IP version selection capabilities in the NR Femto GW to improve flexibility, security, and backward compatibility.

Proposal 1: The NR Femto GW should support IP version selection for NR Femto nodes.

2.2 Terminology Harmonization for "NR Femto" and "NR Femto Node"

To ensure consistent usage throughout the specification, "NR Femto Node" shall refer to an individual physical or logical instance of a New Radio Femto base station; "NR Femto" shall represent the broader concept or system of NR home base stations.

Proposal 2:
The terms shall be used as follows:

- "NR Femto Node": When referring to specific implementations or entities of the base station

- "NR Femto": When discussing the general technology, architecture, or category of home base stations

- Conclusion

Based on the above discussion on the remaining issues for NR Femto, we provide the following observations and proposals:

Proposal 1: The NR Femto GW should support IP version selection for NR Femto nodes.

Proposal 2:
The terms shall be used as follows:

- "NR Femto Node": When referring to specific implementations or entities of the base station

- "NR Femto": When discussing the general technology, architecture, or category of home base stations

- Reference

[1] R3-251524 LS on NR Femto node shared by PLMN and PNI-NPN, SA2.

[2] TS 38.331, NR; Radio Resource Control (RRC) protocol specification Rel-18, 3GPP.

[3] RP-242395, New WID on additional topological enhancements for NR, NTT DOCOMO, AT&T.

[4] RP-241925, TR 38.799 v2.1.0 NR; Study on additional topological enhancements for NR, NTT DOCOMO INC., AT&T.

[5] TS 38.300, NR; NR and NG-RAN Overall description; Stage-2 Rel-18, 3GPP.

[6] TS 38.413, NG-RAN; NG Application Protocol (NGAP) Rel-18, 3GPP.

[7] TS 36.300, E-UTRAN; Overall description; Stage 2 Rel-18, 3GPP.

[8] TS 36.413, E-UTRAN; S1 Application Protocol (S1AP) Rel-18, 3GPP.