3GPP TSG-RAN WG3 Meeting #128	R3-253373

Malta, MT, 19th – 23rd May 2025

Title:	Discussion on continuous MDT measurement

Source:				China Telecom

Agenda item:		11.4

Document for:		Discussion and Decision

1. Introduction

In the previous meetings, the following agreements were made with several FFSs.

Continuous MDT collection targeting the same UE across RRC states RAN3 assumes that the OAM configures Management Based MDT for Continuous MDT towards the NG-RAN. In R19, introducing the Continuous MDT functionality with a solution being applicable to legacy UEs and not only to Rel-19 UEs. Agree to send an LS to SA5 capturing e.g. problem description, solutions requirements, potential solutions (if agreeable). When to send the LS depends on convergence on the content of the LS. RAN3 assumes a solution for continuous MDT should minimize impacts on the current Management Based MDT framework. LS to SA5 in R3-250891

In this contribution, we continue discussing measurement continuity problems based on the reply LS from SA5 [2].

2. Discussion

In R3-253024 [2], SA5 provided response to RAN3’s LS on Continuous Management Based MDT. The details are as follows:

-----------quoted from R3-253024-------------------

For Measurement Continuity Problem, regarding RAN3 agreements:

RAN3: “ The OAM provides to the RAN means to identify that the Management Based MDT is for Continuous MDT, e.g. reusing existing Management Based MDT configuration information.”

Answer from SA5: In SA5 point of view, it is feasible for OAM to provide the configuration mechanisms to RAN to identify Management based MDT for continuous MDT. SA5 expects that RAN3 could provide the configuration details as OAM requirements.

RAN3: “RAN3 will derive solutions for the Measurement Continuity Problem across RRC_Connected mobility. SA5 is requested to focus on the Measurement Continuity Problem across different RRC states.”

Answer from SA5: SA5 would like RAN3 to provide some clarifications on what problem is referred by “the Measurement Continuity Problem across different RRC states”.

RAN3: “ A solution should minimise impacts on the current Management Based MDT framework.”

Answer from SA5: SA5 agrees that the solution should minimise impacts on the current Management Based MDT framework.

For Trace Correlation Problem,

RAN3: “To solve the Trace Correlation Problem, namely to enable correlation of MDT measurements generated by the same UE, it is required to use a temporary identifier for the Continuous MDT, not revealing the UE identity. Such identifier should be made available at the TCE to enable correlation of the management-based MDT reports collected during the Continuous MDT, in a way that minimises the impacts on the Management Based MDT solution.”

Answer from SA5: SA5 agrees that at least TR and TRSR (which does not reveal UE identity) could be used for the correlation of the management-based MDT reports at TCE. SA5 would like RAN3 to elaborate the details of how to avoid overlapping

-----------End quoted-------------------

According to R3-253024 [2], SA5 agrees that it is feasible for OAM to provide the configuration mechanisms to RAN to identify Management based MDT for continuous MDT. In addition, SA5 also agrees that the solution should minimise impacts on the current Management Based MDT framework.

According to the guidance from SA5, to solve the measurement continuity problem, OAM should configure Management Based MDT for continuous MDT to gNBs. The configuration includes specific identifiers (e.g. Trace Record) dedicated to the Continuous MDT process, namely the NG-RAN deduces from such identifier that the Management Based configuration is for Continuous MDT.

Proposal 1: To solve measurement continuity problem, OAM configures Management Based MDT for continuous MDT to gNBs, which include specific identifiers dedicated to the continuous MDT process.

Proposal 2: OAM configures duration for continuous MDT to gNBs, which indicates the validity period of continuous MDT.

In RAN3 previous meeting, we have following agreement:

In R19, introducing the Continuous MDT functionality with a solution being applicable to legacy UEs and not only to Rel-19 UEs.

Therefore, the straightforward solution for handling the Handover case is to include in the Handover signaling an indication to indicate that the UE needs to be selected for continuous MDT.

Proposal 3: To solve measurement continuity problem during handover, Source gNB signals to Target gNB an indication to indicate that the UE needs to be selected for continuous MDT.

3. Conclusion

In this contribution, we further discuss AI/ML based Coverage and Capacity Optimization (CCO). We have the following proposal:

Proposal 1: To solve measurement continuity problem, OAM configures Management Based MDT for continuous MDT to gNBs, which include specific identifiers dedicated to the continuous MDT process.

Proposal 2: OAM configures duration for continuous MDT to gNBs, which indicates the validity period of continuous MDT.

Proposal 3: To solve measurement continuity problem during handover, Source gNB signals to Target gNB an indication to indicate that the UE needs to be selected for continuous MDT.

4. Reference

- R3-250891, LS on Continuous MDT, Ericsson

- R3-253024, Reply LS to LS on Continuous MDT, Huawei