3GPP TSG-RAN3 Meeting #128	R3-253374

# Malta, Malta, 19th May 2025 - 23rd May 2025

Agenda item:	16.2

Source:	Xiaomi

Title:	(TP for 38.413) General aspects on support of AIoT

Document for:	Discussion

# 1	Introduction

In this contribution, we would like to discuss the following open issues on the general aspects to support A-IoT.

- Definition of new A-IoT area

- NG interface management

- Reader selection

- Inventory procedures

- Command procedures

# 2	Discussion

### Definition of new A-IoT area

RAN3 127bis agreements -Introduce a new “A-IoT Area”. The AIOTF may indicate the Requested Service Area as a list of “A-IoT Areas” and/or a list of readers in the Inventory Request message. -It is FFS whether the new A-IoT Area is encoded as GNSS info or new A-IoT Area ID.

RAN3 discussed how to define the new A-IoT area. The intention behind using the A-IoT area ID is to have a concept similar to TAC. However, TAC is configured per cell, whereas for A-IoT, there is no cell concept; instead, a reader ID is used. If the A-IoT area ID corresponds to a list of readers, it seems redundant to introduce both the A-IoT area and the reader ID list in the inventory request. In our view, it's more straightforward to use geographical information as the new A-IoT area. In NTN MBS, the geographical area information has been introduced in the MBS service area information, which includes the information to represent the circle area and the polygon area. Therefore, we believe the A-IoT area can be geographical information, and the encoding can refer to the intended service area information in NTN MBS as shown below.

Reference in BL CR for TS 38.413 in NTN

#### 9.3.1.209 MBS Service Area Information This IE contains MBS service area information. IE/Group Name; Presence; Range; IE type and reference; Semantics description. MBS Service Area Cell List; 0..<maxnoofCellsforMBS>. >NR CGI; M; 9.3.1.7. MBS Service Area TAI List; 0..<maxnoofTAIforMBS>. >TAI; M; 9.3.3.11. MBS Intended Service Area List; 0..<maxnoofIntendedAreasforMBS>. >Intended Service Area Information; M; 9.3.1.x. Range bound; Explanation. maxnoofCellsforMBS; Maximum no. of cells allowed within one MBS Service Area. Value is 8192.. maxnoofTAIforMBS; Maximum no. of TAs allowed within one MBS Service Area. Value is 1024.. maxnoofIntendedAreasforMBS; Maximum no. of intended areas allowed within one MBS Service Area. Value is 65536.. #### 9.3.1.x Intended Service Area Information This IE provides information on the Intended Service Area for MBS Broadcast Service. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Intended Service Area Information; M; OCTET STRING; Encoded in the same format as the IntendedServiceAreaInfo IE as defined in TS 38.331 [18].

Reference in Runing CR for TS 38.300 in NTN

IntendedServiceAreaInfo-r19 ::= SEQUENCE { intendedServiceAreaId-r19 MBS-IntendedAreaID-r19, areaCoordinates-r19 CHOICE { polygonArea OCTET STRING, circleArea SEQUENCE { referenceLocation-r19 ReferenceLocation-r17, distanceRadius-r19 INTEGER(0..65535) } } }

Observation 1, It's not clear how to map the A-IoT area ID to a specific inventory area.

Observation 2, The intended area information introduced in NTN MBS can be referred if introducing geographical area as A-IoT area.

Proposal 1, RAN3 agree to introduce the geographical area as A-IoT area.

### NG interface management

Introducing an AIoT indicator in the NGAP Setup Request message. The detail of this indicator can be further discussed.

RAN3 agreed to introduce an A-IoT indicator in NGAP setup request to ignore the mandatory IEs which are not necessary for A-IoT connection setup.  The following tables shows the IEs in NG setup.

Information in NG setup request

| IE/Group Name | Presence |
| --- | --- |
| Message Type | M |
| Global RAN Node ID | M |
| RAN Node Name | O |
| Supported TA List |  |
| >Supported TA Item |  |
| >>TAC | M |
| >>Broadcast PLMN List |  |
| >>>Broadcast PLMN Item |  |
| >>>>PLMN Identity | M |
| >>>>TAI Slice Support List | M |
| Default Paging DRX | M |

Information in NG setup response

| IE/Group Name | Presence |
| --- | --- |
| Message Type | M |
| AMF Name | M |
| Served GUAMI List |  |
| >Served GUAMI Item |  |
| >>GUAMI | M |
| Relative AMF Capacity | M |
| PLMN Support List |  |
| >PLMN Support Item |  |
| >>PLMN Identity | M |
| >>Slice Support List | M |

As NG setup request can be used for both direct architecture and indirect architecture, and gNB may support only A-IoT or support both A-IoT and NR. The following combinations need to be considered.

| Cases | Direct case | Indirect case |
| --- | --- | --- |
| gNB support only A-IoT | Ignore supported TA, default Paging DRX, AMF related IE, PLMN support list | Ignore supported TA, default Paging DRX, PLMN support list |
| gNB support both A-IoT and NR |  | A-IoT indicator not needed |

It is clear that in the only one case where a gNB supports both A-IoT and NR in an indirect configuration, the indicator is not required. Consequently, it would be unsuitable to name this indicator as an A-IoT support indicator or an A-IoT-only support indicator. The goal is to ignore the inapplicable IEs, a more generic term is suggested, such as "A-IoT Compatibility Indicator"

|  | NG setup request | NG setup response |
| --- | --- | --- |
| Direct case | “A-IoT Compatibility Indicator” is included in the NG setup request, the AIOTF shall ignore the IEs not applicable for A-IoT. | “AIOTF Name” is included in the NG setup response message, the gNB shall ignore the IEs not applicable for A-IoT. |
| Indirect case gNB only support A-IoT | “A-IoT Compatibility Indicator” is included in the NG setup request, the AMF shall ignore the IEs not applicable for A-IoT. | No extra indication. If gNB only support A-IoT, it ignores the IEs not applicable for A-IoT. |

Proposal 2, RAN3 agrees to include the A-IoT Compatibility Indicator IE in the NG setup request message, so that the CN node ignores the the IEs not applicable for A-IoT.

Proposal 3, RAN3 agrees to include the “AIOTF name” in the NG setup response message, so that the gNB can ignore the IEs not applicable for A-IoT.

### Reader selection

FFS on the NGAP signaling support to provide the A-IoT RAN Information (Reader List, Reader location, Supported A-IoT Areas) to the AIOTF.

It has been acknowledged that AIOTF may be aware of the location of readers via OAM configuration. However, the details of the reader's location, such as room number and GNSS information, are still under further study (FFS). Additionally, the signaling support for this information is also FFS.

We believe it is beneficial for AIOTF to be aware of reader information directly from the gNB, rather than solely relying on OAM. This approach offers several advantages: By obtaining reader information from the gNB, AIOTF can receive more accurate and real-time updates on the reader's location and capabilities. This is particularly important as the gNB may have multiple readers, each with fixed locations but varying serving areas depending on transmitting power and other radio conditions. Allowing AIOTF to access reader information from the gNB can lead to more efficient network operations. The gNB can dynamically adjust the serving area and radio capabilities (e.g., bandwidth) of each reader based on current radio conditions, optimizing overall network performance.

The signalling of reader information exchange, there are two signalling options:

- option 1, via (AIoT) NG setup or RAN configuration update.

- option 2, via dedicated reader information exchange

We slightly prefer option 2, as the reader information is common for both direct arch and indirect arch. AMF does not need to know the reader information, the design of the signalling can be the same as other AIoT operation (e.g., inventory or command).

Proposal 4, RAN3 agree to introduce a class 1 procedure for reader information exchange.

Proposal 5, the following information can be included in the reader information provided by gNB for reader selection:

- Reader ID, used to identify a reader.

- Reader location, used to identify the geographical location of a reader

- Supported A-IoT area, used to indicate the supported A-IoT area per reader

- AIoT radio capabilities (frequency, bandwidth, power), used to indicate the radio capabilities of a reader.

### Inventory procedures

About Inventory - Introduce a “Correlation ID” in the respective NGAP inventory at least inside the transfer containers. - WA: Include the Correlation Identifier IE outside of the Inventory related Transfer IEs in all the Inventory related messages. - Include the AIOTF Identifier IE outside of the Inventory related Transfer IEs in all the Inventory related messages. - The Inventory Request Transfer IE, also includes the following: - Device Identification for Paging (to enable paging for single device, a group of devices, all devices) - Requested Service Area Information (A-IoT Area list and/or Reader list) - Inventory Assistance Information (Approximates Number of Target A-IoT Devices). - It is FFS whether the Inventory Assistance Information includes the “Estimate of Expected D2R Message Size”

#### Issue 1, Correlation ID outside of the Inventory related Transfer IEs

There’s a WA “Include the Correlation Identifier IE outside of the Inventory related Transfer IEs in all the Inventory related messages.” in last RAN3 meeting, however, we don’t see the need to include it outside, the intention behind is for indirect case, if we refer the similar architecture in positioning, the NON UE ASSOCIATED NRPPA TRANSPORT message over NGAP, only Routing ID (similar to AIOTF ID) is outside, the NRPPa transaction (similar to correlation ID) is inside the NRPPa-PDU.

#### 9.2.9.3 DOWNLINK NON UE ASSOCIATED NRPPA TRANSPORT This message is sent by the AMF and is used for carrying NRPPa message over the NG interface. Direction: AMF NG-RAN node IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.3.1.1; YES; ignore. Routing ID; M; 9.3.3.13; YES; reject. NRPPa-PDU; M; 9.3.3.14; YES; reject.

Observation 3, the reason to include the correlation ID outside of the inventory related transfer IEs is not clear.

Proposal 6, revert the WA “Include the Correlation Identifier IE outside of the Inventory related Transfer IEs in all the Inventory related messages”.

#### Issue 2, device identification

RAN3 agreed to introduce Device Identification for paging in the inventory request; this information will be further used in A-IoT radio as the Paging ID. RAN3 needs to discuss whether this device identification is readable by the gNB. In previous discussions, most companies believed that the gNB does not need to know the device ID. However, according to the RAN2 agreement, the gNB may needs to read the device identification to decide whether to use CFRA or CBRA, as CFRA is intended for one device, while CBRA is intended for multiple or group devices.

Agreements on CFRA (RAN2#129-bis)

Introduce an explicit 1 bit indication to indicate whether it is CFRA or CBRA per paging message.

On the other hand, according to the latest TS 23.369 [1], SA2 introduces AIoT Device Permanent Identifier and filter information, the filer information can be understood as MASK ID in RFID, which is used for multiple device inventory.

## 5.8 Filtering Information The filtering information is used to identify or filter multiple AIoT Devices and is constructed by one or multiple components of the AIoT Device Permanent Identifier as defined in clause 5.7. The filtering information includes a list of filtering element information. The filtering element information includes: - Information indicating a which component of the AIoT Device Permanent Identifier that is used to match the bitstring. - A bitstring which is used to compare with the component. Each bitstring is corresponding to one component of the AIoT Device Permanent Identifier. NOTE: The bitstring match can start at any position in the corresponding component. Editor's note: Whether and how to secure the filtering information is up to SA WG3. To determine whether an AIoT Device Permanent Identifier matches the filtering information, it is compared with every filtering element information within filtering information by comparing the bitstring in a filtering element information with the indicated component of its AIoT Device Permanent Identifier. If all the compared bitstrings match the AIoT Device Permanent Identifier then an AIoT Device Permanent Identifier matches the filtering information. If an AIoT Device Permanent Identifier does not contain an indicated component then it does not match the filtering information.

Based on the progress in SA2 and RAN2, we think the device identification can be transparent to the gNB, and the type of device identification can be indicated to the gNB by defining a choice IE structure. The following example illustrates how to indicate the device identification type via a choice structure IE, allowing the gNB to determine whether the ID corresponds to a single device or multiple devices, and consequently, whether to use CFRA or CBRA.

| IE/Group Name | Presence |
| --- | --- |
| CHOICE A-IoT Device Identification Information |  |
| >A-IoT Device ID |  |
| >>Device Permanent Identifier | M |
| >Filter Information |  |
| >>Filter Information | M |

Observation 4, The gNB needs to determine whether the inventory request pertains to a single device or multiple devices to decide whether to utilize CBRA or CFRA at the A-IoT radio interface. The gNB can ascertain this either by interpreting the device identification or by recognizing the device ID type.

Proposal 7, RAN3 agrees that the A-IoT CN can signal the device ID type to the gNB by implementing a choice structure for A-IoT Device Identification.

#### Issue 3, end indication

For inventory report, an end indicator is needed, as noted in SA2 TR NOTE 10, AIOTF is expected to receive an end indication from AIoT RAN, signaling the completion of reporting for a specific correlation ID

Proposal 8, RAN3 agrees to introduce an end indication IE in the inventory report message.

### Command procedures

About Command after inventory - WA: NGAP: Command Request procedure is a per single device procedure, and no need to have a Command Report procedure. - WA: Include the Correlation Identifier IE in both inside and outside of the Command related Transfer IEs in all the Command related messages. - Include the AIOTF Identifier IE outside of the Command related Transfer IEs in all the command related messages. - The Command Request Transfer IE, also includes the following: - An A-IoT NAS PDU - Command Assistance Information: (Estimate of Expected D2R Message Size) - It is FFS whether the Command Assistance Information includes the Command Type (read, write, disable…) It is FFS whether to introduce the Device Association or a transaction ID, outside of the Transfer IEs, to allow parallel Command Request procedures for different devices between gNB and AMF.

The device context related issues for command after inventory is discussed in R3-25xxx. In this paper, the following issues for command are focused.

#### Issue 1, per single device command

There’s a WA: NGAP: Command Request procedure is a per single device procedure, and no need to have a Command Report procedure. We think that the command procedure is per single device, although there may be multiple command for multiple devices, we don’t the see the need to aggregate the command response for multiple devices.

Proposal 9, RAN3 turns the “WA: NGAP: Command Request procedure is a per single device procedure, and no need to have a Command Report procedure” to an agreement.

#### Issue 2, IDs outside of the transfer IEs.

For indirect architecture, we think it’s similar to NRPPa as below, the device association is needed. For A-IoT service, the Device NGAP ID is per A-IoT session, so both Device NGAP ID and Correlation ID (to represent the session) should be included.

#### 9.2.9.1 DOWNLINK UE ASSOCIATED NRPPA TRANSPORT This message is sent by the AMF and is used for carrying NRPPa message over the NG interface. Direction: AMF NG-RAN node IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.3.1.1; YES; ignore. AMF UE NGAP ID; M; 9.3.3.1; YES; reject. RAN UE NGAP ID; M; 9.3.3.2; YES; reject. Routing ID; M; 9.3.3.13; YES; reject. NRPPa-PDU; M; 9.3.3.14; YES; reject.

Proposal 10, RAN3 agree to introduce device association including the A-IoT correlation ID and Device NGAP ID outside the transfer IEs.

#### Issue 3, command type

We think gNB needs to know the purpose of the command, so that it can allocate resource and knows how to response the A-IoT CN accordingly.

Proposal 11, RAN3 agree to introduce command type in the command request message.

Based on the above discussion, a TP to BL CR TS 38.413 is provided to reflect the above proposals in the Annex.

# 3	Conclusion

In this contribution, we discussed the issue and potential solutions to support AIoT in topology 1 and have the following observations and proposals.

### Definition of new A-IoT area

Observation 1, It's not clear how to map the A-IoT area ID to a specific inventory area.

Observation 2, The intended area information introduced in NTN MBS can be referred if introducing geographical area as A-IoT area.

Proposal 1, RAN3 agree to introduce the geographical area as A-IoT area.

### NG interface management

Proposal 2, RAN3 agrees to include the A-IoT Compatibility Indicator IE in the NG setup request message, so that the CN node ignores the the IEs not applicable for A-IoT.

Proposal 3, RAN3 agrees to include the “AIOTF name” in the NG setup response message, so that the gNB can ignore the IEs not applicable for A-IoT.

### Reader selection

Proposal 4, RAN3 agree to introduce a class 1 procedure for reader information exchange.

Proposal 5, the following information can be included in the reader information provided by gNB for reader selection:

- Reader ID, used to identify a reader.

- Reader location, used to identify the geographical location of a reader

- Supported A-IoT area, used to indicate the supported A-IoT area per reader

- AIoT radio capabilities (frequency, bandwidth, power), used to indicate the radio capabilities of a reader.

### Inventory procedures

#### Correlation ID issue

Observation 3, the reason to include the correlation ID outside of the inventory related transfer IEs is not clear.

Proposal 6, revert the WA “Include the Correlation Identifier IE outside of the Inventory related Transfer IEs in all the Inventory related messages”.

#### Device ID

Observation 4, The gNB needs to determine whether the inventory request pertains to a single device or multiple devices to decide whether to utilize CBRA or CFRA at the A-IoT radio interface. The gNB can ascertain this either by interpreting the device identification or by recognizing the device ID type.

Proposal7, RAN3 agrees that the A-IoT CN can signal the device ID type to the gNB by implementing a choice structure for A-IoT Device Identification.

#### End indication

Proposal 8, RAN3 agrees to introduce an end indication IE in the inventory report message.

### Command procedures

#### Per device command

Proposal 9, RAN3 turns the “WA: NGAP: Command Request procedure is a per single device procedure, and no need to have a Command Report procedure” to an agreement.

#### IDs outside the transfer IE

Proposal 10, RAN3 agree to introduce device association including the A-IoT correlation ID and Device NGAP ID outside the transfer IEs.

#### Command type

Proposal 11, RAN3 agree to introduce command type in the command request message.

# 4	Reference

[1]	 3GPP TS 23.369, Ambient power-enabled Internet of Things; Stage 2

# 3	Annex (TP for BL CR 38.413)

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change starts>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

8.xx.4	Reader Information Exchange

8.xx.4.1	General

The purpose of the Reader Information Exchange procedure is to allow the AIoT CN node to request the NG-RAN node to provide information for readers hosted by the NG-RAN node. This procedure applies only if the NG-RAN node is a gNB.

8.xx.4.2	Successful Operation

Figure 8.xx.4.2-1: Reader Information Exchange: successful operation

The AIoT CN node initiates the procedure by sending a READER INFORMATION REQUEST message. The NG-RAN node responds with a READER INFORMATION RESPONSE message.

#### 8.xx.4.3	Unsuccessful Operation

Figure 8.xx.4.3-1: Reader Information Exchange: unsuccessful operation

If the NG-RAN node cannot provide any of the requested information for any Reader, it shall consider the procedure as failed and reply with the READER INFORMATION FAILURE message.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

#### 9.2.x.5	COMMAND REQUEST

This message is sent by the A-IoT CN node to the gNB to trigger the command transmission over the A-IoT radio.

Direction: A-IoT CN node  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AIOTF Identifier | M |  | 9.3.3.xx2 |  | YES | reject |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  | YES | reject |
| RAN A-IoT Device NGAP ID | O |  | 9.3.3.yy1 |  | YES | ignore |
| Command Request Transfer | M |  | OCTET STRING | Containing the Command Request Transfer IE specified in subclause 9.3.6.5. | YES | reject |

Editor’s note: FFS whether the Command Request also contains outside of the Command Request Transfer container, the RAN A-IoT device NGAP ID (FFS for CN NGAP device ID) which was received in the Inventory Report message or some additional transaction ID.

#### 9.2.x.6	COMMAND RESPONSE

This message is sent by the NG-RAN node to provide the command response to the A-IoT CN node.

Direction: NG-RAN node  A-IoT CN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AIOTF Identifier | M |  | 9.3.3.xx2 |  | YES | reject |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  | YES | reject |
| RAN A-IoT Device NGAP ID | O |  | 9.3.3.yy1 |  | YES | ignore |
| Command Response Transfer | M |  | OCTET STRING | Containing the Command Response Transfer IE specified in subclause 9.3.6.5. | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

Editor’s note: FFS whether the Command Response message also contains outside of the Command Request Transfer container, the RAN NGAP device ID (FFS for CN NGAP device ID) which was received in the Inventory Report message or some additional transaction ID.

#### 9.2.x.7	COMMAND FAILURE

This message is sent by the NG-RAN node to report the unsuccessful transmission of the command to the A-IoT CN node.

Direction: NG-RAN node  A-IoT CN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AIOTF Identifier | M |  | 9.3.3.xx2 |  | YES | reject |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  | YES | reject |
| RAN A-IoT Device NGAP ID | O |  | 9.3.3.yy1 |  | YES | ignore |
| Command Failure Transfer | M |  | OCTET STRING | Containing the Command Failure Transfer IE specified in subclause 9.3.6.5. | YES | ignore |
| Cause | M[FFS] |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

#### 9.2.x.8	READER INFORMATION REQUEST

This message is sent by the A-IoT CN node to the gNB to request reader information in the gNB.

Direction: A-IoT CN node ® NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Reader Information Request Transfer | M |  | OCTET STRING | Containing the Reader Information Request Transfer IE specified in subclause 9.3.x.8. | YES | reject |

9.2.x.9	READER INFORMATION RESPONSE

This message is sent by the NG-RAN node to provide the reader information response to the A-IoT CN node.

Direction: NG-RAN node ® A-IoT CN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Reader Information Response Transfer | M |  | OCTET STRING | Containing the Reader Information Response Transfer IE specified in subclause 9.3.x.9. | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

9.2.x.10	READER INFORMATION FAILURE

This message is sent by the NG-RAN node to report the unsuccessful providing of the reader information to the A-IoT CN node.

Direction: NG-RAN node ® A-IoT CN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Reader Information Failure Transfer | M |  | OCTET STRING | Containing the Reader Information Failure Transfer IE specified in subclause 9.3.6.5. | YES | reject |
| Cause | M[FFS] |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

#### 9.3.3.xx3	Requested Service Area Information

This IE includes the Requested Service Area Information for the A-IoT inventory.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Requested Reader List |  | 0..1 |  |  |
| >Requested Reader Item |  | 1..<maxnoofReaders> |  |  |
| >>Global gNB ID | M |  | 9.3.1.6 |  |
| >>Reader Index | M |  | 9.3.1.aa1 |  |
| Requested A-IoT Area List |  | 0..1 |  |  |
| >Requested A-IoT Area Item |  | 1..<maxnoofAIoTAreas> |  |  |
| >>A-IoT Area | M |  | 9.3.3.yy4 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofReaders | Maximum no. of Readers. Value is 65535. |
| maxnoofAIoTAreas | Maximum no. of A-IoT Areas. Value is 256. |

#### 9.3.3.xx4	Inventory Assistance Information

This IE includes the assistance information for the A-IoT Inventory Service.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Approximate Number of Target A-IoT Devices | O |  | INTEGER (1..2048, …) |  |
| Estimate of Expected D2R Message Size | O |  | FFS |  |

#### 9.3.3.xx5	A-IoT Device Identification Requested

This IE represents the Requested Identity of the A-IoT device(s).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE A-IoT Device Identification Requested |  |  |  |  |
| >A-IoT Device ID |  |  |  |  |
| >>Device Permanent Identifier | M |  | OCTET STRING |  |
| >Filter Information |  |  |  |  |
| >>Filter Information | M |  | OCTET STRING |  |

#### 9.3.3.yy1	RAN A-IoT Device NGAP ID

This IE uniquely identifies the A-IoT Device over the NG interface within the NG-RAN node within a A-IoT Session, as described in TS 38.401 [2].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RAN A-IoT Device NGAP ID | M |  | INTEGER (0..232 -1) |  |

#### 9.3.3.yy3	Command Assistance Information

This IE includes the assistance information for the A-IoT Command Service.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Estimate of Expected D2R Message Size | O |  | FFS |  |
| Command Type |  |  | ENUMERATED (read, write, disable, ...) |  |

#### 9.3.3.yy4	A-IoT Area

This IE provides information on the A-IoT Area for the A-IoT Inventory Service..

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE A-IoT area | M |  |  |  |
| >circle |  |  |  |  |
| >>Reference Location | M |  | OCTET STRING | tn-ReferenceLocation-r18 as defined in TS 38.331[18] |
| >>Distance Radius | M |  | INTEGER(0..65535) | tn-DistanceRadius-r18 as defined in TS 38.331[18] |
| > polygon |  |  |  |  |
| >> Polygon Points |  | 0..15 |  |  |
| >>>Latitude Sign | M |  | ENUMERATED {north, south} | latitudeSign as defined in TS 37.355 [xx]. |
| >>>Degrees Latitude | M |  | INTEGER (0..8388607) | degreesLatitude as defined in TS 37.355 [xx]. |
| >>>Degrees Longitude | M |  | INTEGER (-8388608..8388607) | degreesLongitude as defined in TS 37.355 [xx]. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 9.3.x	AIOTF Related IEs

#### 9.3.x.1	Inventory Request Transfer

This IE provides the inventory request related information from the AIOTF to the NG-RAN node.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  | YES | reject |
| A-IoT Device Identification Requested | M |  | 9.3.3.xx5 |  | YES | reject |
| Requested Service Area Information | M |  | 9.3.3.xx3 |  | YES | reject |
| Inventory Assistance Information | O |  | 9.3.3.xx4 |  | YES | reject |
| Follow on Command Indication | O |  | ENUMERATED (true, ...) | This IE indicates that there is command(s) to be transmitted. | YES | reject |

#### 9.3.x.2	Inventory Response Transfer

This IE provides the inventory response related information from the NG-RAN node to the AIOTF.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  |

#### 9.3.x.3	Inventory Failure Transfer

This IE provides the inventory failure related information from the NG-RAN node to the AIOTF.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  |
| Cause | M |  | 9.3.1.2 |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  |

#### 9.3.x.4	Inventory Report Transfer

This IE provides the inventory report related information from the NG-RAN node to the AIOTF.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  |
| Global gNB ID | M |  | 9.3.1.6 |  |
| Reader Report List |  | 1 |  |  |
| >Reader Report Item |  | 1..< maxnoofReaders> |  |  |
| >>Reader Index | M |  | 9.3.1.aa1 |  |
| >>Device Report List |  | 1 |  |  |
| >>>Device Report Item |  | 1..<maxnoofDevices> |  |  |
| >>>>A-IoT NAS PDU | M |  | OCTET STRING |  |
| >>>>RAN A-IoT Device NGAP ID | O |  | 9.3.3.yy1 |  |
| End Indication |  |  | ENUMERATED (true, ...) | This IE indicates that the inventory operations is end |

| Range bound | Explanation |
| --- | --- |
| maxnoofReaders | Maximum no. of Readers served by a gNB. Value is 65535. |
| maxnoofDevices | Maximum no. of Devices within a reader. Value is 65535. |

Editor’s Note 1: The A-IoT Correlation Identifier inside of the Transfer IEs in the following Command related messages are included based on RAN3 working assumptions.

Editor’s Note 2: The inclusion of the CN A-IoT Device NGAP ID IE inside of the Transfer IEs in the following Command related messages are FFS.

#### 9.3.x.5	Command Request Transfer

This IE provides the command request related information from the AIOTF to the NG-RAN node.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  | YES | reject |
| RAN A-IoT Device NGAP ID | M |  | 9.3.3.yy1 |  | YES | reject |
| A-IoT NAS PDU | M |  | OCTET STRING |  | YES | reject |
| Command Assistance Information | O |  | 9.3.3.yy3 |  | YES | reject |

#### 9.3.x.6	Command Response Transfer

This IE provides the command response related information from the NG-RAN node to the AIOTF.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  |
| RAN A-IoT Device NGAP ID | M |  | 9.3.3.yy1 |  |
| A-IoT NAS PDU | M |  | OCTET STRING |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  |

#### 9.3.x.7	Command Failure Transfer

This IE provides the command failure related information from the NG-RAN node to the AIOTF.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  |
| RAN A-IoT Device NGAP ID | M |  | 9.3.3.yy1 |  |
| Cause | M |  | 9.3.1.2 |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  |

9.3.x.8	Reader Information Request Transfer

This IE provide the reader information request from the AIOTF to the NG-RAN node.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Reader Information Type List |  | 1 |  |  |
| >Reader Information Type Item |  | 1 .. <maxnoReaderInfoTypes> |  |  |
| >>Reader Information Type Item | M |  | ENUMERATED (geo-coordinates, …, trp type, on-demand prs, trp tx teg, beam antenna info, mobile trp location info, common ta) |  |

9.3.x.9	Reader Information Response Transfer

This IE provides the requested reader information from the NG-RAN node to the AIOTF.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| FFS |  |  |  |  |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< change end>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>