3GPP TSG-RAN3 Meeting #128	R3-253377

# Malta, Malta, 19th May 2025 - 23rd May 2025

Agenda item:	16.3

Source:	Xiaomi

Title:	(TPs for TS 38.300 and TS 38.413) location of the device

Document for:	Other

# 1	Introduction

In this contribution, we’d like to discuss how to support proximity determination with minimum spec impacts.

# 2	Discussion

TR 38.769 [1] outlines two methods for determining proximity in device-reader interactions:

Solution 1: Proximity is confirmed if the reader successfully receives a device-to-reader (D2R) transmission in response to reader-to-device (R2D) signaling.

Solution 2: Proximity is determined based on the reader-side measurements of the received D2R transmission, (which is also triggered by R2D.

The Rel-19 Work Item Description (WID) explicitly limits standardization considerations in TR 38.769 to Solution 1 only, excluding Solution 2 from further study.

For Solution 1, if the A-IoT CN wishes to determine the proximity of an A-IoT device to the reader, a straightforward method involves providing an expected RSSI threshold to the gNB. Consequently, the gNB can use the appropriate transmission power to the A-IoT device based on this threshold. If the gNB receives a response from the device using the determined transmission power, it indicates that the device is sufficiently close to the reader. Conversely, if no response is received, it suggests that the device is not in close proximity.

TPs to TS 38.300 and TS 38.413 are provided in the Annex.

Proposal 1, RAN3 agrees to include the expected RSSI threshold in the inventory request message to assist the gNB in performing proximity determination.

# 3	Conclusion

Based on the above discussion, we have the following proposal.

Proposal 1, RAN3 agree to include the expected RSSI threshold in the inventory request message to assist gNB perform proximity determination.

# 4	Reference

[1] TR 38.769: Study on Ambient IoT Solutions.

# 5	Annex A (TP for TS 38.300) support of device location

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change starts>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 16.xx.x5 Locating A-IoT devices

In this version of specification, the A-IoT device location is reported by the gNB at Reader ID granularity to the AIOTF via Inventory Report procedure. The AIOTF may use the A-IoT device location information in performing A-IoT operations.

To determine the proximity location of an A-IoT device, an expected RSSI threshold may be included in the Inventory request message from the A-IoT CN to the gNB. The gNB may use this expected RSSI threshold to determine the appropriate transmission power for communication with the A-IoT device. With the dedicated transmission power set, if the gNB receives a response from the A-IoT device, it indicates that the A-IoT device is in close proximity to the reader in the gNB.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change ends>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

# 6	Annex B (TP for TS 38.413) support of device location

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change start>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 8.xx.1	Inventory Request

#### 8.xx.1.1	General

The purpose of the Inventory Request procedure is to request the NG-RAN node to perform inventory over the A-IoT radio. This procedure applies only if the NG-RAN node is a gNB.

#### 8.xx.1.2	Successful Operation

Figure 8.xx.1.2-1: Inventory Request, successful operation.

The A-IoT CN node triggers the procedure by sending an INVENTORY REQUEST message to the NG-RAN node. If the NG-RAN node accepts the request, the NG-RAN node responds with the INVENTORY RESPONSE message.

The NG-RAN node shall, include the same values of the AIOTF Identifier IE and A-IoT Correlation Identifier IE in the INVENTORY RESPONSE message as received from the INVENTORY REQUEST message.

If the Inventory Assistance Information IE is included in the INVENTORY REQUEST message, the NG-RAN node may take the information into account as defined in TS 38.300 [8].

If the Expected RSSI Threshold IE is included in the INVENTORY REQUEST message, the NG-RAN node may use it to determine the proximity location of the device as defined in TS 38.300 [8].

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

#### 9.3.x.1	Inventory Request Transfer

This IE provides the inventory request related information from the AIOTF to the NG-RAN node.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  | YES | reject |
| A-IoT Device Identification Requested | M |  | 9.3.3.xx5 |  | YES | reject |
| Requested Service Area Information | M |  | 9.3.3.xx3 |  | YES | reject |
| Inventory Assistance Information | O |  | 9.3.3.xx4 |  | YES | reject |
| Follow on Command Indication | O |  | ENUMERATED (true, ...) | This IE indicates that there is command(s) to be transmitted. | YES | reject |
| Expected RSSI Threshold | O |  | INTEGER (0..76) | Corresponds to the RSRP-Range IE as defined in TS 38.331 [18]. |  |  |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< change end>>>>>>>>>>>>>>>>>>>>>>>>>>>>>