3GPP TSG-RAN3 Meeting #128	R3-253378

# Malta, Malta, 19th May 2025 - 23rd May 2025

Agenda item:	14.3

Source:	Xiaomi

Title:	(TP to TS 38.300) Support of Regenerative Payload

Document for:	Other

1	Introduction

In this contribution, we’d like to discuss the following open issue

NG Connection interruption due to hard feeder link switch over

UE context handling due to NG removal

2	Discussion

NG Connection interruption due to hard feeder link switch over

In case of hard feeder link switch over, the 5GC suspends or resumes the DL signalling/data transmission(s) to the gNB based on the new indicator in RAN configuration update, FFS on the relationship with respect to NG suspend/resume (if any).

According to the open issue left in the last RAN3 meeting, we provide the following table to compare the the two cases of NG suspend/resume:

Case 1, NG transmission suspend/resume due to feeder link switch over

Case 2, NG connection suspend/resume due to satellite movement out of the AMF connection area

|  | Case 1, NG transmission suspend/rsume | Case 2, NG connection suspend/resume |
| --- | --- | --- |
| Cause | Due to feeder link switch over | Due to satellite movement out of the AMF connection area |
| Interruption time | The interruption time is quite low | The interruption time is depends on the satellite speed and longer than the interruption time due to feeder link switch |
| AMF behavior | AMF keeps the UE context and RAN node context. For siganlling, the AMF suspend/resume the signalling transmission accordingly. For user data, the AMF notifies the the SMF, then the SMF notify UPF to suspend/resume data transmission accordingly. | AMF only keeps the RAN node context The UE context in AMF may be updated as UE may be released or handover to other gNB No signalling/data suspend/resume |
| gNB behavior | gNB keeps the UE context and AMF node context | gNB either perform handover or release for the connected UE gNB keeps AMF node context |
| UE service continuity | UE is still be served by the same gNB and CN | UE may be released or handover to other gNBs. |

It is noticed that the distinctions between Case 1 and Case 2 lie in the case, interruption time, AMF/gNB behavior, and UE service continuity. In general, we support NG transmission suspend/resume, it reduce the packet loss.

We also observed that suspending or resuming user data could affect SA2 and CT4. Therefore, they need to devise a method for the AMF to notify the SMF/UPF. To assist with this, we provided a draft LS in [1].

Observation 1: The following outlines the differences between NG transmission suspend/resume and NG connection suspend/resume:

- NG transmission suspend/resume has a shorter interruption time than NG connection suspend/resume.

- NG transmission suspend/resume maintains the UE context in both the gNB and AMF, whereas NG connection suspend/resume release the UE or transfer it to other gNBs.

- NG transmission suspend/resume ensures that the UE continues to be served by the same gNB/AMF, whereas NG connection suspend/resume does not guarantee this.

Proposal 1, RAN3 agrees to support NG transmission suspend/resume due to feeder link switch.

Proposal 2, RAN3 agrees to send SA2, cc CT4 on the core network support of the UP transmission suspend/resume.

UE context handling

RAN3 agreed to introduce NG removal procedure,  to ensure UE service continuity, we think how to handle the UE context due to NG removal should be discussed. When a gNB onboard decides to trigger NG removal, before it sends the NG removal request, it should trigger handovers for the RRC_CONNECETED UEs, and relocate/release the contexts for the RRC_INACTIVE UEs.

Proposal 3, before sending the NG removal request, the RRC_CONNECTED UEs can be moves to other AMF/gNB by performing handover. The context of the RRC_INACTIVE UEs can be either released or relocated to other gNBs.

Proposal 4, RAN3 agree the corresponding TP in the Annex.

3	Conclusion

Based on the discussion, we have the following observations and proposals.

Observation 1: The following outlines the differences between NG transmission suspend/resume and NG connection suspend/resume:

- NG transmission suspend/resume has a shorter interruption time than NG connection suspend/resume.

- NG transmission suspend/resume maintains the UE context in both the gNB and AMF, whereas NG connection suspend/resume release the UE or transfer it to other gNBs.

- NG transmission suspend/resume ensures that the UE continues to be served by the same gNB/AMF, whereas NG connection suspend/resume does not guarantee this.

Proposal 1, RAN3 agree to support NG transmission suspend/resume due to feeder link switch.

Proposal 2, RAN3 agree to send SA2, cc CT4 on the core network support of the UP transmission suspend/resume.

Proposal 3, before sending the NG removal request, the RRC_CONNECTED UEs can be moves to other AMF/gNB by performing handover. The context of the RRC_INACTIVE UEs can be either released or relocated to other gNBs.

Proposal 4, RAN3 agree the corresponding TP in the Annex.

4	Annex (TP to BL CR 38.300)

<<<<<<<<<<<<<<<<<<<< Change starts>>>>>>>>>>>>>>>>>>>>

### 16.14.x	NG Removal

For the case of regenerative payload, when disconnecting from an AMF is required due to e.g. the satellite movement in a LEO constellation, the gNB on board the satellite may request the removal of the NG interface by initiating the NG Removal procedure toward the AMF. Before initiating the NG removal request, the onboard gNB should initiate handovers to move the RRC_CONNECTED UE(s) associated with the NG connection to other AMF/gNB(s). Concurrently, it should either release or relocate the contexts of RRC_INACTIVE UE(s).

Editor’s Note: NG Removal procedure may also be used for WAB; how to capture this for WAB may need further discussion (pending WAB discussion).

<<<<<<<<<<<<<<<<<<<< Change ends>>>>>>>>>>>>>>>>>>>>