3GPP TSG-RAN WG3 Meeting #128	R3-253381

Malta, Malta, 19 – 23 May 2025

Agenda item:	20.2

Source:	Xiaomi

Title:	(TPs for TS 38.305 and TS 38.455) Support of gNB-side model (case 3a)

Document for:	Other

1	Introduction

In this contribution, we’d like to further discuss the data collection for case 3a according to the following RAN3 progress.

RAN3#127bis: Case 3a data collection: An NRPPA based solution could be used to collect UE location information in an opportunistic way, i.e. from UEs that are being positioned. FFS on Whether to introduce an NGAP based procedure to collect UE location information in a proactive way, i.e. from UEs that are being served by the NG-RAN. Pending feedback from SA2. Check progress in RAN1 and solution discussion…

- The format of label

- Opportunistic solution

- Proactive solution

- LMF awareness of AI/ML

2	Discussion

## The format of label

According to RAN1 agreement below, the part B label can be the timing information (i.e., UL-RTOA) and/or the LOS/NLOS indicator.

Agreement(RAN1#120b) For training data collection of Part B in AI/ML based positioning Case 3a, for the case when Part B label includes timing information, support the following for providing label and quality indicator of label, - The corresponding label is provided by LMF to gNB in the format of timing information. “Timing Measurement Quality” in 38.455 can serve as quality indicator of the label. Note: It is assumed that user data privacy of non-PRU UE is preserved. Conclusion For training data collection of Part B in AI/ML based positioning Case 3a, for the case when Part B label includes the LOS/NLOS indicator, - The corresponding label is provided from LMF to the gNB by using the existing IE LOS-NLOS-Indicator-r17. There is no need of a separate field for quality indicator of LOS/NLOS indicator.

The timing information provided by AI/ML may also include gNB Tx-Rx time difference, however, the LMF can only provide UL-RTOA for Rx timing, the Tx timing is only known by gNB, thus, for the part B, the timing information is only refer to UL-RTOA.

Proposal 1, LMF provide the timing information with measurement Quality and LOS/NLOS indicators as part B label to gNB, existing IEs of timing information, measurement Quality and LOS/NLOS indicator.

No matter which solution is used, the following information is needed for data collection, they may be already in the existing signalling, whether enhancements needed can be discussed case by case.

| Data collection requirement information | Data collection response |
| --- | --- |
| - Measurement type (UL-RTOA, LOS/NLOS indicator) (M) - TRP ID (O) - UE ID (O) | - Labels - correlation information - LMF measurement ID - NRPPa transaction ID - timestamps |

## Opportunistic solution (solution 1)

Current agreed Solution 1(i.e., opportunistic data collection solution) relies on the measurement request procedure, which may request multiple different measurement types (e.g., RTOA, AoA) and measurement characteristics (e.g., LOS/NLOS), it can be understood that the gNB can collect UL-RTOA by knowing the SRS configuration, even through the UL-RTOA measurement is not requested in the measurement request.

For this solution, the following information is needed

| Positioning data collection needed information in Measurement response message | Information related to UE location and correlation information |
| --- | --- |
| - Measurement type (UL-RTOA, LOS/NLOS indicator) (M) The TRP ID is already in measurement response, UE ID is not needed in this case. | - UL RTOA with quality - LOS/NLOS - correlation information - LMF measurement ID - NRPPa transaction ID - timestamps |

In addition, LMF performs user consent check or PRU check for the UE correlated to the requested measurement, if the UE is PRU or has user consent, LMF provides the part B to gNB in step 5a, if not, LMF sends the NRPPa POSITIONING DATA COLLECTION FAILURE to gNB with appropriate cause value in step 5b.

TP for TS 38.300 and TS 38.455 are provided in the Annex.

Proposal 2, RAN3 agree the following NRPPa enhancements to support opportunistic data collection

- introduce requested measurement type (UL RTOA and/or LOS/NLOS) in measurement request.

- introduce a class 2 Positioning data collection report message to transfer the requested data.

- introduce the UL RTOA with quality, LOS/NLOS as information related to UE location in the Positioning data collection report message.

- introduce LMF measurement ID, NRRPa transaction ID and timestamps as correlation information in the positioning data collection report message.

- introduce a class 2 message to transfer the Positioning Data Collection Failure message to notify the failure of the data collection.

## Proactive solution

The opportunistic solution may have data collection limitations, as the gNB is unaware whether the UE for the requested measurement is PRU or has user consent. Therefore, we believe that a proactive solution should also be supported.

Currently, there’re two proactive solutions on the table.

- Proactive solution 2-1 (NRPPa based)

- Proactive solution 2-2 (NGAP based)

### Proactive solution 2-1 (NRPPa based)

As the NRPPa transaction is initiated by the LMF, if the LMF supports data collection, it can initiate a data collection requirement procedure to gather the data collection needs from the gNBs within a serving area. The gNB can indicate their needs in the response message or update their needs in the data collection requirement update procedure. The subsequent call flow illustrates how this process functions.

- The LMF sends a NRPPa Data Collection Requirement Request message to a gNB to indicate the data collection is available in LMF.

- The gNB response a NRPPa Data Collection Requirement Response message to the LMF. If data collection is required in the gNB, it includes the data collection requirement information in the NRPPa Data Collection Requirement Response message.

NOTE 1: the LMF may initiates multiple Data Collection Requirement Request Procedure to multiple gNBs.

- The LMF perform UE selection as described in TS 23.273 (e.g. step 1-5 in section 6.22.3) considering to the data collection requirement information received from the gNB.

NOTE 2, The selected UE can be either PRU or UE with user consent.

NOTE 3, The selected UE may be or may be not served by the gNB.

- The LMF may initiate UE positioning procedures for the selected UE as specified in TS 23.273 section 6.11 to obtain the UE location (i.e. the ground truth label).

- The LMF initiates UL positioning procedures for the selected UE to configure UL SRS as specified in TS 23.273 section 6.11.2 and 6.11.3.

- The LMF sends a NRPPa Positioning Data Collection Report message to the gNB. LMF includes the ground truth label(s) along with the measurement ID(s) and timestamps in the NRPPa Data Collection Information message. The gNB correlates the UL SRS measurements obtained in step 5 with the ground truth label obtained in step 6, and use them as data input for AI model training and/or performance monitoring.

- If the data collection requirement in the gNB is updated (e.g. gNB decide to stop data collection), the gNB sends a Data Collection Requirement Update message to the LMF.

Proposal 3, if proactive NRPPa solution is support, the following enhancements are needed:

- introduce one class 1 and one class 2 procedure for data collection requirement exchange

- introduce a class 2 Positioning data collection report message to transfer the requested data.

### Proactive solution 2-2 (NGAP based)

The following solution has been discussed and is checking the feasibility with SA2.

Solution 2: Positioning Information collection for UEs served by the NG-RAN which are not being positioned

#### NGAP solution for non-served UE

Although we are checking the feasibility of solution 2 (a proactive NGAP-based solution), we still believe that the NGAP solution can be utilized for non-served UEs. Multiple gNBs can indicate their data collection needs in an NGAP positioning data collection request, which includes the measurement type, TRP ID, and optionally, the IDs of interested UEs. The gNB can select the AMF based on OAM configuration. Regarding UE selection, we believe that both the AMF and LMF can perform this task.

##### NGAP based solution – LMF perform UE selection

Below is an example of the call flow for UE selection in the LMF.

- A gNB sends a NGAP Data Collection Request message to a AMF when data collection is required

- The AMF performs the LMF selection.

Editor Note 1, how AMF select LMF depends on the SA2.

- The AMF forwards the data collection request of the gNB to the selected LMF.

Editor Note 2, the details are in SA2 scope.

- The LMF perform UE selection as described in TS 23.273 (e.g. step 1-5 in section 6.22.3) considering the data collection requirement information received from the gNB.

NOTE 1, The selected UE can be either PRU or UE with user consent.

NOTE 2, The selected UE may be or may be not served by the gNB.

- The LMF may initiate UE positioning procedures for the selected UE as specified in TS 23.273 section 6.11 to obtain the UE location (i.e. the ground truth label).

- The LMF initiates UL positioning procedures for the selected UE to configure UL SRS as specified in TS 23.273 section 6.11.2 and 6.11.3.

- The LMF response to the AMF with the ground truth label along with the correlation information (e.g. Routing ID, NRPPa Transaction ID, measurement ID(s), timestamps)

- The AMF forewords the the ground truth label along with the correlation information to the gNB in the NGAP Data Collection Response. The gNB correlates the UL SRS measurements obtained in step 5 with the ground truth label obtained in step 8, and use them as data input for AI model training and/or performance monitoring.

##### NGAP based solution – AMF perform UE selection

Below is an example of the call flow for UE selection in the AMF.

AMF can perform UE selection, taking into account UE consent and positioning capability. The selected UE is within the coverage area of the requested gNB but may not be served by the requested gNB. Therefore, it is understood that even non-served UEs can be supported by an NGAP-based solution.

- A gNB sends a NGAP Data Collection Request message to a AMF when data collection is required

- The AMF performs the UE selection.

Editor Note 1, The selected UE can be either PRU or UE with user consent

Editor Note 2, The selected UE can be served or not served by the requested gNB

- The AMF sends the Nlmf_Location_DetermineLocationRequest to LMF for the selected UE.

- The LMF may initiate UE positioning procedures for the selected UE as specified in TS 23.273 section 6.11 to obtain the UE location (i.e. the ground truth label).

- The LMF initiates UL positioning procedures for the selected UE to configure UL SRS as specified in TS 23.273 section 6.11.2 and 6.11.3.

- The LMF response to the AMF with the ground truth label along with the correlation information (e.g. Routing ID, NRPPa Transaction ID, measurement ID(s), timestamps)

- The AMF forewords the ground truth label along with the correlation information to the gNB in the NGAP Data Collection Response. The gNB correlates the UL SRS measurements obtained in step 5 with the ground truth label obtained in step 8, and use them as data input for AI model training and/or performance monitoring.

Observation 1, In NGAP-based solution, either the AMF or LMF can perform UE selection to ensure that data collection covers both served and non-served UEs.

For proactive solution, although both NAGP-based solution and NRPPa-based solution work, we prefer to go with NRPPa solution as it will have less dependency on SA2.

Proposal 4, RAN3 down-select NRPPa solution and NGAP solution to support proactive data collection.

## LMF awareness of AI/ML

Based on discussions in RAN1, timing information, which includes the UL RTOA and the gNB Rx-Tx time difference, as well as the LOS/NLOS, can be produced using either AI/ML methods or traditional approaches. If the LMF is unaware of whether the measurements originate from AI/ML, this could potentially lead to issues in some scenarios. The table below outlines various combinations for the generation of timing information and LOS/NLOS information. It is important to note that timing information derived from AI is based on LOS condition.

| combinations | UL-RTOA/gNB Rx-Tx time difference | LOS/NLOS | From LMF perspective |
| --- | --- | --- | --- |
| Case 1 | Legacy | Legacy | Aligned |
| Case 2 | AI | Legacy | Not-aligned, AI-based UL-RTOA is based on LOS |
| Case 3 | AI | AI | Aligned |
| Case 4 | Legacy | AI | Aligned |

Case 2 could lead to a misunderstanding by the LMF, where the timing is derived from AI, and the LOS/NLOS indication is produced by traditional methods. Since AI/ML-based timing measurements rely on LOS, and the LOS/NLOS indicator might suggest NLOS using conventional means, the positioning accuracy could be affected. This is because the LMF might use not-aligned information for its positioning calculations.

In our understating, the LMF should be aware of whether AI is used for the measurement results (at least for LOS/NLOS, UL RTOA, gNB Tx-Rx time difference), one reason is that LMF needs to know them to ensure the positioning accuracy, another reason is that LMF knows the LCS requirements, LMF can decide wither to use AI or not. We think the following extent awareness of LMF can be considered

- LMF Controls AI Usage: Aligns with LMF’s role as positioning manager.

- gNB Control AI usage with Notification: Not aligned with LMF’s role as the positioning manager; LMF cannot override decisions.

We think option 1 is more algin with LCS principle, that LMF is the manager of the positioning, option 2 only notify LMF, but LMF can do nothing.

Observation 2, The positioning accuracy may be affected due to the misalignment between AI/ML-based timing information and the traditional LOS/NLOS indicator.

Proposal 5, the LMF should be aware of whether the timing information and LOS/NLOS indication are generated by AI/ML.

Proposal 6, RAN3 discuss how to support the LMF awareness.

# 3	Conclusion

In this contribution, we discussed the AI-based positioning data collection for case 3a and have the following observations and proposals.

## The format of label

Proposal 1, LMF provide the timing information with measurement Quality and LOS/NLOS indicators as part B label to gNB, existing IEs of timing information, measurement Quality and LOS/NLOS indicator.

## Opportunistic solution (solution 1)

Proposal 2, RAN3 agree the following NRPPa enhancements to support opportunistic data collection

- introduce requested measurement type (UL RTOA and/or LOS/NLOS) in measurement request.

- introduce a class 2 Positioning data collection report message to transfer the requested data.

- introduce the UL RTOA with quality, LOS/NLOS as information related to UE location in the Positioning data collection report message.

- introduce LMF measurement ID, NRRPa transaction ID and timestamps as correlation information in the positioning data collection report message.

- introduce a class 2 message to transfer the Positioning Data Collection Failure message to notify the failure of the data collection.

## Proactive solution

Proposal 3, if proactive NRPPa solution is support, the following enhancements are needed:

- introduce one class 1 and one class 2 procedure for data collection requirement exchange

- introduce a class 2 Positioning data collection report message to transfer the requested data.

Observation 1, In NGAP-based solution, either the AMF or LMF can perform UE selection to ensure that data collection covers both served and non-served UEs.

Proposal 4, RAN3 down-select NRPPa solution and NGAP solution to support proactive data collection.

## LMF awareness of AI/ML

Observation 2, The positioning accuracy may be affected due to the misalignment between AI/ML-based timing information and the traditional LOS/NLOS indicator.

Proposal 5,  the LMF should be aware of whether the timing information and LOS/NLOS indication are generated by AI/ML.

Proposal 6, RAN3 discuss how to support the LMF awareness.

# 4	Annex A (TP for TS 38.305) opportunistic solution

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change start>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

7.x	AI/ML assisted positioning with gNB side model

7.x.1	General

NG-RAN support for AI/ML positioning requires inputs of ground truth label and related data from LMF and measurements by the gNB for the purpose of management of the corresponding gNB side AI/ML model.

7.x.2	Positioning Data Collection Procedure

Figure 7.x.2-1 shows the Positioning Data Collection procedure used to retrieve positioning information for a measurement that is undergoing a positioning process.

Figure 7.x.2-1: Positioning Data Collection Procedure

1.	The LMF sends the NRPPa MEASUREMENT REQUEST message to one or more gNBs according to positioning procedures.

2.	The gNB(s) determines that data collection is needed for the UE being positioned.

3.	The gNB(s) sends the NRPPa MEASUREMENT RESPONSE message to the LMF and indicates that data collection is needed for the UE being positioned. Optionally, gNB includes the requested measurement type (UL RTOA and/or LOS/NLOS) in the NRPPa MEASUREMENT RESPONSE message

Note:	Steps 1 to 3 may occur while the LMF performs one or more of the positioning procedures described in clauses 6.11.1, 6.11.2 and 6.11.3 of TS 23.273 [35].

4.	LMF performs the user consent check and PRU check for the requested measurement.

5a. If the UE for the requested measurement is PRU or has user consent, the LMF sends the NRPPa POSITIONING DATA COLLECTION REPORT message to the gNB(s) which indicated that positioning data collection is needed for the UE being positioned. The message includes information related to UE location (UL RTOA and/or LOS/NLOS indicator) and correlation information that enables the gNB to correlate the information related to UE location with information related to UL measurements (e.g., LMF Measurement ID and RAN Measurement ID).

5b. If not, the LMF sends the NRPPa POSITIONING DATA COLLECTION FAILURE to the gNB with appropriate cause value.

Editor’s Note: The texts above could be further refined, if needed.

Editor’s Note: FFS on details of the signallings and IEs.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change ends>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

# 5	Annex B (TP for TS 38.455) opportunistic solution

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change start>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

8	NRPPa procedures

8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| E-CID Measurement Initiation | E-CID MEASUREMENT INITIATION REQUEST | E-CID MEASUREMENT INITIATION RESPONSE | E-CID MEASUREMENT INITIATION FAILURE |
| OTDOA Information Exchange | OTDOA INFORMATION REQUEST | OTDOA INFORMATION RESPONSE | OTDOA INFORMATION FAILURE |
| Positioning Information Exchange | POSITIONING INFORMATION REQUEST | POSITIONING INFORMATION RESPONSE | POSITIONING INFORMATION FAILURE |
| TRP Information Exchange | TRP INFORMATION REQUEST | TRP INFORMATION RESPONSE | TRP INFORMATION FAILURE |
| Measurement | MEASUREMENT REQUEST | MEASUREMENT RESPONSE | MEASUREMENT FAILURE |
| Positioning Activation | POSITIONING ACTIVATION REQUEST | POSITIONING ACTIVATION RESPONSE | POSITIONING ACTIVATION FAILURE |
| PRS Configuration Exchange | PRS CONFIGURATION REQUEST | PRS CONFIGURATION RESPONSE | PRS CONFIGURATION FAILURE |
| Measurement Preconfiguration | MEASUREMENT PRECONFIGURATION REQUIRED | MEASUREMENT PRECONFIGURATION CONFIRM | MEASUREMENT PRECONFIGURATION REFUSE |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| E-CID Measurement Failure Indication | E-CID MEASUREMENT FAILURE INDICATION |
| E-CID Measurement Report | E-CID MEASUREMENT REPORT |
| E-CID Measurement Termination | E-CID MEASUREMENT TERMINATION COMMAND |
| Error Indication | ERROR INDICATION |
| Assistance Information Control | ASSISTANCE INFORMATION CONTROL |
| Assistance Information Feedback | ASSISTANCE INFORMATION FEEDBACK |
| Positioning Information Update | POSITIONING INFORMATION UPDATE |
| Measurement Report | MEASUREMENT REPORT |
| Measurement Update | MEASUREMENT UPDATE |
| Measurement Abort | MEASUREMENT ABORT |
| Measurement Failure Indication | MEASUREMENT FAILURE INDICATION |
| Positioning Deactivation | POSITIONING DEACTIVATION |
| Measurement Activation | MEASUREMENT ACTIVATION |
| SRS Information Reservation Notification | SRS INFORMATION RESERVATION NOTIFICATION |
| Positioning Data Collection Report | POSITIONING DATA COLLECTION REPORT |
| Positioning Data Collection Failure | POSITIONING DATA COLLECTION FAILURE |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<next change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

8.2.x1	Positioning Data Collection Report

8.2.x1.1	General

The purpose of the Positioning Data Collection Report procedure is to provide the positioning data from LMF to NG-RAN node.

8.2.x1.2	Successful Operation

Figure 8.2.x1.2-1: Positioning Data Collection Report procedure, successful operation

The LMF initiates the procedure by sending a POSITIONING DATA COLLECTION REPORT message to the NG-RAN node.

8.2.x1.3	Unsuccessful Operation

Not Applicable.

8.2.x1.4	Abnormal Conditions

Void.

8.2.x2	Positioning Data Collection Failure

8.2.x2.1	General

The purpose of the Positioning Data Collection Failure procedure is to indicate the requested positioning data can not be provided from LMF to NG-RAN node.

8.2.x2.2	Successful Operation

Figure 8.2.x2.2-1: Positioning Data Collection Failure procedure, successful operation

The LMF initiates the procedure by sending a POSITIONING DATA COLLECTION FAILURE message to the NG-RAN node.

8.2.x2.3	Unsuccessful Operation

Not Applicable.

8.2.x2.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<next change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

9.1.4.2	MEASUREMENT RESPONSE

This message is sent by the NG-RAN node to report positioning measurements.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Response List |  | 0..1 |  |  | YES | reject |
| >TRP Measurement Response Item |  | 1..<maxnoofMeasTRPs> |  |  | - |  |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>TRP Measurement Result | M |  | 9.2.37 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| >>Data Collection Request Information |  |  | 9.2.x |  |  |  |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one message. Value is 64. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

9.1.4.x1	Positioning Data Collection Report

This message is sent by the LMF to provide positioning data.

Direction: LMF NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Positioning Data List |  | 0..1 |  |  | YES | reject |
| >TRP Positioning Data Item |  | 1..<maxnoofMeasTRPs> |  |  | - |  |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>Positioning Data | M |  | 9.2.x2 |  | - |  |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one message. Value is 64. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

9.2.x1	Data Collection Request Information

The purpose of Data Collection Request Information is to provide the requeted mesurement type for data collection.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Requested Measurment Type | M |  | ENUMERATED (UL RTOA, LOS/NLOS, ..., UL RTOA and LOS/NLOS) |  | - |  |

9.2.x2	Positioning Data

The purpose of Data Collection Request Information is to provide the requeted positioning data.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Positioning Data Item |  | 1 .. <maxnoPosData> |  |  | - |  |
| >CHOICE Positioning Data Value | M |  |  |  |  |  |
| >>UL RTOA |  |  | 9.2.39 |  |  |  |
| >>LOS/NLOS |  |  | 9.2.77 |  |  |  |
| >Time Stamp | M |  | 9.2.42 |  |  |  |
| >Measurement Quality | O |  | 9.2.43 |  |  |  |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change ends>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>