3GPP TSG-RAN3 Meeting #128	R3-253383

# Malta, Malta, 19th May 2025 - 23rd May 2025

Agenda item:	15.2

Source:	Xiaomi

Title:	(TP for TS 36.300)Support of full eNB as regenerative payload

Document for:	Discussion

1	Introduction

In this contribution, we’d like to discuss how to support full eNB as regenerative payload for IoT NTN according to the progresses below.

RAN3 decides to work on Split MME architecture and Full CN on board architecture. Whether eNB only on board architecture is feasible or not can be discussed later. Introduce stage 2 description of Store and Forward operation. Introduce stage 2 description of full eNB as regenerative payload. Introduce the S1 Removal procedure. Check progress in NR NTN and any further impact on IoT NTN…

The following topics are discussed:

- Without S&F satellite operation

- S&F satellite operation

2	Discussion

### Without S&F satellite operation

In NR NTN, soft feeder link switch and hard feeder links switch has been discussed and the following progresses are achieved, according to [1].

TNL address updates due to feeder link switch I case of the TNL addresses are changed due to feeder link switch over, existing procedures may be used. R3-252453 is agreed to add a note for TNL addresses update due to feeder link switch over. NG Connection interruption due to hard feeder link switch over R3-252454 and R3-252409 are agreed to support NG transmission suspend/resume and with the following FFS: Further check the functionality: in case of hard feeder link switch over, the 5GC suspends or resumes the DL signalling/data transmission(s) to the gNB based on the new indicator in RAN configuration update, FFS on the relationship with respect to NG suspend/resume (if any).

For TNL address updates in IoT NTN, it is important to note that the S1 signaling transport does not support multiple SCTP associations, as specified in TS 36.412 [2]. However, it does support SCTP multi-homing, which allows for multiple IP addresses between two endpoints. We believe that TNL address updates due to feeder link switch can also be accommodated by the existing S1 TNL management.

Transport network redundancy may be achieved by SCTP multi-homing between two end-points, of which one or both is assigned with multiple IP addresses. SCTP end-points shall support a multi-homed remote SCTP end-point. For SCTP endpoint redundancy an INIT may be sent from MME or eNB, at any time for an already established SCTP association, which shall be handled as defined in IETF RFC 4960 [5] in subclause 5.2.

For NG interruption due to the hard feeder link switch over , we believe the same changes in NR NTN can also be applied to IoT NTN, it is proposed to introduce an suspend/resume indicator from eNB to MME to suspend/resume the S1 signalling/data transmission.

Observation 1, the TNL address update due to feeder link switch can be supported by SCTP multi-homing.

Proposal 1, In case of TNL address updates due to feeder link switch, existing TNL management may be reused.

Proposal 2, RAN3 agree to introduce suspend/resume indicator from eNB to MME to suspend/resume the S1 signalling/data transmission due to hard feeder link switchover.

### S&F satellite operation

One possible deployment option is that the onboard eNB and CN parts are deployed on different satellites within the same orbit, the distance and connection between the CN and eNB are relatively stable, communicating via inter-satellite links as shown in Figure 1.

Figure 1 example arch for eNB and CN in different satellites

if the onboard eNB and onboard CN deployed in different satellites, we believe that the SA2’s solution can be used and there will be no additional SA2 impacts. One consideration is that the eNB needs to be aware of whether the MME on the other satellite has an available feeder link and whether store and forward (S&F) operation is supported, so that it can decide whether to serve UE or whether to broadcast the store and forward operation to UE.

The following scenarios may exist (assuming the eNB and CN can communicate via inter-satellite link):

Case 1: The eNB has no feeder link, while the MME has a feeder link. The eNB can provide normal service to the UE.

Case 2: Both the eNB and CN have no feeder link, and both support S&F. The eNB can broadcast an S&F indication to the UE.

Case 3: The eNB has a feeder link and can connect to the ground CN, allowing the eNB to provide normal service to the UE.

It is evident that the eNB’s behaviour may vary depending on the feeder link status of both the eNB and the MME. Therefore, it would be beneficial for the eNB to be aware of the feeder link status of the CN on other satellites.

Proposal 3, RAN3 discuss whether to support the deployment option that the onboard eNB and onboard CN are on different satellites but in same orbit.

Proposal 4, if the scenario that onboard eNB and onboard CN are deployed on different satellites is supported, eNB needs to be aware of the feeder link status of CN on other satellites.

3	Conclusion

Based on the discussion, we have the following observations and proposals.

### without S&F satellite operation

Observation 1, the TNL address update due to feeder link switch can be supported by SCTP multi-homing.

Proposal 1, In case of TNL address updates due to feeder link switch, existing TNL management may be reused.

Proposal 2, RAN3 agree to introduce suspend/resume indicator from eNB to MME to suspend/resume the S1 signalling/data transmission due to hard feeder link switchover.

### S&F satellite operation

Proposal 3, RAN3 discuss whether to support the deployment option that the onboard eNB and onboard CN are on different satellites.

Proposal 4, if the scenario that onboard eNB and onboard CN are deployed on different satellites within the same orbit is supported, eNB needs to be aware of the feeder link status of CN on other satellites.

4	Reference

[1] R3-252348, Summary of Offline Discussion on NRNTN, Xiaomi (moderator)

[2]3GPP TS 36.412, S1 signalling transport V18.1.0

5	Annex (TP for TS 36.300)

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change starts>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

23.21.5.3	Procedures

The NTN control function determines the point in time when the feeder link switchover between two eNBs is performed. For BL UEs and UEs in enhanced coverage, the transfer of the affected UE at feeder link switchover is performed by means of handover or TNL functionality, and it depends on the eNBs' implementation and configuration information provided to the eNBs by the NTN Control function.

NOTE:	In case of TNL address change due to feeder link switchover, existing mechanism (e.g., SCTP multi-homing) may be used.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change ends>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>