3GPP TSG-RAN WG3 Meeting #128	R3-253387

St. Julian’s, Malta, May 19th – 23rd 2025

Agenda item:	9.2

Title: 	MN-SN Coordination for Segmentation of QoE Reports

Source: 	Ericsson

Document for:	Agreement

# 1	Introduction

At the RAN3#127-bis meeting, RAN3 discussed segmentation of QoE reports, based on the reply LS from RAN2 in R3-251516. The following was agreed:

Before sending RRC reconfiguration message to the UE, the configuring node for a QoE configuration will have to be in-sync with another node on the RRC segmentation status.

In this contribution, we discuss how to capture the above in stage 3.

# 2	MN-SN Coordination

In the context of configuring the QoE/RVQoE reporting for a UE in NR-DC, the above agreement means that, when a QoE-configuring node wants to offload the reporting the QoE-non-configuring node, it needs to know whether the non-configuring node is willing/able to configure RRC segmentation on its QoE-specific SRB. There are 3 relevant scenarios:

- Scenario 1: The QoE-configuring node configures the UE with a new QoE configuration, and it wants the UE to send the reports via the SRB controlled by the other node.

- Scenario 2: The QoE-configuring node wants to change the reporting leg for an existing QoE configuration (e.g., to offload QoE reports to the other node).

- Scenario 3: A node (MN or SN) wants to change the RRC segmentation for its own SRB (e.g., because it realizes that QoE reports are too big).

In the three scenarios, how the QoE-configuring node decides to configure the UE may depend on whether the non-configuring node is willing to receive segmented QoE reports or not. Therefore, the coordination between MN and SN in these scenarios has two purposes: a) to obtain the knowledge of the RRC segmentation status in the other node, and b) to properly configure the reporting path that the UE will use.

Let’s assume that the MN wants to either change the reporting path for QoE a configuration (Scenario 2) or change the SRB4 segmentation to avoid losing QoE reports (Scenario 3). If the MN wants to change the reporting from SRB4 to SRB5 (Scenario 2), it can ask the SN whether the SN allows segmentation for SRB5. If the SN does not allow this, the MN may decide not to offload the reporting to the SN (and no RRC Reconfiguration is sent to the UE). Alternatively, the MN may decide to allow segmentation for SRB4 (as in Scenario 3).

A common way to address all the scenarios is then the following: the QoE-configuring node requests the QoE non-configuring node whether it allows RRC segmentation for its corresponding SRB and the non-configuring node replies accordingly.

Proposal 1: For a given QoE configuration, the QoE-configuring node requests the QoE non-configuring node to indicate whether it (i.e., the QoE non-configuring node) allows RRC segmentation for its corresponding SRB, or not, and the QoE non-configuring node replies accordingly.

Even though the segmentation status of SRB4 and SRB5 is per-UE, it is clear that MN and SN need to coordinate each time a RRC Reconfiguration has to be sent to the UE for one QoE configuration. In some, albeit unlikely, scenarios, the MN or the SN may want to configure multiple QoE configurations for the UE at the same time. Accordingly, we can consider two approaches to stage-3 design.

A first approach (Option 1) is that, for a given QoE Reference in the QMC Coordination Request IE, the MN (or the SN) requests the SN (or the MN) to indicate whether RRC segmentation is allowed for SRB5 (or SRB4), and the SN (or the MN) replies in the QMC Coordination Response IE.

Another approach (Option 2) is that a UE-level parameter (pertinent to all QoE configurations) is added to the QMC Coordination Request IE, by which the MN (or the SN) requests the SN (or the MN) to indicate whether RRC segmentation is allowed for SRB5 (or SRB4), and the SN (or the MN) replies in the QMC Coordination Response IE.

We propose that RAN3 discusses which option to adopt. In our understanding, the wording of the RAN3#128 agreement “…for a QoE configuration….” seems to be closer to Option 1.

Proposal 2: RAN3 to discuss and downselect between the following two options:

- Option 1: Introduce a per-QoE reference indication in the QMC Coordination Request IE by which the MN/SN requests the SN/MN to indicate whether RRC segmentation is allowed for SRB5/SRB4. The SN/MN sends the response IE in the QMC Coordination Response IE.

- Option 2: Introduce a UE-level indication in the QMC Coordination Request IE by which the MN/SN requests the SN/MN to indicate whether RRC segmentation is allowed for SRB5/SRB4. The SN/MN sends the response IE in the QMC Coordination Response IE.

An example of each option is provided in the tabular below.

Option 1

9.2.3.197	QMC Coordination Request

(…)

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MN to SN QMC Coordination Request List |  | 0..1 |  |  |
| >MN to SN QMC Coordination Request Item |  | 1..<maxnoofUEAppLayerMeas> |  |  |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) | QoE Reference, as defined in clause 5.2 of TS 28.405 [55]. It consists of MCC+MNC+QMC ID, where the MCC and MNC are received with the QMC activation request from the management system to identify one PLMN hosting the management system, and QMC ID is a 3-byte Octet String. |
| >>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<< |  |  |  |  |
| >>Configuration Release Indication | O |  | ENUMERATED (rvqoe, qoe and rvqoe, …) | This IE indicates that the configuration has been released by the M-NG-RAN node. |
| >>Allowed RRC Segmentation SRB5 Request | O |  | ENUMERATED (true, …) | With this IE, the M-NG-RAN node requests the S-NG-RAN node to indicate whether RRC segmentation is allowed for SRB5. |
| SN to MN QMC Coordination Request List |  | 0..1 |  |  |
| >SN to MN QMC Coordination Request Item |  | 1..<maxnoofUEAppLayerMeas> |  |  |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) | QoE Reference, as defined in clause 5.2 of TS 28.405 [55]. It consists of MCC+MNC+QMC ID, where the MCC and MNC are received with the QMC activation request from the management system to identify one PLMN hosting the management system, and QMC ID is a 3-byte Octet String. |
| >>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<< |  |  |  |  |
| >>Configuration Release Indication | O |  | ENUMERATED (rvqoe, qoe and rvqoe, ...) | This IE indicates that the configuration has been released by the S-NG-RAN node. |
| >>Allowed RRC Segmentation SRB4 Request | O |  | ENUMERATED (true, …) | With this IE, the M-NG-RAN node requests the S-NG-RAN node to indicate whether RRC segmentation is allowed for SRB5. |

9.2.3.198	QMC Coordination Response

(…)

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MN to SN QMC Coordination Response List |  | 0..1 |  |  |
| >MN to SN QMC Coordination Response Item |  | 1..<maxnoofUEAppLayerMeas> |  |  |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) | QoE Reference, as defined in clause 5.2 of TS 28.405 [55]. It consists of MCC+MNC+QMC ID, where the MCC and MNC are received with the QMC activation request from the management system to identify one PLMN hosting the management system, and QMC ID is a 3-byte Octet String. |
| >>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<< |  |  |  |  |
| >>Preferred RAN Visible QoE Configuration | O |  | RAN Visible QoE Configuration 9.2.3.201 | The RAN Visible QoE Configuration preference of the M-NG-RAN node. |
| >>Allowed RRC Segmentation SRB4 Response | O |  | ENUMERATED (allowed, not allowed, …) | Indicates whether the M-NG-RAN node allows RRC segmentation for SRB4. |
| SN to MN QMC Coordination Response List |  | 0..1 |  |  |
| >SN to MN QMC Coordination Response Item |  | 1..<maxnoofUEAppLayerMeas> |  |  |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) | QoE Reference, as defined in clause 5.2 of TS 28.405 [55]. It consists of MCC+MNC+QMC ID, where the MCC and MNC are received with the QMC activation request from the management system to identify one PLMN hosting the management system, and QMC ID is a 3-byte Octet String. |
| >>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<< |  |  |  |  |
| >>Preferred RAN Visible QoE Configuration | O |  | RAN Visible QoE Configuration 9.2.3.201 | The RAN Visible QoE Configuration preference of the S-NG-RAN node. |
| >>Allowed RRC Segmentation SRB5 Response | O |  | ENUMERATED (allowed, not allowed, …) | Indicates whether the S-NG-RAN node allows RRC segmentation for SRB5. |

Option 2

9.2.3.197	QMC Coordination Request

(…)

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MN to SN QMC Coordination Request List |  | 0..1 |  |  |
| >MN to SN QMC Coordination Request Item |  | 1..<maxnoofUEAppLayerMeas> |  |  |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) | QoE Reference, as defined in clause 5.2 of TS 28.405 [55]. It consists of MCC+MNC+QMC ID, where the MCC and MNC are received with the QMC activation request from the management system to identify one PLMN hosting the management system, and QMC ID is a 3-byte Octet String. |
| >>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<< |  |  |  |  |
| >>Configuration Release Indication | O |  | ENUMERATED (rvqoe, qoe and rvqoe, …) | This IE indicates that the configuration has been released by the M-NG-RAN node. |
| Allowed RRC Segmentation SRB5 Request | O |  | ENUMERATED (true, …) | With this IE, the M-NG-RAN node requests the S-NG-RAN node to indicate whether RRC segmentation is allowed for SRB5. |
| SN to MN QMC Coordination Request List |  | 0..1 |  |  |
| >SN to MN QMC Coordination Request Item |  | 1..<maxnoofUEAppLayerMeas> |  |  |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) | QoE Reference, as defined in clause 5.2 of TS 28.405 [55]. It consists of MCC+MNC+QMC ID, where the MCC and MNC are received with the QMC activation request from the management system to identify one PLMN hosting the management system, and QMC ID is a 3-byte Octet String. |
| >>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<< |  |  |  |  |
| >>Configuration Release Indication | O |  | ENUMERATED (rvqoe, qoe and rvqoe, ...) | This IE indicates that the configuration has been released by the S-NG-RAN node. |
| Allowed RRC Segmentation SRB4 Request | O |  | ENUMERATED (true, …) | With this IE, the S-NG-RAN node requests the M-NG-RAN node to indicate whether RRC segmentation is allowed for SRB4. |

9.2.3.198	QMC Coordination Response

(…)

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MN to SN QMC Coordination Response List |  | 0..1 |  |  |
| >MN to SN QMC Coordination Response Item |  | 1..<maxnoofUEAppLayerMeas> |  |  |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) | QoE Reference, as defined in clause 5.2 of TS 28.405 [55]. It consists of MCC+MNC+QMC ID, where the MCC and MNC are received with the QMC activation request from the management system to identify one PLMN hosting the management system, and QMC ID is a 3-byte Octet String. |
| >>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<< |  |  |  |  |
| >>Preferred RAN Visible QoE Configuration | O |  | RAN Visible QoE Configuration 9.2.3.201 | The RAN Visible QoE Configuration preference of the M-NG-RAN node. |
| Allowed RRC Segmentation SRB4 Response | O |  | ENUMERATED (allowed, not allowed, …) | Indicates whether the M-NG-RAN node allows RRC segmentation for SRB4. |
| SN to MN QMC Coordination Response List |  | 0..1 |  |  |
| >SN to MN QMC Coordination Response Item |  | 1..<maxnoofUEAppLayerMeas> |  |  |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) | QoE Reference, as defined in clause 5.2 of TS 28.405 [55]. It consists of MCC+MNC+QMC ID, where the MCC and MNC are received with the QMC activation request from the management system to identify one PLMN hosting the management system, and QMC ID is a 3-byte Octet String. |
| >>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<< |  |  |  |  |
| >>Preferred RAN Visible QoE Configuration | O |  | RAN Visible QoE Configuration 9.2.3.201 | The RAN Visible QoE Configuration preference of the S-NG-RAN node. |
| Allowed RRC Segmentation SRB5 Response | O |  | ENUMERATED (allowed, not allowed, …) | Indicates whether the S-NG-RAN node allows RRC segmentation for SRB5. |

# 3	Conclusions

Based on the discussion in this paper, we propose the following:

Proposal 1: For a given QoE configuration, the QoE-configuring node requests the QoE non-configuring node to indicate whether it (i.e., the QoE non-configuring node) allows RRC segmentation for its corresponding SRB, or not, and the QoE non-configuring node replies accordingly.

Proposal 2: RAN3 to discuss and downselect between the following two options:

- Option 1: Introduce a per-QoE reference indication in the QMC Coordination Request IE by which the MN/SN requests the SN/MN to indicate whether RRC segmentation is allowed for SRB5/SRB4. The SN/MN sends the response IE in the QMC Coordination Response IE.

- Option 2: Introduce a UE-level indication in the QMC Coordination Request IE by which the MN/SN requests the SN/MN to indicate whether RRC segmentation is allowed for SRB5/SRB4. The SN/MN sends the response IE in the QMC Coordination Response IE.