3GPP TSG-RAN WG3 #128	          R3-253393

Malta, MT, 19th – 23th May  2025

Agenda item:	15.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on the support of S&F and regenerative architecture

Document for:	Discussion and Decision

# 1	Introduction

Based on the NR NTN progress, this contribution discusses the remaining issues for IoT NTN.

# 2	Discussion on Store & Forward

eNB only on board architecture

In previous RAN3 meeting, RAN3 agreed to work on Split MME architecture and Full CN on board architecture, but no conclusion on whether eNB only on board architecture is feasible or not can be discussed later.

The eNB only on board architecture is agreed for regenerative architecture, but it is not possible to use it for S&F operation. Using this “eNB only on board architecture” for S&F operation has impact to Core Network. It is unclear how the NAS procedure (e.g. Attach) procedure, or MO/MT, etc. are performed. SA2 has agreed to not consider it in Rel-19. So it is impractical for RAN3 to consider this architecture, especially SA2 has already completed Rel-19 5GSAT_ARCH_Ph3 WI.

Proposal 1-1: RAN3 only work on the architecture options selected by SA2, and stop the discussion on “eNB only on board architecture” for S&F.

Split MME Architecture option

SA2 agreed “For data transfer, only Control Plane CIoT EPS Optimization applies.” For Control Plane CIoT EPS optimisation, existing MO/MT procedures defined in TS 36.300 can be reused without modification.  In the below figure, the MME is split into MME-onboard, and MME-Ground. The MO data is buffered in the MME-onboard, when  Feeder Link is unavailable. For MT, the MT data is buffered in MME-onboard when Service Link is unavailable.

Figure 7.3b-1: MO-EDT for Control Plane CIoT EPS Optimisation

Figure 7.3c-1: MT-EDT for Control Plane CIoT EPS Optimisation

In CP CIOT EPS optimization, the DRB and AS security are not used. From RAN3 perspective, the eNB connects with the MME on the same satellite. The MO/MT data is buffered in MME, and does not affect eNB. Via the S1 interface, current MO/MT for CIOT-CP procedure can be reused.

Proposal 1-2: No enhancement is needed for Control Plane CIOT EPS Optimisation in split MME architecture.

Per SA2 decision, RAN3 should not discuss User Plane CIoT EPS Optimization for Split MME architecture.

Proposal 1-3: RAN3 should Not discuss User Plane CIoT EPS Optimization for Split MME architecture.

Full CN on satellite Architecture option

In this architecture option, the whole CN including eNB, MME, SGW, PGW, HSS, E-SMLC, SMSC etc are on board each satellite. The MO/MT for a specific UE is only performed via the same satellite.  In case multiple satellites are used, UE context/information need to be shared among the satellites, which will be in SA2 scope. From RAN3 perspective, the eNB connect with the MME/SGW on the same satellite. Current MO/MT procedure can be reused. It seems no impact to RAN3.

Proposal 1-4: There is no RAN3 impact to support the architecture option with full CN on satellite.

# 3	Discussion on Regenerative architecture

Considering the progress in NR NTN, it is necessary to mirror the NR NTN agreements/TPs to IoT NTN. Last RAN3 meeting agreed the Stage-2/3 TP to support hard FLSO. We propose to update BL CRs to align with NR NTN decision.

Proposal 2-1: update BL CRs to align with NR NTN.

The draft TS 36.300 TP can be found in ([1])

The draft TS 36.413 TP can be found in ([2])

# 4	Discussion on PWS support for NB-IoT NTN

RAN2 has agreed “Introduce the Warning Area Coordinates IE to existing ETWS SIBs”. In current S1AP, the WRITE-REPLACE WARNING REQUEST message includes the optional Warning Area Coordinates IE.

If the Warning Area Coordinates IE is included in the WRITE-REPLACE WARNING REQUEST message, the eNB shall include this information together with the warning message being broadcast according to TS 36.331 [16].

By the Warning Area Coordinates IE received in the eNB, the eNB can add Warning Area Coordinates information to existing ETWS SIBs. So current S1AP can support the RAN2 agreement. No S1AP Stage-3 enhancement is needed at current stage. RAN3 can wait for further RAN2 progress to determine whether any Stage-2 or Stage-3 update are needed.

Observation: Current S1AP can support RAN2 agreement “Introduce the Warning Area Coordinates IE to existing ETWS SIBs”.

Proposal 3: RAN3 wait for RAN2 progress on potential Stage-2/3 enhancement to support PWS in NB-IOT NTN.

# 5	Conclusion

In this contribution, we discussed the remaining issues for IoT NTN. Our proposals are:

For S&F:

Proposal 1-1: RAN3 only work on the architecture options selected by SA2, and stop the discussion on “eNB only on board architecture” for S&F.

Proposal 1-2: No enhancement is needed for Control Plane CIOT EPS Optimisation in split MME architecture.

Proposal 1-3: RAN3 should Not discuss User Plane CIoT EPS Optimization for Split MME architecture.

Proposal 1-4: There is no RAN3 impact to support the architecture option with full CN on satellite.

For Regenerative architecture:

Proposal 2-1: update BL CRs to align with NR NTN.

The draft TS 36.300 TP can be found in ([1])

The draft TS 36.413 TP can be found in ([2])

For PWS:

Proposal 3: RAN3 wait for RAN2 progress on potential Stage-2/3 enhancement to support PWS in NB-IOT NTN.

# References

- R3-253267	(TP for TS 36.300) Align with NR NTN on Hard FLSO for IoT NTN

- R3-253394 (TP for TS 36.413) Align with NR NTN on FLSO