3GPP TSG-RAN WG3 #128	          R3-253397

Malta, MT, 19th – 23th May  2025

Agenda item:	21.3

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP to BL CR for TS 38.415, TS 38.425 and TS 38.413) Update for BSSize/TTNB and available data rate exposure

Document for:	Discussion and Decision

# 1	Introduction

This contribution proposes TP to fix the FFS/EN in the BL CR for TS 38.415, TS 38.425 and TS 38.413.

# TP to BL CR for TS 38.415

-----------------Start of the Changes-------------------

#### 5.5.2.2	UL PDU SESSION INFORMATION (PDU Type 1)

This frame format is defined to allow the UPF to receive some control information elements which are associated with the transfer of a packet over the interface.

The following shows the respective UL PDU SESSION INFORMATION frame.

| Bits |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| PDU Type (=1) |  |  |  | QMP | DL Delay Ind. | UL Delay Ind. | SNP | 1 |
| N3/N9 Delay Ind. | New IE Flag | QoS Flow Identifier |  |  |  |  |  | 1 |
| DL Sending Time Stamp Repeated |  |  |  |  |  |  |  | 0 or 8 |
| DL Received Time Stamp |  |  |  |  |  |  |  | 0 or 8 |
| UL Sending Time Stamp |  |  |  |  |  |  |  | 0 or 8 |
| DL Delay Result |  |  |  |  |  |  |  | 0 or 4 |
| UL Delay Result |  |  |  |  |  |  |  | 0 or 4 |
| UL QFI Sequence Number |  |  |  |  |  |  |  | 0 or 3 |
| N3/N9 Delay Result |  |  |  |  |  |  |  | 0 or 4 |
| New IE flag 7(E) | New IE Flag 6 | New IE Flag 5 | New IE Flag 4 | New IE Flag 3 | New IE Flag 2 | New IE Flag 1 | New IE Flag 0 | 0 or 1 New IE Flags Octet |
| Spare |  |  |  |  |  |  | D1 UL PDCPDelay Result Ind | 0 or 1 |
| UL Congestion Information |  |  |  |  |  |  |  | 0 or 2 |
| DL Congestion Information |  |  |  |  |  |  |  | 0 or 2 |
| UL Available Data Rate |  |  |  |  |  |  |  | 0 or 4 |
| DL Available Data Rate |  |  |  |  |  |  |  | 0 or 4 |
| Padding |  |  |  |  |  |  |  | 0-3 |

Figure 5.5.2.2-1: UL PDU SESSION INFORMATION (PDU Type 1) Format

The New IE Flag in bit 6 of 2nd octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the first octet of New IE Flags Octet is present or not.

Bit 0 of New IE Flags Octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the D1 UL PDCP Delay Result Ind is present (1) or not (0).

Bit 1 of New IE Flags Octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the UL Congestion Information is present (1) or not (0).

Bit 2 of New IE Flags Octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the DL Congestion Information is present (1) or not (0).

Bit 3 of New IE Flags Octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the UL Available Data Rate is present (1) or not (0).

Bit 4 of New IE Flags Octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the DL Available Data Rate is present (1) or not (0).

### 5.5.3	Coding of information elements in frames

#### 5.5.3.1	PDU Type

Description: The PDU Type indicates the structure of the PDU session UP frame. The field takes the value of the PDU Type it identifies; i.e. "0" for PDU Type 0. The PDU type is in bit 4 to bit 7 in the first octet of the frame.

Value range: {0= DL PDU SESSION INFORMATION, 1=UL PDU SESSION INFORMATION, 2-15=reserved for future PDU type extensions}.

Field length: 4 bits.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 5.5.3.26	UL Congestion Information

Description: For the cases of ECN marking at UPF request, this field indicates the percentage of UL IP packets up to two decimal points that should be ECN marked for a QoS flow.

For the case of congestion information request, this field should be interpreted as a percentage of congestion level in UL up to two decimal points for a QoS Flow.

As an example, value 9574 corresponds to a percentage of 95.74%

Value range: {0..10000}.

Field length: 2 octets.

#### 5.5.3.a	UL Available Data Rate Information

Description: This parameter indicates the UL available data rate of the QoS flow. The unit is Kbps.

Value range: {0..4,000,000,000}.

Field length: 4 octets.

#### 5.5.3.b	DL Available Data Rate Information

Description: This parameter indicates the DL available data rate of the QoS flow. The unit is Kbps.

Value range: {0..4,000,000,000}.

Field length: 4 octets.

#### 5.5.3.ca	BSSI (Burst Size Indicator)

Description: This parameter indicates the presence of Burst Size (BSSize).

Value range: {0= BSSize not present, 1= BSSize present}.

Field length: 1 bit.

#### 5.5.3.db	Burst Size (BSSize)

Description: This parameter indicates the total size of the burst.

Value range: {0..224-1}.

Field length: 3 octets.

#### 5.5.3.e	TTNBI (TTNB Indicator)

Description: This parameter indicates the presence of Time To Next Burst (TTNB).

Value range: {0= TTNB not present, 1= TTNB present}.

Field length: 1 bit.

#### 5.5.5.f	Time To Next Burst (TTNB)

Description: This parameter indicates the the approximate time in tenth of milliseconds to the next burst.

Value range: {0..216-1}.

Field length: 2 octets.

-----------------End of the Change-------------------

# TP to BL CR for TS 38.425

-----------------Start of the Changes-------------------

#### 5.5.2.3	ASSISTANCE INFORMATION DATA (PDU Type 2)

This frame format is defined to allow the node hosting the PDCP entity to receive assistance information.

The following shows the respective ASSISTANCE INFORMATION DATA frame.

NOTE 1:	All information elements defined in Figure 5.5.2.3-1 are also applicable to E-UTRA PDCP unless specified otherwise in section 5.5.3.

| Bits |  |  |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 |  | 5 |  | 4 | 3 | 2 | 1 | 0 |  |
| PDU Type (=2) |  |  |  |  |  | PDCP Dupl. Ind. | Assistance Info. Ind. | UL Delay Ind. | DL Delay Ind. | 1 |
| Spare |  | UL Available Data rate Ind |  | DL Available Data rate Ind | DL PSI Based Discard Ind. | DL PSI Based Discard Activation Suggestion | UL Congestion Information Ind. | DL Congestion Information Ind. | PDCP Duplication Activation Suggestion | 1 |
| Number of Assistance Information Fields |  |  |  |  |  |  |  |  |  | 0 or 1 |
| Assistance Information Type |  |  |  |  |  |  |  |  |  | 0 or (2*Number of Assistance Info Fields + sum of Number of octets for Radio Quality Assistance Information Fields) |
| Number of octets for Radio Quality Assistance Information Fields |  |  |  |  |  |  |  |  |  |  |
| Radio Quality Assistance Information |  |  |  |  |  |  |  |  |  |  |
| UL Delay DU Result |  |  |  |  |  |  |  |  |  | 0 or 4 |
| DL Delay DU Result |  |  |  |  |  |  |  |  |  | 0 or 4 |
| UL Congestion Information |  |  |  |  |  |  |  |  |  | 0 or 2 |
| DL Congestion Information |  |  |  |  |  |  |  |  |  | 0 or 2 |
| UL Available Data Rate |  |  |  |  |  |  |  |  |  | 0 or 4 |
| DL Available Data Rate |  |  |  |  |  |  |  |  |  | 0 or 4 |

Figure 5.5.2.3-1: ASSISTANCE INFORMATION DATA (PDU Type 2) Format

NEXT CHANGE

### 5.5.3	Coding of information elements in frames

NOTE 1:	In this section, information elements are also applicable to E-UTRA PDCP unless specified otherwise. With this understanding, each instance of NR PDCP can be replaced by E-UTRA PDCP unless specified otherwise.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 5.5.3.58	Feedback Delay Result

Description: This parameter indicates the feedback delay time measured at the corresponding node in milliseconds from the time of reception of the Polling Frame that triggered transmission of the DL DATA DELIVERY STATUS to the time of transmission of the DL DATA DELIVERY STATUS. It is encoded as an Unsigned32 binary integer value.

Value range: {0..232-1}.

Field length: 4 octets.

#### 5.5.3.x	DL PSI Based Discard Indication

Description: This field indicates the presence of the DL PSI Based Discard Activation Suggestion. This information element is not applicable to E-UTRA PDCP.

Value range: {0= DL PSI Based Discard Activation Suggestion not present, 1= DL PSI Based Discard Activation Suggestion present}.

Field length: 1 bit.

#### 5.5.3.y	DL PSI Based Discard Activation Suggestion

Description: This parameter indicates the suggestion given by the corresponding node on whether DL PSI based PDCP discard should be activated or not. This information element is not applicable to E-UTRA PDCP.

Value range: {0= Not activate, 1= Activate}.

Field length: 1 bit.

#### 5.5.3.x1	UL Available Data Rate Ind

Description: This parameter indicates the presence of UL Available Data Rate. This information element is not applicable to E-UTRA PDCP.

Value range: {0= UL Available Data Rate not present, 1= UL Available Data Rate present}.

Field length: 1 bit.

#### 5.5.3.y1	DL Available Data Rate Ind

Description: This parameter indicates the presence of DL Available Data Rate. This information element is not applicable to E-UTRA PDCP.

Value range: {0= DL Available Data Rate not present, 1= DL Available Data Rate present}.

Field length: 1 bit.

#### 5.5.3.x2	UL Available Data Rate

Description: This field indicates the value of uplink available data rate for the QoS flow. This information element is not applicable to E-UTRA PDCP. The unit is Kbps.

Value range: {0.. 4,000,000,000}.

Field length: 4 octets.

#### 5.5.3.y2	DL Available Data Rate

Description: This field indicates the value of downlink available data rate for the QoS flow. This information element is not applicable to E-UTRA PDCP. The unit is Kbps.

Value range: {0..4,000,000,000}.

Field length: 4 octets.

#### 5.5.3.c	BSSI (Burst Size Indicator)

Description: This parameter indicates the presence of Burst Size (BSSize).

Value range: {0= BSSize not present, 1= BSSize present}.

Field length: 1 bit.

#### 5.5.3.d	Burst Size (BSSize)

Description: This parameter indicates the total size of the burst.

Value range: {0..224-1}.

Field length: 3 octets.

#### 5.5.3.e	TTNBI (TTNB Indicator)

Description: This parameter indicates the presence of Time To Next Burst (TTNB).

Value range: {0= TTNB not present, 1= TTNB present}.

Field length: 1 bit.

#### 5.5.3.f	Time To Next Burst (TTNB)

Description: This parameter indicates the the approximate time in tenth of milliseconds to the next burst.

Value range: {0..216-1}.

Field length: 2 octets.

-----------------End of the Change-------------------

# TP to BL CR for TS 38.413

-----------------Start of the Changes-------------------

#### 9.3.1.x 	Available Data Rate Report Threshold List

This IE contains a list of available data rate report thresholds. It is used for available data rate report for UL and DL as specified in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Available Data Rate Report Threshold Item |  | 1..<maxnoofThresholds> |  |  |
| >Reporting Threshold | M |  | INTEGER (0.. 4000000000) | This IE indicates the Reporting threshold as specified in TS 23.501 [9]. The unit is Kbps. |

| Range bound | Explanation |
| --- | --- |
| maxnoofThresholds | Maximum no. of thresholds allowed to be provided by the SMF. Value is 8. |

-----------------Next Changes-------------------

ReportArea ::= ENUMERATED {

cell,

...

}

ReportingThreshold	::= INTEGER (0..4000000000)

RepetitionPeriod ::= INTEGER (0..131071)

-----------------Next Changes-------------------

maxnoofPartiallyAllowedS-NSSAIs				INTEGER ::= 8

maxnoofRSPPQoSFlows							INTEGER ::= 2048

maxnoofThresholds							INTEGER ::= 8

-----------------End of the Change-------------------