3GPP TSG-RAN WG3 Meeting #128	R3-253422

St. Julian - Malta, 19th – 23rd May 2025

Title: 	AIML enabled CCO - Single predicted CCO issue resolution

Source: 	Ericsson, Jio Platforms, Deutsche Telekom, InterDigital

Agenda item:	11.3

Document Type:	Discussion and agreement

# 1. Introduction

In this contribution we discuss how to resolve a predicted CCO issue across multiple nodes, considering the following open issue from the last meeting:

For the receiving side, whether gNB-CU can also provide to gNB-DU a future CCO state as assistance information to derive a matching future CCO state?

Option1: no need

Opton2: gNB-CU directly forwards the received further CCO state of neighbor cells to gNB-DU.

Option3: gNB-CU generates a recommend further CCO state and send to gNB-DU.

# 2. Discussion

In TS38.300 the CCO functionality is described as follows:

#### 15.5.5.3 Dynamic coverage configuration changes An NG-RAN node may autonomously adjust within and switch between coverage configurations. When a change is executed, a NG-RAN node may notify its neighbour NG-RAN nodes using the NG-RAN NODE CONFIGURATION UPDATE message with the list of cells and SSBs with modified coverage included. The list contains the CGI of each modified cell with its coverage state indicator and optionally the SSB index of each modified SSB with its coverage state indicator. The coverage state indicator may be used at the receiving NG-RAN node to adjust the functions of the Mobility Robustness Optimisation, e.g. by using the coverage state indicator to retrieve a previously stored Mobility Robustness Optimisation state. The coverage state indicator may also be used at the receiving NG-RAN node to adopt coverage configurations matching with neighbouring cells coverage configurations. [...]

Namely, in legacy CCO a source gNB signals its coverage states to a target gNB to enable the target gNB to select Cells/beams coverage states that match those selected by the source gNB. 
It needs to be highlighted that the target gNB can select matching coverage states to those of the source gNB because

- It is informed of the coverage states selected by the source gNB and

- It can collect UE measurements for the source gNB affected cells and beams, hence determining what are the new cell and beam shapes adopted by the source gNB.

Obviously, the same functionality is needed in AI/ML supported CCO. However, so far, the solution captured for AI/ML supported CCO does not enable the target gNB to determine coverage states that match the future coverage states selected by the source gNB. This is because the target gNB cannot collect UE measurements revealing the cell/beam shape changes selected by the source gNB (as they happen in the future) and because the target gNB-DU does not receive any information about the future coverage states selected by the source gNB.

Without addressing this issue the AI/ML supported CCO feature would not function, hence it is important to address this problem in order to achieve support for AI/ML supported CCO.

Proposal 1: RAN3 to agree that the AI/ML supported CCO feature should mirror the legacy CCO feature in enabling a gNB to adopt coverage configurations matching with neighbouring cells coverage configurations.

## Adapt to a future CCO state at neighbour gNB

According to the agreed TP for F1AP, the CU can send to the DU a Predicted CCO Assistance Information IE to indicate “predicted CCO Assistance Information for cells and beams served by the gNB-DU of the same NG-RAN node or for cells and beams not served by the gNB-DU.”.

| Predicted CCO Assistance Information | O |  | 9.3.1.A | Indicates predicted CCO Assistance Information for cells and beams served by the gNB-DU of the same NG-RAN node or for cells and beams not served by the gNB-DU. |
| --- | --- | --- | --- | --- |

As of now, the Predicted CCO Assistance Information IE includes: the Predicted CCO issue, indicating the type of predicted CCO issue, the Predicted Affected Cells and Beams, indicating a list of NR CGIs and SSB Indexes, pointing at the cells and beams predicted to be affected, and the Time for Predicted CCO Issue, indicating the time when the predicted CCO issue will happen.

Except for the timing information, the remaining pieces of information are the same type of information that CU sends to DU for legacy CCO. The same information conveyed in Predicted CCO issue and Predicted Affected Cells and Beams is also sent via Xn from CU1 (gNB1) to CU2 (neighbour gNB2) in the Coverage Modification Cause and the Future Coverage Modification List, together with the Time for Future Coverage State. The CU2 forwards this information to DU2, but this is not enough for DU2 to generate the matching future CCO state.

Let’s now recall the open issue at hand and the options listed to resolve it:

For the receiving side, whether gNB-CU can also provide to gNB-DU a future CCO state as assistance information to derive a matching future CCO state?

Option1: no need

Opton2: gNB-CU directly forwards the received further CCO state of neighbor cells to gNB-DU.

Option3: gNB-CU generates a recommend further CCO state and send to gNB-DU.

In our view, Option 1 is not a valid option. Why? As explained above, for the case of a detected CCO issue (legacy CCO), CU2 at gNB2 does not send to DU2 the coverage states generated by gNB1 (DU1), but the solution works, because DU2 can deduce the coverage state adopted for cells and beams of gNB1, thanks to collected UE measurements and reports that reveal how a cell/beam shape changed. With this information and the configured alternative configurations (received from the OAM), DU2 is able to derive a matching CCO state for cells and beams under its control.

For a predicted CCO issue, however, DU1 has not applied yet the future CCO state, and DU2 cannot deduce it from measurements. Therefore a “do nothing” approach is not a viable solution because it does not provide any solution to the problem of enabling DU2 to select coverage states that match the future coverage states of gNB1.

Observation 1: If, at the receiving side, the CU2 does not provide to DU2 future coverage states as assistance information to derive matching future coverage states, the solution does not work because DU2 cannot deduce future coverage states matching those of gNB1.

If we consider now Option 3, we note that this does not guarantee that the future coverage states derived by the target DU is able to match the future coverage states generated by the source DU. This approach can contradict the way legacy CCO works.

There is no guarantee that the future coverage states generated by the gNB-CU2 at the receiving side will carry information concerning the future CCO state of DU1 in gNB1 or that it will match the future coverage states of gNB1. Instead, in legacy CCO, an NG-RAN node can use the coverage indicator (CCO state) received by a neighbor “to adopt coverage configurations matching with neighbouring cells coverage configurations.” (see TS 38.300). 
It is also important to note that, as per TS38.401, “the gNB-DU may also take into account information received for other cells when adopting the CCO configuration”. 
Therefore, it is part of the gNB.DU functionality to collect information about the coverage states of neighbour cells and derive the coverage state that best matches those. This implies that placing this cuntionality at the gNB-CU breaks the functional distribution established for CCO.
In RAN3 it has been continuously reiterated that the application of AI/ML to an existing feature shall not change the functional framework of the feature, hence this approach is not feasible.

Observation 2: If, at the receiving side, the CU2 generates a recommend future coverage states and sends it to DU2, the solution is not consistent with the functional distribution between CU and DU in legacy CCO, where a gNB can use the coverage states received by a neighbour to adopt coverage configurations matching with neighbouring cells coverage configurations.

In Option 2 the CU2 sends to DU2 the information it already has, i.e., the future coverage states generated by DU1 (which CU2 received from CU1). With this information, DU2 can easily generate matching future coverage states because:

- DU2 can be configured with alternative coverage configurations by OAM, according to which DU2 can select appropriate matching coverage states once it knows the coverage states of neighbour cells/beams

- DU2 can learn which own coverage states best match neighbour cells/beams coverage states and by that it can select matching coverage states.

Observation 3: Sending the future coverage states generated at the gNB where the CCO issue is predicted to a neighbour gNB allows the neighbour gNB to adjust the future coverage states of its cells and beams.

Proposal 2: In a split gNB that receives via Xn uture coverage states for a CCO issue predicted at another gNB, the CU forwards the received future coverage states to the DU, which generates matching future coverage states for its own cells and beams.

Let’s recap the steps to handle one detected CCO issue and one predicted CCO issue if Proposal 1 is agreed.

The logical steps to handle one detected CCO issue are:

- CU1 detects the CCO issue.

- To enable DU1 to generate  coverage states, CU1 sends to DU1 the CCO issue and the affected cells and beams (F1: CCO Assistance Information).

- DU1 generates the coverage states, it applies it immediately and sends the coverage states to CU1 (F1: Coverage Modification Notification).

- To enable gNB2 (DU2) to generate matching coverage states, gNB1 (CU1) sends to gNB2 (CU2) the detected CCO issue (Xn: Coverage Modification Cause) and the coverage states derived at gNB1 (Xn: Cell Coverage State, SSB Coverage State).

- CU2 sends to DU2 the detected CCO issue and the affected cells and beams, not served by DU2 (F1: CCO Assistance Information).

- DU2 generates matching coverage states to adjust the coverage of cells and beams of gNB2 to the coverage states generated by DU1, it applies it immediately and sends the matching CCO state to CU2.

The logical steps to handle one predicted CCO issue are:

- CU1 predicts the CCO issue.

- To enable DU1 to generate future coverage states, CU1 sends to DU1 the predicted CCO issue and the predicted affected cells and beams (F1: Predicted CCO Assistance Information).

- DU1 generates future coverage states and sends it to CU1 with the time when the future coverage states will be applied (F1: Future Coverage Modification Notification).

- To enable gNB2 (DU2) to generate matching future coverage states and to determine when to apply the matching future coverage states, gNB1 (CU1) sends to gNB2 (CU2) the predicted CCO issue (Xn: Predicted Coverage Modification Cause), the future coverage states at gNB1 (Xn: Future Cell Coverage State, Future SSB Coverage State), and timing information (so far, only Xn: Time for Future Coverage State).

- CU2 sends to DU2 the predicted CCO issue, the predicted affected cells and beams (not served by DU2), the future coverage states and the timing information (so far, only F1: Time for Future Coverage State).

- DU2 generates matching future coverage states that will adjust the coverage of cells and beams of gNB2 to the future coverage states generated by DU1 and sends the matching future coverage states to CU2.

From the steps above, it can be observed that, if CU2 receives a predicted CCO issue from a neighbour gNB1, there is no need for CU2 to predict a new CCO issue. The latter would, in fact, create even more problems to enable DU2 to generate matching coverage states as the new predicted CCO issue may be totally different from the CCO issue predicted by gNB1 (hence the future coverage states generated by DU2 may not match those of gNB1.

The only relevant prediction is the predicted CCO issue received from gNB1 which CU2 can send to DU2, together with the Future Coverage State at gNB1, to derive the matching future CCO state.

Observation 4: When CU2 receives a predicted CCO issue from a neighbouring gNB, CU2 does not need to predict a new CCO issue.

Proposal 3: If a CCO issue is predicted at gNB1 (CU1), CU2 sends to DU2 the same predicted CCO issue received from gNB1 (CU1).

## Timing information on future CCO state

A CU1 where a CCO issue is predicted may have more than one underlying DUs, e.g., DU1 and DU2, where DU1 controls cells and beams predicted to be affected by the predicted CCO issue, and DU2 controls cells and beams which are not predicted to be affected by the predicted CCO issue. Due to the future CCO state that will be applied at DU1, the DU2 may need to adjust the coverage of some of its cells and beams. In current specification, the CU1 can indicate to DU2 that some cells and beams not controlled by DU2 will be affected by a predicted CCO issue. However, CU1 cannot provide to DU2 the time of future CCO state (decided by DU1). Therefore, DU2 will not know when it is more appropriate to adapt the coverage of its cells and beams. This problem is not present in legacy CCO, because the new CCO state is applied immediately, but for AI/ML enabled CCO, without a timing information available at DU2, the solution does not work. We propose then that in a split gNB, a CU indicates to a DU the time of future CCO state for cells and beams not controlled by the receiving DU.

We note that the same information is also relevant when the DU2 is a DU of a gNB neighbouring the node where the CCO issue is predicted. If a gNB2 receives the time of future CCO state decided at gNB1 (already supported by current baseline), the CU2 needs to pass the information to its underlying DU2, so that DU2 can understand when it is appropriate to apply a future CCO state matching the future CCO state decided at gNB1.

Proposal 4: The CU indicates to a DU the time of future CCO state for cells and beams not controlled by the receiving DU.

# 3. Conclusions

Based on the discussion in this paper, the following observations and proposals are derived:

Proposal 1: RAN3 to agree that the AI/ML supported CCO feature should mirror the legacy CCO feature in enabling a gNB to adopt coverage configurations matching with neighbouring cells coverage configurations.

Observation 1: If, at the receiving side, the CU2 does not provide to DU2 future coverage states as assistance information to derive matching future coverage states, the solution does not work because DU2 cannot deduce future coverage states matching those of gNB1. Observation 2: If, at the receiving side, the CU2 generates a recommend future coverage states and sends it to DU2, the solution is not consistent with the functional distribution between CU and DU in legacy CCO, where a gNB can use the coverage states received by a neighbour to adopt coverage configurations matching with neighbouring cells coverage configurations.

Observation 3: Sending the future coverage states generated at the gNB where the CCO issue is predicted to a neighbour gNB allows the neighbour gNB to adjust the future coverage states of its cells and beams.

Proposal 2: In a split gNB that receives via Xn uture coverage states for a CCO issue predicted at another gNB, the CU forwards the received future coverage states to the DU, which generates matching future coverage states for its own cells and beams. Observation 4: When CU2 receives a predicted CCO issue from a neighbouring gNB, CU2 does not need to predict a new CCO issue.

Proposal 3: If a CCO issue is predicted at gNB1 (CU1), CU2 sends to DU2 the same predicted CCO issue received from gNB1 (CU1).

Proposal 4: The CU indicates to a DU the time of future CCO state for cells and beams not controlled by the receiving DU.

Proposal 5: Agree the TP to F1AP provided in R3-253423.