3GPP TSG-RAN WG3 Meeting #128	R3-253429

St Julian’s, Malta, May 19th – 23rd, 2025

Title:	AI/ML support for NR-DC

Source:	Ericsson, Jio Platforms, InterDigital

Agenda item:	11.4

Document Type:	Discussion and agreement

# Introduction

At the RAN3#126 meeting, the following agreements and “to-be-continued” were captured for AI/ML assisted mobility optimization for NR-DC in the meeting minutes:

Using the Data Collection Reporting Initiation procedure to configure the collection of UE performance at SN side.

Introduce the Data Collection ID IE into the SN Addition Request message to identify a Data Collection Reporting context.

Introduce predicted PSCell ID in the DC procedure (e.g., SN Addition, MN-initiated SN change) as one of assistance information?

Other information, e.g., predicted SCG bearer?

At RAN3#127, little progress was achieved towards any of the open points above while, at RAN3#127-bis, the following was capture in the meeting minutes:

The reporting of legacy UE performance from SN to MN is performed via the Data Collection Reporting procedure (within the DATA COLLECTION UPDATE message). The details of measurement metrics and any other additional information need to be further checked.

The following list of exit conditions to stop the collection of UE performance at SN side:

- the time since NR-DC was successfully setup with the S-NG-RAN node is equal to the value of the Collection Time Duration for UE Performance IE;

- the NR-DC with the S-NG-RAN node for the UE is released.

UE performance measurements in NR-DC based on the outcome of the UE performance discussion in split architecture?

Whether to introduce assistance information (predicted PSCell ID) from MN to SN?

Whether to enhance the UE Performance IE in the DATA COLLECTION UPDATE message to include the actual PSCell ID?

Whether additional conditions (e.g., SCG activation) for starting the collection of UE performance are needed, pending the outcome of the discussion on UE performance in split architecture?

In this contribution, we discuss the consequences of the above agreements and other aspects related to the enhancements needed in NG-RAN interfaces to support this use case.

# AI/ML support for mobility optimization for NR-DC

## General

Mobility in NR-DC can be optimized by means of AI/ML.

Mobility Optimization for NR-DC is studied by assuming inference at the MN only. The main use case is limited to Dual Connectivity only and Conditional Dual Connectivity procedures are out of scope.

The following is captured in the TR 38.743 in relation to AI/ML-based mobility optimization for NR-DC (clause 5.1.1), where we added the yellow highlight for clarity:

The text clearly states that AI/ML techniques can be used to optimize mobility in NR-DC; furthermore, any inference related to this optimization is located at the MN only. The question is thus, how can AI/ML techniques be used at the MN?

Given the text above and the conclusions captured in the TR 38.743, we would like to focus the discussion on the following types of enhancements for AI/ML-based mobility optimization for NR-DC:

- Enhancements related to AI/ML-based decisions to select dual connectivity for improved performance

- Enhancements related to data collection to assist the AI/ML-based decisions at the MN

We discuss both types of enhancements below.

## Enhancements for configuring dual connectivity

It is reasonable to think that the MN has inferred the action of SN Addition (or SN Change) assuming that adding (or changing) the SN would bring some benefits to the UE performance. We think that it is good if the MN informs the SN about some of the assumptions made, so that the SN can try to provide the best possible service to the UE. We would like to emphasize that a good implementation would not incur in the cost of using NR-DC if this is not benefitial to the UE in some way.

Observation 1:  An MN infers the action to add or change SN based on an assumption that this action will bring some benefits to the UE performance.

The benefits to UE performance brought by dual connectivity mainly stem from the more network resources available to the UE. However, these benefits can only be realized if the additional network resources from the SN are used. In other words, the SN needs to serve some part of the UE traffic for NR-DC to improve the UE performance. If the SN does not serve any traffic, there would be no reason to configure the UE in dual connectivity.

Observation 2:  An MN infers the action to add or change SN based on an assumption that this action will bring some benefits to the UE performance due to the (new) SN serving a certain amount of UE traffic.

In response to the above question of how AI/ML techniques can be applied to this use case, one possible answer is that these techniques can be leveraged to predict UE performance under various configurations. Consequently, dual connectivity should be used when it is expected to improve UE performance. Some companies claim that signaling (to the SN) the PSCell assumed by the MN is enough for mobility optimization in NR-DC. However, as previously noted, the effectiveness of moving a UE to dual connectivity is questionable if the SN does not serve any traffic. If the AI/ML based NR-DC action assumed a certain amount of traffic in the SN, and this is not provided, the final UE performance may be lower than expected. Therefore, if the SN does not know from the MN what traffic it should serve (according to the MN predictions made via AI/ML), the chosen DC configuration might not result in improved UE performance.

Observation 3:  Merely signaling the PSCell that the MN assumes to be used at the SN does not provide sufficient information for the SN to select the best DC configuration that will indeed enhance UE performance.

The argument to only signal the PSCell seems to follow the work in mobility optimization for single connectivity done during Rel-18, which resulted in the introduction of predicted UE trajectory in the handover procedure. The key difference is that, unlike single connectivity where data traffic terminates at a single NG-RAN node, in dual connectivity traffic is distributed between the MN and the SN in a flexible way. Given that the UE maintains a connection with the MN, it is unclear how much UE traffic should be served by the SN once the AI/ML predicted DC configuration for the UE is activated. Therefore, we see a need that both the assumed PSCell and the UE traffic that needs to be served by the SN are made available to the SN as assistance information.

Observation 4:  The assumptions made by the MN when deciding to configure the UE in dual connectivity (i.e., the assumed PSCell and the UE traffic that needs to be served by the SN) need to be available at the SN for the SN to select the best dual connectivity configuration that will enhance UE performance.

For example, when an MN carries out an AI/ML-based SN Addition, it may request the establishment of SN-terminated bearers from the SN. This decision can be based on the assumption that certain QoS flows are served by the SN (as SCG). In the current framework, it is not possible to communicate this assumption. After deriving an AI/ML-based request for resources for SN-terminated bearers, the MN should be able to inform the SN that this request assumes that the SN will handle some or all the QoS flows as SCGs. The SN is still left free to decide whether to serve the QoS flows as either MCG or SCG, but it may now align its decisions with the MN's assumptions, if feasible.

Therefore, we propose that, during SN Addition, the MN could inform the SN that it assumed that a specific cell is selected as PSCell. Together with this, we propose that the MN should inform the SN of its assumptions regarding which QoS flows the SN is expected to serve as SCG. It is crucial to emphasize that both pieces of information—the assumed PSCell and the expected traffic in terms of QoS flows within the SCG—are vital and should be considered together. This combined assistance information can enable more informed and effective decision-making at the SN.

Proposal 1:  Enhance dual connectivity procedures (e.g., SN addition, MN-initiated SN change) to indicate MN assumptions to be considered by the SN, namely the cell the MN assumed as PSCell, the QoS flows the MN assumed to be served as SCGs.

## Enhancements for data collection in dual connectivity

To assist inference at the MN, it is possible to mostly reuse the Rel-18 signaling design established to support AI/ML-assisted use cases. As already agreed in the RAN3#126 and RAN3#127-bis meetings, we can reuse the Data Collection Reporting Initiation procedure and the Data Collection Reporting procedure to allow an MN to request and obtain from a neighbor gNB (acting as SN) the reporting of UE performance.

At the RAN3#127-bis meeting, two additional exit conditions for the collection of UE performance feedback for NR-DC were agreed: (1) the expiration of a timer after NR-DC was successfully setup and (2) the NR-DC with the S-NG-RAN node for the UE is released. The related following open issue was noted:

Whether additional conditions (e.g., SCG activation) for starting the collection of UE performance are needed, pending the outcome of the discussion on UE performance in split architecture?

There is a growing understanding in RAN3 that, for the collection of UE performance in split architecture, the UE performance metrics should be obtained at PDCP level. For consistency, and as hinted in the open issue above, the same should apply to NR-DC, i.e., the UE performance should be collected at PDCP level. Consequently, SCG activation cannot be a condition for starting the collection of UE performance. For example, an SN-terminated MCG bearer would see data flowing through the PDCP entity at the SN even if the SCGs are not active. If “SCG activation” is considered as a starting condition for collection of UE performance, it will not be possible to obtain consistent measurements at PDCP level.

Proposal 2:  For AI/ML-assisted mobility optimization for NR-DC, “SCG activation” should not be considered as a starting condition for collection of UE performance.

Another open issue noted at the last meeting is the following:

Whether to enhance the UE Performance IE in the DATA COLLECTION UPDATE message to include the actual PSCell ID?

In our view, including the PSCell ID in the reported UE performance feedback is a necessary condition for the MN to understand the received information as we explain below.

In the Rel-18 solution, it is specified that the target gNB should stop collecting UE performance when the “UE is handed over to another cell”. UE performance is currently reported from target gNB to source gNB without an explicit indication of the cell they refer to. This is good enough for the Handover case, since the target cell is explicitly signalled in the HANDOVER REQUEST message, and the target gNB knows exactly for which cell it should report the UE performance. Also, since the data collection should stop when the UE is handed over to another cell, there is only one cell the UE performance can refer to. This is not the case in NR-DC.

In NR-DC, the SN has the capability to change the PSCell without notifying the MN. While some companies suggest that the MN can deduce a PSCell change by analyzing received RRC messages, relying on this method is not a good engineering practice. Developing a process that requires decoding RRC messages for the sole purpose of detecting PSCell changes can lead to inefficiencies and increased complexity. Instead, we should strive for a self-contained approach, e.g., by including the PSCell ID in the reported UE performance.

Observation 5:  The MN is not readily aware of the PSCell ID. Forcing the MN to analyze received RRC messages to deduce a PSCell change is not a good engineering practice.

Furthermore, at the RAN3#127-bis meeting, the agreed exit conditions for collecting UE performance feedback do not include an equivalent to the legacy "UE is handed over to another cell," such as a "PSCell change" condition. As a result, UE performance data might be gathered across multiple PSCells of the added SN. In this case, it is essential to include the PSCell ID in the UE Performance IE, which would enable the SN to report the UE performance separately for each PSCell. This can be achieved by including several UE Associated Info Result Item IE for the same UE but with different UE Performance IE, one per PSCell the UE was connected to. This would enable the MN to easily understand that the data was collected from different cells, thus improving the interpretability of the data.

Observation 6:  UE performance data might be gathered across multiple PSCells in NR-DC. Including the PSCell ID in the UE Performance IE would enable the SN to report the UE performance separately for each PSCell.

Proposal 3:  For AI/ML-assisted mobility optimization for NR-DC, extend the UE Performance IE to include the PSCell serving the UE in the SN.

# Conclusions

Based on the discussion in this paper, we derived the following observations and proposals:

Observation 1:  An MN infers the action to add or change SN based on an assumption that this action will bring some benefits to the UE performance.

Observation 2:  An MN infers the action to add or change SN based on an assumption that this action will bring some benefits to the UE performance due to the (new) SN serving a certain amount of UE traffic.

Observation 3:  Merely signaling the PSCell that the MN assumes to be used at the SN does not provide sufficient information for the SN to select the best DC configuration that will indeed enhance UE performance.

Observation 4:  The assumptions made by the MN when deciding to configure the UE in dual connectivity (i.e., the assumed PSCell and the UE traffic that needs to be served by the SN) need to be available at the SN for the SN to select the best dual connectivity configuration that will enhance UE performance.

Proposal 1:  Enhance dual connectivity procedures (e.g., SN addition, MN-initiated SN change) to indicate MN assumptions to be considered by the SN, namely the cell the MN assumed as PSCell, the QoS flows the MN assumed to be served as SCGs.

Proposal 2:  For AI/ML-assisted mobility optimization for NR-DC, “SCG activation” should not be considered as a starting condition for collection of UE performance.

Observation 5:  The MN is not readily aware of the PSCell ID. Forcing the MN to analyze received RRC messages to deduce a PSCell change is not a good engineering practice.

Observation 6:  UE performance data might be gathered across multiple PSCells in NR-DC. Including the PSCell ID in the UE Performance IE would enable the SN to report the UE performance separately for each PSCell.

Proposal 3:  For AI/ML-assisted mobility optimization for NR-DC, extend the UE Performance IE to include the PSCell serving the UE in the SN.

A TP mirroring the above proposals is presented in R3-25xxxx.