3GPP TSG-RAN WG3 #128                                                    R3-253440

Malta, MT, 19th – 23rd May, 2025

Agenda Item:	10.2

Source:	CATT

Title:	(TP for 38.423 and 38.300) MRO for CHO with Candidate SCG(s) and S-CPAC

Document for:	other

Introduction

In last RAN3 meeting, the following agreements are captured:

MRO for CHO with Candidate SCG(s)

No additional information is needed in handover report message.

Check stage 3 TP on the Target cell CGI related in handover report message.

One SCGFailureInfomation is included in SCG Failure Indication message, no need aggregation.

Respective MN in stage2 is at least a candidate MN.

Information needed in the SCG Failure Indication message: The Xn UE AP ID in the receiving node.

Additional information reported from the UE in SCGFailureInformation: The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the SCG failure is encountered.

No support of near failure case in R19.

Whether Handover Report message is needed for case 7a?

In addition to SCGFailureInformation, information needed in the SCG Failure Indication message?

- CPC failure type (CPC failure type is needed in case of SCG Failure Indication is sent from the candidate to the source?)

- Suitable PCell ID

MRO for S-CPAC：

The description on S-CPAC information forwarding is technically correct. Cleanup of the text is needed.

Whether to enhance UHI in case of S-CPAC? The motivation and benefits need to be clarified.

Based on above agreements, here we provide some analysis on MRO for R18 mobility mechanisms.

Discussion

  - MRO for CHO with candidate SCGs

    - Analysis for MCG failure case

In last RAN3 meeting, for case 7a, whether handover report message is needed was discussed and no consensus was achieved:

whether Handover Report message is needed for case 7a

For case 7, the description is captured below:

| Case 7 |  | Both CHO and CPAC execution conditions are met. 7a) CHO execution to a candidate PCell failure. 7b) CPAC to a candidate PSCell failure. 7c) Execution to SCG failure and toMCG failure. |
| --- | --- | --- |

For case 7a, only CHO failure is detected, so only source MN is required to perform optimization. In this case, handover report message is not needed to candidate MN for CPC optimization. If the optimization on CHO impacts the CHO with candidate SCG execution, the optimization on CPC or decoupled CHO with candidate SCG can be done by next failure happens.

Proposal 1: It is proposed to not send handover report message from source MN to candidate MN for case 7a.

    - Analysis for SCG failure case

For SCG failure case, a new SCG Failure Indication message has been introduced between MNs to transfer SCGFailureInfomation for a UE.

The candidate MN should perform CPC optimization when receiving SCG Failure Indication message.

In candidate MN, the CPC optimization should be performed based on the PCell.

The following IE in HANDOVER REQUEST ACKNOWLEDGE message can reflect that the configuration on PSCell should base on PCell as a pair.

| Conditional Handover Information Acknowledge | O |  |  |  |
| --- | --- | --- | --- | --- |
| >Requested Target Cell ID | M | Target Cell Global ID 9.2.3.25 | Target cell indicated in the corresponding HANDOVER REQUEST message | PCell |
| >Maximum Number of CHO Preparations | O | 9.2.3.101 |  |  |
| >CHO-CPAC Information | O | 9.2.3.202 |  | Optimization on CPC is based on PCell. |

So, the suitable PCell should be included in SCG Failure Indication message for the same reason as the suitable PCell is included in handover report

Proposal 2: Source MN sends suitable PCell to candidate MN in SCG Failure Indication message.

In addition to SCGFailureInformation, information needed in the new message?

- CPC failure type

CPC failure type is detected based on the next suitable PSCell and MRO configuration, for example, Tstore_UE_cntxt. Since source MN and candidate MN may have different configuration, if candidate MN detect failure type e.g., too late, different failure type may be detected in source MN e.g., to wrong cell. So, CPC failure typeneeds to be provided from source MN to candidate MN.

For too late CPC, the source MN as the last serving MN should perform initial analysis, and further send failure type to target or candidate MN.

Proposal 3: It is proposed for source MN to send CPC failure type to candidate MN.

After receiving SCG Failure Indication message from source MN, candidate MN performs analysis and may ask candidate SN to make optimization. Legacy SCG FAILURE INFORMATION REPORT message could be reused, but stage 2 text may be updated to include SN node.

In case there is an SCG failure when UE is configured with CHO with candidate SCG(s), the serving M-NG-RAN node receiving SCGFailureInformation from the UE performs initial analysis. If the SCG failure has been caused by a different NG-RAN node, the serving M-NG-RAN node transfers the SCGFailureInformation to that NG-RAN node.

Proposal 4: The “M-NG-RAN node” should be changed to “NG-RAN node” in stage 2 to include both candidate MN and SN.

    - SCG failure occurs shortly after successful CHO with candidate SCGs

When SCG failure occurs shortly after successful CHO with candidate SCGs, the target MN would receive SCG failure information message from UE.

The related agreement and FFS is captured below:

Respective MN in stage 2 is at least a candidate MN.

Check whether respective MN is only a candidate?

For CHO with candidate SCGs, after receiving SCG failure information message, the target MN would perform MRO analysis. If the failure is caused by other MNs, a message should be sent to source MN.

After receiving message from target MN, source MN may also inform other candidate MN to perform optimization on candidate PSCell list selection.

Note that the candidate MN may be not the target MN and target MN may not have Xn interface with candidate MN. So, the MRO procedure may be target MN->source MN->candidate MN.

Proposal 5: For SCG failure occurs shortly after successful CHO with candidate SCGs, the MRO procedure may be target MN->source MN->candidate MN. The respective MN also can be source MN.

For the message from target MN to source MN, new SCG Failure Indication message should also be used. And as discussed in above clause, CPC failure type should be included in SCG Failure Indication message. For the case of SCG failure occurs shortly after successful CHO with candidate SCGs, target MN as the last serving node perform MRO analysis, CPC failure type should also been included for the case.

Proposal 6: It is proposed for target MN to send CPC failure type to source MN via SCG Failure Indication message.

Proposal 7: It is proposed to discuss TP for TS38.300 /TS38.423 in annex.

  - MRO for Subsequent CPAC

UHI

There is an issue raised in RAN3 meeting as below:

Whether to enhance UHI in case of S-CPAC? The motivation and benefits need to be clarified.

Support UHI and SCG UE History Information transmission during S-CPAC procedure to avoid the ping-pong or unnecessary PSCell change

To solve the issue on ping-pong or unnecessary PSCell change, there are mainly two steps:

The first step is how to record UHI and detect ping-pong occurrence.

The second step is to optimize configuration to solve the issue.

For the first step, UHI could be updated by MN during S-CPAC procedure. The related text in TS37.340 is as below:

The UE starts evaluating the execution conditions. If the execution conditionof one candidate PSCell is satisfied, the UE applies RRCReconfiguration* message corresponding to the selected candidate PSCell, and sends an MN RRCReconfigurationComplete* message, including an RRCReconfigurationComplete** messagefor the selected candidate PSCell, and information enabling the MN to identify the SN of the selected candidate PSCell.

Since MN is aware of the each PSCell selected during S-CPAC procedure according to above text, MN could correlate PCell and PSCell, i.e. update the correlated UHI which includes PCell and PSCell information.

After the analysis of UHI, MN can detect ping-pong issue occurs during S-CPAC procedure. To solve the issue, i.e. the second step, there may be the following solutions:

Solution 1: MN sends UHI to every candidate SN each time UHI is updated.

Solution 2: MN sends UHI to every candidate SN when ping-pong issue is detected by MN.

Solution 3: MN sends UHI to the candidate SN which ping-pong issue is related to when detecting ping-pong issue by MN.

Solution 4: It is not needed to solve ping-pong issue immediately, i.e. the next time configuring S-CPAC, ping-pong issue will be solved according to the previous collected UHI.

Considering above solutions, we think not every candidate SN should be informed because they may be not involved in ping-pong issue, and it is also not needed to inform SN each time UHI is updated because ping-pong issue may not exist. So, solution 1 and 2 is not preferred.

For solution 3, after detecting ping-pong issue, MN informs the related SN about the updated UHI. The candidate cell add/release may be optimized at once which may lead to many interactions between every candidate cells because the change on each candidate cell should be informed to every other candidate cells in S-CPAC procedure.

For solution 4, considering the complexity to perform the modification of S-CPAC configuration, UHI can be used for the next S-CPAC, i.e. not perform optimization for current S-CPAC. MN could save the UHI and make optimization for the next S-CPAC.

First, MN could include updated UHI in SN ADDITION REQUEST to candidate SN when UE connect this problem SN again and it will perform optimization to address ping-pong issue.

Second, SN MODIFICATION REQUEST message includes UE History Information from the UE IE as below. MN can send it to source SN to address ping-pong issue.

VisitedCellInfo-r16 ::=  SEQUENCE {

visitedCellId-r16        CHOICE {

nr-CellId-r16            CHOICE {

cgi-Info                 CGI-Info-Logging-r16,

pci-arfcn-r16            PCI-ARFCN-NR-r16

},

eutra-CellId-r16         CHOICE {

cellGlobalId-r16         CGI-InfoEUTRA,

pci-arfcn-r16                PCI-ARFCN-EUTRA-r16

}

}                                        OPTIONAL,

timeSpent-r16            INTEGER (0..4095),

...,

[[

visitedPSCellInfoListReport-r17    VisitedPSCellInfoList-r17                   OPTIONAL

]]

}

In this way, both initiating SN and candidate SN could be informed to solve ping-pong issue with current specification. Comparing with solution 3, solution 4 has no specification impact. So, solution 4 is preferred.

Proposal 8: The current specification can solve the ping-pong issue for the follow reason:

- UHI can be send from MN to SN when UE connect to the problem SN again via SN addition request.

- SN MODIFICATION REQUEST message includes UE History Information from the UE which can also be used to solve ping-pong issue.

Information in SCGfailureInformation message

There is a possible case, for the initiating node, it may be not selected as candidate cell and cannot save UE context when UE is not connected to this node. So, it is hard for network to save PSCell information in initiating node in case of SN initiating S-CPAC case.

The initiating node, specifically, initiating PSCell may be required to perform MRO e.g., candidate PSCell list selection. Therefore, SCGfailureInformation may be enhanced to include source initiating PSCell ID.

Proposal 9: It is proposed to include initiating PSCell ID in SCGfailureInformation message.

If above case is available, LS to RAN2 may be needed. The LS is attached in annex.

Proposal 10: It is proposed for RAN3 to discuss the LS to RAN2 in annex.

Conclusions

MRO for CHO with candidate SCGs:

Proposal 1: It is proposed to not send handover report message from source MN to candidate MN for case 7a.

Proposal 2: Source MN sends suitable PCell to candidate MN in SCG Failure Indication message.

Proposal 3: It is proposed for source MN to send CPC failure type to candidate MN.

Proposal 4: The “M-NG-RAN node” should be changed to “NG-RAN node” in stage 2 to include both candidate MN and SN.

Proposal 4: The “M-NG-RAN node” should be changed to “NG-RAN node” in stage 2 to include both candidate MN and SN.

Proposal 5: For SCG failure occurs shortly after successful CHO with candidate SCGs, the MRO procedure may be target MN->source MN->candidate MN. The respective MN also can be source MN.

Proposal 6: It is proposed for target MN to send CPC failure type to source MN via SCG Failure Indication message.

Proposal 7: It is proposed to discuss TP for TS38.300 /TS38.423 in annex.

MRO for Subsequent CPAC:

Proposal 8: The current specification can solve the ping-pong issue for the follow reason:

- UHI can be send from MN to SN when UE connect to the problem SN again via SN addition request.

- SN MODIFICATION REQUEST message includes UE History Information from the UE which can also be used to solve ping-pong issue.

Proposal 9: It is proposed to include initiating PSCell ID in SCGfailureInformation message.

Proposal 10: It is proposed for RAN3 to discuss the LS to RAN2 in annex.

Reference

[1]

TP for TS38.423 on CHO with candidate SCG(s)

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

- 8.4.x	SCG Failure Indication

- 8.4.x.1	General

The purpose of the SCG Failure Indication procedure is to transfer SCG failure related information between NG-RAN nodes.

The procedure uses non UE-associated signalling.

- 8.4.x.2	Successful Operation

Figure 8.4.x.2-1: SCG Failure Indication, successful operation

NG-RAN node1 initiates the procedure by sending the SCG FAILURE INDICATION message to NG-RAN node2. When receiving the message NG-RAN node2 shall assume that a SCG failure related problem was detected.

If the Handover Report Type IE is set to "Too Late CPC", “Too Early CPC”,”CPCExecution to Wrong Cell”,NG-RAN node1 indicates to NG-RAN node2the target PCell and PSCell, NG-RAN node2makes MRO optimization according to TS 38.300 [9]. The detection of Too Late CPCevent,Too Early CPCenent and CPCExecution to Wrong Cellare made according to TS 38.300 [9].

If received, NG-RAN node2 uses the above information according to TS 38.300 [9].

- 8.4.x.3	Unsuccessful Operation

Not applicable.

- 8.4.x.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<<Next Change >>>>>>>>>>>>>>>>>>>>

- 9.1.3.xx	SCG Failure Indication

This message is sent by NG-RAN node1 to NG-RAN node2 to report a SCG failure event that occurred while CHO with candidate SCG was configured.

Direction: NG-RAN node 1NG-RAN node 2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the candidate target NG-RAN node |  |  |
| SCG Failure Report Container | M |  | OCTET STRING | Contains the SCGFailureInformationmessage as defined in TS 38.331 [10] | YES | ignore |
| Handover Report Type | M |  | ENUMERATED (Too Late CPC, Too Early CPC, CPC Execution to Wrong Cell…) |  | YES | ignore |
| Target PCellCGI | M |  | Global NG-RAN Cell Identity 9.2.2.27 | The suitable PCell | YES | ignore |

<<<<<<<<<<<<<<<<<<End of the changes >>>>>>>>>>>>>>>>>>>>>>>

TP for TS 38.300 on CHO with candidate SCG(s)

- 15.5.2.6	PSCell addition/change failure

For analysis of PSCell addition/change failures, the UE makes the SCG Failure Informationavailable to the MN. If the MN can perform an initial analysis, it transfers the SCG Failure Information together with the analysis results to the relevant SN as defined in TS 37.340 [21].

In case there is an SCG failure when UE is configured with CHO with candidate SCG(s), the serving M-NG-RAN node receiving SCGFailureInformation from the UE performs initial analysis. If the SCG failure has been caused by a different NG-RAN node, the serving M-NG-RAN node transfers the SCGFailureInformation to that NG-RAN node. In case of SCG failure occurs shortly after successful CHO with candidate SCGs, serving M-NG-RAN node may transfer SCGFailureInformation to candidate M-NG-RAN node via source M-NG-RAN node.

Draft LS

Title:	[Draft] LS on SON for Subsequent CPAC

Response to:

Release:	Rel-19

Work Item:	NR_ENDC_SON_MDT_Ph4-Core

Source:	RAN3

To:	RAN2

Cc:

Contact Person:

Name:	ziqiaoliu

E-mail Address:	liuziqiao@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

There is a possible case the initiating node may be not selected as candidate cell when configuring Subsequent CPAC. In this case, the initiating SN cannot save UE context after UE successfully changes to other SNs. So, the initiating PSCell ID in the initiating SN cannot be saved in network which is necessary for MRO for Subsequent CPAC.

RAN3 have acknowledged the issue respectfully asks RAN2 to consider defining a UE-based solution for this.

2. Actions:

To RAN2:

ACTION: 	RAN3 kindly asks RAN2 to take the above issue into consideration.

3. Date of Next TSG-RAN3 Meetings:

3GPP TSG RAN WG3#129     25 – 29 Aug 2025				India (TBC), IN