3GPP TSG RAN WG3 Meeting #128	R3-253460

St Julian, Malta, 19 - 23 May 2025

Title:	Reply LS on signalling feasibility of dataset and parameter sharing

Response to:	R3-253010 (R2-2503169)

Release:	Release 19

Work Item:	NR_AIML_air-Core

Source:	RAN3

To:	RAN1, RAN2, SA2, SA5

Cc:	SA3

Contact person:	Angelo Centonza

Angelo.centonza@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# Overall description

RAN3 would like to thank RAN2 for their LS on signalling feasibility of dataset and parameter sharing.

In their LS RAN2 described the following approach for non-OTA transferring of dataset/model parameters:

Alternative 1 (non-OTA approach): gNB -> NW dataset/model parameters collection entity -> UE training entity (a server inside MNO or an OTT server) NOTE: The data transfer (up to RAN1 further details on what to transfer) between gNB and NW dataset/model parameters collection entity (OAM/CN) in Alternative 1, if needed, is up to RAN3/SA2/SA5.

RAN2 went on to describe a number of candidate solutions as follows:

Table 1. non-OTA candidate solutions

| Option | Impacted WG | Specification impact/Implementation impact |
| --- | --- | --- |
| OAM -> UE-side training entity (a server inside MNO or an OTT server), where OAM is NW-side dataset/model parameter collection entity | SA5, SA3 | Up to SA5 (any intermediate node between OAM and UE-side OTT server is up to SA5; CN involvement if needed is up to SA2/SA5 discussion) |
| CN -> UE-side training entity (a server inside MNO or an OTT server), where CN is NW-side dataset/model parameter collection entity | SA2, SA3 | Up to SA2 (any intermediate node between CN and UE-side OTT server is up to SA2) |
| gNB -> OAM/CN -> UE-side training entity (a server inside MNO or an OTT server), where gNB is NW-side dataset/model parameter collection entity | RAN3, SA2, SA5, SA3 | Up to RAN3, SA2, SA5 (any intermediate node between gNB/OAM, OAM/UE-side OTT server, CN/UE-side OTT server is up to RAN3/SA2/SA5) |

RAN3 would like to clarify that as shown in the Alternative 1 figure, the gNB is always the entity that starts transferring of the dataset/model parameters.

Therefore, for the candidate solution 1 the transferring follows the order:

gNB -> OAM -> UE-side training entity;

while for the candidate solution 2 the transferring follows the order:

gNB -> CN -> UE-side training entity.

With the above assumption, RAN3 would like to clarify that candidate solution 3 in table 1 is not evaluated, since it is seen as a combination of candidate solutions 1 and candidate solution 2.

Concerning Candidate Solution 1:

RAN3 would like to clarify that TS38.300 specifies the following:

“AI/ML algorithms and models are out of 3GPP scope, and the details of model performance feedback are also out of 3GPP scope.”

Therefore, transferring of model parameters shall not be enabled, as it would disclose parts of the model implementation, which is deemed out of 3GPP scope.

Furthermore, from a RAN3 point of view, it would be feasible to enable transferring of dataset parameters from the RAN to the OAM as part of current management level performance measurement collection procedures. Such procedures are defined in the SA5 specifications nd it is up to SA5 to ensure that support for dataset/model parameters transfer is enabled.

Concerning Candidate Solution 2:

RAN3 would like to state that there are currently no procedures over the NGAP interface between the NG-RAN and the 5GC to transfer AI/ML associated information and neither there are discussions in Rel19 to enable such procedures. Therefore, the candidate solution is not feasible for Rel19.

# Action

To RAN2, SA2, SA5:

Action: RAN3 respectfully asks RAN2/SA2/SA5 to take the above feedback into account.

# Dates of the next TSG RAN WG3 meetings

TSG-RAN3 Meeting #129			August 25-28, 2025			Bangalore, India

TSG-RAN3 Meeting #129-bis		October 13-17, 2025			Prague, Czech Republic