3GPP TSG-RAN WG3 Meeting #128	R3-253469

Malta, MT, May 19th – 23rd, 2025

Agenda Item:	13.2

Source:	CATT

Title:	(TP for 37.340) Discussion for Inter-CU LTM in DC

Document for:	Decision

- Introduction

In this paper, we will analyse the potential issues in inter-CU LTM.

- Discussion

2.1	Inter SCG LTM without MN change

2.1.1 SN CHANGE REQUIRED PROCEDURE

This message is use for source SN to trigger SN initial inter-SCG LTM and reflect as step 1 in 37.340.

Include the following information in SN Change Required (Source SN to MN) message:

- LTM request indication (explicit way)

- Suggested PScell list (implicit way in INM, FFS on explicit way)

- Measurement results (implicit way in INM)

- Upper limit number of PScell(s) for each candidate SN (FFS)

- CSI Resource configuration request (explicit way)

- Early sync information request (explicit way)

In SN initial inter-SN S-CPAC, the MN shall inform SN the max number candidate Pscell can be configured based on inter node message(maxNumberCPCCandidates) and then SN can decide the upper limit of PSCell is provided for each candidate SN. However, in LTM case, intra-SCG LTM, inter-SCG LTM and intra-MCG LTM can be configured at the same time, and the maxnumber of candidate cell can be configured to UE is still limited as 8. Considering these three types of coexistence, the S-SN cannot properly decide the upper limit of PSCells due to its lack of knowledge regarding how many candidate PCells the MN has already configured or is preparing to configure.

In REL-18 LTM, this cordinaration controled by MN and introduce a maxNumberLTM-CandidatesSCG IE in inter node message, the direction is from MN to SN, but this is only can be used for intra-SCG LTM.

maxNumberLTM-CandidatesSCG

Indicates the maximum number of LTM candidate configurations that the SN is allowed to configure. If the field is absent the SN is not allowed to configure LTM candidate configurations. This field is only used in NR-DC.

There are two possible way:

- Source SN decide the Upper limit number of PScell(s) for each candidate SN.

It requires MN to indicate source SN the maximum number of inter-SCG LTM candidate cells that can be configured. The maxNumberLTM-CandidatesSCG may can be reused and need the confirm from RAN2, i.e., enhance this IE means how many SCG LTM(both intra SN and inter SN) can be configured by SN.

- MN decided the Upper limit number of PScell(s) for each candidate SN by implemention.

The S-SN sends a recommended PSCell list to the MN, and the MN checks whether the number of recommend PSCell is exceed the max number can be configured to the UE.For example, if the SN requests 6 candidate PSCells in the recommeded PSCell list , but the MN already support 3 intra-MCG candidate PCells, the MN needs to adjust the upper limit number to 5 and include it in the SN ADDITION REQUEST to the candidate SN(assuming only one candidate SN).

Proposal 1: Discuss how to decide the Upper limit number of PScell(s) for each candidate SN

- Source SN decided upper limit of PSCells of each candidate PScell based on indication(e.g., extended maxNumberLTM-CandidatesSCG) from MN.

- MN to decided the upper limit of PSCells of each candidate PScell by implemention.

As for the FFS on explicit way transfer Suggested PScell list, consider the request for candidate cell relative to UE specific information (early sync request, CSI resource requet), the suggested PSCell list shall be include in the SN addition request message explicitly.

Proposal 2: Include the suggested PSCell list in the SN addition request message explicitly.

2.1.2 SN ADDITION REQUEST/RESPONSE

This message is used for MN request the candidate SN to perform admission control for LTM candidate PSCell and providing corresponding candidate PSCell configuration. It is reflected in steps 2 and 3 of the BLCR procedure in 3GPP TS 37.340.

Include the following information in SN Addition Request (MN to Candidate SN) message:

- LTM request indication (explicit way)

- Suggested PScell list (implicit way in INM, FFS on explicit way)

- Measurement results (implicit way in INM)

- Upper limit number of PScell(s) for the candidate SN (explicit way)

- CSI Resource configuration request (explicit way)

- Early sync information request (explicit way)

Additionally, since the request is based on a per-cell bias, the Suggested PScell list should also be included in the SN addition request message.

Proposal 3: Include the suggested PSCell list in the SN ADDITION REQUEST message explicitly.

Moreover, the security-related information should also be included in the SN addition request message.

In RAN2, the following agreements have been reached:

- NW configures the corresponding sk-Counter in all LTM candidate configurations, and UE uses the configured value for generating the SN key when security key update is performed in MCG.

- A list of sk-counters is linked to a Rel-19 set ID configured by the SN.

From the UE perspective, it shall receive the sk-Counter list for each candidate SN in LTM preparation phrase. Once LTM occurred, UE shall derive the S-KSN based on the first unused sk-Counter in the list and KNG-RAN of MN.

From the network perspective, there are two possible solutions:

- S-CPAC like solution:

- MN shall send the { S-KSN,sk-Counter } list to each candidate SN via SN ADDITION REQUEST message, and SN shall use the first unused S-KSN for when UE access.

- Once UE send RRCReconfigurationcomplete to the MN, MN shall include the, sk-Counter in S-NODE RECONFIGURATION COMPLETE message,

If go for S-CPAC like solution, a new LTM Security Configurations List IE shall be included in the SN ADDITION REQUEST message and SN MODIFICATION REQUEST message.

- Legacy network based HO solution:

- MN shall send the S-KSN to the each candidate SN,

- Once UE switch from SN1 to SN2, the MN shall resend a new S-KSN to the SN1 for subsequent inter SCG LTM. the MN shall guarantee the S-KSN used in SN and derived by UE should be the same one.

If go for legacy network based HO solution, the legacy S-NG-RAN node Security Key IE can be reused and no stage 3 impact.

Proposal 4: discussion how to relalize SN key handing:

- Alt1:MN shall send the { S-KSN,sk-Counter } list to each candidate SN

- Alt2:MN shall send the S-KSN to the each candidate SN

    - SN MODIFICATION REQUEST/RESPONSE

From MN to source SN:

This procedure is use for MN provide feedback or collected information to the source SN, which is reflected in step 5.6 in BLCR for 37.340.

When examining the S-CPAC case, we noticed it also support MN to request reference configuration from source SN. S-CPAC Reference Configuration Request IE is already included in the SN MODIFICATION REQUEST message. Thus, it is propose to allowed MN to request reference configuration in LTM

| >S-CPAC Reference Configuration Request | O |  | ENUMERATED (request, …) | Indicates that the reference configuration for S-CPAC is requested. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

From MN to candidate SN:

This procedure is use for MN provide collected information to candidate SN, which reflect in step 7.8 in BLCR for 37.340.

As the candidate SN may provide the delta configuration or fully configuration, a Full Configuration indicator IE for LTM shall be include in SN MODIFICATION RESPONSE message.

Proposal 5: Introduce a LTM reference configuration request IE in SN MODIFICATION REQUEST message.

Proposal 6: Introduce a LTM full configuration IE in SN MODIFICATION RESPONSE message

    - SN MODIFICATION REQUEST/RESPONSE

SP-CSI RS can also be supported in the DC scenario, and the CSI-RS COORDINATION REQUEST/RESPONSE messages can be reused for the DC case. The source SN should send the CSI-RS COORDINATION REQUEST message to the MN, which will then forward it to the candidate SN.

Proposal 7: CSI-RS COORDINATION REQUEST message to the target SN including the CSI-RS resource to be activated/deactivated of the candidate cell(s) via MN .

2.1.5	 ACCESS SUCCESS

For the information after completion of SN initiated inter-CU SCG LTM, there are three options:

- Option 1: Reuse the SN MODIFICATION REQUEST message to inform the source SN that UE successfully accessed the target PSCell.

- Option 2: Reuse the HANDOVER SUCCESS message to inform the source SN the UE successfully accessed the target PSCell.

- Option 3: No need to inform the access success information to the source SN.

The key issue is whether need and how to inform S-SN that the UE access to T-SN successfully. In S-CPAC case, the MN will send a SN MODIFICATION REQUEST message to S-SN after receive the RRCReconfigurationcomplete from UE, the main purpose is to initial late data forwarding and update the data forwarding address based on the proposed mapping. However, in LTM, as normal data forwarding is already supported by S-SN implementation, suggest RAN3 to discuss whether need introduce a SN MODIFICATION REQUEST message to inform S-SN that the UE has access to T-SN successfully.

Proposal 8: RAN3 shall discuss whether to introduce an SN MODIFICATION REQUEST message to inform the S-SN that the UE has successfully accessed the T-SN, prefer option 1 or option 3.

2.1.6	 Data forwarding

Early data forwarding can be supported for inter-CU LTM.

Normal data forwarding may be initiated after the source gNB decides to trigger the LTM cell switch for the UE, and when exactly it is initiated is left to implementation.

For normal or early data forwarding, the same principles as those in the SA scenario can be applied. Normal data forwarding can be initiated after the source SN decides to trigger the LTM cell switch for the UE. The exact timing of when it is initiated is left to the implementation.

Considering the following note in stage2

In case of SN terminated bearers, early data forwarding may take place. For the early data forwarding of SN terminated bearers, the MN forwards the PDCP SDU to the candidate SN(s). For the early transmission of MN terminated split/SCG bearers, the MN forwards the PDCP PDU to the candidate SN(s)

Proposal 9: Early data forwarding can be support in inter-CU LTM in DC.

Proposal 10: Normal data forwarding can be initiated after the source SN decides to trigger the LTM cell switch for the UE. The exact timing of its initiation is left to implementation.

    - Signalling flow

23~25. The MN triggers the MN initiated SN Modification procedure to inform the last serving SN UE has success access to the target PSCell, to switch to the prepared state, and if applicable, to allow provisioning of new data forwarding addresses based on the data forwarding proposals of the MN and the selected candidate SN. If applicable, the MN triggers the Xn-U Address Indication procedure to inform the last serving SN the address of the SN of the selected candidate PSCell, to start late data forwarding

26~27. If PDCP termination point is changed for bearers using RLC AM, the SN sends the SN Status Transfer message to MN, which the MN sends then to the SN of the selected candidate PSCell, if needed.

28. If applicable, data forwarding from the last serving SN takes place..

29. If applicable, a PDU Session path update procedure is triggered by the MN.

Proposal 11: It is proposal to discuss the step 23-29 based one above procedure text.

  - Inter MCG LTM with SN kept or release

RAN2 has introduced the signalling flow for inter MCG LTM with SN kept or release in endorsed BLCR R2-2502234. RAN3 should also add this part in RAN3 BLCR and check RAN3-related part.

Proposal 16: RAN3 shall include the signalling flow for inter MCG LTM in R2-2502234 in RAN3 BLCR and further check RAN3-related part.

.	If the candidate MN decides to keep the UE context in the SN, the candidate MN sends the SN Addition Request message to the SN including the SN UE XnAP ID as a reference to the UE context in the SN that was established by the source MN. If the candidate MN decides to change the SN allowing delta configuration, the candidate MN sends the SN Addition Request message to the candidate SN including the UE context in the source SN that was established by the source MN. Otherwise, the candidate MN may send the SN Addition Request message to the candidate SN including neither the SN UE XnAP ID nor the UE context in the source SN that was established by the source MN. Within the SN Addition Request message, the candidate MN also includes the LTM related information, i.e., the source MN ID and the MN UE XnAP ID in the source MN, in order to indicate that the SN Addition Preparation procedure is triggered in relation to an LTM and to enable the SN to identify requests related to the same UE.

Based on the RAN2 BLCR, it assumed that the MN may request candidate SN with LTM related information. However, form the perspective of SN, the behaviour is same for whether the MN executes CHO or LTM, so, there is no need for MN to indicate SN the request is for inter-MCG LTM.

Proposal 12: From the perspective of the SN, the behavior is the same whether the MN executes CHO or LTM. There is no need to indicate that the SN addition request is for inter-MCG LTM.

3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals:

Proposal 1: Discuss how to decide the Upper limit number of PScell(s) for each candidate SN

Proposal 2: Include the suggested PSCell list in the SN addition request message explicitly.

Proposal 3: Include the suggested PSCell list in the SN ADDITION REQUEST message explicitly.

Proposal 4: discussion how to relalize SN key handing:

- Alt1:MN shall send the { S-KSN,sk-Counter } list to each candidate SN

- Alt2:MN shall send the S-KSN to the each candidate SN

Proposal 5: Introduce a LTM reference configuration request IE in SN MODIFICATION REQUEST message.

Proposal 6: Introduce a LTM full configuration IE in SN MODIFICATION RESPONSE message

Proposal 7: CSI-RS COORDINATION REQUEST message to the target SN including the CSI-RS resource to be activated/deactivated of the candidate cell(s) via MN .

Proposal 8: RAN3 shall discuss whether to introduce an SN MODIFICATION REQUEST message to inform the S-SN that the UE has successfully accessed the T-SN, prefer option 1 or option 3.

Proposal 9: Early data forwarding can be support in inter-CU LTM in DC.

Proposal 10: Normal data forwarding can be initiated after the source SN decides to trigger the LTM cell switch for the UE. The exact timing of its initiation is left to implementation.

Proposal 11: It is proposal to discuss the step 23-29 based one above procedure text.

Proposal 12: From the perspective of the SN, the behavior is the same whether the MN executes CHO or LTM. There is no need to indicate that the SN addition request is for inter-MCG LTM.

5. TP for BLCR

## 10.5	Secondary Node Change (MN/SN initiated)

*// skip unrelated part //*

### 10.5.2	MR-DC with 5GC

*// skip unrelated part //*

SN initiated inter-SN SCG LTM

The SN initiated inter-SN SCG LTM procedure is used to configure inter-SN SCG LTM.

Figure 10.5.2-5 SN initiated inter-SN SCG LTM

Figure 10.5.2-5 shows an example signalling flow for the inter-SN SCG LTM initiated by the SN:

Editor’s Note: RAN2 assumes that the figure should be anchored together with RAN3. The final figure is up to the progresses made in RAN3.

1.	The source SN initiates the inter-SN SCG LTM procedure by sending the SN Change Required with a LTM Request Indication to the MN providing a list of candidate PSCell(s) for inter-SN SCG LTM preparation. The message may include an SCG reference configuration. Source SN may additionally send measurement results of candidate PSCells to the MN. Source SN may send CSI resource configuration request and/or early sync information request for suggested candidate PSCell(s).

Editor’s Note: RAN2 assumes that how to indicate the list of candidate PSCells from source SN to MN is up to RAN3.

2.	The MN requests each candidate SN to allocate resources for the UE by means of the SN Addition procedure. The MN also provides a list of KSN and associated sk-Counter values for each candidate SN, and forwards the received measurement results to each candidate SN(s). The MN may transfer CSI resource configuration request and/or early sync information for suggested candidate PSCell(s) to SN. The MN also provides upper limit for the number of PSCells to each candidate SN. The MN may select one of the candidate SN(s) and request providing the reference SCG configuration as part of the SN Addition procedure. Once obtained, the MN provides the reference configuration to other candidate SN(s).

NOTE X:	The MN may trigger the MN-initiated SN Modification procedure (to the source SN) to request a reference configuration for the subsequent inter-SN SCG LTM before step 2, if not provided in step 1.

3.	The candidate SN(s) determines the LTM candidate cells based on the measurement results (if provided) and the upper limit for the number of PSCells that can be prepared by each candidate SN. The candidate SN provides the SCG part configuration of each candidate PSCell, and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL/DL sync configuration, by sending SN Addition Request Acknowledge message to the MN.

4.	For SN terminated bearers using MCG resources, the MN provides Xn-U DL TNL address information in the Xn-U Address Indication message to the candidate SN(s).

5.	The MN sends the received L1 RS configuration, early UL/DL sync configuration of candidate cells to the source SN via SN Modification Request message.

6.	The source SN generates the common CSI resource configuration for L1 measurement on candidate PSCells. The source SN sends the generated common CSI resource configuration and the updated source SCG configuration to the MN via SN modification request acknowledge message. The source SN sends the configuration ID mapping list for the accepted candidate PSCell(s) to the MN for subsequent inter-SN SCG LTM.

7.	he MN transfers, during the LTM preparation phase, configuration ID mapping list, the common CSI resource configuration and the collected information of candidate cells to each candidate SN via the SN Modification Request message.

8.	The candidate SN(s) responds with the updated candidate SCG configuration to the MN via the SN Modification Request Acknowledge message.

9.	The MN generates the reference configuration for SN initiated inter-CU SCG LTM, which may include the MCG part of the reference configuration generated by the MN (if any), together with the SCG part of the reference configuration generated by the source or candidate SN. The MN sends an RRCReconfiguration message to the UE including the inter-SN SCG LTM configuration, i.e. a list of RRCReconfiguration* messages, in which each RRCReconfiguration* message contains the SCG configuration in the RRCReconfiguration** message received from the candidate SN in step 3 and possibly an MCG configuration. Besides, the RRCReconfiguration message can also include an updated MCG configuration, as well as the NR RRCReconfiguration*** message generated by the source SN, e.g., to configure the required measurements.

10.	The UE applies the RRCReconfiguration message received in step 9, stores the inter-SN SCG LTM configuration and replies to the MN with an RRCReconfigurationComplete message, including an SN RRCReconfigurationComplete*** message to the source SN.

11.	The MN informs the source SN with the SN RRCReconfigurationComplete*** message via SN Change Confirm message, to indicate that SCG LTM is prepared.

12.	. In case of SN terminated bearers, early data forwarding may take place. For the early data forwarding of SN terminated bearers, the MN forwards the PDCP SDU to the candidate SN(s). For the early transmission of MN terminated split/SCG bearers, the MN forwards the PDCP PDU to the candidate SN(s).

13.	The UE may perform DL synchronization with LTM candidate cell(s) before receiving the cell switch command, as specified in clause 9.2.3.5.2 in TS 38.300 [3].

NOTE XX:	If SP-CSI RS is used in candidate cell, the source SN may send a CSI-RS COORDINATION REQUEST message to the target SN including the CSI-RS resource to be activated/deactivated of the candidate cell(s) via MN .

14a.The UE may perform UL synchronization with LTM candidate cell(s) before receiving the cell switch command, as specified in clause 9.2.3.5.2 in TS 38.300 [3].

14b. The candidate SN(s) transfers the calculated TA value of candidate cell(s) to the source SN via the MN via the TA Information Transfer Message.

Editor’s Note: Details for TA information transfer procedure is up to RAN3 discussion.

The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the source SN, if the L1 measurement configuration in RRCReconfiguration is received in step 9.

16.	The source SN decides to execute cell switch to a target PSCell.

transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID and other related information for the target PSCell, as specified in clause 9.2.3.5.2 in TS 38.300 [3]. The UE applies the candidate configuration indicated by the target configuration ID including PSCell change.

18/19.	The source SN sends Cell Switch Notification Message by Cell Switch Notification procedure, to indicate the triggering of LTM to the UE , including the target PScell ID and the TCI state ID(s).

20.	The UE performs the random access procedure towards the target PSCell, if the UE does not have valid TA of the target PSCell.

21. The UE sends an MN RRCReconfigurationComplete* message to the MN, which includes an SN RRCReconfigurationComplete** message to the target SN.

22.	The MN informs the target SN that the UE has completed the reconfiguration procedure successfully via SN Reconfiguration Complete message, including the SN RRCReconfigurationComplete** message.

23~25. The MN triggers the MN initiated SN Modification procedure to inform the last serving SN UE has success access to the target PSCell, to switch to the prepared state, and if applicable, to allow provisioning of new data forwarding addresses based on the data forwarding proposals of the MN and the selected candidate SN. If applicable, the MN triggers the Xn-U Address Indication procedure to inform the last serving SN the address of the SN of the selected candidate PSCell, to start late data forwarding

26~27. If PDCP termination point is changed for bearers using RLC AM, the SN sends the SN Status Transfer message to MN, which the MN sends then to the SN of the selected candidate PSCell, if needed.

28. If applicable, data forwarding from the last serving SN takes place..

29. If applicable, a PDU Session path update procedure is triggered by the MN.

23~29. Editor’s Note: Details for related procedures and descriptions are up to RAN3 discussion.

NOTE X:	The steps 13-29 can be performed multiple times for subsequent SCG LTM cell switch execution using the SCG LTM candidate configuration(s) provided in step 9.

Editor’s Note: In the above descriptions and signalling flow for the overall procedure of SN-initiated inter-SN SCG LTM, the detail procedures and messages between MN, source or candidate SN(s) are up to RAN3 discussion.

NEXT CHANGE

## 10.6	PSCell change

In MR-DC, a PSCell change does not always require a security key change.

If a security key change is required, this is performed through a synchronous SCG reconfiguration procedure towards the UE involving random access on PSCell and a security key change, during which the MAC entity configured for SCG is reset and RLC configured for SCG is re-established regardless of the bearer type(s) established on SCG. For SN terminated bearers, PDCP is re-established. In all MR-DC options, to perform this procedure within the same SN, the SN Modification procedure as described in clause 10.3 is used, setting the PDCP Change Indication to indicate that a S-KgNB (for EN-DC, NGEN-DC and NR-DC) or S-KeNB (for NE-DC) update is required when the procedure is initiated by the SN or including the SgNB Security Key / SN Security Key when the procedure is initiated by the MN. In all MR-DC options, to perform a PSCell change between different SN nodes, the SN Change procedure as described in clause 10.5 is used.

If a security key change is not required (only possible in EN-DC, NGEN-DC and NR-DC), this is performed through a synchronous SCG reconfiguration procedure without security key change towards the UE involving random access on PSCell, during which the MAC entity configured for SCG is reset and RLC configured for SCG is re-established regardless of the bearer type(s) established on SCG. For DRBs using RLC AM mode PDCP data recovery applies, and for DRBs using RLC UM no action is performed in PDCP. For SRB3 PDCP may discard all stored SDUs and PDUs. Unless MN terminated SCG or split bearers are configured, this does not require MN involvement. In this case, if location information was requested for the UE, the SN informs the MN about the PSCell change (as part of location information) using the SN initiated SN modification procedure independently from the reconfiguration of the UE. In case of MN terminated SCG or split bearers, the SN initiated SN Modification procedure as described in clause 10.3 is used, setting the PDCP Change Indication to indicate that a PDCP data recovery is required. If the MN subscribes to PSCell changes to retrieve the SCG UE history information, the SN informs the MN about the SCG UE history information using the SN initiated SN modification procedure when the SCG UE history information changes.

A Conditional PSCell Change (CPC) is defined as a PSCell change that is executed by the UE when execution condition(s) is met. The UE starts evaluating the execution condition(s) upon receiving the CPC configuration, and stops evaluating the execution condition(s) once PSCell change or PCell change is triggered. Intra-SN CPC without MN involvement, inter-SN CPC initiated either by MN or SN are supported.

The following principles apply to CPC:

-	The CPC configuration contains the configuration of CPC candidate PSCell(s) and execution condition(s) and may contain the MCG configuration for inter-SN CPC, to be applied when CPC execution is triggered.

-	An execution condition may consist of one or two trigger condition(s) (see CondEvent, as defined in TS 38.331 [4] or TS 36.331 [10]). Only single RS type and at most two different trigger quantities (e.g. RSRP and RSRQ, RSRP and SINR, etc.) can be used for the evaluation of CPC execution condition of a single candidate PSCell.

-	Before any CPC execution condition is satisfied, upon reception of PSCell change command or PCell change command, the UE executes the PSCell change procedure as described in clause 10.3 and 10.5 or the PCell change procedure as described in clause 9.2.3.2 in TS 38.300[3] or clause 10.1.2.1 in TS 36.300 [2], regardless of any previously received CPC configuration. Upon the successful completion of PSCell change procedure or PCell change procedure, the UE releases all stored CPC configurations.

-	While executing CPC, the UE is not required to continue evaluating the execution condition of other candidate PSCell(s) or PCell(s).

-	Once the CPC procedure is executed successfully, the UE releases all stored conditional reconfigurations (i.e. for CPC and for CHO, as specified in TS 38.300 [3] or TS 36.300 [2]) except for subsequent CPAC.

-	Upon the release of SCG, the UE releases the stored CPC configurations.

-	MN can inform SN of the maximum number of conditional reconfigurations the SN is allowed to configure for SN initiated CPC including both intra-SN and inter-SN CPC.

CPC configuration in HO command, in PSCell addition/change command or within any conditional reconfiguration (i.e. CPA, CPC or CHO configuration) is not supported.

An SCG LTM is defined as a PSCell cell switch procedure that the network triggers via MAC CE. Intra-SN SCG LTM without MN involvement and SCG LTM initiated by SN with or without MN changed are supported. Inter-MN MCG LTM with intra-SN PSCell change is supported.

NEXT CHANGE

## 10.7	Inter-Master Node handover with/without Secondary Node change

*// skip unrelated part //*

### 10.7.2	MR-DC with 5GC

Inter-MN handover with/without MN initiated SN change is used to transfer UE context data from a source MN to a target MN while the UE context at the SN is kept or moved to another SN. During an Inter-Master Node handover, the target MN decides whether to keep or change the SN (or release the SN, as described in clause 10.8). Only intra-RAT Inter-Master node handover with/without SN change is supported (e.g. no transition from NGEN-DC to NR-DC). Inter-MN MCG LTM with SCG release or with SCG configuration change in the same SN, inter-MN or intra-MN MCG LTM with SCG addition are supported.

Editor’s Note: FFS whether capture the MCG LTM with SCG addition/SCG change here or add a new section as the CHO with SCG configuration.

Figure 10.7.2-1: Inter-MN handover with/without MN initiated SN change procedure

Figure 10.7.2-1 shows an example signalling flow for inter-MN handover with or without MN initiated SN change:

NOTE 1:	For an Inter-Master Node handover without Secondary Node change, the source SN and the target SN shown in Figure 10.7.2-1 are the same node.

1.	The source MN starts the handover procedure by initiating the Xn Handover Preparation procedure including both MCG and SCG configuration. The source MN includes the source SN UE XnAP ID, SN ID and the UE context in the source SN in the Handover Request message.

NOTE 2:	The source MN may trigger the MN-initiated SN Modification procedure (to the source SN) to retrieve the current SCG configuration and the QMC configuration information managed by the SN and to allow provision of data forwarding related information before step 1.

2.	If the target MN decides to keep the UE context in source SN, the target MN sends SN Addition Request to the SN including the SN UE XnAP ID as a reference to the UE context in the SN that was established by the source MN. If the target MN decides to change the SN allowing delta configuration, the target MN sends the SN Addition Request to the target SN including the UE context in the source SN that was established by the source MN. Otherwise, the target MN may send the SN Addition Request to the target SN including neither the SN UE XnAP ID nor the UE context in the source SN that was established by the source MN.

3.	The (target) SN replies with SN Addition Request Acknowledge. The (target) SN may include the indication of the full or delta RRC configuration.

NOTE 2a0: Void.

3a.	For SN terminated bearers using MCG resources, the target MN provides Xn-U DL TNL address information in the Xn-U Address Indication message.

4.	The target MN includes within the Handover Request Acknowledge message the MN RRC reconfiguration message to be sent to the UE in order to perform the handover, and may also provide forwarding addresses to the source MN. If PDU session split is performed in the target side during handover procedure, more than one data forwarding addresses corresponding to each node are included in the Handover Request Acknowledge message. The target MN indicates to the source MN that the UE context in the SN is kept if the target MN and the SN decided to keep the UE context in the SN in step 2 and step 3.

5a/5b.	The source MN sends SN Release Request message to the (source) SN including a Cause indicating MCG mobility. The (source) SN acknowledges the release request. The source MN indicates to the (source) SN that the UE context in SN is kept, if it receives the indication from the target MN. If the indication as the UE context kept in SN is included, the SN keeps the UE context.

5c.	The source MN sends XN-U Address Indication message to the (source) SN to transfer data forwarding information. More than one data forwarding addresses may be provided if the PDU session is split in the target side.

6.	The source MN triggers the UE to perform handover and apply the new configuration.

7/8.	The UE synchronizes to the target MN and replies with MN RRC reconfiguration complete message.

9.	If configured with bearers requiring SCG radio resources, the UE synchronizes to the (target) SN.

NOTE 2a1:	The order the UE performs Random Access towards the MN (step 7) and performs the Random Access procedure towards the SN (step 9) is not defined.

10.	If the RRC connection reconfiguration procedure was successful, the target MN informs the (target) SN via SN Reconfiguration Complete message.

11a. The source SN sends the Secondary RAT Data Usage Report message to the source MN and includes the data volumes delivered to and received from the UE over the NR/E-UTRA radio as described in clause 10.11.2.

NOTE 2a2:	The order the source SN sends the Secondary RAT Data Usage Report message and performs data forwarding with MN/target SN is not defined. The SN may send the report when the transmission of the related QoS is stopped.

11b. The source MN sends the Secondary RAT Report message to AMF to provide information on the used NR/E-UTRA resource.

12.	For bearers using RLC AM, the source MN sends the SN Status Transfer message to the target MN, including, if needed, SN Status received from the source SN. The target forwards the SN Status to the target SN, if needed.

13.	If applicable, data forwarding takes place from the source side. If the SN is kept, data forwarding may be omitted for SN terminated bearers or QoS flows kept in the SN.

14-17.	The target MN initiates the Path Switch procedure. If the target MN includes multiple DL TEIDs for one PDU session in the Path Switch Request message, multiple UL TEID of the UPF for the PDU session should be included in the Path Switch Ack message in case there is TEID update in UPF.

NOTE 3:	If new UL TEIDs of the UPF for SN are included, the target MN performs MN initiated SN Modification procedure to provide them to the SN.

18.	The target MN initiates the UE Context Release procedure towards the source MN.

19.	Upon reception of the UE Context Release message from source MN, the (source) SN releases C-plane related resources associated to the UE context towards the source MN. Any ongoing data forwarding may continue. The SN shall not release the UE context associated with the target MN if the UE contest kept indication was included in the SN Release Request message in step 5.

Inter-MN MCG LTM with SN

Figure 10.7.2-2: Inter-MN MCG LTM with Secondary Node

Figure 10.7.2-2 shows an example signaling flow for Inter-MN MCG LTM with Secondary Node.

Editor’s Note: RAN2 assumes that the figure and procedure descriptions should be anchored together with RAN3. The final figure and procedure descriptions are up to the progresses made in RAN3.

NOTE 1:	For an Inter-MN MCG LTM without SN change, the source SN and the target SN shown in Figure 10.7.2-2 are the same node.

NOTE 2:	For an Inter-MN MCG LTM with SN addition, the source SN and steps involving the source SN in Figure 10.7.2-2 are ignored.

NOTE 3:	Figure 10.7.2-2 is not applied for an Inter-MN MCG LTM with SN changed to another SN, which is not supported.

1.	The source MN starts LTM praperation by initiating the Xn Handover Preparation procedure including MCG configuration and, if the UE is configured with an SCG, SCG configuration. The source MN includes the source SN UE XnAP ID, SN ID and the UE context in the source SN in the Handover Request message.

2.	If the candidate MN decides to keep the UE context in the SN, the candidate MN sends the SN Addition Request message to the SN including the SN UE XnAP ID as a reference to the UE context in the SN that was established by the source MN. If the candidate MN decides to change the SN allowing delta configuration, the candidate MN sends the SN Addition Request message to the candidate SN including the UE context in the source SN that was established by the source MN. Otherwise, the candidate MN may send the SN Addition Request message to the candidate SN including neither the SN UE XnAP ID nor the UE context in the source SN that was established by the source MN.

3.	The (candidate) SN replies with the SN Addition Request Acknowledge message. The (candidate) SN may include the indication of the full or delta RRC configuration.

3a.	For the SN terminated bearers using MCG resources, the candidate MN provides Xn-U DL TNL address information in the Xn-U Address Indication message.

4.	The candidate MN includes within the Handover Request Acknowledge message the MN RRC reconfiguration message to be sent to the UE in order to perform LTM, and may also provide forwarding addresses to the source MN. If PDU session split is performed in the target side during handover procedure, more than one data forwarding addresses corresponding to each node are included in the Handover Request Acknowledge message. The candidate MN indicates to the source MN that the UE context in the SN is kept if the candidate MN and the SN decided to keep the UE context in the SN in step 2 and step 3.

4a.	The source MN sends the Xn-U Address Indication message to the (source) SN. This Xn-U Address Indication message notifies LTM to the (source) SN, which may decide to perform, if applicable, early data forwarding for SN-terminated bearers, together with the sending of an Early Status Transfer message to the source MN.

5.	The source MN sends an LTM Configuration Update message to the candidate MN(s) to update the LTM configurations of candidate cell(s). The source MN may include the common CSI-RS resource configuration.

6.	The candidate MN(s) sends the LTM Configuration Update Acknowledge message to the source MN. The candidate MN(s) may also provide the CSI-RS report configuration.

7.	The source MN sends an RRC reconfiguration message to the UE, including the LTM candidate configuration, i.e. a list of RRCReconfiguration* messages, in which each RRCReconfiguration* message contains an MCG configuration and possibly an SCG configuration in the RRCReconfiguration** message received from the candidate SN in step 3.

8.	The UE applies the RRC reconfiguration message received in step 7, stores the LTM condidate configuration and replies to the MN with an RRCReconfigurationComplete message.

9.	The UE may perform DL synchronization with LTM candidate cell(s) before receiving the cell switch command, as specified in clause 9.2.3.5.2 in TS 38.300 [3].

10a. The UE may perform UL synchronization with LTM candidate cell(s) before receiving the cell switch command, as specified in clause 9.2.3.5.2 in TS 38.300 [3].

10b. The candidate MN(s) sends the TA Information Transfer message to the source MN if early TA acquisition is performed to the candidate MN(s).

11.	The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the source MN, if the L1 measurement configuration in RRCReconfiguration is received in step 7.

12.	The source MN decides to execute cell switch to a target cell.

13.	The source MN transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID and other related information for the target cell, as specified in clause 9.2.3.5.2 in TS 38.300 [3]. The UE switches to the target cell and applies the candidate configuration indicated by the target configuration ID.

14.	The source MN sends the Cell Switch Notification message to the target MN to indicate the initiation of Cell Switch command to the UE.

15.	The UE performs the random access procedure towards the target cell, if UE does not have valid TA of the target cell as specified in clause 5.18.35 of TS 38.321[6].

16. The UE sends an MN RRCReconfigurationComplete* message to the target MN, which may include an SN RRCReconfigurationComplete** message to the target SN.

17.	If configured with bearers requiring SCG radio resources, the UE synchronizes to the (target) SN.

NOTE x:	The order the UE performs Random Access towards the MN (step 15) and performs the Random Access procedure towards the (target) SN (step 17) is not defined.

18.	If the RRC connection reconfiguration procedure was successful, the target MN informs the (target) SN via SN Reconfiguration Complete message.

19.	The target MN sends the Handover Success message to the source MN to inform that the UE has successfully accessed the target cell.

20a/b.	The source MN sends SN Release Request message to the (source) SN including a Cause indicating MCG mobility. The source MN indicates to the (source) SN that the UE context in SN is kept, if it receives the indication from the target MN. The (source) SN acknowledges the release request.

20c. The source MN sends XN-U Address Indication message to the (source) SN to transfer data forwarding information. More than one data forwarding addresses may be provided if the PDU session is split in the target side.

20d/e. Editor’s Note: FFS up to RAN3. RAN2 assumes that 20d/e are not needed for inter-MN LTM with SN.

21a/b. Editor’s Note: FFS up to RAN3.

22a/b. Editor’s Note: FFS up to RAN3.

23.	For bearers using RLC AM, the source MN sends the SN Status Transfer message to the target MN, including, if needed, SN Status received from the source SN. The target MN forwards the SN Status to the target SN, if needed.

24.	If applicable, data forwarding takes place from the source side (i.e. source MN or source SN). If the SN is kept, data forwarding may be omitted for the SN terminated bearers or QoS flows kept in the SN.

25-27.	The target MN initiates the Path Switch procedure. If the target MN includes multiple DL TEIDs for one PDU session in the Path Switch Request message, multiple UL TEID of the UPF for the PDU session should be included in the Path Switch Ack message in case there is TEID update in UPF.

28. The target MN (i.e. the new source MN) sends the LTM Configuration Update message to the candidate MNs.

29. The candidate MN(s) responds the LTM Configuration Update Acknowledge message to the source MN.

30. The target MN may send the UE Context Release message to inform the source MN to release radio and C-plane related resources associated to the UE context if no LTM candidate cell(s) exist in the source MN. Any ongoing data forwarding may continue.

31.	Upon reception of the UE Context Release message from source MN, the (source) SN releases C-plane related resources associated to the UE context towards the source MN. Any ongoing data forwarding may continue. The SN shall not release the UE context associated with the target MN if the UE context kept indication was included in the SN Release Request message in step 20a.

Editor’s Note: In the above descriptions and signalling flow for the overall procedure of inter-MN MCG LTM with SN, the detail procedures and messages between MN, source or candidate SN(s) are up to RAN3 discussion.

CHANGE END