3GPP TSG RAN WG3 Meeting #128	R3-253478

Malta, MT, 19-23 May 2025

Title: 	(TP to 37.320) - Continuous MDT collection targeting the same UE across RRC states

Source: 	Ericsson, InterDigital, Deutsche Telekom, BT

Agenda item:	11.4

Document Type:	Discussion and agreement

# 1. Introduction

In this paper we present a TP to 37.320 for Continuous MDT collection targeting the same UE across RRC states R3-253476.

# 2. TP to Radio measurement collection for MDT – Continuous MDT collection targeting the same UE across RRC states

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

### 5.1.x	Continuous MDT procedures

The Continuous data collection for management-based MDT is described as follows: a UE in the NG-RAN is configured with management-based Logged MDT when in RRC_Idle and RRC_Inactive states and with management-based Immediate MDT when in RRC_Connected state. Differently from signalling-based MDT, in management-based MDT, a UE is not uniquely identified in the MDT activation. Therefore, when a UE transits to RRC_Connected state from RRC_Idle/RRC_Inactive (during which Logged MDT data have been collected) or when a UE is handed over between gNBs, the network needs to select again the same UE for continuous MDT for subsequent MDT data collection. Such solution is based on the encoding of the Trace Reference ID signalled from the management system to the RAN and reported by the UE in the relevant MDT logs. When the UE reports the logs, the UE reports the TR, which implicitly flag that the UE is a Continuous MDT UE, and its RAN assigned TRSR,  which should be re-selected again for the continuous MDT process for the UE.

The Continuous MDT operation is provided by the following figure and steps:

Figure 5.1.x-1: Example of “continuous MDT” signalling support

- RAN receives two Management Based MDT configurations: an Immediate MDT configuration and a Logged MDT configuration which are associated to a range of TRs that identify Continuous MDT configurations. TR 1 and TR2 are assigned respectively to the Management Based Immediate MDT job and to the Management Based Logged MDT job.

- The RAN may configure the UE with Immediate MDT and logged, The Immediate MDT configuration corresponds to TR1, TRSR A and the logged MDT configuration corresponds to TR2 and TRSR A. The same TRSR is assigned to the UE during the course of the continuous MDT session.

- The UE reports Immediate MDT measurements collected while in RRC_CONNECTED and according to the Immediate MDT configuration received.

- NG-RAN1 reports the measurements corresponding to the Management Based Immediate MDT configuration for TR1 and TRSR A to the TCE.

- The UE moves to RRC_IDLE/INACTIVE. The UE will collect logged MDT measurements and store them as per received Logged MDT configuration. Logs include TR2 and TRSR A assigned by the source gNB.

- When the UE moves from RRC_IDLE/INACTIVE to RRC_CONNECTED, it reports Logged MDT logs, including TR2 and  + TRSR A. NG-RAN 2 realizes that the UE shall be configured with continuous MDT by means of reading the TR (TR2).

- NG-RAN 2 reports to the TCE the Trace Records including the logged MDT measurements. These include TR 2 and TRSR 2. By checking the TRSR the TCE can correlate the previous immediate MDT measurements (for TR 1 and TRSR A) with the logged MDT measurements (for TR 2 and TRSR A).

- Step 2, 3 and 4 are repeated at NG-RAN 2. The UE is configured with Management Based Immediate MDT with TR 1 and TRSR A and immediate MDT measurements are signalled to the TCE.  By checking the TRSR the TCE can correlate the previous logged MDT measurements (for TR 2 and TRSR A) with the immediate MDT measurements (for TR 1 and TRSR A)

- During connected mode mobility NG-RAN 2 icludes in the Handover Preparation signalling towards NG-RAN 1 TR1 and TRSR A assigned to the management based Immediate MDT process for the UE. NG-RAN 1 is able to deduce that the UE has to be configured with continuous MDT (by checking the TR) and that the same TRSR received from NG-RAN 2 shall be applied to the UE.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>