3GPP TSG-RAN WG3 Meeting #127bis	R3-253480

Wuhan, China, 07 - 11 April, 2025

Agenda Item:	19.2

Source:	Ericsson, Jio Platforms

Title:	Discussion on RAN3 impacts to support enhancements for CLI handling for SBFD

Document for:	Discussion, Approval

# 1	Introduction

RAN1 has started work on “Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, as per WID in RP-241614.

As part of this WID, RAN3 has been identified as one of the WGs that needs to contribute to solutions on enhancements for CLI handling, as per the excerpt below (from RP-241614):

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - … - Note: Without dedicated optimization for dynamic/flexible TDD. - … - Note: RAN3 will not specify enhancements to network signalling to support inter-operator coordination for CLI handling - Notes: Following are assumed for information exchange among gNBs - No normative behavior is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signaling can be discussed in RAN3 - The information exchanges are subject to RAN3’s assessment of feasibility and specification impact

Additionally, RAN1 sent an LS to RAN3 in R3-245006, where the objectives quoted above were clarified as follows:

While the WID objective lists only RAN3 as responsible working group, RAN1 has discussed the sub-bullets of the objective and would like to clarify several PHY/L1-related aspects that are not typically discussed in RAN3. RAN1 would like to inform RAN3 (RAN2 is cc’d) of the following RAN1 agreement. Agreement Send an LS to RAN3 with the following information (cc: RAN2) From RAN1 perspective, exchange among gNBs of the configuration info for a set of one or more periodic NZP CSI-RS resources (relevant IE in 38.331 are NZP-CSI-RS-Resource, NZP-CSI-RS-ResourceSet) is supported. Strongest DL beam information in the context of NZP-CSI-RS is defined as a CSI-RS Resource Indicator (CRI) value. CRI is a relative index in the range [1 .. N], where N is the number of CSI-RS resources within a set of resources for which the configuration is provided to a gNB. Strongest DL beam information in the context of SSBs is defined as an SSB index. SSB index is an absolute index among the SSB(s) for which the configuration is provided to a gNB. Note: RAN1 will not discuss information exchange among gNBs for gNB-gNB CLI handling unless there is an LS from RAN3. # 2 Actions To TSG RAN3 ACTION: RAN1 respectfully asks RAN3 to take the above information into account in their further work.

During the last RAN3 meetings discussinos on CLI mitigation were carried out. For clarity, we report below the agreements and open points taken so far in RAN3:

RAN3-125bis:

RAN3 agrees to Exchange the following information over Xn interface and F1 interface:

- Semi-static cell-specific SBFD time and frequency location configuration with cell granularity (FFS on container or explicit IEs over XnAP and F1AP)

- Measurement resource configuration

- Strongest DL beam information (FFS on the details)

- CLI-mitigation request (FFS on the details)

RAN3-126:

The SBFD time and frequency location configuration with cell granularity in Served Cell Information NR IE shall be introduced in XnAP XN SETUP REQUEST message, XnAP XN SETUP RESPONSE message, XnAP NG-RAN NODE CONFIGURATION UPDATE message and XnAP NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message.

Exchange reference signalling configuration(SSB and NZP CSI-RS) in interface management procedures: Xn Setup and NG-RAN node Configuration Update procedure?

RAN3-127:

RAN3 to confirm that SSB resource configuration has already been supported to be exchanged between gNBs in Served Cell Information NR IE.

gNB-DU provides the SBFD time and frequency configuration to gNB-CU in Served Cell Information IE, and gNB-CU provides the SBFD time and frequency configuration info received over Xn interface from gNB-CU to gNB-DU, in Served Cell Information IE in Served Cell Information IE in gNB-CU Configuration Update message.

The SBFD time and frequency configuration is encoded as container defined by RAN2.

For exchange of SSB info over Xn, nothing to be done, already supported in RAN3 spec.

For exchange of NZP CSI-RS over Xn, to add a new IE in Served Cell Information NR IE with reference to RAN2 definition?

For exchange of strongest DL beam information over Xn, SSB info is defined as SSB-Index INTEGER (0..63), NZP CSI-RS info is defined as NZP CSI-RS Resource Indicator INTEGER (1..64).

To define CLI-mitigation request as an optional IE. FFS on the details.

How to exchange the strongest DL beam information and the CLI-mitigation request over Xn, e.g. to reuse existing procedures such as NG-RAN node Configuration Update procedure or Resource Status Reporting procedure, or, to introduce new procedures?

During RAN3-127, RAN3 agreed an LS to RAN2 in R3-250888, where the following was stated:

RAN3 has started to work on the SBFD information exchange among gNBs for CLI mitigation, according to the received LS in R1-2407533 from RAN1. RAN3 has agreed that SBFD time and frequency configuration info should be exchanged between neighbour gNBs in Served Cell Information NR IE, and also between gNB-DU and gNB-CU in case of split architecture. RAN3 also reached the common understanding that RAN3 needs RAN2 to provide the concrete IE design on SBFD time and frequency configuration information, which should be based on parameters provided by RAN1. With the agreement and common understanding reached in RAN3, RAN3 would like to ask RAN2 to work on the IE design of SBFD time and frequency configuration information similar to the TDD-UL-DL-ConfigurationCommon IE which is signalled over Xn and F1. ACTION: RAN3 respectfully asks RAN2 to work on the IE design of SBFD time and frequency configuration informaiton.

At the last RAN3 meeting (RAN3-127bis), RAN3 took some important agreements that made the discussion move considerably forward. The agreements are listed here and have been captured in corresponding BLCRs, namely R3-252472 for the F1AP and R3-252470 for the XnAP.

Introduce NZP-CSI-RS resource configuration in Served Cell Information NR IE of XnAP and F1AP specifications, which refers to the NZP-CSI-RS-Resource IE and NZP-CSI-RS-ResourceSet IE defined in TS 38.331. Introduce new procedure to provide the strongest DL beam information and the CLI-mitigation request over XnAP and F1AP. gNB-DU provides the SBFD time and frequency configuration to gNB-CU in Served Cell Information IE, and gNB-CU provides the SBFD time and frequency configuration info received over Xn interface from gNB-CU to gNB-DU, in Neighbour Cell Information List IE in gNB-CU Configuration Update message. Exchange of SBFD configuration The SBFD time and frequency location configuration with cell granularity in Served Cell Information IE is introduced in F1AP F1 SETUP REQUEST message and F1AP gNB-DU CONFIGURATION UPDATE message The SBFD time and frequency location configuration with cell granularity in the Neighbour Cell Information List IE is introduced in F1AP gNB-CU CONFIGURATION UPDATE message Exchange of measurement configuration For exchange of NZP CSI-RS over F1, to put a new IE in Served Cell Information IE from gNB-DU to gNB-CU, and in Neighbour Cell Information List IE from gNB-CU to gNB-DU with reference to RAN2 definition In order to transfer the SSB info from gNB-CU to gNB-DU, to introduce an explicit IE in the Neighbour Cell Information List IE. Exchange of measurement result & Exchange of CLI-mitigation request To introduce a class 2 procedure, with tentative message name “CLI MEASUREMENT UPDATE”, in which both the strongest DL beam information and the CLI-mitigation request could be included as two optional IEs.

In this contribution an analysis of the remaining work needed in RAN3 is made and conclusions on a way forward are derived. TPs for TS 38.423 and TS38.473 are provided in R3-253481.

# 2	Discussion

## 2.1	Semi-static cell-specific SBFD time and frequency location configuration

The first objective to analyse concerns the exchange of following information between gNBs:

–	Semi-static cell-specific SBFD time and frequency location configuration

In the agreed BLCRs for TS38.423 and TS38.473, solutions to address this objective have been captured by means of the following:

- For TS38.423 (XnAP), a new SBFD Configuration IE has been included in the Served Cell Information NR IE. The SBFD Configuration IE is an OCTET string that will represent the RRC IE defined by RAN2 to represent the SBFD time and frequency configuration

- For TS38-473 (F1AP), a new SBFD Configuration IE has been included in the Served Cell Information IE and in the Neighbour Cell Information List Item IE. The SBFD Configuration IE is an OCTET string that will represent the RRC IE defined by RAN2 to represent the SBFD time and frequency configuration

With the above enhancements it is possible to signal the SBFD time and frequency configuration in the following cases

- From a gNB-DU serving the cell to which the SBFD configuration applies to its connected gNB-CU,

- From a gNB serving the cell to which the SBFD configuration applies to its neighbouring gNBs

- From a gNB-CU to a gNB-DU of a gNB receiving SBFD time and frequency information of cells of a neighbour gNB and forwarding SBFD configuration information to its connected gBNB-DU, for cells that neighbour the gNB-DU served cells.

With the above, the objective is completed with the exception of adding the exact reference to the SBFD Time and Frequency configuration IE from TS38.331 in the BLCRs for TS38.423 and TS38.473.

Conclusion 1: The WI objective concerning “Semi-static cell-specific SBFD time and frequency location configuration” is completed, with the exception of adding to the BLCRs for TS38.423 and TS.473 the xact reference to the SBFD time and frequency configuration IE to be defined by RAN2 in TS38.331

## 2.2	Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS

The second objective to analyse concerns the exchange of following information between gNBs:

–	Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS

In the agreed BLCRs for TS38.423 and TS38.473, solutions to address this objective have been captured by means of the following:

- For signalling of SSB cofigurations of served cells, RAN3 agreed that the information is already included in the F1AP and XnAP protocols and that nothing needs to be added to current signalling.

- For TS38.423 (XnAP), a new NZP CSI-RS Resources Configuration IE has been included in the Served Cell Information NR IE. The NZP CSI-RS Resources Configuration IE references to the NZP-CSI-RS-ResourceSet IE and to the NZP-CSI-RS-Resource IE defined in TS38.331.

- For TS38-473 (F1AP), a new NZP CSI-RS Resources Configuration IE has been included in the Served Cell Information NR IE and a new SSB resource configuration IE and NZP CSI-RS Resources Configuration IE have been included in the Neighbour Cell Information List Item IE. NZP CSI-RS Resources Configuration IE references to the NZP-CSI-RS-ResourceSet IE and to the NZP-CSI-RS-Resource IE defined in TS38.331 while the SSB resource configuration IE refers to the MeasTimingList IE defined in TS38.331.

With the above enhancements it is possible to signal the SSB and/or NZP CSI-RS configurations in the following cases

-	From a gNB-DU serving the cell to which the NZP CSI-RS Resources Configuration applies to its connected gNB-CU,

-	From a gNB serving the cell to which the NZP CSI-RS Resources Configuration applies to its neighbouring gNBs

-	From a gNB-CU to a gNB-DU of a gNB receiving NZP CSI-RS Resources Configuration of cells of a neighbour gNB and forwarding NZP CSI-RS Resources Configuration information to its connected gBNB-DU, for cells that neighbour the gNB-DU served cells.

With the above changes, the objective is completed.

Conclusion 2: The objective concerning “Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS” has ben completed by RAN3

## 2.3	Strongest DL beam information and CLI mitigation request

The third objective to analyse concerns the exchange of following information between gNBs:

–	Strongest DL beam information

–	CLI-mitigation request

With respect to this objective RAN3 has agreed to include in the F1AP and XnAP protocols a new Class 2 procedure to signal the following information:

- Over the XnAP: A CRI and/or an SSB Index and/or a CLI Mitigation Request, per cell. The information is signalled from a source gNB to a target gNB

- Over the F1AP: A CRI and/or an SSB Index and/or a CLI Mitigation Request, per cell. The information is signalled from a gNB-DU to a gNB-CU, for the cells served by the gNB-DU

- Over the F1AP: A CRI and/or an SSB Index and/or a CLI Mitigation Request, per cell. The information is signalled from a gNB-CU to a gNB-DU, for cells that are neighbourig the cells served by the gNB-DU

The details of the newly added procedure are still subject to further studies. In particular, the new proceure has been named “CLI Measurement Update”.

However, it should be noted that the use of “Update” in Class 2 procedures is usually assigned to procedures that are triggered by a previous Class 1 procedure. The achronim “CLI” is also not suited for a procedure name. We therefore suggest to change the name of the procedure to “Interference Indication”, which is in line with similar class 2 procedures, such as the Xn: Access and Mobility Indication procedure.

Proposal 1. Modify the name of the new Class 2 procedure for the F1AP and XnAP into “Interference Indication”

Some updates to the procedure description are also needed. They are presented in the TPs in R3-25xxxx and R3-25xxxx.

With the above changes the objective concerning the exchange of Strongest DL beam information and CLI-mitigation request can be considered complete.

Conclusion 3: With the exception of Stage 3 procedure description rewording, the objective on exchange of information concerning Strongest DL beam information and CLI-mitigation request can be considered completed

# 3	Conclusion

In this paper an analysis of the RAN3´s objectives on Evolution of enhancements for CLI handling, as part of the WI “NR duplex operation: Sub-band full duplex (SBFD)”, has been carried out.

The paper derived the following proposals and conclusions:

Conclusion 1: The WI objective concerning “Semi-static cell-specific SBFD time and frequency location configuration” is completed, with the exception of adding to the BLCRs for TS38.423 and TS.473 the xact reference to the SBFD time and frequency configuration IE to be defined by RAN2 in TS38.331

Conclusion 2: The objective concerning “Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS” has ben completed by RAN3

Proposal 1. Modify the name of the new Class 2 procedure for the F1AP and XnAP into “Interference Indication”

Conclusion 3: With the exception of Stage 3 procedure description rewording, the objective on exchange of information concerning Strongest DL beam information and CLI-mitigation request can be considered completed

TPs mirroring the above proposals are available for agreement in R3-253481.

# 4	References

- RP-241614, WID on Evolution of NR duplex operation: Sub-band full duplex (SBFD)