3GPP TSG-RAN WG3 Meeting #128	R3-253495

St. Julian’s, Malta, 19th – 23rd May, 2025

Title: 	Discussion on assistance information for SN configuration in AI/ML-based Mobility Optimization in NR-DC

Source: 	Huawei

Agenda item:	11.4

Document Type:	Discussion and Decision

# 1. Introduction

In the last meeting RAN3 kept on discussing the objectives of the Rel-19 WI “Revised WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [1], which follows the SI “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [2] and whose outcome is summarized in TR 38.743 [3].

One of the objectives of the WI is to introduce support of the leftovers in Rel-18 AI/ML for NG-RAN, one of which is related to the mobility optimization for NR-DC.

This contribution analyzes the technical enablers for specifying support over the Xn interface for mobility optimization in NR-DC, and provides proposals for discussion based on the outcome from the last RAN3 meeting.

# 2. Discussion

The outcome of the last RAN3 meeting concerning the mobility optimization in NR-DC is reported below [4]:

The reporting of legacy UE performance from SN to MN is performed via the Data Collection Reporting procedure (within the DATA COLLECTION UPDATE message). The details of measurement metrics and any other additional information need to be further checked. The following list of exit conditions to stop the collection of UE performance at SN side: - the time since NR-DC was successfully setup with the S-NG-RAN node is equal to the value of the Collection Time Duration for UE Performance IE; - the NR-DC with the S-NG-RAN node for the UE is released. UE performance measurements in NR-DC based on the outcome of the UE performance discussion in split architecture? Whether to introduce assistance information (predicted PSCell ID) from MN to SN? Whether to enhance the UE Performance IE in the DATA COLLECTION UPDATE message to include the actual PSCell ID? Whether additional conditions (e.g., SCG activation) for starting the collection of UE performance are needed, pending the outcome of the discussion on UE performance in split architecture?

In the next sections we discussed the open issue listed above related to the need for assistance information to be provided from the MN to SN.

## 2.1 Assistance information for SN configuration

In the last RAN3 meeting the following FFSs were captured in the Chair’s meeting minutes, but there was no progress due to lack of time [4]:

Whether to introduce assistance information (predicted PSCell ID) from MN to SN? Whether to enhance the UE Performance IE in the DATA COLLECTION UPDATE message to include the actual PSCell ID?

The above follows the discussion already occurred in RAN3#126 meeting, with following FFSs being captured in the meeting minutes, which are related to the specification of assistance information that could be sent over Xn from Master Node (MN) to Secondary Node (SN) for subsequent SN configuration [5]:

Introduce predicted PSCell ID in the DC procedure (e.g., SN Addition, MN-initiated SN change) as one of assistance information? Other information, e.g., predicted SCG bearer?

There were some contributions submitted since RAN3#127 meeting, e.g. [7][8], proposing to indicate the PSCell ID predicted (or assumed, as labelled in [8]) by the MN to help the SN in choosing and configuring the Secondary Cell Group (SCG). First of all, it should be recalled that RAN3 had a discussion on providing predicted information from the MN to the SN during the SI, and discussed in particular the predicted UE trajectory in the SN (across PSCells). Back then, there was no consensus in RAN3 on transferring such prediction over the Xn interface as captured in RAN3#125 meeting [9]:

There is no consensus to transfer the predicted/measured UE trajectory over NR-DC. Check online whether to support it during normative phase.

and in our view the predicted PSCell ID is a particular case where the predicted UE trajectory in the SN is only limited to a single PSCell. Moreover, in our understanding, in the legacy NR-DC the MN does not provide any information to the SN to help it in selecting the PSCell of the SCG, and we see no technical reasons to deviate from this behaviour, also considering that among proposing companies there is no clear view on whether the PSCell ID information to be transferred to the SN is either a prediction or an assumption, and what is the difference between the two.

Our understanding is that the predicted PSCell ID (as proposed in [7]) is an output of the AI/ML model used in the MN – which, by the way, has not been agreed to be included in TR 38.743 [3] in clause 5.1.2 for the potential standard impact – and hence this will require RAN3 to also discuss the feedback information which is needed to validate such prediction. Proponents of [8], instead, refer to the PSCell ID to be transferred to the SN as an assumption made by the MN. Still based on our understanding, this information should be derived according to actual PSCell serving the UE that contribution [8] proposes in Proposal 2 to be transferred to the MN within the existing Rel-18 UE Performance IE. Hence, if a UE was served well by an SN on a certain PSCell in the past, the AI/ML model would likely predict that specific SN as the best option for the UE. It is however not clear how the MN would use past information for the inference process in order to predict the best SN to setup NR-DC with. And RAN3 had no discussions about this aspect during the SI.

Proposal 1: RAN3 to agree not to transfer predicted/assumed PSCell ID from MN to SN as part of NR-DC procedures (SN Addition and MN-initiated SN change).

Proposal 2: RAN3 to agree that there is no need to enhance the UE Performance IE to include the actual PSCell ID in use in the SN.

In a similar way, [8] also proposes to transfer the list of QoS flows that are assumed to be served by the SN as SCG bearers. This is not the current NR-DC behaviour, where an SN may decide that all the QoS flows for which the MN requests SN-terminated bearers will be served via the Master Cell Group (MCG) as MCG bearers, and informs the MN about this decision – refer to QoS Flows Not Admitted List IE within the PDU Session Resource Setup Response Info – SN terminated IE of the S-NODE ADDITION REQUEST ACKNOWLEDGE message in TS 38.423 [10]. Differently from the assumed PSCell ID (where the PSCell actually selected by the SN after the Addition procedure is not known), the MN may have historical information from the previous SN Addition procedures for the UE from which it may deduce whether a certain SN was a good choice or not by checking the above mentioned QoS Flows Not Admitted List IE. And this information may be used during the inference process in the MN to determine the best SN to add as an input information (if available) locally available. In this way the MN may already be able to predict the best SN to add by “simply” discarding those SNs which, in the past, had less chances to serve SN-terminated bearers via the SCG. Therefore, we think there is no need for the MN to send to the SN the list of QoS flows that are assumed to be served by the SN as SCG bearers.

Proposal 3: RAN3 to agree not to transfer the list of QoS flows that are assumed to be served by the SN as SCG bearers from MN to SN as part of NR-DC procedures (SN Addition and MN-initiated SN change).

# 3. Conclusion

Based on the above discussion, we have the following proposals:

Proposal 1: RAN3 to agree not to transfer predicted/assumed PSCell ID from MN to SN as part of NR-DC procedures (SN Addition and MN-initiated SN change).

Proposal 2: RAN3 to agree that there is no need to enhance the UE Performance IE to include the actual PSCell ID in use in the SN.

Proposal 3: RAN3 to agree not to transfer the list of QoS flows that are assumed to be served by the SN as SCG bearers from MN to SN as part of NR-DC procedures (SN Addition and MN-initiated SN change).

# 4. Reference

- RP-250326, Revised New WID on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN, 3GPP RAN#107

- RP-240323, “Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#103

- 3GPP TR 38.743 “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, Rel-19

- 3GPP RAN3#127bis Chairman’s notes (RAN3_127bis_agenda_20250415_EOM1.doc)

- 3GPP RAN3#126 Chairman’s notes (RAN3_126_agenda_20241122_EOM1.doc)

- 3GPP RAN3#127 Chairman’s notes (RAN3_127_agenda_20250221_1300_EOM.doc)

- R3-250664, Discussion on AIML based Mobility Optimization for NR-DC, CMCC, CATT, ZTE Corporation, Lenovo

- R3-250459, AI/ML support for NR-DC, Ericsson, Jio Platforms (JPL), InterDigital, FiberCop

- 3GPP RAN3#125 Chairman’s notes (RAN3_125_agenda_20240823EOM1.doc)

- 3GPP TS 38.423, NG-RAN; Xn Application Protocol (XnAP), V18.4.0 (2024-12)