3GPP TSG-RAN WG3 Meeting #128	R3-253503

St. Julian’s, Malta, 19th – 23rd May, 2025

Title: 	[DRAFT] Reply LS from SA5 on Continuous MDT for AI/ML

Source: 	Huawei, FiberCop, Jio Platforms

Agenda item:	11.4

Document Type:	Discussion and decision

# 1. Introduction

This is a draft reply LS to SA5 on Continuous MDT based on our contribution in [1].

# 2. Reference

- R3-253502, Discussion on the Reply LS from SA5 on Continuous MDT for AI/ML, Huawei, FiberCop, Jio Platforms

# 3. Draft Reply LS to SA5

3GPP TSG-RAN3 Meeting #128	R3-25abcd

St. Julian’s, Malta, 19th – 23rd May, 2025

Title:	[Draft] Reply LS to LS on Continuous MDT

Response to:	R3-253024 (S5-252114 Reply LS to LS on Continuous MDT)

Release:	Rel-19

Work Item:	NR_AIML_NGRAN_enh-core

Source:	Huawei [to be RAN3]

To:	SA5

Cc:

Contact person:	Damiano Rapone

damiano.rapone@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN3 thanks SA5 for their Reply LS to LS on Continuous MDT. RAN3 discussed the Continuous MDT topic further and provides comments to the SA5 answers as follows.

1st input

RAN3: “- The OAM provides to the RAN means to identify that the Management Based MDT is for Continuous MDT, e.g. reusing existing Management Based MDT configuration information.” Answer from SA5: In SA5 point of view, it is feasible for OAM to provide the configuration mechanisms to RAN to identify Management based MDT for continuous MDT. SA5 expects that RAN3 could provide the configuration details as OAM requirements.

[RAN3 input on OAM requirements] RAN3 believes that OAM needs to provide the gNB with a configuration encompassing both Immediate MDT and Logged MDT measurements and indicates that this configuration is used for Continuous MDT. To this end, RAN3 would ask SA5 to discuss further based on this requirement and the summary of solutions proposed in RAN3 in the following Table, and provide guidance to RAN3 on solution(s):

| Solution ID | Description | Measurement Continuity | Trace correlation | Pros. & Cons. |
| --- | --- | --- | --- | --- |
| Solution 1 (R3-247537) | - OAM configures Management Based MDT to the gNBs taking part in Continuous MDT, including TRs dedicated to Continuous MDT; - Serving gNB selects a UE and configures TRSR which is reassigned to the UE across RRC states and during connected mobility. Involved domains: RAN and OAM | - explicit indication in handover signalling that UE is selected for Continuous MDT (e.g., assigned TR/TRSR); - presence of TR/TRSR in the Logged MDT report sent to the serving gNB after idle-to-connected transition. | Reusing part/all of the identifiers in the Trace Record header, e.g. the TRSR, for the whole duration of the Continuous MDT trace. | Pros: only RAN and OAM involved. Cons: - Dedicated TRs for Continuous MDT; - gNB coordination in TRSR assignment to enable trace correlation. |
| Solution 2 (R3-247419) | - OAM configures Management Based MDT with a new Job Type “Immediate MDT and Logged MDT” and assigns a unique TR to this MDT session (valid for all UEs in the area of gNBs where this configuration is provided); - The gNB selects a UE and assigns a TRSR which is UE-specific; - At connected-to-idle transition the MDT configuration (at least Job Type and TRSR) is sent by the serving gNB to the 5GC/AMF which stores this information; when the UE resumes the (new) serving gNB receives from 5GC/AMF the stored MDT configuration; the presence of the MDT configuration is an indication for the (new) serving gNB that the UE needs to be reselected for Continuous MDT - At handover, the source gNB sends the MDT configuration (at least Job Type and TRSR) to the target gNB; the presence of the MDT configuration is an indication for the target gNB that the UE needs to be reselected for Continuous MDT. Involved domains: RAN, OAM and 5GC/AMF | - source gNB provides the target gNB with MDT configuration which is an indication for the target gNB to re-select the UE for Continuous MDT; - serving gNB receiving from 5GC/AMF the MDT configuration previously stored in the 5GC/AMF by the old serving gNB, so that the same UE can be re-selected. | Ensured by correlating at TCE the MDT reports having the same couple TR/TRSR: the TR links the MDT reports one to each other (as they both belong to the same MDT session) while the TRSR links the MDT reports to a specific UE. | Pros: no need for gNB coordination in TRSR assignment. TRSR conflicts at the target gNB (or new serving gNB) are solved by keeping the mapping relationship of TRSRs allocated to the UE (and sent to the TCE). Cons: OAM impacted due to the need to define a new Job Type. |
| Solution 3 (R3-247393) | - OAM configures gNBs in an area with same immediate MDT configuration (TR1) and logged MDT configuration (TR2); - A gNB chooses UE and configures the UE for Immediate MDT and Logged MDT separately (with separate TRSRs, i.e. TRSR1 and TRSR2); - At connected-to-idle transition the serving gNB sends the Immediate MDT report (TR1/TRSR1) to TCE by also including the TR2/TRSR2 assigned for the Logged MDT; - At idle-to-connected transition the (new) serving gNB sends the Logged MDT report (TR2/TRSR2) to TCE by also including the TR1/TRSR3 assigned for the Immediate MDT. Involved domains: RAN and OAM | - target gNB receives at handover the UE’s MDT configuration used in the source gNB; - (new) serving gNB receives the UE’s MDT configuration previously stored in the 5GC/AMF. | Ensured by correlating at TCE the Immediate MDT report (TR1/TRSR1) which includes also TR2/TRSR2 of the Logged MDT session with the Logged MDT report (TR2/TRSR2). | Pros: no need for gNB coordination in TRSR assignment. Cons: Multiple TRSRs are assigned to the same UE even if there are no TRSR conflicts. |

2nd input

RAN3: “- RAN3 will derive solutions for the Measurement Continuity Problem across RRC_Connected mobility. SA5 is requested to focus on the Measurement Continuity Problem across different RRC states.” Answer from SA5: SA5 would like RAN3 to provide some clarifications on what problem is referred by “the Measurement Continuity Problem across different RRC states”.

[RAN3 input on Measurement Continuity Problem across different RRC states] RAN3 would like to refer to TR 38.743 clause 5.4 for the description of the Measurement Continuity Problem, complemented by the following minimal call flow showing the interaction among the involved network entities and which has no relation with a particular solution for Continuous MDT in the Table above.

Figure 1 - Call flow showing the Measurement Continuity Problem across different UE’s RRC states.

- The OAM provides gNBs in a certain area with a Continuous MDT configuration identified by parameters including Trace Reference(s), Job Type(s), etc.;

- gNB1 selects a connected mode UE (UE1) for Continuous MDT and assigns Trace Recording Session Reference(s) to UE1;

- gNB1 configures UE1 with MDT measurements (both immediate and logged measurements) according to the Continuous MDT configuration as received in step 1.;

- UE1 performs Immediate MDT measurements according to the configuration received from gNB1 in step 3. and reports such measurements to gNB1;

- gNB1 sends to TCE a Trace Record file containing Immediate MDT measurements of UE1 including mean(s) to enable trace correlation at TCE (which “mean(s)” is FFS);

- UE1 moves to RRC idle/inactive and starts Logged MDT measurements according to the configuration received from gNB1 in step 3.;

- After some time, UE1 goes back to RRC connected under the coverage of a different gNB (gNB2);

NOTE: The call flow works also if UE1 goes back to RRC connected under the coverage of the same gNB (gNB1).

- Based on the definition of Continuous MDT, UE1 needs to be reselected by the new serving gNB2 (FFS how gNB2 understands that UE1 was previously selected by gNB1 for Continuous MDT) and it is assigned with Trace Recording Session Reference(s) (FFS how to solve the issue of TRSR(s) allocation conflict);

- gNB2 configures UE1 with MDT measurements (both immediate and logged measurements) according to the Continuous MDT configuration as received in step 1.;

- gNB2 retrieves the Logged MDT measurements from UE1;

- gNB1 sends to TCE a Trace Record file containing Logged MDT measurements of UE1 including mean(s) to enable trace correlation at TCE (which “mean(s)” is FFS);

- TCE correlates Trace Record files relying on “mean(s)” to enable trace correlation (which “mean(s)” is FFS).

3rd input

RAN3: “To solve the Trace Correlation Problem, namely to enable correlation of MDT measurements generated by the same UE, it is required to use a temporary identifier for the Continuous MDT, not revealing the UE identity. Such identifier should be made available at the TCE to enable correlation of the management-based MDT reports collected during the Continuous MDT, in a way that minimises the impacts on the Management Based MDT solution.” Answer from SA5: SA5 agrees that at least TR and TRSR (which does not reveal UE identity) could be used for the correlation of the management-based MDT reports at TCE. SA5 would like RAN3 to elaborate the details of how to avoid overlapping

[RAN3 input on Trace Correlation Problem] RAN3 would like to check with SA5 about the meaning of “overlapping”. RAN3 discussed this aspect and a possible interpretation could be related to a conflict in the allocation of the TRSR to the UE by the serving gNB after a mobility event of the UE. RAN3 asks SA5 to confirm this understanding.

# 2	Actions

To SA5

ACTION: 	RAN3 respectfully asks SA5 to take the above RAN3 inputs into consideration and provide feedback to RAN3 on possible solution(s) for Continuous MDT as soon as possible.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#129				2025-08-25 – 2025-05-29				Bangalore, IN

RAN3#129bis			2025-10-13 – 2025-10-17				Prague, CZ