3GPP TSG-RAN WG3 Meeting #128	R3-253507

St. Julian’s, Malta, 19th – 23rd May, 2025

Title:	Discussion on the reply LS from SA5 on UE throughput calculation in Rel-18

Agenda Item:	8.1

Source:	Huawei, Lenovo, Deutsche Telekom, BT

Document for:	Discussion

# 1	Introduction

Since RAN3#124 meeting (May 2024), RAN3 is discussing how to properly derive UE-level UE performance metrics introduced as part of the Rel-18 WI “Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [1] and a clear conclusion is still missing.

In this contribution we discuss further the topic, in light of the reply LS from SA5 in [2] pertaining the interpretation of the UE throughput measurements as currently specified in TS 28.558 [6].

# 2	Discussion

## 2.1 Background

In the context of the Rel-18 “AI/ML for NG-RAN” RAN3 introduced in TS 38.423 [3] a set of metrics representing the UE performance that could be requested by the source gNB to be reported by the target gNB via the Data Collection Reporting (Initiation) procedures after an AI/ML-driven mobility decision, i.e., handover. Such UE performance could be used as feedback information to the source gNB for further optimization of the AI/ML model which recommended the previous UE’s mobility action from source gNB to target gNB.

#### 9.2.3.179 UE Performance This IE indicates the UE performance measurements metrics. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description. Average UE Throughput DL; O; Bit Rate 9.2.3.4. Average UE Throughput UL; O; Bit Rate 9.2.3.4. Average Packet Delay; O; 9.2.3.187. Average Packet Loss DL; O; Packet Loss Rate 9.2.3.11. **** Skip non-relevant text ****

The above metrics, i.e., Average UE Throughput DL, Average UE Throughput UL, Average Packet Delay and Average Packet Loss DL, are intended to be collected by the target gNB within a short time window starting immediately after the handover and on a per-UE basis. However, there is no clear understanding on how a gNB could perform such per-UE measurements because the corresponding measurement definitions in relevant specifications, e.g., TS 28.558 and TS 37.320, are specified for a different granularity, i.e., not per-UE.

Observation 1: UE performance metrics in TS 38.423 are collected on a per-UE basis. However, there is no clear understanding in RAN3 on how a gNB could derive such per-UE measurements.

In RAN3#124 meeting (May 2024) the contribution in [4] made an attempt to address the issue of how the gNB could derive the UE-level UE performance metrics introduced by RAN3 and, among the proposed options, it was captured in RAN3#124 meeting minutes the common understanding that, for Rel-18, UE level performance is derived as the average value of per-DRB level measurements of the UE performance metrics [5].

#### UE level performance is derived as the average value of per DRB level measurements of the UE. **** Skip non-relevant text ****

There have been attempts to capture the above common understanding in TS 38.423 clause 9.2.3.179 clarifying that the value of each IE of the UE Performance results from averaging the corresponding DRB level measurements over all configured DRBs in the UE, and this only applies to UE-level UE performance metrics. However, it was argued in RAN3 that TS 28.558 [7], clause 6.3.1.4.1 (6.3.1.4.2), already refers to the Average DL UE Throughput in gNB (Average UL UE Throughput in gNB) as a per-UE measurement, even though the measurement name starts with the prefix “DRB.”.

Observation 2: By following the definitions of Average UE Throughput DL/UL in TS 28.558 clauses 6.3.1.4.1/6.3.1.4.2, it can be argued that these measurements are already specified as per-UE, even though the measurement name starts with the prefix “DRB.”.

## 2.2 On the reply LS from SA5 in [2]

Seeking for some clarification from SA5 on what has been actually specified in TS 28.558, RAN3 agreed to send the LS to SA5 in [7] during RAN3#126 [8]. An excerpt from the RAN3 LS is reported below for convenience:

*** skip unmodified parts *** As part of the Rel-18 Work Item on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN, RAN3 agreed to specify UE performance metrics in TS 38.423 to be exchanged over the Xn interface from target gNB to source gNB after a handover event occurs. Among the specified UE performance metrics, Average UE Throughput DL and Average UE Throughput UL are considered to be reported by the target gNB with a per-UE granularity. For the Average UE Throughput DL and Average UE Throughput UL, RAN3 would like to ask SA5 whether the formulas in fields c) of clauses 6.3.1.4.1 and 6.3.1.4.2 in TS 28.558, respectively, refer to measurements that are specified with a per-UE granularity or with a per-DRB granularity as the measurement name in field e) of the same mentioned clauses starts with “DRB.”. RAN3 would appreciate an explanation of why the measurement name starts with the “DRB.” prefix. *** skip unmodified parts *** To SA5 ACTION: RAN3 respectfully asks SA5 to provide feedback on the above question.

In their reply LS in [2] SA5 states the following:

*** skip unmodified parts *** SA5 confirms that the formulas specified in fields (c) of clauses 6.3.1.4.1 and 6.3.1.4.2 in TS 28.558 calculate average UE throughput based on per DRB level measurement for a UE. The formulas aggregate data across DRBs for each UE, ensuring that the reported throughput measurement represents the overall throughput per UE rather than being limited to individual DRBs. The rationale for using the "DRB." prefix in the measurement names, as listed in field (e), is to indicate that the measurements originate from data transmissions occurring at the DRB level. However, these measurements are aggregated to provide a per-UE throughput metric. Specifically: - The formulas include a summation over DRBs, ensuring that the result represents the total throughput per UE rather than individual DRB-level values. - The "DRB." prefix in field (e) indicates that the contributing measurements originate from DRBs, but they are aggregated to reflect per-UE throughput. - The definitions in the tables in field (c) in clauses 6.3.1.4.1 and 6.3.1.4.2 are based on DRB level measurements, but the summation ensures that the final metric is at the UE level. *** skip unmodified parts ***

From the above, SA5 confirms that measurements specified in TS 28.558 clauses 6.3.1.4.1/6.3.1.4.2 for the Average UE Throughput DL/UL are on a per-UE granularity, and originates from per-DRB measurements for the UE.

Observation 3: SA5 confirmed that Average UE Throughput DL/UL in TS 28.558 clauses 6.3.1.4.1/6.3.1.4.2 are per-UE measurements, originating from corresponding per-DRB measurements being aggregated to reflect a per-UE throughput.

In our view, measurements as specified in TS 28.558 clauses 6.3.1.4.1 and 6.3.1.4.2 for the Average UE Throughput DL and Average UE Throughput UL, respectively, can be referred in TS 38.423 for the UE Performance IE. From a pure technical perspective, it should be recalled that, in order to monitor the per-UE UE performance as a consequence of an AI/ML-driven handover, it could be needed to collect per-UE UE performance metrics before the handover (hence by the source node) which will then be compared to the same metrics but collected after the handover (hence by the target node). To make the comparison meaningful, it needs to be ensured that both source and target nodes apply the same approach in measuring UE performance metrics (average UE throughput in the discussion so far). We think that this purpose may be achieved by mandating the source and target nodes to refer to measurements in TS 28.558.

Therefore, we propose the following:

Proposal 1: RAN3 to agree to introduce a reference to TS 28.558 for Average UE Throughput DL and Average UE Throughput UL measurements in TS 38.423 pertaining the throughput measurements as part of the UE Performance IE.

## 2.3 On per-UE Average Packet Delay and Average Packet Loss DL measurements

It should be discussed how to avoid any misunderstanding in the derivation of the remaining per-UE UE performance metrics, i.e., Average Packet Delay and Average Packet Loss DL. In our understanding, such measurements are currently defined in TS 28.558 on a per-DRB per-UE granularity, meaning that if corresponding per-UE measurements are needed the only way to derive them is by following the RAN3 common understanding as captured in RAN3#124 meeting.

Observation 4: Average Packet Delay and Average Packet Loss DL are defined in TS 28.558 on a per-DRB per-UE granularity. The corresponding per-UE measurements may be derived by averaging per-DRB measurements over all the configured DRBs for the UE (as per the common understanding from RAN3#124).

As part of the ongoing discussion in Rel-19 on the granularity of UE performance metrics, there is an agreement related to whether the Average Packet Loss DL is a meaningful measurement in the way it has been specified in Rel-18, as per the outcome of the discussion in RAN3#127bis [9]:

A Rel18 correction to the Average Packet Loss DL IE may be made when agreements on whether and how to specify Average Packet Loss in Rel19 are taken.

Following the same approach, the applicability of the RAN3 common understanding on how to derive per-UE Average Packet Loss DL from per-DRB per-UE measurements may be deferred once the related discussion in Rel-19 is concluded. But for the Average Packet Delay DL/UL, we think that a clarification in the semantics description stating that the corresponding per-UE measurement is derived following the RAN3 common understanding (i.e., averaging per-DRB measurements across all configured DRBs for the UE).

Proposal 2: RAN3 to agree to clarify in the semantics description of the Average Packet Delay IE that the corresponding per-UE measurement is derived by averaging per-DRB measurements across all configured DRBs for the UE.

A XnAP CR covering the above proposals is in [10].

# 3	Conclusion

This document proposes the following observation and proposals:

Observation 1: UE performance metrics in TS 38.423 are collected on a per-UE basis. However, there is no clear understanding in RAN3 on how a gNB could derive such per-UE measurements.

Observation 2: By following the definitions of Average UE Throughput DL/UL in TS 28.558 clauses 6.3.1.4.1/6.3.1.4.2, it can be argued that these measurements are already specified as per-UE, even though the measurement name starts with the prefix “DRB.”.

Observation 3: SA5 confirmed that Average UE Throughput DL/UL in TS 28.558 clauses 6.3.1.4.1/6.3.1.4.2 are per-UE measurements, originating from corresponding per-DRB measurements being aggregated to reflect a per-UE throughput.

Proposal 1: RAN3 to agree to introduce a reference to TS 28.558 for Average UE Throughput DL and Average UE Throughput UL measurements in TS 38.423 pertaining the throughput measurements as part of the UE Performance IE.

Observation 4: Average Packet Delay and Average Packet Loss DL are defined in TS 28.558 on a per-DRB per-UE granularity. The corresponding per-UE measurements may be derived by averaging per-DRB measurements over all the configured DRBs for the UE (as per the common understanding from RAN3#124).

Proposal 2: RAN3 to agree to clarify in the semantics description of the Average Packet Delay IE that the corresponding per-UE measurement is derived by averaging per-DRB measurements across all configured DRBs for the UE.

A XnAP CR covering the above proposals is in [10].

# 4	References

- RP-233441, “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (CMCC, Ericsson), 3GPP RAN#102

- R3-253023 (S5-252056), Reply LS on per-UE UE performance metrics (Response to R3-247775), Rel-19, SA5 (NEC, Intel, Ericsson)

- 3GPP TS 38.423, NG-RAN; Xn Application Protocol (XnAP), Rel-18

- R3-243578, Discussion on definition of UE performance metrics, CATT, Deutsche Telekom, Nokia, China Telecom, China Unicom

- 3GPP RAN3#124 Chairman’s notes (RAN3_124_agenda_20240524_eom1)

- 3GPP TS 28.558, Management and orchestration; UE level measurements for 5G system, Rel-18, v18.0.0

- R3-247775, LS on per-UE UE performance metrics, Rel-18, RAN3 (Huawei)

- 3GPP RAN3#126 Chairman’s notes (RAN3_126_agenda_20241122_EOM1)

- 3GPP RAN3#127bis Chairman’s notes (RAN3_127bis_agenda_20250415_EOM1)

- R3-253508, Correction on UE performance feedback, CR#1495 to TS 38.423, Huawei, Lenovo, Deutsche Telekom, BT