3GPP TSG-RAN WG3 Meeting #128	R3-253509

St. Julian’s, Malta, 19th – 23rd May, 2025

Title:	Further discussion on UAV altitude information reporting

Agenda Item:	8.1

Source:	Huawei, Nokia

Document for:	Discussion

# 1	Introduction

This contribution further discusses RAN aspects and impacts to enable the UAV altitude reporting based on the latest updates to TS 23.256 as agreed by SA2 which follow the (reply) “LS on UAV regulation from RAN3” [1] that RAN3 sent to SA2 in RAN3#127 meeting.

# 2	Discussion

## 2.1 Background

In RAN3#127 meeting (Athens 2025) RAN3 discussed further the network-based mechanism for identifying, detecting and reporting the flight status information associated to Rel-18 UAV UEs based on the feedback from SA2 in [2]. Such feedback from SA2 is included in the reply LS to the LS sent by RAN3 to SA2 (Cc: RAN, SA, SA1) in [3], where RAN3 asked SA2 for some guidance on whether there was a requirement for the NG-RAN node to provide flight information to CN for both Rel-18 UAV UEs and pre Rel-18 non-UAV devices.

As part of its feedback, SA2 indicated in [2] that, although no requirements exist, the flight status information reporting from NG-RAN to the CN pertaining to Rel-18 UAV UEs could be supported based on the current specification work that SA2 is conducting as part of the Rel-19 WI “Uncrewed Aerial System Phase 3” (UAS_Ph3) [4]. More specifically, SA2 specified new features in Rel-19, in particular the “NEF Assisted In-flight Monitoring” which is detailed in clause 5.12.3 of TS 23.256 [5]. For these features, SA2 also specified means to instruct the UAV UEs to perform altitude reporting in clause 5.16 of TS 23.256, which can be done via application layer or via node-level signalling.

The typical scenario where the network-based mechanism mentioned above may be needed can refer to the “NEF Assisted In-flight Monitoring” feature specified in Rel-19 by SA2 based on the node-level signalling solution in TS 23.256, for which SA2 already foresaw the NG-RAN to be required to report flight status information to the CN/AMF which, in turn, will report such information to the UAV monitoring platform via the NEF. The general assumption for the node-level signalling solution is that the NG-RAN relies on the existing mobility measurements in RRC to trigger a UE report when the altitude has reached a certain threshold, i.e., events H1 and H2 as specified in clause 5.5.4.21 and clause 5.5.4.22 of TS 38.331 [6].

The outcome of the discussion in RAN3#127 meeting is reported as follows [7]:

No TU on UAV WI in RAN WGs in Rel-19. The enhancement on this topic shall be limited without any impact on RAN2 and reusing the NGAP location reporting procedure. LS to SA2 on UAV regulation from RAN3 in R3-250865 - Update to final LS format Rev in R3-250876 Agreed unseen Stage2 CR on Correction on altitude information reporting for aerial UE in R3-250866 Correction on altitude information reporting for aerial UE for TS38.413 in R3-250867 Correction on altitude information reporting for aerial UE for TS38.423 in R3-250868 Taking the above CRs as the baseline for TEI19 UAV enhancements, RAN3 CRs are TEI19, Cat.B with unique TEI identifier (using SA2 WI code). The existing Location Report Control and Location Report procedures can be reused to configure and perform altitude reporting. Three report types i.e., periodical, threshold-based, event-based, can be supported to report altitude information based on NG-RAN node implementation.

As per the above, RAN3 agreed to reuse the existing Location Report Control and Location Report procedures defined in TS 38.413 [8] to configure and perform the UAV altitude reporting, respectively – meaning that it is feasible from RAN3 perspective that the NG-RAN can provide the UAV UE altitude information to CN upon configuration – and that the UAV altitude reporting can be configured to be periodic, threshold-based or event-based based on NG-RAN implementation. The agreements have been captured in a new (reply) LS in [1] to make SA2 aware of the discussion status in RAN3 and to ask for further feedback from SA2 on whether the CN needs to indicate derived altitude thresholds and reporting periodicity for the UAV UE's altitude reporting.

## 2.2 Latest progress in SA2

In SA2#168 meeting (April 2025) SA2 discussed the LS from RAN3 in [1] and, based on this LS, SA2 agreed the CR to TS 23.256 in [9] to clarify that the existing NGAP’s LOCATION REPORTING CONTROL message sent from AMF to NG-RAN node is used for the UAV altitude reporting configuration. Moreover, for the actual reporting from NG-RAN to AMF, it is clarified that the N2 message used to transfer the UAV altitude information is the existing NGAP’s LOCATION REPORT message. To complete the picture, it is also worth to note that SA2 agreed to change the previously used terminology of “node-level signalling” and adopted the “3GPP-defined signalling” term to indicate the network-based mechanism enabling the UAV UE’s altitude reporting.

Observation 1: SA2 agreed to clarify in TS 23.256 that the UAV altitude reporting configuration is provided from AMF to the NG-RAN within the LOCATION REPORTING CONTROL message, as per RAN3 agreement.

Observation 2: SA2 also agreed to clarify in TS 23.256 that the reporting of UAV altitude information is performed via the LOCATION REPORT message, as per RAN3 agreement.

On top of that, SA2 also agreed another CR to TS 23.256 in [10] which introduces a new clause in the TS pertaining a new procedure to instruct a UAV UE to stop and/or update the altitude reporting due to a trigger from NG-RAN and/or a request from the UAV monitoring platform. The main motivation for this is that it is not clear how the UAV UE’s altitude reporting is stopped and/or updated, e.g., when the UAV UE reaches the destination point or when the 5GC needs to change the derived altitude reporting thresholds. Also in this case SA2 specified means to address this scenario via either application layer or 3GPP-defined signalling (formerly known as “node-level signalling”), and only the latter one may have RAN3 impacts.

Observation 3: SA2 agreed to introduce in TS 23.256 a new clause describing a new procedure to instruct a UAV UE to stop and/or update the altitude reporting due to a trigger from NG-RAN and/or a request from the UAV monitoring platform.

Observation 4: The 3GPP-defined signalling option may have RAN3 impacts.

As part of this new procedure for instructing a UAV UE to stop/update the altitude reporting, for the 3GPP-defined signalling option, it is stated in TS 23.256 that the existing NGAP’s LOCATION INFORMATION FAILURE INDICATION message is used to provide the AMF with information about the configured altitude reporting. In our understanding, the 5GC would be able to assess whether to stop and/or update the UAV’s altitude reporting based on the content of the LOCATION REPORTING FAILURE INDICATION message. As a consequence of the 5GC decision, the procedure includes a step where the AMF sends a LOCATION REPORTING CONTROL message to the NG-RAN node which, in turn, will trigger the release of the configured altitude reporting for the UAV UE (hence stopping the reporting).

Observation 5: SA2 specified in the new procedure for instructing a UAV UE to stop/update the altitude reporting that the NGAP’s LOCATION INFORMATION FAILURE INDICATION message is used to provide the AMF with information about the configured altitude reporting, based on which the 5GC would assess whether to stop and/or update the UAV’s altitude reporting.

SA2 agreed to send the reply LS in [11] to make RAN3 aware of the SA2 updates to TS 23.256, attaching the two CRs mentioned above. Moreover, in the reply LS in [11], SA2 asks RAN3 to provide feedback on whether the NG-RAN node can add a timestamp when it receives the UAV UE’s altitude report, indicating when the report has been received from the UE. We will discuss this aspect in section 2.3 of this document, along with some considerations on the RAN3 impacts due to the two CRs attached in [11] and briefly described in this section.

## 2.3 RAN3 impacts due to the latest changes in TS 23.256 from SA2 to enable UAV UE’s altitude reporting from NG-RAN to CN

On S2-2504470 (CR#0199 to TS 23.256) in [9]

For the CR to TS 23.256 in [9], it is our understanding that it follows the outcome of the RAN3 discussion from RAN3#127 meeting, where the existing Location Report Control and Location Report procedures defined in TS 38.413 [8] are reused to configure and perform the UAV altitude reporting, respectively.

Moreover, in the reply LS from SA2 in [11], SA2 provided feedback on whether the CN needs to indicate derived altitude thresholds and reporting periodicity for the aerial UE's altitude reporting – refer to the following excerpt from [11]:

1. Overall Description: SA2 thanks RAN3 for the LS on UAV regulation for further clarification. SA2 would like to provide the following reply. Question1: With regards to the “node-level signaling” solution configuration in clause 5.16.2, RAN3 would like to ask for further feedback from SA2 on whether the CN needs to indicate derived altitude thresholds and reporting periodicity for the aerial UE's altitude reporting. SA2 reply: For SA2 perspective, the Core Network may indicate the derived altitude thresholds and reporting periodicity for the aerial UE's altitude reporting. This aspect is further clarified in the attached CRs (#0199 and #0194 for TS 23.256). *** skip non-relevant text ***

From the above highlighted text in yellow, it can be concluded that, as part of the UAV UE’s altitude reporting configuration, the AMF may include the derived altitude thresholds and, optionally, the reporting periodicity when sending the LOCATION REPORTING CONTROL message to all applicable NG-RAN node(s) – this is reflected in the updated step 4b. of clause 5.16.2 Instructing aerial UEs to perform altitude reporting in TS 23.256 from [9]:

### 5.16.2 Instructing aerial UEs to perform altitude reporting Procedure shown in Figure 5.16.2-1 specifies how the 5GS instructs the aerial UEs (UAVs) to perform altitude reporting, considering requests from a USS/UTM for assistance with UAV's flight. This procedure can be used in conjunction with the NEF-assisted pre-flight planning, in-flight monitoring and USS changeover as specified in clauses 5.12.2, 5.12.3 and 5.13, respectively. *** skip non-relevant text *** Option B: 3GPP-defined signalling. Editor's note: This requires RAN to provide further enhancement of the procedure under RAN responsibility. 3b. The UAS NF/NEF may send an Namf_EventExposure_Subscribe request to the AMF to get notified about events of UAV's deviation from the assigned trajectory (i.e. Event ID = "Assigned Trajectory") as specified in clause 5.2.2.3.1 of TS 23.502 [3]. Inside the subscription request, the UAS NF/NEF may include the derived altitude thresholds and indication about reporting periodicity for the UE's altitude reporting. Additionally, the UAS NF/NEF may also include an indication to perform UE's altitude reporting for all aerial UEs in a specific TA(s) or NG-RAN node(s). 4b. The AMF sends the corresponding N2 messages (LOCATION REPORTING CONTROL message, see clause 8.12.1.2 of TS 38.413 [24]) to all applicable NG-RAN node(s), including the derived altitude thresholds and optionally reporting periodicity. 5b. The NG-RAN node(s) instructs the relevant UE(s) to perform altitude reporting as requested by the UAS NF/NEF (via AMF). NOTE 3: Aspects related to AMF informing NG-RAN regarding instructing UEs to perform altitude reporting are specified in TS 38.413 [24].

Observation 6: SA2 clarified that, as part of the procedure to instruct the UAV UE’s altitude reporting, the AMF may include the derived altitude thresholds and, optionally, the reporting periodicity within the LOCATION REPORTING CONTROL message to the NG-RAN node.

Proposal 1: RAN3 to agree a CR to TS 38.413 where the LOCATION REPORTING CONTROL and LOCATION REPORT messages are enhanced to include relevant information to enable the UAV UE’s altitude reporting.

To address Proposal 1, the NGAP CR in [12] needs to be used as the starting point (as captured in the RAN3#127 meeting minutes [7]) to include further changes pending the outcome of the SA2 discussion in their last meeting. The current NGAP’s LOCATION REPORTING CONTROL message contains the Location Reporting Request Type IE which, in turns, contains the Event Type IE. As per the NGAP CR in [12] we think that two new codepoints need to be added to start and stop the UAV UE’s altitude reporting.

Proposal 2: RAN3 to agree that new codepoints to start and stop the UAV UE’s altitude reporting are added in the existing Event Type IE of the Location Reporting Request Type IE within the LOCATION REPORTING CONTROL message.

Moreover, as per SA2 CR in [9], the LOCATION REPORTING CONTROL message should also include the 5GC-derived altitude thresholds and, optionally, the reporting periodicity, which may be used by the NG-RAN node for configuring the UAV UE to report its altitude information. We think that these pieces of information can be transferred by means of a new Aerial UE Altitude Information Reporting Configuration IE within the Location Reporting Request Type IE of the LOCATION REPORTING CONTROL message. For the encoding of the new IE, we can:

- refer to the Altitude IE as defined in TS 38.331 [6], which is an INTEGER (- 420..10000,…) representing the UAV UE’s altitude in meters, for the 5GC-derived minimum and maximum altitude thresholds;

- refer to the ReportInterval IE as defined in TS 38.331 [6], which is an ENUMERATED type of IE representing the interval between periodical reports, for the reporting periodicity.

Proposal 3: RAN3 to agree that a new Aerial UE Altitude Information Reporting Configuration IE is added in the Location Reporting Request Type IE of the LOCATION REPORTING CONTROL message.

Proposal 3bis: RAN3 to agree that the minimum and maximum altitude thresholds as well as reporting periodicity are encoded as defined in TS 38.331 for the Altitude IE and ReportInterval IE, respectively.

The current NGAP’s LOCATION REPORT message contains the User Location Information IE. In our view, and as part of the NR user location information, a new IE specific for the UAV UE’s altitude report can be introduced, as per the NGAP CR in [12]. For the encoding of the new IE, we can again refer to the Altitude IE as defined in TS 38.331 [6].

Proposal 4: RAN3 to agree that a new IE specific for the UAV UE’s altitude report can be introduced within the User Location Information IE of the LOCATION REPORT message.

Proposal 4bis: The new IE specific for the UAV UE’s altitude report is encoded as the Altitude IE as defined in TS 38.331.

On the timestamp of the UAV UE’s altitude report

As briefly mentioned before, in the reply LS in [11], SA2 asks RAN3 to provide feedback on whether the NG-RAN node can add a timestamp when it receives the UAV UE’s altitude report, indicating when the report has been received from the UE – refer to the following excerpt from [11]:

*** skip non-relevant text *** Additionally, SA2 has specified that USS can benefit from knowing when altitude was reported from the UE and would like to check whether RAN3 has reached the agreement on NG-RAN providing a timestamp for UAV UE’s altitude information (see Step 2b in clause 5.16.3 of TS 23.256). #---excerpt from clause 5.16.3 of TS 23.256--- 2b. After receiving a report with the altitude information from the UE, the NG-RAN node should, for instance, add a time stamp indicating when the altitude information has been received from the UE; the NG-RAN sends the UE's altitude information report in an N2 message to the AMF. #--- *** skip non-relevant text ***

From the above excerpt, it can be understood that there is a SA2 requirement for the NG-RAN node to provide a timestamp indicating when the NG-RAN node has received the altitude report from the UAV UE. There is the possibility that the AMF may derive such timing indication based on the time the AMF received the LOCATION REPORT message from the NG-RAN node, however the timestamp generated by the gNB is for sure more precise, since the gNB could not guarantee to report the altitude information immediately upon reception of the UE report. Therefore we think RAN3 should agree to send the timestamp along with the altitude report within the LOCATION REPORT message.

Proposal 5: RAN3 to agree to define the timestamp of the altitude report as the point in time at which the NG-RAN node received the altitude report from the UAV UE, following the SA2 definition in TS 23.256.

Proposal 6: RAN3 to agree to send the timestamp of the altitude report within the LOCATION REPORT message, along with the UAV UE’s altitude report, from NG-RAN to AMF.

The current User Location Information IE within the LOCATION REPORT message contains the Age of Location IE. Such IE is already encoded as Time Stamp IE (refer to clause 9.3.1.75 of TS 38.413), and we think that it can be reused as timestamp of the altitude report sent within the LOCATION REPORT message.

Proposal 7: RAN3 to agree that, for the timestamp of the altitude report, the existing Age of Location IE of the User Location Information IE within the LOCATION REPORT message can be reused.

The NGAP CR reflecting the above proposals can be found in [13].

A draftCR to TS 38.300 and a CR to TS 38.410 are also provided in [17] and [18], respectively.

In addition to the NGAP CR in [12], in RAN3#127 also the XnAP CR in [14] was proposed to be agreed and, similarly, it was captured in RAN3#127 meeting minutes [7] that such XnAP CR should be taken as a starting point to include further changes pending the outcome of the SA2 discussion in their last meeting. In our understanding the XnAP specification should be enhanced to enable the UAV UE to keep on reporting its altitude information during mobility and, for this purpose, the existing Location Reporting Information IE included in both the HANDOVER REQUEST and RETRIEVE UE CONTEXT RESPONSE messages should be enhanced.

Proposal 8: RAN3 to agree a CR to TS 38.423 where the existing Location Reporting Information IE included in both the HANDOVER REQUEST and RETRIEVE UE CONTEXT RESPONSE messages is enhanced to include relevant information to enable the UAV UE’s altitude reporting during mobility events.

In our view the current Event Type IE within the Location Reporting Information IE should be enhanced to only include the new codepoint needed to start the UAV UE’s altitude reporting. This because it should be the AMF (5GC) which eventually decides to stop the reporting also after the mobility event, not the source (or old) gNB.

Proposal 9: RAN3 to agree that a new codepoint to start the UAV UE’s altitude reporting is added in the existing Event Type IE of the Location Reporting Information IE within the HANDOVER REQUEST and RETRIEVE UE CONTEXT RESPONSE messages.

And finally we think that a new Aerial UE Altitude Information Reporting Configuration IE within the Location Reporting Information IE of the HANDOVER REQUEST and RETRIEVE UE CONTEXT RESPONSE messages should be introduced to propagate over Xn the configuration to be used by the target (new) gNB to configure altitude thresholds for reporting and, optionally, reporting periodicity to the UAV UE. The encoding of this new IE follows the same considerations previously done for the NGAP CR.

Proposal 10: RAN3 to agree that a new Aerial UE Altitude Information Reporting Configuration IE is added in the Location Reporting Information IE of the HANDOVER REQUEST and RETRIEVE UE CONTEXT RESPONSE messages.

Proposal 10bis: RAN3 to agree that the minimum and maximum altitude thresholds as well as reporting periodicity are encoded as defined in TS 38.331 for the Altitude IE and ReportInterval IE, respectively.

The XnAP CR reflecting the above proposals can be found in [15].

On S2-2504375 (CR#0194 to TS 23.256) in [10]

For the CR to TS 23.256 in [10], this is related to the new clause in the SA2 TS pertaining a procedure to instruct a UAV UE to stop and/or update the altitude reporting due to a trigger from NG-RAN and/or a request from the UAV monitoring platform. As mentioned in section 2.2, such procedure can be via either application layer or 3GPP-defined signalling (formerly known as “node-level signalling”), and only the latter one may have RAN3 impacts.

To be specific, for the 3GPP-defined signalling option, it is stated in TS 23.256 that the existing NGAP’s LOCATION INFORMATION FAILURE INDICATION message is used to provide the AMF with information about the configured altitude reporting. In our understanding, the 5GC would be able to assess whether to stop and/or update the UAV’s altitude reporting based on the content of the LOCATION REPORTING FAILURE INDICATION. As a consequence of the 5GC decision, the procedure includes a step where the AMF sends a LOCATION REPORTING CONTROL message to the NG-RAN node which, in turn, will trigger the release of the configured altitude reporting for the UAV UE (hence stopping the reporting) – this is reflected in the step 2b. and step 7b-8b. of clause 5.16.X Stop and update altitude reporting for aerial UEs in TS 23.256 from [10], respectively:

### 5.16.X Stop and update altitude reporting for aerial UEs Procedure shown in Figure 5.16.X-1 specifies how the 5GS instructs the aerial UEs to stop and/or update their altitude reporting due to a trigger from NG-RAN and/or a request from the USS. *** skip non-relevant text *** 0. The UAV UE is requested to perform altitude information reporting as specified in clause 5.16.2, and the UAV UE performs its altitude information reporting as specified in clause 5.16.3. 1. In case the UAV UE fails to report the altitude information and/or the NG-RAN to receive the altitude information, the UAS NF/NEF or the USS may decide to stop/update the UAV UE’s altitude reporting. NOTE X1: The UAS NF/ NEF or the serving USS can also decide to stop UAV UE’s altitude reporting in cases when the UAV UE deviates from the assigned flight trajectory, which can be detected during the in-flight monitoring (clause 5.12.3) or the USS changeover (clause 5.13.2), or when the DAA procedure is triggered to avoid a collision (clause 5.14). *** skip non-relevant text *** Option B: 3GPP-defined signalling. 2b. The NG-RAN node sends an N2 message(LOCATION REPORTING FAILURE INDICATION message, see clause 8.12.2.2 of TS 38.413 [24]) to the AMF with information about the configured altitude information reporting. *** skip non-relevant text *** 7b-8b. The AMF sends the corresponding N2 message (LOCATION REPORTING CONTROL message, see clause 8.12.1.2 of TS 38.413 [24]) to the NG-RAN node, and the NG-RAN triggers the release of the configured altitude reporting for the UAV UE, thereby stop the altitude reporting, as specified in TS 38.331 [23]. NOTE X3: Aspects related to AMF informing NG-RAN regarding start/stop of UAV UE’s altitude reporting are specified in TS 38.413 [24].

Firstly, it should be clarified by SA2 how, in step 1., the UAS NF/NEF or the USS may decide to stop/update the UAV UE’s altitude reporting in case the UAV UE fails to report the altitude information and/or the NG-RAN is not able to receive the altitude information from the UAV UE. In other words, should the missing reporting from the NG-RAN at the AMF be interpreted as an implicit trigger for the UAS NF/NEF or the USS to take a proper decision on the ongoing altitude reporting?

Observation 7: It is unclear how, in step 1., the UAS NF/NEF or the USS may decide to stop/update the UAV UE’s altitude reporting in case the UAV UE fails to report the altitude information and/or the NG-RAN is not able to receive the altitude information from the UAV UE.

Then, in the step 2b. of the SA2 procedure in [10], the NGAP’s LOCATION REPORTING FAILURE INDICATION message is mentioned. Such message is defined in TS 38.413 [8] as part of the Location Reporting Failure Indication procedure, whose purpose is to allow the NG-RAN node to inform the AMF that the location reporting request contained in the Location Reporting Control procedure has failed – see highlighted text in yellow below:

### 8.12.2 Location Reporting Failure Indication #### 8.12.2.1 General The purpose of the Location Reporting Failure Indication procedure is to allow the NG-RAN node to inform the AMF that the location reporting request contained in the Location Reporting Control procedure, the Handover Resource Allocation procedure or the Initial Context Setup procedure has failed. The procedure uses UE-associated signalling. *** skip non-relevant text ***

However, based on the SA2 CR in [10], it is our understanding that the intention is to address the scenario where the NG-RAN triggers the 5GC to stop/update an ongoing measurement (UAV UE’s altitude) reporting(s) via an appropriate triggering condition included in the LOCATION REPORTING FAILURE INDICATION message.

We think that there is an issue if the NG-RAN trigger is provided by means of the Location Reporting Failure Indication procedure. According to the current NGAP, such procedure is initiated by the NG-RAN node when a location reporting request fails, meaning that there cannot be ongoing measurement (UAV UE’s altitude) reporting(s) prior to the sending of the LOCATION REPORTING FAILURE INDICATION message.

Observation 8: The use of the LOCATION REPORTING FAILURE INDICATION message within the SA2 procedure to instruct a UAV UE to stop and/or update the altitude reporting violates the purpose of the NGAP’s Location Reporting Failure Indication procedure as currently specified in TS 38.413.

On top of that, the reason for which, in step 2b.,  the AMF should be provided with information about the configured altitude information reporting is also unclear, and we suggest SA2 clarifying this aspect as well.

In our view, we should make SA2 aware of the concerns related to the new SA2 procedure to instruct the UE to stop/update the altitude reporting and ask to provide further clarifications on the scenario that SA2 intends to address with such procedure. Indeed, there could be multiple scenarios that may need to be supported which motivate the need for a stop/update of altitude information reporting, e.g., the UAV UE no longer reports because it has reached the flight’s destination, or a “normal” failure of the reporting from the UAV UE occurs, or the stop/update of the UAV UE is triggered by the 5GC autonomously, etc.

To this purpose, a draft reply LS to SA2 can be found in [16] which include an update on the RAN3 progress and the concern on how the NGAP’s Location Reporting Failure procedure is used in the context of the SA2 procedure to instruct a UAV UE to stop and/or update the altitude reporting.

Proposal 11: RAN3 to agree to send a reply LS to SA2 to provide an update of the RAN3 progress and highlighting the concern on how the NGAP’s Location Reporting Failure procedure is used in the context of the SA2 procedure to instruct a UAV UE to stop and/or update the altitude reporting.

A draft reply LS to SA2 is in [16].

# 3	Conclusion

This document proposes the following observation and proposals:

Latest progress in SA2

Observation 1: SA2 agreed to clarify in TS 23.256 that the UAV altitude reporting configuration is provided from AMF to the NG-RAN within the LOCATION REPORTING CONTROL message, as per RAN3 agreement.

Observation 2: SA2 also agreed to clarify in TS 23.256 that the reporting of UAV altitude information is performed via the LOCATION REPORT message, as per RAN3 agreement.

Observation 3: SA2 agreed to introduce in TS 23.256 a new clause describing a new procedure to instruct a UAV UE to stop and/or update the altitude reporting due to a trigger from NG-RAN and/or a request from the UAV monitoring platform.

Observation 4: The 3GPP-defined signalling option may have RAN3 impacts.

Observation 5: SA2 specified in the new procedure for instructing a UAV UE to stop/update the altitude reporting that the NGAP’s LOCATION INFORMATION FAILURE INDICATION message is used to provide the AMF with information about the configured altitude reporting, based on which the 5GC would assess whether to stop and/or update the UAV’s altitude reporting.

On the updated SA2 procedure to instruct the UAV UE to report the altitude reporting

Observation 6: SA2 clarified that, as part of the procedure to instruct the UAV UE’s altitude reporting, the AMF may include the derived altitude thresholds and, optionally, the reporting periodicity within the LOCATION REPORTING CONTROL message to the NG-RAN node.

Proposal 1: RAN3 to agree a CR to TS 38.413 where the LOCATION REPORTING CONTROL and LOCATION REPORT messages are enhanced to include relevant information to enable the UAV UE’s altitude reporting.

Proposal 2: RAN3 to agree that new codepoints to start and stop the UAV UE’s altitude reporting are added in the existing Event Type IE of the Location Reporting Request Type IE within the LOCATION REPORTING CONTROL message.

Proposal 3: RAN3 to agree that a new Aerial UE Altitude Information Reporting Configuration IE is added in the Location Reporting Request Type IE of the LOCATION REPORTING CONTROL message.

Proposal 3bis: RAN3 to agree that the minimum and maximum altitude thresholds as well as reporting periodicity are encoded as defined in TS 38.331 for the Altitude IE and ReportInterval IE, respectively.

Proposal 4: RAN3 to agree that a new IE specific for the UAV UE’s altitude report can be introduced within the User Location Information IE of the LOCATION REPORT message.

Proposal 4bis: The new IE specific for the UAV UE’s altitude report is encoded as the Altitude IE as defined in TS 38.331.

On the timestamp of the UAV UE’s altitude report

Proposal 5: RAN3 to agree to define the timestamp of the altitude report as the point in time at which the NG-RAN node received the altitude report from the UAV UE, following the SA2 definition in TS 23.256.

Proposal 6: RAN3 to agree to send the timestamp of the altitude report within the LOCATION REPORT message, along with the UAV UE’s altitude report, from NG-RAN to AMF.

Proposal 7: RAN3 to agree that, for the timestamp of the altitude report, the existing Age of Location IE of the User Location Information IE within the LOCATION REPORT message can be reused.

The NGAP CR reflecting the above proposals can be found in [13].

A draftCR to TS 38.300 and a CR to TS 38.410 are also provided in [17] and [18], respectively.

Proposal 8: RAN3 to agree a CR to TS 38.423 where the existing Location Reporting Information IE included in both the HANDOVER REQUEST and RETRIEVE UE CONTEXT RESPONSE messages is enhanced to include relevant information to enable the UAV UE’s altitude reporting during mobility events.

Proposal 9: RAN3 to agree that a new codepoint to start the UAV UE’s altitude reporting is added in the existing Event Type IE of the Location Reporting Information IE within the HANDOVER REQUEST and RETRIEVE UE CONTEXT RESPONSE messages.

Proposal 10: RAN3 to agree that a new Aerial UE Altitude Information Reporting Configuration IE is added in the Location Reporting Information IE of the HANDOVER REQUEST and RETRIEVE UE CONTEXT RESPONSE messages.

Proposal 10bis: RAN3 to agree that the minimum and maximum altitude thresholds as well as reporting periodicity are encoded as defined in TS 38.331 for the Altitude IE and ReportInterval IE, respectively.

The XnAP CR reflecting the above proposals can be found in [15].

On the new SA2 procedure to instruct the UAV UE to stop/update the altitude reporting

Observation 7: It is unclear how, in step 1., the UAS NF/NEF or the USS may decide to stop/update the UAV UE’s altitude reporting in case the UAV UE fails to report the altitude information and/or the NG-RAN is not able to receive the altitude information from the UAV UE.

Observation 8: The use of the LOCATION REPORTING FAILURE INDICATION message within the SA2 procedure to instruct a UAV UE to stop and/or update the altitude reporting violates the purpose of the NGAP’s Location Reporting Failure Indication procedure as currently specified in TS 38.413.

Proposal 11: RAN3 to agree to send a reply LS to SA2 to provide an update of the RAN3 progress and highlighting the concern on how the NGAP’s Location Reporting Failure procedure is used in the context of the SA2 procedure to instruct a UAV UE to stop and/or update the altitude reporting.

A draft reply LS to SA2 is in [16].

# 4	References

- R3-250876, LS on UAV regulation from RAN3 (Response to R3-250022/S2-2413011), Rel-19, RAN3 (CMCC)

- R3-250022 (S2-2413011), Reply LS on UAV regulation, Samsung

- R3-245815, LS on UAV regulation, CMCC

- SP-230518, Uncrewed Aerial System Phase 3, Rel-19

- 3GPP TS 23.256 “Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking; Stage 2”, Rel-19

- 3GPP TS 38.331 “NR; Radio Resource Control (RRC); Protocol specification”, Rel-18

- 3GPP RAN3#127 Chairman’s notes (RAN3_127_agenda_20250221_1300_EOM.doc)

- 3GPP TS 38.413 “NG-RAN; NG Application Protocol (NGAP)”, Rel-18

- S2-2504470, The indication on altitude thresholds and reporting periodicity for the aerial UE's altitude reporting, SA2 (China Mobile, LG Electronics), CR#0199 to TS 23.256, Rel-19

- S2-2504375, Instructing a UAV to stop the altitude reporting, SA2 (Ericsson, CATT), CR#0194 to TS 23.256, Rel-19

- R3-253020 (S2-2504471), Reply LS on clarification of UAV regulation from RAN3 (Response to R3-250876), Rel-19, SA2 (CMCC)

- R3-250867, Correction on altitude information reporting for aerial UE, Ericsson, CR#1249 to TS 38.413, Rel-19

- R3-253510, Introduction of aerial UE’s altitude information reporting, Huawei, Nokia, CR#1284 to TS 38.413, Rel-19

- R3-250868, Correction on altitude information reporting for aerial UE, ZTE Corporation, CR#1460 to TS 38.423, Rel-19

- R3-253512, Introduction of aerial UE’s altitude information reporting, Huawei, Nokia, CR#1496 to TS 38.423, Rel-19

- R3-253514, [DRAFT] Reply LS on clarification of UAV regulation from RAN3, Response to R3-253020 (S2-2504471), Rel-19, Huawei, Nokia

- R3-253513, Introduction of aerial UE’s altitude information reporting, Huawei, Nokia, draftCR to TS 38.300, Rel-19

- R3-253511, Introduction of aerial UE’s altitude information reporting, Huawei, Nokia, CR#0054 to TS 38.410, Rel-19