3GPP TSG-RAN WG3 Meeting #128	R3-253513

St. Julian’s, Malta, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | draftCR | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of aerial UE’s altitude information reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3, TEI19 |  |  |  |  |  | Date: |  |  | 2025-05-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on TS 23.256, for the aerial UEs, SA2 specified new procedures to enable the altitude reporting from NG-RAN to CN. Moreover, RAN3 agreed in RAN3#127 to reuse the Location Reporting Control and Location Report procedures to configure and perform the aerial UE’s altitude reporting, respectively, and that the altitude reporting can be configured to be periodic, threshold-based or event-based based on NG-RAN implementation. This CR follows the above mentioned agreements and introduces the Stage 2 description pertaining the aerial UE’s altiture reporting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The Stage 2 description for the configuration and reporting of the aerial UE’s altitude information is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Stage 2 description of the aerial UE’s altitude information reporting and configuration is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.18.1, 16.18.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.410 CR 0054 TS 38.413 CR 1284 TS 38.423 CR 1496 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Start of Changes ***

## 16.18	Support for Aerial UE Communication

### 16.18.1	General

NR connectivity for UEs capable of Aerial communication is supported via the following functionalities:

-	subscription-based Aerial UE identification and authorization, as specified in TS 23.502 [22], clause 5.2.3.3.1;

-	altitude reporting based on the measurement event(s) where the UE's altitude has crossed a network-configured reference altitude threshold;

-	altitude-dependent configurations which apply only to specific altitude regions;

-	interference detection based on a measurement reporting that is triggered when a configured number of cells (i.e. larger than one) fulfils the triggering criteria simultaneously;

-	signalling of flight path information from UE to NG-RAN and from the source gNB to target gNB during handover;

-	location information reporting, including UE's horizontal and vertical velocity;

-	transmitting of BRID and DAA messages via PC5 interface;

-	aerial UE’s altitude information reporting, as specified in TS 23.256 [60], clause 5.16.

### 16.18.2	Subscription-based Identification of Aerial UE

Support for Aerial UE functions is stored in the user's subscription information in UDM. UDM transfers this information to the AMF during Registration, Service Request and Mobility Registration Update procedures.

The Aerial UE subscription information can be provided by the AMF to the NG-RAN node via the NGAP INITIAL CONTEXT SETUP REQUEST message during the Registration, Mobility Registration Update and Service Request procedures. The subscription information can also be updated via the NGAP UE Context Modification procedure and NGAP Path Switch Request procedure. In addition, for Xn-based handover, the source NG-RAN node can include the Aerial UE subscription information in the XnAP HANDOVER REQUEST message and RETRIEVE UE CONTEXT RESPONSE message to the target NG-RAN node.

For intra- and inter-AMF NG-based handover, the AMF provides the Aerial UE subscription information to the target NG-RAN node after the handover procedure.

*** skip unmodified parts ***

### 16.18.8	BRID and DAA Support via A2X Communication

The Aerial UE supports A2X communication, as defined in 3.2. BRID relies on broadcasting while DAA can be provided either via unicast or broadcast transmissions in NR sidelink. BRID and DAA message transmission is supported in both in-coverage and out-of-coverage scenarios and relies on UE autonomous resource selection for NR sidelink communication.

BRID and DAA follow the QoS framework defined for NR sidelink and dedicated A2X PQI values are specified in table 6.2.4.1-1 of TS 23.256 [60]. The NG-RAN can configure separate SL Tx resource pool(s) for BRID and/or DAA. The procedure for SL Tx pool selection for A2X is described in TS 38.321 [6], clause 5.22.

*** Next Change ***

### 16.18.x Aerial UE’s altitude information reporting

The aerial UE’s altitude information reporting can be configured by the CN to NG-RAN based on altitude thresholds and, optionally, reporting periodicity. The Location Reporting Control procedure as specified in TS 38.413 [26] is used to provide the NG-RAN with the configuration for the aerial UE’s altitude information reporting. The Location Report procedure, as specified in TS 38.413 [26], is used to report the aerial UE’s altitude information to the CN in either a threshold-based, event-based or periodic manner based on NG-RAN implementation.

*** End of Changes ***