3GPP TSG-RAN WG3 Meeting #128	R3-253531

Malta, 19 – 23 May, 2025

Agenda Item:	11.3

Source:	Nokia

Title:	(TP for BLCR to TS 38.473) Discussion on open points for AI/ML-based CCO

Document for:	Discussion and TP

# 1	Introduction

Agreements taken for AI/ML-based CCO at previous RAN3 meetings have all been integrated in the XnAP and F1AP BL CRs [1,2].

We here provide further discussion on the following open points and propose stage 3 solutions where relevant:

- future CCO state information provided from gNB-CU to gNB-DU, e.g. whether the gNB-CU that received future CCO state from neighbour nodesprovides to its served gNB-DU, the information relative to the future CCO state

- potential additional UE performance measurement metrics

- timing information for predicted CCO issue over Xn

- update of the predicted CCO issue and/or future CCO state

# 2	Discussion

## 2.1	Future CCO state information provided from gNB-CU to gNB-DU

According to the current F1AP BLCR [2], the predicted CCO issue information is sent from the gNB-CU to the gNB-DU (in the Predicted CCO Assistance Information IE), while the future CCO state information is sent from the gNB-DU to the gNB-CU (in the Future Coverage Modification Notification IE). And, according to the current XnAP BLCR [1], the future CCO state information is sent over Xn from the gNB 1 that has decided to apply the future CCO state change to a gNB 2 controlling neighbour cells (Future Coverage Modification List IE).

An open point relative to the gNB 2 was captured at the last RAN3 meeting:

For the receiving side, whether gNB-CU can also provide to gNB-DU a future CCO state as assistance information to derive a matching future CCO state?

Option 1: no need

Option 2: gNB-CU directly forwards the received future CCO state of neighbor cells to gNB-DU.

Option 3: gNB-CU generates a recommend future CCO state and send to gNB-DU.

The considered scenario can be illustrated with the following example: gNB-CU1 (CU1) is serving three gNB-DUs, namely gNB-DU1 (DU1), DU2 and DU3, while CU2 serves DU’ as illustrated in Figure 1:

Figure 1: Illustration of scenario where CU1 is serving three DUs, namely DU1, DU2 and DU3. Similarly, CU2 is serving DU’.

Figure 2: F1 and Xn signalling for CCO issue detected by CU1 in a cell served by DU1, with neighbour cells served by other DUs in same gNB and neighbour gNB.

Figure 2 shows F1 and Xn signalling triggered by CCO issue within a cell served by DU1, with neighbour cells served by other DUs in same gNB (under CU1) and neighbour gNB (CU2). Along with predicted CCO issue, CU1 predicts the start time denoted as t0. The CU1 sends the predicted CCO issue and start time t0 to DU1 via Predicted CCO Assistance Information IE in the gNB-CU Configuration Update message. DU1 determines its own future coverage state c1 along with Time for Future Coverage Modification t1. DU1 provides the c1 and t1 information to CU1 via Future Coverage Modification Notification IE in gNB-DU Configuration Update message. At this point, CU1 sends the Predicted CCO Assistance Information IE in the gNB-CU Configuration Update to the remaining DUs involved in the predicted CCO issue i.e. DU2 and DU3. Additionally, it should be noted that DU2 and DU3 needs to know when DU1 is applying its selected Future coverage configuration i.e. Time for Future Coverage Modification t1 selected by DU1. If t1 is not provided to DU2 and DU3 then the time when DU2 and DU3 applies their matching coverage configuration will be different than t1, and therefore, leading to incompatible coverage configurations at different DUs. Therefore, it seems needed to inform the gNB-DU at least about the Time for Future Coverage Modification.

Note: As shown in Figure 2, the value of Time for Future Coverage Modification sent from CU1 to DU2 and DU3, denoted as t1’, may be different from t1 due to time required for message exchanges and processing at CU1 as it is expected to be expressed as relative time from the reception of gNB-DU Configuration Update message by CU1 from DU1. Similarly, the value Time for Future Coverage Modification sent from CU1 to CU2, denoted as t1’’, may be different from t1.

As shown in Figure 2, CU1 waits to receive the Future Coverage Modification Notification from the DUs involved in the predicted CCO issue before it initiates the XnAP NG-RAN Node Configuration Update which conveys the Future Coverage Modification List to the concerned neighboring nodes. We assume that the CU1 triggers a single XnAP procedure per predicted CCO issue, e.g. avoiding exposing the internal architecture of gNB1 to the neighboring nodes via the Xn interface.

Proposal 1: Inform gNB-DUs controlling neighbour cells at least about the Time for Future Coverage Modification.

As shown in Figure 2, the future coverage configuration is shared by CU1 to CU2 over Xn in Future Coverage Modification List IE. Furthermore, from the received information CU2 needs to provide information to the DU’ enabling it to select a matching coverage state. Simple forwarding of the Future Coverage Modification information from the CU2 to the DU’ (option 2) seems to be a straightforward approach to achieve this. While option 3 would require the CU2 to have a trained AI/ML model to generate a recommended future CCO state based on an issue detected in neighbour node.

Therefore, option 2 is preferred, where the receiving gNB-CU directly forwards the received future CCO state of neighboring cells to the gNB-DU.

Proposal 2: RAN3 to adopt option 2, where the receiving gNB-CU directly forwards the received future CCO state of neighboring cells to the gNB-DU.

## 2.2	Xn aspects

In the last meesting there was following open point:

Timing information for predicted CCO issue is NOT needed to exchange over Xn?

Currently, the Future Coverage Modification List IE in the NG-RAN NODE CONFIGURATION UPDATE message includes the "Time for Future Coverage State" IE, which specifies the relative time when the Future Cell Coverage State(s) and/or the Future SSB Coverage State(s) will be applied by the NG-RAN node1. The intention of this timing information is, at the time instant specified in Time for Future Coverage State IE, the receiving NG-RAN node2 shall match its own coverage. However, providing additional timing information related to the coverage modification cause i.e. time for predicted CCO issue, does not benefit but rather creates problems discussed below.

The only way for NG-RAN node2 to use the time for the predicted CCO issue is to apply the matching coverage configuration after the "Time for Future Coverage State" but before “time for predicted CCO issue” i.e between the time instant when NG-RAN node1 applies Future coverage state and time instant when predicted CCO issue occurs. By providing time for predicted CCO issue to NG-RAN node2, one allows NG-RAN node2 to delay the application of matching coverage configuration to after the neighbouring NG-RAN node1 has already applied the new coverage state. This delay results in NG-RAN nodes having different and likely incompatible coverage configurations for a certain duration (from the "Time for Future Coverage State" to the time when NG-RAN node2 applies the matching coverage configuration). Such a mismatch in the CCO configuration between neighboring nodes could negatively impact the UE and overall network performance.

Furthermore, by delaying the application of matching coverage configuration at NG-RAN node2, it could happen that a new CCO issue (immediate CCO issue or a CCO issue prediction) could emerge between the same NG RAN nodes (NG-RAN node1 and NG-RAN node2). The reason of emergence of this new CCO issue could be the mismatch in the CCO configuration between NG-RAN node1 and NG-RAN node2 node.  Similarly, there is also a possibility that a new CCO issue (immediate CCO issue or a CCO issue prediction) emerged can involve one or both of NG-RAN node1 and NG-RAN node2 and different NG RAN node(s). Again, the cause of emergence of this new CCO issue could be the mismatch in the CCO configuration between NG-RAN node1 and NG-RAN node2 node. Thus, there is a risk that uncoordinated application of the matching coverage configurations becomes the origin of a new CCO issue. The issue created is similar to the one discussed in section 2.1 for the case where different DUs are served under the same CU.

Observation 1: There is a risk that uncoordinated application of the matching coverage configurations becomes the origin of a new CCO issue.

Another issue that arises by delaying the application of matching coverage configuration at NG-RAN node2, is that NG RAN node1 will not know when NG-RAN node2 has applied the matching coverage state. Due to this the data collection at NG-RAN node1 that may be required to validate the outcome of the CCO issue prediction (selection of Future CCO state) will be hindered as NG-RAN node1 cannot determine the time of change of coverage configuration at NG-RAN node2.

Observation 2: In order to evaluate the outcome of the CCO issue prediction (selection of Future CCO state), the NG RAN node1 should know when NG-RAN node2 has applied the matching coverage state.

To overcome the issues mentioned above, it is proposed that additional timing information related to the coverage modification cause i.e. time for predicted CCO issue should not be provided over Xn.

Proposal 3: It is not beneficial to send the time for predicted CCO issue over Xn.

## 2.3	Update of predicted CCO issue or future CCO state

We here address the following open point:

Whether the predicted CCO issue and /or future CCO state can be updated over Xn?

We see two main scenarios where a predicted CCO / future CCO state decision may need to be changed.

The first scenario is that the gNB first predicts and sends a CCO issue that will happen some time in future (say t). Then the gNB determines and sends a real-time (legacy) CCO issue before expiry of t. In order to solve the real-time CCO issue, new cell and/or beam configuration will be applied. This means that the situation at which the CCO prediction was made is no longer be valid, and thus, the prediction itself will be invalid. In other words, real-time CCO issue resolution creates a new situation, rendering the initial CCO prediction obsolete. This scenario can be solved based on existing signalling, i.e. that a real-time CCO issue detected after a predicted CCO issue will cancel the prediction, and we believe this should be clarified in the specification (e.g. stage 2).

Proposal 4: Clarify in the specification that a real-time CCO issue detected after a predicted CCO issue will cancel the prediction.

The second scenario is when a gNB-A predicts a CCO issue and informs a gNB-B about a future CCO state change to take place at time t. However, in the meantime, a real-time CCO issue detected at a third node (gNB-C) which requires gNB-A to adopt a matching coverage configuration state. In this case the gNB-A should be able to inform gNB-B about the new coverage state. Such notification can be done by adding a new coverage modification cause, reflecting the CCO state change prompted by a third gNB.

Proposal 5: Introduce a new coverage modification cause, reflecting CCO state change prompted by a third gNB.

Such scenario is illustrated in Figure 3 below.

Figure 3: Illustration for update of CCO issue/states involving multiple NG RAN nodes.

We believe that by the 2 proposals above there is no need for additional mechanism to update predicted CCO issue and/or future CCO state over Xn.

## 2.4	Information to validate the prediction

Concerning the following agreement open point:

Legacy UE performance measurement metrics can be reused for CCO.

Whether/what additional UE performance measurement metrics is needed?

This aspect was also discussed during the Rel-19 study item, and the following conclusion was reached (TR 38.743):

#### 4.2.2.4	Feedback of AI/ML based CCO

To optimize the performance of AI/ML-based CCO model, following feedback can be considered to be collected from gNBs:

-	Measured radio resource status

-	Legacy UE performance feedback for those UEs handed over from the source gNB

SON Reports (e.g., RLF, CEF, RA)

There has still been further discussion on this aspect during Rel-19 work item phase, however with limited progress. The challenges encountered and the lack of substantial advancements indicate that it may be preferable to allocate remaining time to more critial topics, and close discussion on feedback information in line with study item outcome.

Proposal 6: RAN3 to close discussion on feedback information in line with study item outcome and agreement from RAN3#127bis.

## 2.4	ASN.1 aspects

Extension of Future Coverage Modification Cause

In the current BLCR for F1AP [2] as shown below both futureCoverageModificationCause IE and predicted-CCO-issue-detection IE are using enumeration Predicted-CCO-issue-detection that has two codepoints coverage and cell-edge-capacity.

Due to this, futureCoverageModificationCause and predicted-CCO-issue-detection IEs are bound to the enumerated Predicted-CCO-issue-detection. This necessarily means that change in the code point of one IE cannot be done independently, for e.g. the possibility to add new code point to futureCoverageModificationCause IE without adding it to and predicted-CCO-issue-detection IE is not possible. Therefore, we propose that futureCoverageModificationCause IE should have separate enumeration. i.e. not using Predicted-CCO-issue-detection.

Proposal 7: RAN3 to define separate ENUMERATED for futureCoverageModificationCause IE i.e. not using the Predicted-CCO-issue-detection ENUMERATED.

Proposal 8: RAN3 to agree the proposed ASN.1 changes shown in the TP below.

#### 9.3.1.A	Predicted CCO Assistance Information

This IE provides predicted assistance information for the future Coverage and Capacity Optimisation (CCO) actions for predicted CCO issues.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Predicted CCO issue | O |  | ENUMERATED (coverage, cell edge capacity, ...) | Indicates the type of predicted CCO issue. |
| <<<<<<<<<<<<<<<<<<<<Skipped Unrelated part >>>>>>>>>>>>>>>>>>>> |  |  |  |  |

#### 9.3.1.B	Future Coverage Modification Notification

This IE includes a list of cells and/or SS/PBCH block indexes with the corresponding future coverage configuration selected by a gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| <<<<<<<<<<<<<<<<<<<<Skipped Unrelated part >>>>>>>>>>>>>>>>>>>> |  |  |  |  |
| >>>Future SSB Modification Notification Item |  | 1..<maxnoofSSBAreas> |  |  |
| <<<<<<<<<<<<<<<<<<<<Skipped Unrelated part >>>>>>>>>>>>>>>>>>>> |  |  |  |  |
| >>Future Coverage Modification Cause | O |  | ENUMERATED(coverage, cell edge capacity, …) |  |

# 3	Conclusion

We have made the following proposals:

Proposal 1: Inform gNB-DUs controlling neighbour cells at least about the Time for Future Coverage Modification.

Proposal 2: RAN3 to adopt option 2, where the receiving gNB-CU directly forwards the received future CCO state of neighboring cells to the gNB-DU.

Observation 1: There is a risk that uncoordinated application of the matching coverage configurations becomes the origin of a new CCO issue.

Observation 2: In order to evaluate the outcome of the CCO issue prediction (selection of Future CCO state), the NG RAN node1 should know when NG-RAN node2 has applied the matching coverage state.

Proposal 3: It is not beneficial to send the time for predicted CCO issue over Xn.

Proposal 4: Clarify in the specification that a real-time CCO issue detected after a predicted CCO issue will cancel the prediction.

Proposal 5: Introduce a new coverage modification cause, reflecting CCO state change prompted by a third gNB.

Proposal 6: RAN3 to close discussion on feedback information in line with study item outcome and agreement from RAN3#127bis.

Proposal 7: RAN3 to define separate ENUMERATED for futureCoverageModificationCause IE i.e. not using the Predicted-CCO-issue-detection ENUMERATED.

Proposal 8: RAN3 to agree the proposed ASN.1 changes shown in the TP below.

A corresponding F1AP TP is provided in annex of this paper.

# References

[1]		R3-252504, (BL CR to 38.423) Support of enhancements on AI/ML for NG-RAN

[2]		R3-252505, (BL CR to 38.473) Support of enhancements on AI/ML for NG-RAN

# Annex	- TP for BLCR to TS 38.473

<<< start of changes >>>

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 8.2.4	gNB-DU Configuration Update

#### 8.2.4.1	General

The purpose of the gNB-DU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and the gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

NOTE:	Update of application level configuration data also applies between the gNB-DU and the gNB-CU in case the DU does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [7]. How to use this information when this option is used is not explicitly specified.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: gNB-DU Configuration Update procedure: Successful Operation

The gNB-DU initiates the procedure by sending a GNB-DU CONFIGURATION UPDATE message to the gNB-CU including an appropriate set of updated configuration data that it has just taken into operational use. The gNB-CU responds with GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall interpret that the corresponding configuration data is not changed and shall continue to operate the F1-C interface with the existing related configuration data.

<<<<SKIP UNRELATED PART>>>>

If the Coverage Modification Notification IE is contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, take it into account for Coverage and Capacity Optimization and network energy saving. If the Coverage Modification Cause IE is set to the "network energy saving", gNB-CU may consider those deactivated SSB beams are due to network energy saving.

If the Cells for SON IE is present in the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-DU may store or update this information and behaves as follows:

-	For each served cell indicated by the NR CGI IE included within the Cells for SON Item IE, the gNB-DU may adjust the PRACH configuration of this served cell.

-	If the Neighbour NR Cells for SON List IE is present in the Cells for SON Item IE, the gNB-DU may take the PRACH configuration of neighbour cells included in the Neighbour NR Cells for SON List IE into consideration when adjusting the PRACH configuration of the served cell.

If the RedCap Broadcast Information IE is contained in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store and use this information to determine a suitable target in case of subsequent outgoing mobility involving RedCap UEs.

If the eRedCap Broadcast Information IE is contained in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store and use this information to determine a suitable target in case of subsequent outgoing mobility involving eRedCap UEs.

If the TAI NSAG Support List IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, use this information as specified in TS 23.501 [21].

If the gNB-DU Name IE is included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-DU. If the Extended gNB-DU Name IE is included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-DU and shall ignore the gNB-DU Name IE if also included.

If the RRC Terminating IAB-Donor Related Info IE is included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, consider that the BAP address indicated by the Mobile IAB-MT BAP Address IE is assigned by the gNB-CU of the RRC-terminating IAB-donor indicated  by the RRC Terminating IAB-Donor gNB-ID IE, and it shall use this  BAP address and gNB ID for the subsequent IAB Transport Migration Management procedure towards the RRC-terminating IAB-donor of the mobile IAB-node as needed, as specified in TS 38.423 [28].

If the GNB-DU CONFIGURATION UPDATE message contains the Mobile IAB-MT User Location Information IE, the gNB-CU shall, if supported, take it into account when reporting UE location information to the AMF for a UE served by the mobile IAB-node.

If the XR Broadcast Information IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, consider the indicated cell does not allow 2Rx XR UEs in case of subsequent outgoing mobility involving XR UEs.

If the Barring Exemption for Emergency Call Information IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store the information and consider the indicated cell allows emergency bearer services for UEs who would otherwise consider the cell as barred as specified in TS 38.304 [24].

If the Future Coverage Modification Notification IE is contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, take it into account for Coverage and Capacity Optimization.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.2.5	gNB-CU Configuration Update

#### 8.2.5.1	General

The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

#### 8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU Configuration Update procedure: Successful Operation

The gNB-CU initiates the procedure by sending a GNB-CU CONFIGURATION UPDATE message including the appropriate updated configuration data to the gNB-DU. The gNB-DU responds with a GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall interpret that the corresponding configuration data is not changed and shall continue to operate the F1-C interface with the existing related configuration data.

<<<<SKIP UNRELATED PART>>>>

If the CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message, and the NR CGI IE contained in the Affected Cells and Beams IE is served by the gNB-DU, the gNB-DU may use it to determine a new cell and/or beam configuration.

If the CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message and the NR CGI IE contained in the Affected Cells and Beams IE is not served by the gNB-DU, the gNB-DU may use it to adjust coverage of its cells.If the CCO issue detection IE set to "network energy saving" is included in the CCO Assistance Information IE, the gNB-DU may consider the indicated SSB beams by the Affected Cells and Beam IE are deactivated due to network energy saving.

If the Cells for SON IE is present in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may store or update this information and it behaves as follows:

-	For each served cell indicated by the NR CGI IE included within the Cells for SON Item IE, the gNB-DU may adjust the PRACH configuration of this served cell.

-	If the Neighbour NR Cells for SON List IE is present in the Cells for SON Item IE, the gNB-DU may take the PRACH configuration of neighbour cells included in the Neighbour NR Cells for SON List IE into consideration when adjusting the PRACH configuration of the served cell.

If the gNB-CU Name IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-CU. If the Extended gNB-CU Name IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-CU and shall ignore the gNB-CU Name IE if also included.

If the Mobile IAB Barred IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider it as an indication of whether the cell allows mobile IAB-node access.

If the Predicted CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message, and the NR CGI IE contained in the Predicted Affected Cells and Beams IE is served by the gNB-DU, the gNB-DU may use it to determine a future cell and/or beam configuration.

If the Predicted CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message and the NR CGI IE contained in the Affected Cells and Beams IE is not served by the gNB-DU, the gNB-DU may use it to adjust the coverage of its cells. 
Editor´s note: to be further discussed whether any more details need to be added to the above paragraph.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.1.7	GNB-DU CONFIGURATION UPDATE

This message is sent by the gNB-DU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| <<<<SKIP UNRELATED PART>>>> |  |  |  |  |  |  |
| Coverage Modification Notification | O |  | 9.3.1.213 |  | YES | ignore |
| gNB-DU Name | O |  | PrintableString(SIZE(1..150,...)) | Human readable name of the gNB-DU. | YES | ignore |
| Extended gNB-DU Name | O |  | 9.3.1.205 |  | YES | ignore |
| RRC Terminating IAB-Donor Related Info | O |  | 9.3.1.306 | Indicates the information related to a mobile IAB-node’s RRC-terminating IAB-donor. | YES | reject |
| Mobile IAB-MT User Location Information | O |  | 9.3.1.307 |  | YES | ignore |
| Future Coverage Modification Notification | O |  | 9.3.1.B |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofUEIDs | Maximum no. of UEs that can be served by a gNB-DU. Value is 65536. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.1.10	GNB-CU CONFIGURATION UPDATE

This message is sent by the gNB-CU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| <<<<SKIP UNRELATED PART>>>> |  |  |  |  |  |  |
| CCO Assistance Information | O |  | 9.3.1.211 | Indicates CCO Assistance Information for cells and beams served by the gNB-DU of the same NG-RAN node or for cells and beams not served by the gNB-DU. | YES | ignore |
| Cells for SON List | O |  | 9.3.1.214 |  | YES | ignore |
| gNB-CU Name | O |  | PrintableString(SIZE(1..150,...)) | Human readable name of the gNB-CU. | YES | ignore |
| Extended gNB-CU Name | O |  | 9.3.1.206 |  | YES | ignore |
| Cells Allowed to be Deactivated List |  | 0..1 |  |  | YES | ignore |
| >Cells Allowed to be Deactivated List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| Predicted CCO Assistance Information | O |  | 9.3.1.A | Indicates predicted CCO Assistance Information for cells and beams served by the gNB-DU of the same NG-RAN node or for cells and beams not served by the gNB-DU. | YES | Ignore |
| Future Coverage Modification Notification | O |  | 9.3.1.B |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum numbers of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.212	Affected Cells and Beams

This IE includes a list of cells and/or SS/PBCH block indexes affected by the detected CCO issue or predicted to be affected by the predicted CCO issue.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Affected Cell List |  | 1 .. < maxAffectedCells> |  |  |
| >NR CGI | M |  | 9.3.1.12 |  |
| >Affected SSB List |  | 0..<maxnoofSSBAreas> |  |  |
| >>SSB Index | M |  | INTEGER (0..63) |  |

| Range bound | Explanation |
| --- | --- |
| maxAffectedCells | Maximum numbers of cells affected by a CCO issue. Value is 32. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.A	Predicted CCO Assistance Information

This IE provides predicted assistance information for the future Coverage and Capacity Optimisation (CCO) actions for predicted CCO issues.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Predicted CCO issue | O |  | ENUMERATED (coverage, cell edge capacity, ...) | Indicates the type of predicted CCO issue. |
| Predicted Affected Cells and Beams | O |  | 9.3.1.212 |  |
| Time for Predicted CCO Issue | O | INTEGER (1..60, ...) |  | Indicates the time when the predicted CCO issue will happen from the time of receiving this information, in seconds. |

#### 9.3.1.B	Future Coverage Modification Notification

This IE includes a list of cells and/or SS/PBCH block indexes with the corresponding future coverage configuration selected by a gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Future Coverage Modification Notification List |  | 1 |  |  |
| >Future Coverage Modification Notification Item |  | 1..<maxCellingNBDU> |  |  |
| >>NG CGI | M |  | 9.3.1.12 |  |
| >>Future Cell Coverage State | M |  | INTEGER (0..63, ...) | Value ‘0’ indicates that the cell will be inactive. Other values Indicate that the cell will be active and also indicate the future coverage configuration of the concerned cell. |
| >>Future SSB Modification Notification List |  | 0..1 |  |  |
| >>>Future SSB Modification Notification Item |  | 1..<maxnoofSSBAreas> |  |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  |
| >>>>Future SSB Coverage State | M |  | INTEGER (0..15, ...) | Value ‘0’ indicates that the SSB beam will be inactive. Other values indicate that the SSB beams will be active and also indicates the future coverage configuration of the concerned SSB beams. |
| >>Time for Future Coverage Modification | O |  | INTEGER (1..60, ...) | Indicates the time when the Future Cell Coverage State(s) and/or the Future SSB Coverage State(s) will be applied by the gNB-DU relative to the time of receiving this information, in seconds. |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofSSBAreas | Maximum numbers of SSB Areas that can be served by a NG-RAN node cell. Value is 64. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

-- ASN1START

<<<<<<<<<<<<<<<<<<<<Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

#### -- F

Future-Coverage-Modification-Notification ::= SEQUENCE {

future-coverage-Modification-List			Future-Coverage-Modification-List,

iE-Extensions								ProtocolExtensionContainer { { Future-Coverage-Modification-Notification-ExtIEs} }	OPTIONAL,

...

}

Future-Coverage-Modification-Notification-ExtIEs F1AP-PROTOCOL-EXTENSION ::={

...

}

Future-Coverage-Modification-List ::= SEQUENCE (SIZE (1..maxCellingNBDU)) OF Future-Coverage-Modification-Item

Future-Coverage-Modification-Item ::= SEQUENCE {

nRCGI								NRCGI,

futurecellCoverageState				FutureCellCoverageState,

futureSSBCoverageModificationList	FutureSSBCoverageModification-List	OPTIONAL,

timeforFutureCoverageModificaiton	TimeforFutureCoverageModification	OPTIONAL,

futureCoverageModificationCause		 FutureCoverageModificationCause		OPTIONAL,

iE-Extension			ProtocolExtensionContainer { { Future-Coverage-Modification-Item-ExtIEs} } 				OPTIONAL,

...

}

Future-Coverage-Modification-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<<Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

#### -- P

Predicted-CCO-Assistance-Information ::= SEQUENCE {

predicted-CCO-issue-detection	Predicted-CCO-issue-detection	OPTIONAL,

predictedAffectedCellsAndBeams-List		AffectedCellsAndBeams-List 		OPTIONAL,

timeforPredictedCCOIssue		TimeforPredictedCCOIssue,	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Predicted-CCO-Assistance-Information-ExtIEs} }	OPTIONAL,

...

}

Predicted-CCO-Assistance-Information-ExtIEs F1AP-PROTOCOL-EXTENSION ::={

...

}

Predicted-CCO-issue-detection	::=	ENUMERATED {coverage, cell-edge-capacity, ...}

FutureCoverageModificationCause	::=	ENUMERATED {coverage, cell-edge-capacity, ...}

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>