3GPP TSG-RAN WG3 Meeting #128	R3-253540

Malta, MT, 19 – 23 May, 2025

Title:	Discussion on the support for PWS over satellite NGRAN in Rel-17 and beyond

Agenda Item:	8.3

Source:	Huawei, Ericsson, Jio Platforms, Nokia, Nokia Shanghai Bell, 	Qualcomm Incorporated, Thales

Document for:	Discussion

# 1	Introduction

The previous CT#153 meeting sent the LS on the support for PWS over satellite NGRAN in Rel-17, and copied to RAN3 as follows:

1. Overall Description: SA1 has agreed to introduce a new requirement for the support for PWS over satellite NG-RAN (CR#0081 to TS 22.268 in S1-244777) from Rel-17. CT1 has discussed the stage 2 aspects of this requirement and determined that this requirement would have UE impacts that would create backward compatibility issues (e.g. country-specific duplicate detection timers for PLMNs using non-country specific PLMN ID with MCC 9xx over satellite access). Consequently, CT1 has decided to introduce the support for PWS over satellite NG-RAN in Rel-19. This work will be done under the new CT-wide Rel-19 work item on the support for PWS over satellite E-UTRAN and satellite NG-RAN. CT1 kindly asks SA1 to align their specifications accordingly. 2. Actions: To SA1 group. ACTION: CT1 kindly asks SA1 to align the stage 1 specifications to reflect the support for PWS over satellite NG-RAN from Rel-19.

This paper provides our further considerations and proposals according to the LS.

# 2	Discussion

This paper starts from the scratch to introduce the whole process on the PWS support over satellite.

First, RAN2 begins to discuss the intended areas for broadcast services over NR NTN. The discussion first focuses on the MBS broadcast service and then extends to the PWS including both the CMAS and ETWS. For CMAS, RAN2 specification already supports the geofencing capability to a given area. However, the geofencing is not supported for ETWS.  Hence RAN2 sent a LS [2] to ask SA1 whether to support the geofencing capability for ETWS with the following contents.

# 1. Overall Description: RAN2 is working on a solution to provide UEs with geographical information of where within an NTN cell a broadcast service is applicable. CMAS supports the capability to geofence emergency messages to an area. However, this is currently not supported for ETWS by RAN2 specification. RAN2 would like to know whether the capability to geofence emergency messages to an area should be supported for ETWS in a NTN cell, considering the delay requirements for ETWS primary notification. # 2. Actions: # To SA WG 1: # RAN2 kindly request SA1 to take the above information into account and provide feedback whether the capability to geofence emergency messages to an area should be supported for ETWS in a NTN cell, considering the delay requirements for ETWS primary notification.

In response, the SA1#108 meeting made further agreements as follows:

- The CR [3] to support the geofencing functionality to ETWS over Satellite E-UTRAN and Satellite NG-RAN since Rel-19.

- The CRs [4][5] to support the CMAS over Satellite NG-RAN from Rel-17 and Rel-18, respectively.

The CT1#153 meeting monitored the SA1 agreed CR in [4] and sent the above LS to SA1 and also copied to RAN3. It can be observed that the CT1 could only support the PWS including ETWS and CMAS over satellite E-UTRAN and satellite NG-RAN since Rel-19. That means, the SA1 and RAN3 shall not support the PWS over satellite E-UTRAN and satellite NG-RAN earlier than Rel-19.

However, in section 16.14.5 for NR NTN, it is specified that the Cell Identity used for PWS corresponds to a mapped cell ID, which hints that the PWS could be supported from earlier release than Rel-19. Also from stage 3 specification, it seems the PWS can be supported for NR NTN as well. The same is true for IoT NTN.

The Cell Identity, as defined in TS 38.413 [26] and TS 38.423 [50], used in following cases corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported: - The Cell Identity indicated by the gNB to the Core Network as part of the User Location Information; - The Cell Identity used for Paging Optimization in NG interface; - The Cell Identity used for Area of Interest; - The Cell Identity used for PWS.

Hence, there are some misalignments between the CT1 and RAN3 in these earlier releases. It is better for RAN3 to remove the support of PWS for NR NTN and IoT NTN in Rel-17 and Rel-18, then work together with RAN2, to support PWS in Rel-19.

- The support of PWS should be removed from Rel17/18 TS 38.300 and TS 36.300 for NR NTN and IoT NTN.

# 3	Conclusion

In this paper, we provide the following observations and proposals:

- The support of PWS should be removed from Rel17/18 TS 38.300 and TS 36.300 for NR NTN and IoT NTN.

The CRs can be found in [6-9]:

R3-253541	Remove the support for NR NTN PWS, rel-18, Huawei, …

R3-253449	Remove the support for NR NTN PWS, rel-17, Ericsson, …

R3-253538	Remove the support for NR IoT NTN PWS, rel-18, Qualcomm Incorporated, …

R3-253389	Remove the support for NR IoT NTN PWS, rel-17, Nokia, Nokia Shanghai Bell

# 4	Reference

- C1-2450715/ R3-251505, LS on stage 1 requirements for the support for PWS over satellite NGRAN in Rel-17, CT1

- R2-2407823, LS on requirements for ETWS primary notification, RAN2

- S1-244753,  Warning notification with geofencing for ETWS over satellite access, Qualcomm et al.

- S1-244777,  PWS corrections_R17 CR, Qualcomm et al.

- S1-244778,  PWS corrections_R18 CR, Qualcomm et al.

- R3-253541	Remove the support for NR NTN PWS, rel-18

- R3-253449	Remove the support for NR NTN PWS, rel-17

- R3-253538	Remove the support for NR IoT NTN PWS, rel-18

- R3-253389	Remove the support for NR IoT NTN PWS, rel-17