3GPP TSG-RAN WG3 Meeting #128	R3-253561

St. Julian, Malta, May 19th - May 23rd, 2025

Agenda item:	13.2

Source:	Qualcomm Incorporated

Title:	Signalling enhancements for Inter-CU LTM handover

WID/SID:	NR_Mob_Ph4-Core – Release 19

Document for:	Discussion and decision

# Introduction

In the last RAN3 meeting (RAN3 #127-bis [1]), RAN3 made good progress, with the goal of completing the specification work for Inter-CU LTM in two more meetings. The list of agreements RAN3 made is as below.

RAN3 #127-bis agreements:

Reuse the LTM configuration update procedure to transfer UE’s 5G security capabilities and/or UE’s UP security policy from the new serving gNB to the other candidate gNBs in subsequent LTM.

The new NCC value needs to be sent from gNB-CU to the gNB-DU via UE context modification procedure.

Introduce two new Class-1 UE-associated procedures over F1AP to support the source gNB-DU to trigger the activation/deactivation of the SP CSI-RS transmission in the candidate cell(s).

Introduce a new class-1 UE-associated procedures over XnAP for the SP CSI-RS activation/deactivation.

For the initial inter-CU LTM preparation, the source gNB computes the KgNB*(s) (per candidate cell) for the candidate gNB, and forwards the {KgNB*, NCC} pair(s) to the candidate gNB via a Handover Request message as legacy.

Turn the WA into agreement: Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU.

Include the following information in the TA information transfer message:

- Candidate Cell ID

- TA value

- Preamble index

- RA-RNTI

- FFS on the XnAP IDs.

Turn the WA into agreement: Use a single UE association (e.g. identified by a pair of {Source NG-RAN node UE XnAP ID IE and Target NG-RAN node UE XnAP ID IE}) for multiple LTM handover request to the same candidate gNB.

For subsequent LTM, the candidate gNB uses the old target UE XnAP ID (the target UE AP ID it previously allocated between it and the old serving gNB) to identify the UE context when receiving the LTM Configuration Update message from the new serving gNB. FFS on how to deliver the old target UE XnAP ID(s) to the new serving gNB.

Data Forwarding Information IE is a list including multiple data forwarding addresses from each candidate gNB(s).

Include the following information in SN Change Required (Source SN to MN) message:

- LTM request indication (explicit way)

- Suggested PScell list (implicit way in INM, FFS on explicit way)

- Measurement results (implicit way in INM)

- Upper limit number of PScell(s) for each candidate SN (FFS)

- CSI Resource configuration request (explicit way)

- Early sync information request (explicit way)

Include the following information in SN Addition Request (MN to Candidate SN) message:

- LTM request indication (explicit way)

- Suggested PScell list (implicit way in INM, FFS on explicit way)

- Measurement results (implicit way in INM)

- Upper limit number of PScell(s) for the candidate SN (explicit way)

- CSI Resource configuration request (explicit way)

- Early sync information request (explicit way)

It is the source SN to generate the LTM configuration ID mapping list for the accepted candidate PSCell(s), and send it to the MN for subsequent LTM via SN modification request acknowledge message. Then the MN forwards the LTM configuration ID mapping list to the candidate SN via SN Modification Request message.

For SN-initiated inter-CU SCG LTM, the source SN can provide the SCG reference configuration via SN Change Required message. If the source SN does not provide the SCG reference configuration, MN can request SCG reference configuration from one of the candidate SN(s), FFS on whether MN can request SCG reference configuration from the source SN.

The TA information transfer message can be reused and enhanced to transfer the TA information from the candidate SN to the source SN via the MN.

Include the target PSCell ID and corresponding TCI State ID(s) in the Cell Switch Notification message.

Continue to work on the solutions comparison and details…

In this contribution, we discuss further signalling enhancements for inter-CU LTM handover, focusing on the following procedures and scenarios:

- UE association information signalling upon LTM cell switch,

- Security key information signalling for subsequent LTM,

- Semi-persistent CSI-RS resource handling upon LTM cell switch,

- Support of CSI-RS resources for LTM CSI acquisition for candidate cells,

- Remaining issues in Early TA RACH configuration and TA Information Transfer procedures,

- LTM reference configuration,

- Remaining aspects of the Cell Switch Notification procedure,

- SN initiated Inter-CU SCG LTM.

# 2 UE association information signalling upon LTM cell switch

In the last RAN3 meeting (RAN3 #127-bis [1]), the following agreement was reached regarding UE association information signaling upon LTM cell switch. Signaling of this information from the new serving gNB to a candidate gNB enables the candidate gNB to identify the UE context.

For subsequent LTM, the candidate gNB uses the old target UE XnAP ID (the target UE AP ID it previously allocated between it and the old serving gNB) to identify the UE context when receiving the LTM Configuration Update message from the new serving gNB. FFS on how to deliver the old target UE XnAP ID(s) to the new serving gNB.

We address the highlighted FFS issue. Upon LTM cell switch, the old serving gNB can use the Cell Switch Notification message to transmit the old target UE XnAP IDs to the new serving gNB. It is assumed that, during and after LTM cell switch, each target gNB stores the old target UE XnAP ID that it used with the old serving gNB.

Proposal 1. Upon LTM cell switch, the old serving gNB uses the Cell Switch Notification message to transmit the old target UE XnAP IDs to the new serving gNB, for supporting subsequent LTM.

# 3 Security key information signalling for subsequent LTM

For inter-CU LTM cell switch during subsequent LTM, the security key information signaling implements the solution proposed by SA3.

For inter-CU LTM cell switch during subsequent LTM, the new serving gNB computes the KgNB*(s) per LTM candidate cell for each candidate gNB, and forwards the {KgNB*, NCC} pair(s) to the candidate gNB, using the LTM Configuration Update procedure. The same NCC value is provided to all candidate gNBs.

Proposal 2. For inter-CU LTM cell switch during subsequent LTM, the new serving gNB computes the KgNB*(s) per LTM candidate cell for each candidate gNB, and forwards the {KgNB*, NCC} pair(s) to the corresponding candidate gNB, using the LTM Configuration Update procedure.

Each candidate gNB-CU forwards the NCC value to its underlying candidate gNB-DU. The candidate gNB-DU then transmits the NCC value to the UE using a MAC CE during an inter-CU LTM cell switch.

Proposal 3. For subsequent LTM, a candidate gNB-CU transmits to its underlying candidate gNB-DUs the new NCC value using the UE Context Modification procedure.

# 4 Semi-persistent (SP) CSI-RS resource handling upon LTM cell switch

In the last RAN2 meeting (RAN2 #129-bis [2]), on the handling of the activated SP CSI-RS resources after LTM cell switch, the following solution options were discussed. RAN2 did not reach a conclusion on which option to adopt as a solution.

Option 1: UE stops measuring the activated semi-persistent CSI-RS resources after cell switch.

Option 2: UE continues to measure the activated semi-persistent CSI-RS resources after cell switch.

Option 3: UE stops measuring the activated semi-persistent CSI-RS resources of the candidate cells (i.e., other than target cell) after cell switch. UE continues to measure the activated semi-persistent CSI-RS resources of the target cell after cell switch.

Below we evaluate the RAN3 impacts of the above options.

Option 1 enables the target gNB-DU to later determine the activation or deactivation of SP CSI-RS resources when the UE is operating in the target cell after the LTM cell switch and based on receiving L1 measurement results from the UE in the target cell. In this case, upon LTM cell switch, the source gNB-DU transmits an SP CSI-RS deactivation message to each of the candidate gNB-DUs to deactivate SP CSI-RS transmission on the LTM candidate cells.

In the case of Option 2, the source gNB-DU needs to inform the target gNB-DU of the activation or deactivation status of the configured SP CSI-RS resources in the candidate cells other than the candidate cells of the target gNB-DU, since the target gNB-DU is only aware of the activation or deactivation status of its own candidate cells. The source gNB-DU can use the Cell Switch Notification procedure to provide the information.

In the case of Option 3, upon LTM cell switch, the source gNB-DU transmits an SP CSI-RS deactivation message to each of the candidate gNB-DUs to deactivate SP CSI-RS transmission on the LTM candidate cells, except for the target cell.

Observation 1. There are RAN3 impacts for handling activation or deactivation of configured SP CSI-RS resources upon LTM cell switch. RAN3 to wait for RAN2’s conclusion on the solution for the issue, before working on the RAN3 specification impacts.

# 5 Support of CSI-RS resources for LTM CSI acquisition for candidate cells

Agreement:

For candidate cell CSI acquisition:

- In addition to periodic CSI-RS resource, semi-persistent CSI-RS resource is supported.

O Support of semi-persistent CSI-RS resource is subject to separate UE capability.

- MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource similarly to the legacy mechanism for a serving cell which will be specified in RAN2.

As captured in the agreement, RAN1 sees the spec impact in RAN2 to define the MAC CE. Also, RAN1 foresees the RAN3 spec impact to enable this functionality for inter-DU/inter-CU case.

In an LS to RAN2, RAN3 [3], RAN1 informed that it was agreed that periodic and semi-persistent (SP) CSI-RS resources can be configured to the UE for the purpose of CSI acquisition of LTM candidate cells.

We discuss below the RAN3 impacts for supporting this aspect.

Support for configuration of periodic and semi-persistent (SP) CSI-RS resources for the purposes of CSI acquisition on LTM candidate cells is introduced in the LTM preparation procedure in the same way as for CSI-RS resources for the purposes of L1 measurement and reporting on candidate cells.

Proposal 4. The source gNB-CU requests its underlying candidate DUs and a candidate gNB-CU for configuration of periodic and SP CSI-RS resources for CSI acquisition on the candidate cells in the Handover Req message.

A candidate gNB-CU, in turn, requests its underlying candidate DUs for configuration of periodic and SP CSI-RS resources for CSI acquisition on the candidate cells in the UE Context Setup Req message.

Proposal 5. The underlying candidate DUs provide the configurations for periodic and SP CSI-RS resources for CSI acquisition for each prepared LTM candidate cell in the UE Context Setup Resp message to the source gNB-CU and the candidate gNB-CU.

The candidate gNB-CU forwards the configurations to the source gNB-CU in the Handover Req Ack message.

Proposal 6. The source gNB-CU includes the common CSI-RS configuration, which includes the CSI-RS configurations for L1 measurements and the CSI-RS configurations for CSI acquisition in the LTM Configuration Update message to a candidate gNB-CU, which forwards this information to its underlying DUs using the UE Context Modification procedure.

Proposal 7. Each candidate DU provides the CSI report configuration corresponding to the common CSI-RS configuration, to the candidate gNB-CU using the UE Context Modification procedure. The candidate gNB-CU forwards the CSI report configurations to the source gNB-CU using the Handover Req Ack message.

The source gNB-CU transmits the CSI-RS configurations for CSI acquisition to the UE in the RRC reconfiguration message containing the LTM configuration.

The signaling mechanism for activation or deactivation of SP CSI-RS resources for CSI acquisition is the same as for the SP CSI-RS resources for L1 measurements and reporting. For the overall signaling flow, see Figure 1.

Proposal 8. The signaling mechanism for activation or deactivation of SP CSI-RS resources for a list of LTM candidate cells for CSI acquisition purposes is the same as for the SP CSI-RS resources for L1 measurements and reporting.

Figure 1: CSI-RS resources for CSI acquisition for LTM candidate cells

The UE performs measurements on the periodic CSI-RS and the activated SP CSI-RS resources of a candidate cell and transmits the CSI reports (e.g., containing different combinations of CQI, rank indicator (RI), and precoder-matrix indicator (PMI) information) to the serving gNB-DU on PUCCH. The transmitted CSI reports are per candidate cell. Upon receiving the CSI reports, the serving gNB-DU forwards them to the corresponding candidate gNB-DU. The CSI reports for a candidate cell enable the candidate gNB-DU to have the information needed to schedule the UE as soon as possible after the LTM cell switch.

Proposal 9. Upon receiving a CSI report from the UE for an LTM candidate cell, the source gNB-DU (or serving gNB-DU) forwards the report to the corresponding candidate gNB-DU, via the source gNB-CU (or serving gNB-CU) and the candidate gNB-CU.

# 6 Remaining issues in Early TA RACH configuration and TA Information Transfer procedures

In the RAN3 meetings so far, Early TA RACH configuration for initial LTM was covered. Below we discuss some aspects of the procedure for configuration for subsequent LTM.

For subsequent LTM, the serving gNB-DU needs the Early TA RACH configuration for all the configured LTM candidate cells.

Reuse the LTM Configuration Update procedure to sync up configurations among candidate gNBs for subsequent LTM, including early sync configuration, configuration ID, and data forwarding addresses, etc.

It is up to the network implementation when the sync up is performed: during the preparation step, or during the cell switch execution step, or after successful cell switch.

It was agreed in a previous meeting RAN3 #127 [4] that, to sync up the Early TA RACH configurations between the candidate gNBs, the LTM Configuration Update procedure may be triggered at any later point in time depending on network implementation.

For obtaining the Early TA RACH configuration, in Handover Request Ack, each candidate gNB-CU provides (candidate gNB ID, List of IDs of Early TA RACH configuration).

Proposal 10. For obtaining the Early TA RACH configuration, each candidate gNB-CU provides the pair (candidate gNB ID, List of IDs of Early TA RACH configuration) to the source gNB-CU in Handover Request Ack.

The source gNB-CU includes the information regarding the pair of (candidate gNB ID, List of IDs of Early TA RACH configuration) provided by a candidate gNB-CU, in the LTM Configuration Update procedure with the other candidate gNB-CUs, for obtaining the Early TA RACH configuration for subsequent LTM.

Include the following information in the TA information transfer message:

- Candidate Cell ID

- TA value

- Preamble index

- RA-RNTI

- FFS on the XnAP IDs.

In the last RAN3 127-bis meeting [1], including information regarding XnAP IDs in the TA Information Transfer message over Xn was left FFS.

Since the TA Information Transfer message over Xn was agreed to be a non-UE associated procedure, the XnAP IDs do not need to be included in the message.

We think that the following additional information needs to be included in the TA Information Transfer messages over Xn and over F1 from a candidate gNB-DU to the candidate gNB-CU, so that the messages are forwarded to the correct source gNB and source gNB-DU.

- ID of Early RACH configuration,

- Source gNB ID.

Proposal 11. Since the TA Information Transfer message over Xn was agreed to be a non-UE associated procedure, the XnAP IDs are not included in the message.

Proposal 12. Beside the information already agreed, the following additional information needs to be included in the TA Information Transfer messages over Xn and over F1 from a candidate gNB-DU to the candidate gNB-CU, so that the messages are forwarded to the correct source gNB and source gNB-DU:

- ID of Early RACH configuration,

- Source gNB ID.

# 7 LTM reference configuration

FFS on whether the source gNB can request a candidate gNB to provide a reference configuration.

In the previous RAN3 #127 meeting [4] the following issue was kept FFS.

Our thinking is that the source gNB generating the reference configuration for LTM is a good enough solution.

Companies proposing that a candidate gNB may also provide the LTM reference configuration were considering the case where a group of the candidate gNBs belong to a different network vendor than the source gNB. In that case, they argued that a candidate gNB belonging to the group may be able to generate a reference configuration that works for the configured candidate cells that belong to the group.

However, in that case the signaling may become more complicated, and the benefits are not clear. The source gNB can ask a candidate gNB from the group to provide the reference configuration. Then another two rounds of signaling is required so that the source gNB can send a request including the reference configuration to the other candidate gNBs, and then each candidate gNB responds with either a delta configuration with respect to the reference or with a complete configuration. Additionally source gNB itself will be from another infra vendor and candidate gNB provided configuration will not work for source gNB and source gNB has to provide full configuration for its cells. In 3GPP specification, source gNB does not know whether candidate gNBs are from same vendor or different vendor unless some OAM based approach is used, which is undesirable.

Observation 2. The case where the source gNB requests a candidate gNB to provide a reference configuration involves more signaling and complexity.

Proposal 13. The agreed solution where the source gNB generates a reference configuration for LTM is enough, and the source gNB does not request a candidate gNB to provide a reference configuration.

# 8 Remaining aspects of the Cell Switch Notification procedure

A new XnAP class 2 procedure, namely LTM Cell Switch Notification is introduced on Xn to forward the target Cell ID and target TCI state ID(s) from the source gNB-CU to the target gNB-CU.

The following agreement was reached in a previous meeting on the Cell Switch Notification procedure.

As in Rel-18 Intra-CU LTM, it is useful to transmit information regarding the valid TA values of LTM candidate cells from the source gNB-DU to the target gNB-DU upon LTM cell switch, since the UE does not need to invoke the Early TA acquisition procedure when it is in the target cell, for those LTM candidate cells for which the TA information is already available. This impacts the Cell Switch Notification procedure since this information needs to be carried over it.

Proposal 14. The Xn Cell Switch Notification message includes the TA information list, which consists of a list of pairs of (LTM candidate cell ID, TA value). The source gNB-DU ensures that only valid TA values are transmitted, and in the message provides the TA validity timer values associated with the TA values.

# 9 SN initiated Inter-CU SCG LTM

Include the following information in SN Change Required (Source SN to MN) message:

- LTM request indication (explicit way)

- Suggested PScell list (implicit way in INM, FFS on explicit way)

- Measurement results (implicit way in INM)

- Upper limit number of PScell(s) for each candidate SN (FFS)

- CSI Resource configuration request (explicit way)

- Early sync information request (explicit way)

Include the following information in SN Addition Request (MN to Candidate SN) message:

- LTM request indication (explicit way)

- Suggested PScell list (implicit way in INM, FFS on explicit way)

- Measurement results (implicit way in INM)

- Upper limit number of PScell(s) for the candidate SN (explicit way)

- CSI Resource configuration request (explicit way)

- Early sync information request (explicit way)

It is the source SN to generate the LTM configuration ID mapping list for the accepted candidate PSCell(s), and send it to the MN for subsequent LTM via SN modification request acknowledge message. Then the MN forwards the LTM configuration ID mapping list to the candidate SN via SN Modification Request message.

For SN-initiated inter-CU SCG LTM, the source SN can provide the SCG reference configuration via SN Change Required message. If the source SN does not provide the SCG reference configuration, MN can request SCG reference configuration from one of the candidate SN(s), FFS on whether MN can request SCG reference configuration from the source SN.

For SN-initiated inter-CU SCG LTM, the source SN can provide the SCG reference configuration via SN Change Required message. If the source SN does not provide the SCG reference configuration, MN can request SCG reference configuration from one of the candidate SN(s), FFS on whether MN can request SCG reference configuration from the source SN.

For the LTM preparation procedure, the following agreements were reached in the last RAN3 #127-bis meeting [1], and some FFS issues were identified. We discuss the FFS issues below.

Based on the maximum number of 8 candidate PSCells that can be configured for Inter-CU SCG LTM, the source SN can indicate an upper limit on the number of PSCells that can be configured for each candidate SN.

Proposal 15. In the SN Change Required message, the source SN indicates in an explicit way, for each candidate SN, an upper limit on the number of PSCells that can be configured.

There does not seem to be any reason that the Suggested PSCell list should be transmitted in an explicit way in the SN Change Required or the SN Addition Request message.

Proposal 16. The Suggested PSCell list is not transmitted in an explicit way in the SN Change Required or the SN Addition Request message.

It was agreed that the source SN can provide the SCG reference configuration in SN Change Required. If the source SN does not provide the SCG reference configuration in SN Change Required, then, for simplicity, the MN need not request the source SN to provide an SCG reference configuration later, e.g., in case that the SCG reference configuration is not provided by a candidate SN upon request by MN. Otherwise, the resulting procedure is more complex as it involves additional processing at the source SN and the candidate SNs to generate delta SCG configurations in the subsequent steps.

Proposal 17. If the source SN does not provide the SCG reference configuration in SN Change Required, then the MN does not request the source SN to provide an SCG reference configuration later, i.e., in a subsequent message.

# 10 Conclusion

Based on the above discussion, we recommend that RAN3 discuss the following observations and proposals.

UE association information signalling upon LTM cell switch

Proposal 1. Upon LTM cell switch, the old serving gNB uses the Cell Switch Notification message to transmit the old target UE XnAP IDs to the new serving gNB, for supporting subsequent LTM.

Security key information signalling for subsequent LTM

Proposal 2. For inter-CU LTM cell switch during subsequent LTM, the new serving gNB computes the KgNB*(s) per LTM candidate cell for each candidate gNB, and forwards the {KgNB*, NCC} pair(s) to the corresponding candidate gNB, using the LTM Configuration Update procedure.

Proposal 3. For subsequent LTM, a candidate gNB-CU transmits to its underlying candidate gNB-DUs the new NCC value using the UE Context Modification procedure.

Semi-persistent CSI-RS resource handling upon LTM cell switch

Observation 1. There are RAN3 impacts for handling activation or deactivation of configured SP CSI-RS resources upon LTM cell switch. RAN3 to wait for RAN2’s conclusion on the solution for the issue, before working on the RAN3 specification impacts.

Support of CSI-RS resources for LTM CSI acquisition for candidate cells

Proposal 4. The source gNB-CU requests its underlying candidate DUs and a candidate gNB-CU for configuration of periodic and SP CSI-RS resources for CSI acquisition on the candidate cells in the Handover Req message.

A candidate gNB-CU, in turn, requests its underlying candidate DUs for configuration of periodic and SP CSI-RS resources for CSI acquisition on the candidate cells in the UE Context Setup Req message.

Proposal 5. The underlying candidate DUs provide the configurations for periodic and SP CSI-RS resources for CSI acquisition for each prepared LTM candidate cell in the UE Context Setup Resp message to the source gNB-CU and the candidate gNB-CU.

The candidate gNB-CU forwards the configurations to the source gNB-CU in the Handover Req Ack message.

Proposal 6. The source gNB-CU includes the common CSI-RS configuration, which includes the CSI-RS configurations for L1 measurements and the CSI-RS configurations for CSI acquisition in the LTM Configuration Update message to a candidate gNB-CU, which forwards this information to its underlying DUs using the UE Context Modification procedure.

Proposal 7. Each candidate DU provides the CSI report configuration corresponding to the common CSI-RS configuration, to the candidate gNB-CU using the UE Context Modification procedure. The candidate gNB-CU forwards the CSI report configurations to the source gNB-CU using the Handover Req Ack message.

Proposal 8. The signaling mechanism for activation or deactivation of SP CSI-RS resources for a list of LTM candidate cells for CSI acquisition purposes is the same as for the SP CSI-RS resources for L1 measurements and reporting.

Proposal 9. Upon receiving a CSI report from the UE for an LTM candidate cell, the source gNB-DU (or serving gNB-DU) forwards the report to the corresponding candidate gNB-DU, via the source gNB-CU (or serving gNB-CU) and the candidate gNB-CU.

Remaining issues in Early TA RACH configuration and TA Information Transfer procedures

Proposal 10. For obtaining the Early TA RACH configuration, each candidate gNB-CU provides the pair (candidate gNB ID, List of IDs of Early TA RACH configuration) to the source gNB-CU in Handover Request Ack.

Proposal 11. Since the TA Information Transfer message over Xn was agreed to be a non-UE associated procedure, the XnAP IDs are not included in the message.

Proposal 12. Beside the information already agreed, the following additional information needs to be included in the TA Information Transfer messages over Xn and over F1 from a candidate gNB-DU to the candidate gNB-CU, so that the messages are forwarded to the correct source gNB and source gNB-DU:

- ID of Early RACH configuration,

- Source gNB ID.

LTM reference configuration

Observation 2. The case where the source gNB requests a candidate gNB to provide a reference configuration involves more signaling and complexity.

Proposal 13. The agreed solution where the source gNB generates a reference configuration for LTM is enough, and the source gNB does not request a candidate gNB to provide a reference configuration.

Remaining aspects of the Cell Switch Notification procedure

Proposal 14. The Xn Cell Switch Notification message includes the TA information list, which consists of a list of pairs of (LTM candidate cell ID, TA value). The source gNB-DU ensures that only valid TA values are transmitted, and in the message provides the TA validity timer values associated with the TA values.

SN initiated Inter-CU SCG LTM

Proposal 15. In the SN Change Required message, the source SN indicates in an explicit way, for each candidate SN, an upper limit on the number of PSCells that can be configured.

Proposal 16. The Suggested PSCell list is not transmitted in an explicit way in the SN Change Required or the SN Addition Request message.

Proposal 17. If the source SN does not provide the SCG reference configuration in SN Change Required, then the MN does not request the source SN to provide an SCG reference configuration later, i.e., in a subsequent message.

# References

[1] Chair notes, RAN3 #127-bis meeting.

[2] Chair notes, RAN2 #129-bis meeting.

[3] R1-2503079, LS on the support of semi-persistent CSI-RS resource for LTM CSI acquisition for candidate cells, RAN1 LS to RAN2, RAN3.

[4] Chair notes, RAN3 #127 meeting.