3GPP TSG-RAN WG3 Meeting #128	R3-253562

St.Julian, Malta, May 19th - May 23rd, 2025

Agenda item:	13.3

Source:	Qualcomm Incorporated

Title:	Signalling enhancements for Intra-CU Conditional LTM

WID/SID:	NR_Mob_Ph4-Core – Release 19

Document for:	Discussion and decision

# Introduction

In the last RAN3 meeting (RAN3 #127-bis [1]), there was good progress on the specification work for conditional Intra-CU LTM. The following set of basic agreements were made.

RAN3 #127-bis agreements:

For CLTM focus firstly on scenarios without CU-UP change.

The gNB-DU signals the generated L1 execution conditions over F1AP for CLTM via a new IE in the UE CONTEXT SETUP RESPONSE and UE CONTEXT MODIFICATION RESPONSE messages. Details FFS to be continued.

Reuse the ACCESS SUCCESS message from Candidate DU to CU to inform the cell change.

For Inter-DU CLTM, reuse the UE CONTEXT MODIFICATION REQUEST message from CU to Source DU to inform the cell change and stop data transmission.

For CLTM, the existing LTM network signaling for LTM Cell Switch Completion phase is reused.

FFS whether a cell can be simultaneously configured for LTM and CLTM.

FFS whether its needed that for each candidate cell, the CU indicates to the source DU and the candidate DUs in the LTM Configuration ID Mapping List the need of generating L1 execution condition.

Whether needs to indicate from CU to source DU and candidate DU when the cell is configured with L3 CLTM, separately from L1 CLTM.

Continue to work on the solutions comparison and details…

In this contribution, we continue the discussion on the signalling aspects of intra-CU conditional LTM (CLTM).

# 2 Signalling aspects of the LTM preparation phase of intra-CU conditional LTM (CLTM)

In the previous RAN2 #129 meeting [2], it was agreed that: “From a given source cell, network configures either L1 execution condition or L3 execution condition for a C-LTM candidate cell”.

Consequently, upon receiving an RRC measurement report from the UE, when the gNB-CU determines an initial list of candidate cells for Intra-CU CLTM, it also determines, for each candidate cell, whether to configure L1 execution condition or L3 execution condition for it.

For the case when the gNB-CU determines that an L3 execution condition should be configured for a candidate cell, the gNB-CU only transmits an Intra-CU CLTM indication to the source gNB-DU or the candidate gNB-DU, during LTM preparation. Since the gNB-CU is responsible for generating the L3 execution condition, the receiving DU entity does not need to generate any execution condition for the candidate cell. However, since the DU entity has a distinct behaviour for CLTM, e.g., it does not need to transmit an LTM cell switch command to the UE, the DU entity needs the Intra-CU CLTM indication.

For the case when the gNB-CU determines that an L1 execution condition should be configured for a candidate cell, the gNB-CU transmits an Intra-CU CLTM indication as well as an L1 execution condition request indication, to the source gNB-DU or the candidate gNB-DU, during LTM preparation. Upon receiving the message, the source gNB-DU or the candidate gNB-DU generates the L1 execution condition configuration for the candidate cell and transmits it to the gNB-CU in the response.

For the case when the gNB-CU determines that an L1 execution condition should be configured for a candidate cell, another possible option that might be considered is an implicit L1 execution condition request indication, in which the gNB-CU transmits to the source gNB-DU or a candidate gNB-DU a list of the suggested LTM candidate cells, so that the source gNB-DU or the candidate gNB-DU can generate L1 execution conditions from the prepared candidate cell to other candidate cells. However, this approach does not work, since the LTM configuration ID mapping list is always transmitted during LTM preparation by the gNB-CU to the source gNB-DU or the candidate gNB-DU following Rel-18 LTM, which already includes the list of the suggested LTM candidate cells, and so if this approach is used, it is not possible to distinguish a request for L1 execution condition configuration from a request for L3 execution condition configuration.

There can be two options for configuration of the L1 execution conditions:

Option 1: The configuration of the L1 execution conditions is performed during the first round of the CLTM preparation procedure. The corresponding signalling flow is shown in Figure 1. In this case, the gNB-CU needs to process the L1 execution condition configurations received from the underlying DUs and remove the execution conditions corresponding to candidate cells that are not prepared, before sending the overall CLTM configuration to the UE.

Option 2: The configuration of the L1 execution conditions is performed during the second round of the CLTM preparation procedure. The corresponding signalling flow is shown in Figure 2. In this case, after information regarding all the prepared candidate cells is available, this information can be provided by the gNB-CU to the underlying DUs in the LTM configuration ID mapping list, which includes the L1 execution condition request indication.

Observation 1. There can be two options for configuration of the L1 execution conditions:

Option 1: The configuration of the L1 execution conditions is performed during the first round of the CLTM preparation.

Option 2: The configuration of the L1 execution conditions is performed during the second round of the CLTM preparation. In this case, information regarding all the prepared candidate cells is provided by the gNB-CU to the underlying DUs in the LTM configuration ID mapping list, which includes the L1 execution condition request indication.

Below we provide proposals corresponding to the above two options.

Proposal 1a. In case of Option 1, during Intra-CU CLTM preparation, if the gNB-CU determines that an L1 execution condition should be configured for a candidate cell, the gNB-CU includes an L1 execution condition request indication for the candidate cell, to the source gNB-DU or the candidate gNB-DU, in the UE Context Modification Request or the UE Context Setup Request message, respectively.

Proposal 2a. The L1 execution condition request indication can be included in the LTM configuration ID mapping list IE, which contains the list of all suggested LTM candidate cells, in the UE Context Modification Request or the UE Context Setup Request message.

Proposal 3a. In case of Option 1, the UE Context Setup Request or the UE Context Modification Request message contains the following relevant IEs:

- Intra-CU CLTM indication,

- Suggested candidate cell ID to be prepared,

- LTM configuration ID mapping list:

  - LTM candidate cell ID (NR CGI),

  - LTM Configuration ID,

  - L1 execution condition request indication.

The L1 execution condition request indication is absent if L3 execution condition is to be configured for the candidate cell.

If the source gNB-DU or the candidate gNB-DU accepts the LTM configuration request, it generates and transmits the lower layer configuration of the prepared LTM candidate cell in its response to the gNB-CU.

The source gNB-DU or the candidate gNB-DU selects the candidate cells for generating the L1 execution conditions, e.g., the execution conditions are only generated for the neighbour cells of the prepared candidate cell. The source gNB-DU or the candidate gNB-DU transmits the generated L1 execution conditions in its response to the gNB-CU.

Proposal 4a. If the source gNB-DU or the candidate gNB-DU accepts the LTM configuration request, the UE Context Setup Response from a candidate gNB-DU or the UE Context Modification Response from the source gNB-DU contains the following related information:

- Prepared candidate cell ID,

- Lower layer configuration of the prepared candidate cell,

- L1 execution conditions from the prepared cell to other candidate cells.

Proposal 5a. The source gNB-DU generates the L1 execution conditions for the initial LTM cell switch and may also generate the L1 execution conditions for a subsequent LTM cell switch.

A candidate gNB-DU generates the L1 execution conditions for subsequent LTM cell switch.

When the parallel preparation of LTM candidate cells is completed, i.e., upon completion of Steps 5a and 5b in Figure 1, the gNB-CU knows about all the prepared candidate cells. The gNB-CU can then generate the L3 execution conditions based on this information and the UE RRC measurement reports.

Proposal 6a. Upon receiving information regarding the prepared LTM candidate cells from its underlying DUs, the gNB-CU generates the L3 execution conditions for Intra-CU CLTM:

- The gNB-CU can generate the L3 execution conditions from the source cell to other LTM candidate cells, as well as the execution conditions from an LTM candidate cell to other LTM candidate cells, i.e., for subsequent LTM.

Proposal 1b. For Option 2, the initial round of LTM preparation is the same as that of Rel-18 Intra-CU LTM, except that in the UE Context Setup Request or the UE Context Modification Request message, the Intra-CU CLTM indication is included.

Proposal 2b. For Option 2, upon receiving information regarding the prepared LTM candidate cells from its underlying DUs, the gNB-CU generates the L3 execution conditions for Intra-CU CLTM.

Proposal 3b. For Option 2, the gNB-CU then transmits the UE Context Modification Request message to the underlying DUs including:

- LTM configuration ID mapping list:

  - LTM candidate cell ID (NR CGI),

  - LTM Configuration ID,

  - L1 execution condition request indication.

The L1 execution condition request indication is absent if L3 execution condition was configured by the gNB-CU for the candidate cell.

Proposal 4b. In the response message UE Context Modification Response, each underlying DU then provides:

- L1 execution condition configurations,

- Lower layer configurations of the prepared candidate cells.

Observation 2. An assumption in the above proposals is that the DU complies with an L1 execution condition configuration request from the CU. If this assumption does not hold good, then further modifications to the procedure are required.

Figure 1: Option 1 Signalling flow for Intra-CU CLTM with L1 and L3 execution condition configuration

The gNB-CU transmits the L3 execution conditions and the L1 execution conditions to the UE in the RRC reconfiguration message containing the LTM configuration for Intra-CU CLTM.

Figure 2: Option 2 Signalling flow for Intra-CU CLTM with L1 and L3 execution condition configuration

5. RACH-less Conditional intra-CU LTM is supported.

7. UE based TA measurement mechanism is supported for conditional intra-CU LTM.

8. PDCCH ordered early TA acquisition is supported for conditional LTM.

8. The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4.

9. The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.

10. Candidate cell TA is maintained by a new timer.

RAN2 made the following agreements regarding Early TA acquisition for Intra-CU CLTM.

Based on the agreements above, we think that for the Early TA RACH configuration procedure, the related Rel-18 intra-CU LTM procedures can be simply reused.

Proposal 7. The Rel-18 Intra-CU LTM Early TA RACH configuration procedure can be reused for Early TA acquisition for Intra-CU CLTM.

For Intra-CU CLTM, it was agreed in RAN2 that a TA timer value associated with the TA value of a candidate cell needs to be signalled to the UE, since the UE is responsible for checking the validity of the TA. A TA timer value needs to be provided to the UE for each candidate cell, and the gNB-CU provides the TA timer value to the UE through RRC signalling.

For this purpose, we think that the candidate gNB-DU can determine the TA timer value associated with a TA value for a candidate cell that is managed by the candidate gNB-DU, and transmits it to the source gNB-DU and the gNB-CU.

Proposal 8. The candidate gNB-DU can determine the TA timer value associated with a TA value for each candidate cell. This TA timer value for each candidate cell is signalled to the source gNB-DU and the gNB-CU.

The gNB-CU then provides the TA timer value for each candidate cell to the UE through RRC signalling.

Similarly, the Rel-18 LTM CSI configuration procedure can be reused for providing the UE with configuration for performing L1 measurements and reporting. However, the Rel-18 LTM CSI configuration procedure considers CSI-RS resources corresponding to SSBs only. The CSI-RS resources corresponding to CSI-RSs to enable R19 L1 CSI-RS Resource based measurements need to be added to the Rel-18 LTM CSI configuration procedure, and the corresponding procedure is as follows:

- The CU requests a candidate gNB-DU and the source gNB-DU to provide the CSI-RS resources and configuration for the UE, for a suggested candidate cell, in UE Context Setup Request and UE Context Modification Request, respectively.

- A candidate gNB-DU and the source gNB-DU provide the CSI-RS resources and configuration for the UE for the prepared candidate cell, in UE Context Setup Response and UE Context Modification Response, respectively.

- The CU generates the list of CSI-RS resources for all the LTM candidate cells and provide them to a candidate gNB-DU and the source gNB-DU, which then respond with the corresponding CSI reporting configurations for each LTM candidate cell.

Proposal 9. The Rel-18 LTM CSI configuration procedure can be reused for providing the UE with configuration based on SSBs for performing L1 measurements and reporting.

Proposal 10. The Rel-18 LTM CSI configuration procedure needs to be enhanced to provide the UE with configuration based on CSI-RS resources for performing L1 measurements and reporting, and the procedure consists of:

- The CU requests a candidate gNB-DU and the source gNB-DU to provide the CSI-RS resources and configuration for the UE, for a suggested candidate cell, in UE Context Setup Request and UE Context Modification Request, respectively.

- A candidate gNB-DU and the source gNB-DU provide the CSI-RS resources and configuration for the UE for the prepared candidate cell, in UE Context Setup Response and UE Context Modification Response, respectively.

- The CU generates the common CSI configuration, i.e., the list of CSI-RS resources for all the LTM candidate cells and provides them to a candidate gNB-DU and the source gNB-DU, which then respond with the corresponding CSI reporting configurations for each LTM candidate cell.

# 3 Conclusion

Based on the above discussion, we recommend that RAN3 discuss the following observations and proposals.

Observation 1. There can be two options for configuration of the L1 execution conditions:

Option 1: The configuration of the L1 execution conditions is performed during the first round of the CLTM preparation.

Option 2: The configuration of the L1 execution conditions is performed during the second round of the CLTM preparation. In this case, information regarding all the prepared candidate cells is provided by the gNB-CU to the underlying DUs in the LTM configuration ID mapping list, which includes the L1 execution condition request indication.

Option 1 Signalling for configuration of the L1 execution conditions

Proposal 1a. In case of Option 1, during Intra-CU CLTM preparation, if the gNB-CU determines that an L1 execution condition should be configured for a candidate cell, the gNB-CU includes an L1 execution condition request indication for the candidate cell, to the source gNB-DU or the candidate gNB-DU, in the UE Context Modification Request or the UE Context Setup Request message, respectively.

Proposal 2a. The L1 execution condition request indication can be included in the LTM configuration ID mapping list IE, which contains the list of all suggested LTM candidate cells, in the UE Context Modification Request or the UE Context Setup Request message.

Proposal 3a. In case of Option 1, the UE Context Setup Request or the UE Context Modification Request message contains the following relevant IEs:

- Intra-CU CLTM indication,

- Suggested candidate cell ID to be prepared,

- LTM configuration ID mapping list:

  - LTM candidate cell ID (NR CGI),

  - LTM Configuration ID,

  - L1 execution condition request indication.

The L1 execution condition request indication is absent if L3 execution condition is to be configured for the candidate cell.

Proposal 4a. If the source gNB-DU or the candidate gNB-DU accepts the LTM configuration request, the UE Context Setup Response from a candidate gNB-DU or the UE Context Modification Response from the source gNB-DU contains the following related information:

- Prepared candidate cell ID,

- Lower layer configuration of the prepared candidate cell,

- L1 execution conditions from the prepared cell to other candidate cells.

Proposal 5a. The source gNB-DU generates the L1 execution conditions for the initial LTM cell switch and may also generate the L1 execution conditions for a subsequent LTM cell switch.

A candidate gNB-DU generates the L1 execution conditions for subsequent LTM cell switch.

Proposal 6a. Upon receiving information regarding the prepared LTM candidate cells from its underlying DUs, the gNB-CU generates the L3 execution conditions for Intra-CU CLTM:

- The gNB-CU can generate the L3 execution conditions from the source cell to other LTM candidate cells, as well as the execution conditions from an LTM candidate cell to other LTM candidate cells, i.e., for subsequent LTM.

Option 2 Signalling for configuration of the L1 execution conditions

Proposal 1b. For Option 2, the initial round of LTM preparation is the same as that of Rel-18 Intra-CU LTM, except that in the UE Context Setup Request or the UE Context Modification Request message, the Intra-CU CLTM indication is included.

Proposal 2b. For Option 2, upon receiving information regarding the prepared LTM candidate cells from its underlying DUs, the gNB-CU generates the L3 execution conditions for Intra-CU CLTM.

Proposal 3b. For Option 2, the gNB-CU then transmits the UE Context Modification Request message to the underlying DUs including:

- LTM configuration ID mapping list:

  - LTM candidate cell ID (NR CGI),

  - LTM Configuration ID,

  - L1 execution condition request indication.

The L1 execution condition request indication is absent if L3 execution condition was configured by the gNB-CU for the candidate cell.

Proposal 4b. In the response message UE Context Modification Response, each underlying DU then provides:

- L1 execution condition configurations,

- Lower layer configurations of the prepared candidate cells.

Observation 2. An assumption in the above proposals is that the DU complies with an L1 execution condition configuration request from the CU. If this assumption does not hold good, then further modifications to the procedure are required.

Proposal 7. The Rel-18 Intra-CU LTM Early TA RACH configuration procedure can be reused for Early TA acquisition for Intra-CU CLTM.

Proposal 8. The candidate gNB-DU can determine the TA timer value associated with a TA value for each candidate cell. This TA timer value for each candidate cell is signalled to the source gNB-DU and the gNB-CU.

The gNB-CU then provides the TA timer value for each candidate cell to the UE through RRC signalling.

Proposal 9. The Rel-18 LTM CSI configuration procedure can be reused for providing the UE with configuration based on SSBs for performing L1 measurements and reporting.

Proposal 10. The Rel-18 LTM CSI configuration procedure needs to be enhanced to provide the UE with configuration based on CSI-RS resources for performing L1 measurements and reporting, and the procedure consists of:

- The CU requests a candidate gNB-DU and the source gNB-DU to provide the CSI-RS resources and configuration for the UE, for a suggested candidate cell, in UE Context Setup Request and UE Context Modification Request, respectively.

- A candidate gNB-DU and the source gNB-DU provide the CSI-RS resources and configuration for the UE for the prepared candidate cell, in UE Context Setup Response and UE Context Modification Response, respectively.

- The CU generates the common CSI configuration, i.e., the list of CSI-RS resources for all the LTM candidate cells and provides them to a candidate gNB-DU and the source gNB-DU, which then respond with the corresponding CSI reporting configurations for each LTM candidate cell.

# References

[1] Chair notes, RAN3 #127-bis meeting.

[2] Chair notes, RAN2 #129 meeting.