3GPP TSG-RAN WG3 Meeting #128	R3-253584

St Julian’s, Malta, 19 – 23 May, 2025

Agenda item:	11.2

Source:	Nokia, FiberCop, Deutsche Telekom, Jio Platforms (JPL)

Title:	(TP to TS 38.423) AI/ML Slicing

Document for:	Text Proposal

# 1	Introduction

In this paper we provide a Text Proposal, reflecting our proposals R3-253583.

# TP for BL CR to TS 38.423

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

#### 9.1.3.27	DATA COLLECTION RESPONSE

This message is sent by NG-RAN node2 to NG-RAN node1 to indicate that the requested information, for all or part of the measurement objects included in the reporting, is successfully initiated.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Node Measurement Initiation Result List |  | 0..1 |  | List of measurement objects that failed to be initiated in the node. | YES | reject |
| >Node Measurement Initiation Result Item |  | 1 .. <maxFailedMeasPerNode> |  |  | – |  |
| >>Node Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Energy Cost, Second Bit = Average UE Throughput DL, Third Bit = Average UE Throughput UL, Fourth Bit = Average Packet Delay, Fifth Bit = Average Packet Loss DL, Sixth Bit = Measured UE Trajectory Seventh Bit = Slice Average UE Throughput DL, Eighth Bit = Slice Average UE Throughput UL, Ninth Bit = Slice Average Packet Delay Tenth Bit = Slice Average Packet Loss DL. Other bits are ignored by the NG-RAN node1. | – |  |
| >>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| Cell Measurement Initiation Result List |  | 0..1 |  | List of measurement objects that failed to be initiated per cell. | YES | reject |
| >Cell Measurement Initiation Result Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Cell Measurement Failure Cause List |  | 0..1 |  | Indicates that NG-RAN node2 could not initiate the measurement for at least one of the requested measurement objects in the cell. | – |  |
| >>>Cell Measurement Failure Cause Item |  | 1 .. <maxFailedCellMeasObjects> |  |  | – |  |
| >>>>Cell Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections. Other bits are ignored by the NG-RAN node1. | – |  |
| >>>>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| >>Slice Measurement Initiation Result | O |  | 9.2.3.x1 | List of measurement objects that failed to be initiated per slice. | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxFailedCellMeasObjects | Maximum number of measurement objects that can fail per cell. Value is 124. |
| maxFailedMeasPerNode | Maximum number of measurement objects that can fail per node. Value is 124. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.x2	Slice UE Performance

This IE indicates per Slice UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| S-NSSAI UE Performance List |  | 1 |  | Indicates the UE performance per network slice (details are FFS) |
| >S-NSSAI UE Performance Item |  | 1 .. < maxnoofSliceItemsForUEPerformanceReporting> |  |  |
| >>S-NSSAI | M |  | 9.2.3.21 |  |
| >>Slice Based UE Performance | M |  | UE Performance 9.2.3.179 | [It is FFS how the slice based UE performance metrics are derived] [It is FFS whether and how the Slice Average Packet Loss DL IE is defined] |

| Range bound | Explanation |
| --- | --- |
| maxnoofSliceItemsForUEPerformanceReporting | Maximum no. of signalled slice support items. Value is 256 (FFS). |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.3.5	Information Element definitions

<<<<<<<<<<<<<<<<<<<< Unmodified Text Skipped >>>>>>>>>>>>>>>>>>>>

#### -- S

SliceToReportForDataCollection-List-Item	::= SEQUENCE {

pLMNIdentity				PLMN-Identity,

sNSSAIlist					SNSSAI-list,

iE-Extensions				ProtocolExtensionContainer { { SliceToReportForDataCollection-List-Item-ExtIEs} }	OPTIONAL,

...

}

SliceToReportForDataCollection-List-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SliceMeasurementInitiationResult::= SEQUENCE {

sliceMeasurementInitiationResult-List		SliceMeasurementInitiationResult-List,

iE-Extensions					ProtocolExtensionContainer { {SliceMeasurementInitiationResult-ExtIEs} } OPTIONAL,

...

}

SliceMeasurementInitiationResult-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SliceMeasurementInitiationResult-List ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF SliceMeasurementInitiationResult-Item

SliceMeasurementInitiationResult-Item ::= SEQUENCE {

pLMNIdentity									PLMN-Identity,

sNSSAIMeasurementInitiationResult-List			SNSSAIMeasurementInitiationResult-List,

iE-Extensions									ProtocolExtensionContainer { { SliceMeasurementInitiationResult-Item-ExtIEs} }	OPTIONAL,

...

}

SliceMeasurementInitiationResult-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SNSSAIMeasurementInitiationResult-List ::= SEQUENCE (SIZE(1.. maxnoofSliceItemsForUEPerformanceReporting)) OF SNSSAIMeasurementInitiationResult-Item

SNSSAIMeasurementInitiationResult-Item ::= SEQUENCE {

sNSSAI								S-NSSAI,

sNSSAIMeasurementFailureCause-List		SNSSAIMeasurementFailureCause-List		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { SNSSAIMeasurementInitiationResult-Item-ExtIEs} }	OPTIONAL,

...

}

SNSSAIMeasurementInitiationResult-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SNSSAIMeasurementFailureCause-List ::=SEQUENCE (SIZE(1..maxnoofFailedSliceMeasObjects)) OF SNSSAIMeasurementFailureCause-Item

SNSSAIMeasurementFailureCause-Item ::= SEQUENCE {

sNSSAIMeasurementFailedReportCharacteristics	BIT STRING(SIZE(32)),

cause											Cause,

iE-Extensions									ProtocolExtensionContainer { { SNSSAIMeasurementFailureCause-Item-ExtIEs} }		OPTIONAL,

...

}

SNSSAIMeasurementFailureCause-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SliceUEPerformance ::= SEQUENCE {

s-NSSAIUEPerformance-List                   S-NSSAIUEPerformance-List,

iE-Extensions								ProtocolExtensionContainer { { SliceUEPerformance-ExtIEs} } OPTIONAL,

...

}

SliceUEPerformance-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

S-NSSAIUEPerformance-List ::= SEQUENCE (SIZE(1..maxnoofSliceItemsForUEPerformanceReporting)) OF S-NSSAIUEPerformance-List-Item

S-NSSAIUEPerformance-List-Item	::= SEQUENCE {

sNSSAI					            S-NSSAI,

sliceBasedUEPerformance             UEPerformance,

iE-Extensions						ProtocolExtensionContainer { { S-NSSAIUEPerformance-List-Item-ExtIEs} }	OPTIONAL,

...

}

S-NSSAIUEPerformance-List-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### -- U

UEAssociatedInfoResult-List ::= SEQUENCE (SIZE(1..maxnoofUEReports)) OF UEAssociatedInfoResult-Item

UEAssociatedInfoResult-Item ::= SEQUENCE {

uEAssistantIdentifier						NG-RANnodeUEXnAPID,

uEPerformance								UEPerformance			OPTIONAL,

measuredUETrajectory						MeasuredUETrajectory	OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { UEAssociatedInfoResult-Item-ExtIEs} } OPTIONAL,

...

}

UEAssociatedInfoResult-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-SliceUEPerformance	CRITICALITY ignore	EXTENSION SliceUEPerformance PRESENCE optional },

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxEARFCN									INTEGER ::= 262143

maxnoofAllowedAreas							INTEGER ::= 16

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

maxnoofSecurityConfigurations				INTEGER ::= 8

maxnoofRSPPQoSFlows							INTEGER ::= 2048

maxnoofFailedSliceMeasObjects				INTEGER ::= 124

maxnoofSliceItemsForUEPerformanceReporting  	INTEGER ::= 256 --FFS

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>