3GPP TSG RAN WG3 Meeting #128	R3-253607

St Julian’s, Malta, 19 - 23 May, 2025

Agenda Item:	14.3

Source:	Jio Platforms Limited (JPL)

Title:	Discussion on Support for Cross-Link Interference (CLI) Handling

Document for:	Discussion

## 1. Introduction

This document presents comprehensive proposals to enhance handling of Cross-Link Interference (CLI) in New Radio (NR) networks, particularly for Release 19 under the NR Duplex Evolution work item.

The primary objectives are:

- To enable semi-static or dynamic coordination of TDD/SBFD (Symmetric Band Full Duplex) patterns across neighboring gNBs.

- To introduce explicit signaling procedures that allow a victim cell to request and negotiate interference mitigation from an aggressor cell, rather than relying solely on static or semi-static sharing of TDD patterns.

- To ensure that both the Xn interface (inter-gNB) and the F1 interface (intra-gNB: gNB-DU ↔ gNB-CU) can carry the necessary information (e.g., measurement resources, beam info, SBFD configuration, and mitigation requests).

The ultimate aim is to improve UL performance in partially synchronized or unsynchronized TDD networks, by giving network operators a more direct and dynamic mechanism to combat cross-link interference in real-time.

## 2. Background and Observations

- Semi-static SBFD Time/Frequency Configuration

  - In Release 15–18, some level of TDD pattern coordination exists (e.g., via the Xn or SON solutions). However, these often apply to static or semi-static patterns.

  - With SBFD (Symmetric Band Full Duplex), advanced usage scenarios require more granular sharing of the time–frequency location. For instance, specifying exactly which subframes or OFDM symbols are allocated for UL vs. DL in each cell.

- Measurement Resource Configuration

  - NR already allows exchanging certain cell configuration parameters, such as SSB beams, via the Served Cell Information NR IE in XnAP/F1AP.

  - For NZP CSI-RS, explicit cross-gNB sharing is currently less defined. This is important for CLI detection (the victim cell can detect the neighbor’s CSI-RS as an interference reference).

- Strongest DL Beam Information

  - In beamformed TDD systems, cross-link interference may come predominantly from the strongest neighbor beam. Hence, the victim cell benefits from knowing which neighbor beam (SSB index or NZP CSI-RS resource) is actually creating interference.

- CLI-Mitigation Request

  - Current specs do not define a dedicated procedure to request an aggressor cell to dynamically modify its transmissions. Instead, solutions tend to rely on sharing TDD patterns or “RIM-RS detection” signals.

  - A more explicit handshake (request → response) can accelerate and clarify interference mitigation, enabling partial muting, power reduction, or dynamic pattern adjustments, on demand.

## 3. Description

### 3.1 Semi-static Cell-Specific SBFD Time and Frequency Location Configuration

Purpose:

- Provide each gNB (and its CU/DU) with an up-to-date time/frequency map that indicates when/where a neighbor cell is in UL vs. DL mode. This is crucial for SBFD or advanced TDD usage.

Exchange Over Xn and F1:

- F1 Interface (within a single gNB):

  - The gNB-DU shall report its SBFD configuration to the gNB-CU (e.g., in the Served Cell Information IE in F1 SETUP REQUEST or gNB-DU CONFIGURATION UPDATE).

  - When the gNB-CU receives SBFD configuration from neighbors (over Xn), it can pass that info to the DU via Neighbour Cell Information List IE in a gNB-CU CONFIGURATION UPDATE.

- Xn Interface (between different gNBs):

  - The gNB-CU includes SBFD parameters in Served Cell Information NR IE during Xn Setup or Configuration Update procedures.

Implementation Details:

- A new or extended IE, e.g. SBFD-TimeFrequencyConfiguration, container defined by RAN2, is introduced in the relevant Served Cell Information NR IE.

- In XnAP:

  - Xn SETUP REQUEST/RESPONSE and

  - NG-RAN NODE CONFIGURATION UPDATE/ACK
are updated to carry this IE.

- In F1AP:

  - F1 SETUP REQUEST and

  - gNB-DU CONFIGURATION UPDATE
carry the same IE or container.

Benefit:

- Neighboring cells can better schedule or adapt transmissions if they know each other’s UL/DL allocation, especially in partially aligned TDD setups.

### 3.2 Measurement Resource Configuration (SSB and NZP CSI-RS)

Purpose:

- Provide a standardized method for gNBs to share which SSB beams or NZP CSI-RS resources are used for interference measurements.

Current Status:

- SSB resource configuration exchange is already partly supported (in Served Cell Information NR IE).

- NZP CSI-RS configuration is not fully standardized for cross-gNB sharing.

Enhancement:

- Introduce new IEs in both XnAP and F1AP to share NZP CSI-RS resource configurations.

- The DU can inform the CU about which NZP CSI-RS resources it uses for CLI detection; the CU can forward this to neighbors over Xn.

- Conversely, a neighbor’s NZP CSI-RS configuration can be sent to the DU, so the DU knows how to detect or measure that neighbor’s transmissions.

Implementation Details:

- XnAP: Under Served Cell Information NR IE, add or extend an IE referencing NZP-CSI-RS-Resource / NZP-CSI-RS-ResourceSet from TS 38.331.

- F1AP: Similarly, add these resources to the Served Cell Information IE and Neighbour Cell Information List IE.

Benefit:

- The victim cell can better correlate interference with specific neighbor beams or subcarriers, enabling more fine-grained mitigation (e.g., partial frequency muting).

### 3.3 Strongest DL Beam Information

Purpose:

- If the victim cell can identify the neighbor’s strongest DL beam (which is presumably causing the worst interference), it can request targeted mitigation on that beam rather than a blanket request on all beams.

Information Exchange:

- Over Xn: The neighbor gNB can share which beam index (SSB index or CSI-RS resource) is currently used with the highest power or is most frequently scheduled.

- Over F1: The CU can propagate relevant beam info down to the DU, or the DU can report any local detection of the neighbor’s beam ID.

Implementation Details:

- New IE: StrongestDLBeamInfo, containing either:

  - SSB-Index (0..63), or

  - NZP-CSI-RS Resource Indicator (1..64).

- Potentially included in a new or existing neighbor cell configuration message.

Benefit:

- More targeted CLI mitigation requests (e.g., “reduce power on SSB index 12 for 5 ms”).

### 3.4 Dynamic CLI-Mitigation Request Procedure

Motivation:

- Moving beyond static or semi-static patterns, a victim cell can dynamically request real-time mitigation from a neighbor. This is especially important if interference spikes suddenly (e.g., the neighbor schedules heavy DL transmissions unexpectedly).

Overall Flow:

#### 3.4.1 Step-by-Step Description

- Interference Detection (DU)

  - The gNB-DU at the victim cell detects cross-link interference via elevated UL noise, RIM-RS detection, or other measurement triggers (e.g., CLI-RSSI above threshold).

- DU → CU (F1: Mitigation Request)

  - The DU sends a DU-CU RADIO INFORMATION TRANSFER message, possibly extended with a new IE requestMitigation = TRUE.

  - It indicates the aggressor cell ID (if known) and the interference level.

- CU Aggregation & Decision

  - The CU can consolidate multiple requests (if multiple victim DUs report the same aggressor). Based on policy/thresholds, it decides to request mitigation from the neighbor gNB.

- Xn CLI Mitigation Request

  - The victim CU sends a new XnAP message: CLI Mitigation Request to the aggressor CU, containing:

    - Victim Cell ID

    - Aggressor Cell ID

    - Interference Level (high/medium/low or actual dBm)

    - Requested Action (mute DL, reduce power, reconfigure TDD pattern, etc.)

    - Duration or subframe pattern

- Xn CLI Mitigation Response

  - The aggressor CU responds with Accepted, Partially Accepted, or Rejected.

  - If accepted/partial, details of the actual action (e.g., “mute for 5 ms in subframes 2–3,” or “reduce Tx power by 6 dB”).

  - If rejected, a cause is provided (e.g., “Cannot comply due to ongoing critical traffic”).

- CU → DU (F1: Mitigation Outcome)

  - The victim CU informs its DU that the neighbor’s action was accepted, rejected, or partially accepted.

  - If accepted, the DU can schedule UEs aggressively in UL during the muted subframes.

- Aggressor CU → Aggressor DU

  - The aggressor CU instructs its DU to enforce the agreed mitigation (mute pattern, reduced power, or a TDD reconfiguration) within the specified time.

- Monitoring & Completion

  - The victim DU monitors if interference actually decreases.

  - Optionally, the victim CU can send a CLI Mitigation Complete message to close the loop, or rely on a timer if a fixed duration was agreed.

## 4. Proposed Specification Updates

### 4.1 XnAP (TS 38.423)

- Served Cell Information NR IE

  - Extend to include SBFD-TimeFrequencyConfiguration (cell granularity).

  - Introduce NZP-CSI-RS-ResourceConfiguration referencing TS 38.331.

  - (Optional) Provide a field for StrongestDLBeamInfo to convey which beam is most frequently used in DL.

- New Procedure: CLI Mitigation Request

  - Messages:

    - CLI Mitigation Request (Initiating)

    - CLI Mitigation Response (Successful Outcome)

  - Request IE:

    - Victim Cell ID (NRCGI)

    - Aggressor Cell ID (NRCGI)

    - Interference Level / cause

    - RequestedAction (mute, reduceTxPower, reconfigurePattern)

    - Duration or subframe pattern bitmask

  - Response IE:

    - Outcome: Accepted / Rejected / Partially Accepted

    - AcceptedAction (if not fully matching RequestedAction, specify partial details)

    - Cause (if rejected or partial)

- Backward Compatibility

  - These new IEs are optional. If a node does not support them, it ignores them.

### 4.2 F1AP (TS 38.473)

- SBFD Configuration

  - In F1 SETUP REQUEST and gNB-DU CONFIGURATION UPDATE, incorporate SBFD-TimeFrequencyConfiguration in Served Cell Information NR IE.

  - In gNB-CU CONFIGURATION UPDATE → Neighbour Cell Information List IE, replicate neighbor cells’ SBFD info.

- NZP CSI-RS Resource Configuration

  - Similarly, add references to NZP-CSI-RS-Resource or NZP-CSI-RS-ResourceSet in Served Cell Information IE.

- DU-CU RIM Information

  - Extend existing IE with a new requestMitigation (flag) and interferenceLevel.

  - When the DU sets requestMitigation = TRUE, it signals the CU to initiate CLI mitigation procedures over Xn if needed.

- CU-DU RIM Information

  - Potentially inform the DU about the outcome of the neighbor’s response.

  - Could reuse “RIM-RS Detection Status = disappeared” to indicate that the neighbor has muted or drastically reduced DL transmissions, but a more explicit “MitigationOutcome” IE might be clearer.

## 5. Detailed Sequence Diagram

Below is an illustrative sequence diagram showing the new procedure in more detail.
(A) Interference Detected: The victim DU observes high UL interference (RIM-RS or CLI-RSSI threshold breach).

- (B) DU → CU: The DU sends an F1 DU-CU Radio Information Transfer with requestMitigation=TRUE.

- (C) CU Aggregation: The victim CU checks whether it has enough data to proceed with a CLI request to the aggressor.

- Xn CLI Mitigation Request: The victim CU sends a newly defined XnAP message specifying the type of mitigation sought.

- (D) Aggressor CU decides if it can comply. Possibly it partially accepts or rejects if it has critical traffic.

- (E) Xn CLI Mitigation Response: Communicates acceptance, partial acceptance (with new parameters), or rejection.

- (F) Aggressor CU → DU: The aggressor CU instructs its DU to apply the agreed measure (mute subframes, reduce power, or reconfigure TDD pattern).

- (G) Victim DU sees an immediate reduction in interference.

- (H) (Optional): The victim can send a “Mitigation Complete” message to confirm or provide final measurements.

## 6. Conclusions

### Proposal 1: Semi-static SBFD Time/Frequency Configuration in Served Cell Information

- Detail:

  - Introduce a new IE (e.g., SBFD-TimeFrequencyConfiguration) in Served Cell Information NR IE for both XnAP and F1AP.

  - Over Xn, this is exchanged in Xn SETUP and NG-RAN NODE CONFIG UPDATE. Over F1, in F1 SETUP REQUEST and gNB-DU CONFIGURATION UPDATE.

  - RAN2 will define the container format; RAN3 will incorporate it into these interfaces.

### Proposal 2: NZP CSI-RS Resource Configuration Exchange

- Detail:

  - Extend Served Cell Information NR IE with references to NZP-CSI-RS-Resource and NZP-CSI-RS-ResourceSet (from TS 38.331).

  - This enables cross-gNB knowledge of which CSI-RS resources are used for interference detection or beam measurement, assisting in CLI identification.

### Proposal 3: Strongest DL Beam Information

- Detail:

  - Provide an IE to indicate the neighbor cell’s strongest DL beam index (SSB or NZP CSI-RS).

  - This allows a victim to pinpoint the likely cause of interference and request partial beam-based mitigation.

### Proposal 4: Enhance F1 Interface for DU-Initiated Mitigation Request

- Detail:

  - Add a requestMitigation flag in the existing DU-CU RIM Information procedure.

  - The DU can also provide an interferenceLevel to indicate severity.

  - The CU is then triggered to coordinate with neighbors over Xn if needed.

### Proposal 5: New Xn Procedure for CLI Mitigation (Request/Response)

- Detail:

  - Define “CLI Mitigation Request” and “CLI Mitigation Response” messages in XnAP.

  - The victim CU can specify the requested action (mute DL, reduce TX power, change TDD pattern) and duration or subframe pattern.

  - The aggressor CU responds with acceptance, partial acceptance (with modifications), or rejection (with cause).

  - This explicit handshake supports dynamic, on-demand CLI resolution.

### Proposal 6:

- RAN3 members are to review these proposals in detail, especially:

  - The structure of the new IE for NZP-CSI-RS resource configuration.

  - The feasibility of partial DL muting or dynamic TDD changes (discussion needed with RAN1/RAN2).

  - ASN.1 definitions for new XnAP messages (Proposal 5) and additions to F1AP.

- Action Items:

  - Align with RAN2 on container definition for SBFD time/frequency.

  - Coordinate with RAN1 on partial DL muting and beam-based interference reduction mechanisms.

  - Ensure backward compatibility: older gNBs can ignore or reject unknown procedures gracefully.