3GPP RAN WG3 Meeting #128	R3-253618

# Malta, 19th – 23th May, 2025

Source:	CATT

Title:	(TP for TS38.423) Discussion on Per S-NSSAI UE performance metrics

Agenda Item:	11.2

Document for:	Discussion and decision

- Introduction

How to calculate Per S-NSSAI per UE performance was still FFS in last meeting as below:

Per S-NSSAI per UE performance metrics is reported as part of the Slice UE Performance IE in the Data Collection Update message. An FFS on how the metrics are calculated will be added in the TP. If no SA5 definition on how to calculate the metrics is available, RAN3 to decide the metrics are derived by an average or weighted average measurement.

This document provides some analysis on above issue.

- Discussion

During previous RAN3 meeting, LS has been sent to SA5 on how to calculate UE level throughput and the feedback from SA5[1] is to use weighted average mechanism. In RAN3 side, we ever had a common understanding that UE level performance was derived by averaging DRB level performance. So, clearly, there are two options on how to derive UE level performance.

One opinion is to define them as simple average, i.e. sum of each result per-DRB average delay divided by number of radio bearers:

DelayperUE=eachDRB_ForTheUEDelayperDRBeachDRB_ForTheUE1

Whereas another opinion is to define them as average weighted by number of packets, i.e. sum of delay of each packet divided by the total number of packets, regardless of the DRB used to deliver:

DelayperUE=eachDRB_ForTheUEDelayperDRBeachPacketInTheDRB1eachDRB_ForTheUEeachPacketInTheDRB1=eachPacketForTheUEDelayperPacketeachPacketForTheUE1

Observation 1: There are two major opinions for the definition of per-slice level UE performance: simple average, or average weighted by number of packets.

As is well known, overall packet delay is the sum of delay in Uu interface, in DU, in F1 and in CU-UP. And for each of the delay component, it seems SA5 already define how to calculate. Take delay in Uu as an example, it is stated in 28.558 as below:

This measurement is obtained as: sum of (point in time when the last part of an RLC SDU packet was sent to the UE which was consequently confirmed by reception of HARQ ACK from UE for UM mode or point in time when the last part of an RLC SDU packet was sent to the UE which was consequently confirmed by reception of RLC ACK for AM mode, minus time when corresponding RLC SDU part arriving at MAC layer) divided by total number of RLC SDUs transmitted to UE successfully. The measurement is performed per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI.

Based on above statement, slice-level delay could be calculated as follow:

Delayperslicing=eachPacketForTheslicingDelayperPacketeachPacketInTheslicing1

Observation 2: In SA5 spec, slice-level packet delay could be derived via weighted averaging DRB level packet delay in case DRB level measurement is already available.

In fact, if no flow-to-DRB remapping occurs during the handover procedure or if there is no flows with small data volume, there is minimal discrepancy between the two methods of calculating per-UE or per-slice-level UE performance. However, if there is flow-to-DRB remapping or if there are flows with small data volume, the calculation outcomes will diverge significantly.

The following table shows the example of QoS flow remapping during handover. QoS flow2 is remapped, and packet delay and data rate are not changed for each QoS flow.

| Before handover |  |  | After handover |  |  |
| --- | --- | --- | --- | --- | --- |
| QoS and DRB mapping | Packet delay | Data rate | QoS and DRB mapping | Packet delay | Data rate |
| DRB1: QoS flow1 | 20ms | 1M bit/s | DRB1: QoS flow1 | 20ms | 1M bit/s |
| DRB2: QoS flow2 | 20ms | 1M bit/s | DRB1: QoS flow2 | 20ms | 1M bit/s |
| DRB3: QoS flow3 | 80ms | 1M bit/s | DRB2: QoS flow3 | 80ms | 1M bit/s |
| Average delay | (20+20+80)/3 = 60ms |  | Average delay | (20+80)/2= 50ms |  |
| Weighted Average delay | (20+20+80)/3 = 60ms |  | Weighted Average delay | (20+20+80)/3 = 60ms |  |

Given that packet delay remains constant and data rate is unchanged, the average packed delay should theoretically remain stable. However, during flow to DRB remapping, average delay changes from 60ms to 50ms under average condition. While weighted average packet delay remains unaffected. This indicates that flow-to-DRB remapping does not impact the weighted average User Equipment (UE) performance, whereas the average UE performance is influenced.

Observation 3: In case of QoS flow remapping, the weighted average delay metric provides more accurate insights compared to the unweighted average.

Another case is the scenario involving DRBs caring small data volume. For instance, if date rate in DRB1 (mapped from QoS flow 1) is low, statistics analyses based on such small sample may have a relatively large fluctuation range. In the following example, the packet delay of DRB1 could fluctuate between 20ms and 60ms. Consequently, the arithmetic mean delay would vary between 50ms and 70ms whereas the weighted Average remains stable. With this, it seems the weighted average is less susceptible to fluctuation caused by small data volume, making it a more reliable metric in such scenario。

| Before handover |  |  | After handover |  |  |
| --- | --- | --- | --- | --- | --- |
| QoS and DRB mapping | Packet delay | Data rate | QoS and DRB mapping | Packet delay | Data rate |
| DRB1: QoS flow1 | 20ms~60ms | 1k bit/s | DRB1: QoS flow1 | 20ms~60ms | 1k bit/s |
| DRB2: QoS flow2 | 80ms | 2M bit/s | DRB2: QoS flow2 | 80ms | 2M bit/s |
| Average delay | 50ms~70ms |  | Average delay | 50ms~70ms |  |
| Weighted Average delay | ((20~60)*1+ 80*1000)/1001≈80ms |  | Weighted Average delay | ((20~60)*1+ 80*1000)/1001≈80ms |  |

Observation 4: Average delay may fluctuate randomly due to the interference of small data, while average weighted can keep stable.

Proposal 1: It is proposed to define Per S-NSSAI UE performance metrics with weighted average measurement.

- Conclusion

Observation 1: There are two major opinions for the definition of per-slice level UE performance: simple average, or average weighted by number of packets.

Observation 2: In SA5 spec, slice-level packet delay could be derived via weighted averaging DRB level packet delay in case DRB level measurement is already available.

Observation 3: In case of QoS flow remapping, the weighted average delay metric provides more accurate insights compared to the unweighted average.

Observation 4: Average delay may fluctuate randomly due to the interference of small data, while average weighted can keep stable.

Proposal 1: It is proposed to define Per S-NSSAI UE performance metrics with weighted average measurement.

- Reference

[1] R3-253023 “Reply LS on per-UE UE performance metrics” RAN3 #128

- TP for TS 38.423

- 9.2.3.x2 Slice UE Performance

This IE indicates per Slice UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Slice UE Performance |  | 1..<maxnoofBPLMNs> |  |  |
| >PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN |
| >S-NSSAIUE Performance List |  | 1 |  | Indicates the UE performance per network slice (details are FFS) |
| >>S-NSSAIUE Performance Item |  | 1 ..<maxnoofSliceItems> |  |  |
| >>>S-NSSAI | M |  | 9.2.3.21 |  |
| >>>Slice Based UE Performance | M |  | UE Performance 9.2.3.179 | Slice Based UE Performance are derived by weighted average of each DRB measurement. |

| Range bound | Explanation |
| --- | --- |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |
| maxnoofBPLMNs | Maximum no. of PLMNIds.broadcast in a cell. Value is 12. |