3GPP RAN WG3 Meeting #128	R3-253621

# Malta, 19th – 23rd May 2025

Source:	CATT

Title:	Use non-UE procedure to request UE performance measurement in gNB-DU

Agenda Item:	11.4

Document for:	Discussion and decision

- Introduction

This document proposes using a non-UE-associated F1AP procedure to request per-UE performance measurement in gNB-DU.

- Discussion

Periodical reporting is supported for UE performance reports (agreed in RAN3#119-bis and #121). And it was a common understanding during the discussion that a RAN node sends only one report per report periodicity even if there are multiple UEs subject of UE performance reporting.

Our interpretation of such common understanding includes that the UE performance results included in each periodical UPDATE message is the intersection of the “Collection Time Duration” of the UE and the duration from the time one periodicity before the UPDATE message is sent and the time when the UPDATE message is sent. For example:

- Assume that the reporting periodicity is configured as 5s, and the “Collection Time Duration for UE Performance” is configured as 10s (for simple).

- There are two UEs subject of UE performance reporting, with UE1 handed over at t=0s and UE2 handed over at t=3s. Assume that both UE are measured until the configured duration expires (e.g. no RRC release).

- One UPDATE is sent at t=5s, including UE performance result for UE1 for 0–5s, and the one for UE2 for 3–5s.

- One UPDATE is sent at t=10s, including UE performance result for UE1 for 5–10s, and the one for UE2 for 5–10s.

- One UPDATE is sent at t=15s, including UE performance result for UE2 for 10–13s.

Proposal 1: Discuss the issue what duration a UE performance result is for in periodical reporting (considering that multiple UEs are handed over at different time), and confirm that it is for the “intersection” of the “Collection Time Duration” of the UE and the duration from the time one periodicity before the UPDATE message is sent and the time when the UPDATE message is sent.

And now we have split gNBs.

Some UE performance is reported first from the target gNB-CU-UP to the target gNB-CU-CP, and then to the source gNB. The reporting periods must be synchronised between E1AP and XnAP, or otherwise (continue to use the abovementioned example):

- Now it is t=5s. One XnAP UPDATE should be sent, including the UE performance result for UE1 for 0–5s, and the one for UE2 for 3–5s. (One gNB-CU-CP may connect to many gNB-CU-UPs, so it cannot adjust the periodicity offset.)

- But one E1AP UPDATE was sent at t=3s, including only the UE performance result for UE1 for 0–3s.

- The next E1AP UPDATE will be sent at t=8s including much information, but not now.

- Thus, the target gNB-CU-CP has no idea on what happened in 3–5s.

- Thus, the target gNB-CU-CP cannot fill the XnAP UPDATE message.

This is obviously not acceptable. So, they must be synchronised.

Synchronised to what?

The time of the first UE subject of data collection is a bad idea. It is possible that UE1 (set up at t=0s) uses gNB-CU-UP1 whereas UE2 (set up at t=3s) uses gNB-CU-UP2. But both gNB-CU-UP should report at t=5s and 10s. The problem still exists.

Therefore, we propose synchronising with the time when the initiation procedure is performed—the gNB-CU-CP can anyway trigger E1AP initiation procedures to all its connected gNB-CU-UPs once upon receiving the XnAP data collection request message.

One may ask what if two neighbour gNB requests UE performance toward this gNB-CU-CP, one at t=0s and the other at t=3s? Well, that is easy to handle: trigger two E1AP data collection initiation procedure with different measurement ID pairs, one at t=0s and the other at t=3s, and include in BEARER CONTEXT SETUP message the corresponding measurement ID pair for the UEs handed over from each given neighbour gNB.

Proposal 2: Confirm that the report periods on E1AP must be synchronised with the report periodicity on XnAP. They should both synchronised according to the time of (XnAP and E1AP) data collection initiation procedures.

The gNB-DU also measures some information of UE performance (to the gNB-CU-UP at least). The measurement period should be synchronised with XnAP as well. A non-UE-associated F1AP procedure is thus needed.

Nevertheless, we are open on what F1AP non-UE-associated message should be used.

Proposal 3: A F1AP non-UE-associated procedure should be used to configure UE performance measurement at the gNB-DU, so that it can be a time reference to synchronise measurement periods with Xn.

The last thing is associating including the measurement ID pair into the UE-associated message. In CU/DU-split gNB, it is the gNB-CU(-CP) to decide the SpCell, i.e. it is the gNB-CU(-CP) which triggers inter-gNB-DU and intra-gNB-DU handover or PSCell change. So, both inter-gNB-DU scenario or intra-gNB-DU scenario may require collecting UE performance. Therefore, the measurement ID pair should be introduced into both the UE CONTEXT SETUP REQUEST message and the UE CONTEXT MODIFICATION REQUEST message.

Proposal 4: The description should cover both handover and PSCell change.

Proposal 5: The measurement ID pair should be introduced into both the UE CONTEXT SETUP REQUEST message and the UE CONTEXT MODIFICATION REQUEST message.

- Conclusion

Proposal 1: Discuss the issue what duration a UE performance result is for in periodical reporting (considering that multiple UEs are handed over at different time), and confirm that it is for the “intersection” of the “Collection Time Duration” of the UE and the duration from the time one periodicity before the UPDATE message is sent and the time when the UPDATE message is sent.

Proposal 2: Confirm that the report periods on E1AP must be synchronised with the report periodicity on XnAP. They should both synchronised according to the time of (XnAP and E1AP) data collection initiation procedures.

Proposal 3: A F1AP non-UE-associated procedure should be used to configure UE performance measurement at the gNB-DU, so that it can be a time reference to synchronise measurement periods with Xn.

Proposal 4: The description should cover both handover and PSCell change.

Proposal 5: The measurement ID pair should be introduced into both the UE CONTEXT SETUP REQUEST message and the UE CONTEXT MODIFICATION REQUEST message.

- Reference

[1] R3-253622; (TP on 38.473) AI/ML in split scenario; CATT.