3GPP RAN WG3 Meeting #128	R3-253623

# Malta, 19th – 23rd May 2025

Title: 	Discussion on the definition of UE Throughput

Source: 	CATT,ZTE,Samsung

Agenda item:	9.2

Document Type:	Discussion and agreement

1. Introduction

In this contribution, we discuss aspects related to the Average UE Throughput metric, which is part of UE performance measurements. In the discussion below we show that there exist a mismatch in definition of the Average UE Throughput metric.

2. Discussion

According to TS 28.558 clauses 6.3.1.4.1 and 6.3.1.4.2., the Average UE Throughput DL and UL are calculated through the Data Volume in RLC level:

##### 6.3.1.4.1 Average DL UE throughput in gNB a) This measurement provides the average UE throughput in downlink. This measurement is intended for data bursts that are large enough to require transmissions to be split across multiple slots. The UE data volume refers to the total volume scheduled for each UE regardless of if using only primary- or also supplemental aggregated carriers. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in EN-DC architecture [16]) and subcounters per supported S-NSSAI. This measurement is also referred to as DL M5 in TS 37.320 [9]. b) DER(N=1) c) This measurement is obtained according to the following formula based on the "ThpVolDl" and "ThpTimeDl" defined below. Separate counters are maintained for each mapped 5QI (or QCI for EN-DC architecture [16]) and for each supported S-NSSAI. If ThpTimeDl>0, ThpVolDlThpTimeDl×1000 [kbit/s] If ThpTimeDl=0, 0 [kbit/s] For small data bursts, where all buffered data is included in one initial HARQ transmission,, otherwise ThpTimeDl; The time to transmit a data burst excluding the data transmitted in the slot when the buffer is emptied. A sample of "ThpTimeDl" for each time the DL buffer for one Data Radio Bearer (DRB) is emptied.. The point in time after T2 when data up until the second last piece of data in the transmitted data burst which emptied the RLC SDU available for transmission for the particular DRB was successfully transmitted, as acknowledged by the UE.. The point in time when the first transmission begins after a RLC SDU becomes available for transmission, where previously no RLC SDUs were available for transmission for the particular DRB.. The RLC level volume of a data burst, excluding the data transmitted in the slot when the buffer is emptied. A sample for ThpVolDl is the data volume, counted on RLC SDU level, in kbit successfully transmitted (acknowledged by UE) in DL for one DRB during a sample of ThpTimeDl. (It shall exclude the volume of the last piece of data emptying the buffer).. d) Each measurement is a real value representing the throughput in kbit per second. The number of measurements is equal to one. If the optional QoS level subcounter and S-NSSAI subcounter are performed, the number of measurements is equal to the number of mapped 5QIs and the number of supported S-NSSAIs. e) The measurement name has the form DRB.UEThpDl, or optionally DRB.UEThpDl.QoS, where QoS identifies the target quality of service class, and DRB.UEThpDl.SNSSAI, where SNSSAI identifies the S-NSSAI. f) NRCellDU g) 5G-S-TMSI h) One usage of this measurement is to support ML training and performance evaluation.

And the Average UE throughput DL and Average UE throughput UL defined during Release 18 had the following definition (TS 38.423, clause 9.2.3.4).

#### 9.2.3.179 UE Performance This IE indicates the UE performance measurements metrics. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description. Average UE Throughput DL; O; Bit Rate 9.2.3.4. Average UE Throughput UL; O; Bit Rate 9.2.3.4. Average Packet Delay; O; 9.2.3.187. Average Packet Loss DL; O; Packet Loss Rate 9.2.3.11. #### 9.2.3.4 Bit Rate This IE indicates the number of bits delivered by NG-RAN in UL or to NG-RAN in DL or by the UE in sidelink within a period of time, divided by the duration of the period. It is used, for example, to indicate the maximum or guaranteed bit rate for a GBR QoS flow, or an aggregate maximum bit rate. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Bit Rate; M; INTEGER (0..4,000,000,000,000,…); The unit is: bit/s.

Consider the following example shown in Figure 1: For a single UE, the measurement period is 5s, the transmitted data volume is 10Mbit, and it takes 2s to deliver. The time length when the RLC buffer is not empty is 2s.

Figure 1. Average UE Throughput

The definition of ThpTime is the time to transmit a data burst excluding the data transmitted in the slot when the buffer is emptied, so the ThpTime is 2s. Applying the above formula ThpVolDlThpTimeDl×1000 [kbit/s] results in a UE-level throughput of 10 Mbit / 2 s = 5Mbps.

However, The IE of throughput introduced in Rel-18 refers to TS 38.423, clause 9.2.3.4“Bit Rate”, and the description of “Bit Rate” states that: “This IE indicates the number of bits delivered by NG-RAN in UL or to NG-RAN in DL or by the UE in sidelink within a period of time, divided by the duration of the period”. It seems to imply that the divisor is the measurement period, i.e. 5s, rather than the ThpTime. The result is thus impled as Total Data volumeThe Measurement period, i.e. 10Mbit/5s = 2Mbps, same as the “Average Bit Rate”.

|  | Formula | Denominator(Time Scope) | Example (10 Mbit in 5s, 2s active) |
| --- | --- | --- | --- |
| Average UE Throughput | ThpVolThpTime×1000 [kbit/s] | Only when RLC buffer is non-empty (2s) | 10 Mbit / 2 s = 5 Mbps |
| Average Bit Rate | Total Data volumeThe Measurement period | Entire period (5s) | 10 Mbit / 5 s = 2 Mbps |

Therefore, the reference of “9.2.3.4 Bit Rate” incurred a mismatch in the definition of the Average UE Throughput.

Observation 1: The reference of “9.2.3.4 Bit Rate” as the IE type of UE throughputs incurred a mismatch in the definition of the Average UE Throughput in TS 28.558 and the definition in TS 38.423.

There are two solutions available for the aforementioned issues with Average UE Throughput. The first solution is to introduce a new IE type for the Average UE Throughput. This solution is non backwards compatible, but only impacts the AI/ML for NG-RAN. The second solution is to revise the description of the Bit Rate IE in TS 38.423 to make it applicable for Average UE Throughput, i.e. indicating that it can also be used to indicate Average UE Throughput. RAN3 needs to discuss the above solutions.

Proposal 1: RAN3 needs to discuss the following solutions:

- Solution 1: Introduce a new IE type for the Average UE Throughput.

- Solution 2: Revise the description of the Bit Rate IE in TS 38.423 to explicitly specify that it can also be used for Average UE Throughput

CR reflecting the main proposals in this paper are presented in [1] and [2].

3. Conclusions

This paper makes the following observations and proposals:

Observation 1: The reference of “9.2.3.4 Bit Rate” as the IE type of UE throughputs incurred a mismatch in the definition of the Average UE Throughput in TS 28.558 and the definition in TS 38.423.

Proposal 1: RAN3 needs to discuss the following solutions:

- Solution 1: Introduce a new IE type for the Average UE Throughput.

- Solution 2: Revise the description of the Bit Rate IE in TS 38.423 to explicitly specify that it can also be used for Average UE Throughput

4. Reference

[1] R3-253624 Correction on the definition on UE throughput CATT.ZTE,Samsung

[2] R3-253625  Correction on the definition on UE throughput CATT.ZTE,Samsung