3GPP TSG RAN3 meeting #128   	         	                                            R3-253632

19th – 23rd May 2025  Malta, MT

Agenda item: 	    10.2

Source: 	China Unicom

Title: 	Discussion on MRO enhancements

Document for:	Discussion

1	Introduction

At RAN#102, the new Work Item on Data Collection for SON_MDT in NR standalone and MR-DC phase 4 was agreed [1] which introduced the following objectives in the core part:

The objective of this work item is to specify data collection enhancement in NR standalone and MR-DC for SON/MDT purpose. The specific objectives of this work item are: - MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]: - Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3] - Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2] - Support of SON/MDT enhancements for [RAN3, RAN2]: - Intra-NTN mobility - Network Slicing - Support of the leftovers in Rel-18 SON/MDT [RAN3, RAN2]: - RACH optimization for SDT - MHI Enhancement for SCG Deactivation/Activation - MRO for MR-DC SCG failure If needed, co-operate with RAN1, SA2, SA5, CT4.

This contribution provides our views on the scenarios of MRO enhancements which can be supported in Rel-19.

2	Discussion

2.1	LTM

UHI and ping-pong issue UHI and ping-pong issue: For intra-CU case, CU detects the potential LTM ping-pong issue based on local information, DU decides and solves the LTM ping-pong issue.

At the RAN3#127bis meeting, we have been made above agreements and discussed LTM ping-pong avoidance.

For ping-pong issue in LTM, since CU determines the LTM configuration and candidate cell list then sends to target candidate gNB-DU(s), CU should be responsible for avoiding LTM cell switch ping-pong with all the UHI information when choosing the candidate LTM cells. When inappropriate cell switch triggering made by DU, DU should be responsible for the issue. CU can do the initial analyse then sends the UHI to DUs when inappropriate cell switch triggering made by DU. Furthermore, considering which message to be used to propagate the UE history information, we prefer to use CU-DU CELL SWITCH NOTIFICATION message when the cell switch command executed.

Proposal 1: CU sends the UHI to source DU when inappropriate cell switch triggering made by DU via CU-DU CELL SWITCH NOTIFICATION message.

LTM failure due to outdated TA:

MRO will cover the scenario that RACH-less LTM fails including outdate TA and UE performs RACH based LTM.

RAN3 will not consider the case of LTM cell switch failure due to outdated TA calculated by UE.

For PDCCH order triggered early TA acquisition, RAN3 consider the case of RACH-less LTM fails due to outdate TA and UE performs RACH-based LTM failure recovery or RRC re-establishment to the same cell.

Whether UE based solution or network based solution is used for LTM failure due to outdated TA?

Whether LTM failure due to outdated TA is defined as a new failure case or it can covered by existing failure case?

In last meeting, we have made some progress on LTM failure due to outdated TA. As for which solutions to be used, we prefer network-based solution since network could identify the outdated TA without any UE impact. In that case, CU should be the node to identify LTM cell switch failure due to outdated TA with the information provided by source DU and target DU.

Proposal 2: Network-based solution is preferred for LTM failure due to outdated TA.

Proposal 3: CU identifies LTM cell switch failure due to outdated TA.

2.2	CHO with candidate SCGs

In last RAN3#127bis meeting, we have the following agreements.

No additional information is needed in handover report message.

Check stage 3 TP on the Target cell CGI related in handover report message.

One SCGFailureInfomation is included in SCG Failure Indication message, no need aggregation.

Respective MN in stage2 is at least a candidate MN.

Information needed in the SCG Failure Indication message: The Xn UE AP ID in the receiving node.

Additional information reported from the UE in SCGFailureInformation: The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the SCG failure is encountered.

No support of near failure case in R19.

Whether Handover Report message is needed for case 7a?

In addition to SCGFailureInformation, information needed in the SCG Failure Indication message?

- CPC failure type (CPC failure type is needed in case of SCG Failure Indication is sent from the candidate to the source?)

- Suitable PCell ID

We agreed to reuse Handover Report message to allow the last serving MN to inform the candidate MN of a failure in the case of a too late CHO. But here in case 7a, it is a too early HO or wrong cell handover, we think the handover report is not needed for case 7a.

Proposal 4: The is no need to use Handover Report message for case 7a.

2.3	Subsequent CPAC

The description on S-CPAC information forwarding is technically correct. Cleanup of the text is needed.

Whether to enhance UHI in case of S-CPAC? The motivation and benefits need to be clarified.

Ping-pong issue

For pingpong issue addressed in S-CPAC, when considering SN initiated S-CPAC case, it is beneficial for SN to have the whole accurate UE history information to do the optimization. As in subsequent CPAC, UHI cannot be updated to SN after the first initiating phase, MN need to propagate the updated UHI back to the initiating SN to do the further optimization.

Proposal 5: For SN initiated S-CPAC, MN sends UE history information to initiating SN to optimize the S-CPAC configuration.

For MN initiated S-CPAC, MN has the whole picture of UHI, and it could identify the pingpong issue and optimize itself without spec impacts.

Proposal 6: For MN initiated S-CPAC,  there is no spec impacts to solve the ping-pong issue.

3	Conclusion

Proposal 1: CU sends the UHI to source DU when inappropriate cell switch triggering made by DU via CU-DU CELL SWITCH NOTIFICATION message.

Proposal 2: Network-based solution is preferred for LTM failure due to outdated TA.

Proposal 3: CU identifies LTM cell switch failure due to outdated TA.

Proposal 4: The is no need to use Handover Report message for case 7a.

Proposal 5: For SN initiated S-CPAC, MN sends UE history information to initiating SN to optimize the S-CPAC configuration.

Proposal 6: For MN initiated S-CPAC,  there is no spec impacts to solve the ping-pong issue.

4   References

- RP-234038: New WID: Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) Phase 4; TSG RAN Meeting #102, Edinburgh, Scotland, Dec 11-15, 2 023