3GPP TSG-RAN WG3 #128                                       R3-253654

Malta, MT, 19 - 23 May, 2025

Agenda item:	13.2

Source:	Samsung

Title:	Discussion on inter-gNB-CU LTM

Document for:	Discussion & Decision

# 1 Introduction

RAN3 had some discussions on inter-CU LTM in previous meetings, and below agreements are achieved in last meeting:

RAN3#127-bis: Inter-CU LTM Semi-Persistent CSI-RS coordination - Introduce two new Class-1 UE-associated procedures over F1AP to support the source gNB-DU to trigger the activation/deactivation of the SP CSI-RS transmission in the candidate cell(s). - Introduce a new class-1 UE-associated procedures over XnAP for the SP CSI-RS activation/deactivation. - Open issues on the granularity of the CSI-RS resource/resource set, and need to be checked with RAN2 progress. Security Mechanism in inter-CU LTM - For the initial inter-CU LTM preparation, the source gNB computes the KgNB*(s) (per candidate cell) for the candidate gNB, and forwards the {KgNB*, NCC} pair(s) to the candidate gNB via a Handover Request message as legacy. - The new source gNB needs to provide the new KgNB*(s) to the corresponding candidate gNB via a LTM configuration update procedure. FFS on the procedure design either per cell or per gNB. - The new NCC value needs to be sent from gNB-CU to the gNB-DU via UE context modification procedure. Whether to define additional information from CU to DU to deliver NCC value. TA information transfer - Turn the WA into agreement: Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU. Include the following information in the TA information transfer message: - Candidate Cell ID - TA value - Preamble index - RA-RNTI - FFS on the XnAP IDs. LTM Preparation for Parallel messages - Turn the WA into agreement: Use a single UE association (e.g. identified by a pair of {Source NG-RAN node UE XnAP ID IE and Target NG-RAN node UE XnAP ID IE}) for multiple LTM handover request to the same candidate gNB. UE associated context setup for subsequent LTM - For subsequent LTM, the candidate gNB uses the old target UE XnAP ID (the target UE AP ID it previously allocated between it and the old serving gNB) to identify the UE context when receiving the LTM Configuration Update message from the new serving gNB. FFS on how to deliver the old target UE XnAP ID(s) to the new serving gNB. Data Forwarding Info IE design - Data Forwarding Information IE is a list including multiple data forwarding addresses from each candidate gNB(s). Inter-CU LTM with DC - Include the following information in SN Change Required (Source SN to MN) message: - LTM request indication (explicit way) - Suggested PScell list (implicit way in INM, FFS on explicit way) - Measurement results (implicit way in INM) - Upper limit number of PScell(s) for each candidate SN (FFS) - CSI Resource configuration request (explicit way) - Early sync information request (explicit way) - Include the following information in SN Addition Request (MN to Candidate SN) message: - LTM request indication (explicit way) - Suggested PScell list (implicit way in INM, FFS on explicit way) - Measurement results (implicit way in INM) - Upper limit number of PScell(s) for the candidate SN (explicit way) - CSI Resource configuration request (explicit way) - Early sync information request (explicit way) - It is the source SN to generate the LTM configuration ID mapping list for the accepted candidate PSCell(s), and send it to the MN for subsequent LTM via SN modification request acknowledge message. Then the MN forwards the LTM configuration ID mapping list to the candidate SN via SN Modification Request message. - For SN-initiated inter-CU SCG LTM, the source SN can provide the SCG reference configuration via SN Change Required message. If the source SN does not provide the SCG reference configuration, MN can request SCG reference configuration from one of the candidate SN(s), FFS on whether MN can request SCG reference configuration from the source SN. - The TA information transfer message can be reused and enhanced to transfer the TA information from the candidate SN to the source SN via the MN. - Include the target PSCell ID and corresponding TCI State ID(s) in the Cell Switch Notification message.

In this paper, we will further discuss the basic procedure for inter-gNB-CU LTM based on agreed BLCR in [1], [2] and [3], and discuss the potential issues on Xn/F1 interface.

# 2 Discussion

To achieve the analysis for network interface impact for the scenario of inter-gNB-CU LTM, we draw the potential message flows of the whole inter-gNB-CU LTM process as shown in the below Fig.1 to provide the basic description for each involved message flows and our view for the issues raised in stage 2/3 BLCR. After that, we finally summarize our proposals.

Fig.1 Message Flows for Inter-gNB-CU LTM

The procedure for inter-gNB-CU LTM is as follows:

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB-DU containing measurements of inter-gNB neighbouring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

2.	The source gNB-CU determines to initiate inter-gNB-CU LTM configuration.

3.	The source gNB requests inter-gNB LTM for one or more candidate cells belonging to one or more candidate gNB(s). The source gNB initiates a HANDOVER REQUEST message per candidate cell containing one candidate cell ID and the CSI resource configuration for subsequent LTM. The source gNB may request the candidate gNB(s) to provide the CSI-RS configuration.

4.  The candidate gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU(s) for each candidate cell to request LTM configurations.

5.	If the candidate gNB-DU accepts the request of LTM configuration, it responds with a UE CONTEXT SETUP RESPONSE message including the LTM lower layer RRC configurations.

6.  The candidate gNB(s) prepares the LTM configuration(s) and sends inter-gNB response (HO REQUEST ACKNOWLEDGE) to the source gNB including the generated RRC configurations for the accepted candidate cell. The candidate gNB(s) may also include the CSI-RS configuration upon request.

7.	The source gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU including the information related to early sync and the LTM configuration IDs for the accepted target candidate cell(s) in other candidate gNB-CU(s).

8.	The source gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message which includes an updated lower layer configuration, e.g., containing the updated CSI report configuration of the source cell.

9.	The source gNB sends an LTM CONFIGURATION UPDATE to the candidate gNB(s) to update the LTM configurations of candidate cell(s). The source gNB may include the common CSI-RS resource configuration.

10.	The candidate gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the candidate gNB-DU(s) containing the information of candidate cells for subsequent LTM. The candidate gNB-CU may include the common CSI-RS resource configuration.

11.	The candidate gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message. The candidate gNB-DU may also provide the CSI-RS report configuration.

12.	The candidate gNB(s) sends the LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the source gNB. The candidate gNB(s) may also provide the CSI-RS report configuration.

Editor’s Note: FFS on whether step 9 and 12 are mandatory or optional.

NOTE: Step 9 may also be triggered after step 24, or after step 32 by implementation for subsequent LTM.

13.	The source gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message with the LTM configuration.

14.	The source gNB-DU forwards the received RRCReconfiguration message to the UE.

15.	The UE stores the LTM candidate configurations and sends an RRCReconfigurationComplete message to the source gNB-DU.

16.	The source gNB-DU forwards the RRCReconfigurationComplete message to the source gNB-CU via an UL RRC MESSAGE TRANSFER message.

17. If early data forwarding is applied, the source gNB sends the EARLY STATUS TRANSFER message to other candidate gNB(s).

NOTE: Early data forwarding can be triggered before the source gNB triggers a MAC CE Cell Switch Command to the UE to change cell, timing is left to implementation.

18.	Early synchronization to the target candidate cell(s) may be performed.

19. The candidate gNB-DU sends a DU-CU TA INFORMATION TRANSFER message to the candidate gNB-CU, which includes the TA value, the associated PRACH resource information, and etc.

20.	The candidate gNB-CU forwards the TA value and the associated CFRA resource information to source gNB-CU via the TA INFORMATION TRANSFER message.

21. The source gNB-CU forwards the TA value, and the associated CFRA resource information to the source gNB-DU in the CU-DU TA INFORMATION TRANSFER message.

22.	The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the source gNB-DU. L1 measurement should be performed as long as RRC reconfiguration (step 14) is applicable.

23.	The source gNB-DU determines to initiate inter-gNB-CU LTM.

24.	The source gNB-DU decides to execute cell switch to a target cell and transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID which indicates the index of the candidate configuration of the target cell, a beam indicated with a TCI state or beams indicated with DL and UL TCI states, and a timing advance command for the target cell, if available. The UE switches to the target cell and applies the candidate configuration indicated by the target configuration ID.

NOTE :	Up to implementation, data forwarding and SN Status Transfer may be initiated once the source gNB triggers the inter-gNB LTM cell switch for the UE in Step 24.

25.	The source gNB-DU sends the DU-CU CELL SWITCH NOTIFICATION message to source gNB-CU to indicate the initiation of the Cell Switch Command to the UE including the target cell ID and the TCI state ID(s).

26.	The source gNB-CU sends the CELL SWITCH NOTIFICATION message to the target gNB-CU to indicate the initiation of Cell Switch command to the UE.

27.	The target gNB-CU forwards the target cell ID and the TCI state ID to the target gNB-DU in the CU-DU CELL SWITCH NOTIFICATION message.

28.	The target gNB-DU detects the UE access.

29.	The target gNB-DU sends the ACCESS SUCCESS message to the target gNB-CU with the target cell ID.

30.	The UE sends an RRCReconfigurationComplete message to the target gNB-DU.

31.	The target gNB-DU forwards the RRCReconfigurationComplete message to the target gNB-CU via an UL RRC MESSAGE TRANSFER message.

32/33. The target gNB-CU sends the HANDOVER SUCCESS message to the source gNB-CU to inform that the UE has successfully accessed the target cell. In return, the source gNB-CU sends the SN STATUS TRANSFER message following the principles described in step 7 of Intra-AMF/UPF Handover in clause 9.2.3.2.1.

NOTE: Late data forwarding may be initiated as soon as the source gNB receives the HANDOVER SUCCESS message.

34. The target gNB-CU sends a PATH SWITCH REQUEST message to 5GC to trigger 5GC to switch the DL data path towards the target gNB-CU and to establish an NG-C interface instance towards the target gNB-CU.

35.	The 5GC confirms the PATH SWITCH REQUEST message with the PATH SWITCH REQUEST ACKNOWLEDGE message to target gNB-CU.

36.	The new source gNB (i.e., the target gNB) sends the LTM CONFIGURATION UPDATE message to the candidate gNBs. This message may include the new security key(s) to be used with the UE.

37. The candidate gNB(s) responds the LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the new source gNB.

38. The new source gNB may send the UE CONTEXT RELEASE message to inform the old source gNB to release radio and C-plane related resources associated to the UE context if no LTM candidate cell(s) exist in the old source gNB. Any ongoing data forwarding may continue.

39.	The source gNB-CU may send the UE CONTEXT RELEASE COMMAND message to the source gNB-DU to release the resources of source cell.

40.	The source gNB-DU responds with a UE CONTEXT RELEASE COMPLETE message.

NOTE: The steps 17-40 can be performed multiple times for subsequent LTM cell switch executions.

From the above overview of inter-gNB-CU LTM procedure, the impacts on Xn/F1 interface are mainly reflected in the following aspects:

## Issue 1: Early TA RACH resource allocation

In RAN3#127 meeting, the following agreement was reached on this issue:

- The source gNB-CU sends the pair of (gNB ID, new ID of early RACH configuration resource (to be further discussed)) to the candidate gNB-CU to request early RACH configuration. - The format of the new introduced IE is same with the gNB-DU ID.

From the RAN3’s agreements, we note that early RACH configuration of each candidate cell is allocated based on gNB-DU which is replaced with an implicit ID (new ID of early RACH configuration resource) in XnAP specification. And the dedicated different early RACH configuration is allocated by candidate cell for each candidate gNB-DU. RACH configuration could be shared only among the UEs in a single gNB-DU to avoid the RACH access conflict between UEs from different gNB-DU.

However, another case exists. If one LTM gNB is not CU-DU split architecture, how does source gNB request early RACH configuration to this LTM gNB for each candidate cell? We think RACH configuration allocation shoult not be based on gNB-DU, should be based on gNB in case of aggregated gNB.

Therefore, we should announce below mechanism on early RACH configuration allocation.

- Case#1: If one LTM gNB is CU-DU split architecture, early RACH configuration of candidate cell is allocated based on gNB-DU.

- Case#2: If one LTM gNB is not CU-DU split architecture, i.e., aggregated gNB, early RACH configuration of candidate cell is allocated based on gNB.

Proposal 1-1: RAN3 support the early TA RACH resource allocation is based on gNB-DU in case of split gNB and/or based on gNB in case of aggregated gNB.

Additionally, in case#1, we should note before the step that the source gNB-CU sends the pair of (gNB ID, new ID of early RACH configuration resource) to the candidate gNB-CU to request early RACH configuration is initiated from the source gNB-CU, the source gNB-CU should be aware of the gNB-DU ID of each candidate cell belonging.

Then, how does source gNB-CU acquire the gNB-DU ID information to which inter-CU candidate cell belongs? Reviewing current specification as below highlight from TS 38473, it cannot be achieved via Cell ID information due to there is no connection between gNB-DU ID and cell identifiers.

9.3.1.9 gNB-DU ID [F1 IE] The gNB-DU ID uniquely identifies the gNB-DU at least within a gNB-CU. IE/Group Name; Presence; Range; IE type and reference; Semantics description. gNB-DU ID; M; INTEGER (0 .. 236-1); The gNB-DU ID is independently configured from cell identifiers, i.e. no connection between gNB-DU ID and cell identifiers..

Therefore, RAN3 should discuss this issue on how does source gNB-CU acquire the gNB-DU ID information to which inter-CU candidate cell belongs.

Proposal 1-2: RAN3 should discuss how source gNB-CU acquires the gNB-DU ID information to which inter-CU candidate cell belongs before initiation of LTM preparation.

Based on the mechanism of early TA RACH resource allocation proposed by Proposal 1-1, in the Case#2: If one LTM gNB is not CU-DU split architecture, i.e., aggregated gNB, early RACH configuration of candidate cell is allocated based on gNB. Let’s assume the case that source gNB is aggregated gNB, and one candidate gNB with one candidate cell is CU-DU split gNB. When source gNB initiates the HO REQ to candidate gNB with Early Sync Information Request IE for requesting early TA RACH resource from candidate cell, the IE of Early RACH Resources Requester ID seem to be reasonable as an optional IE since source gNB is an aggregated gNB.

Below IEs are from BLCR in [2] #### 9.2.1.xx3 Early Sync Information Request This IE contains a request for Early synchronization information. Editor’s note: Details on IEs need to be continued. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Early Sync Configuration Request; M; ENUMERATED (true, …). Early RACH Resources List; 0..1. >Early RACH Resources Item; 1..< maxnoofEarlyRACHResourcesID>. >>Global gNB ID; M; 9.2.2.1. >>Early RACH Resources Requester ID; M; 9.2.1.xx9; Identifies the entity requesting Early RACH resources.. Range bound; Explanation. maxnoofEarlyRACHResourcesID; Maximum no. of Early RACH resources identities allowed to be configured with LTM towards one UE, the maximum value is 8..

Furthermore, when candidate gNB-CU initiates UE Context Setup Request to candidate gNB-DU with Early Sync Information Request IE for requesting early TA RACH resource from candidate cell, it will introduce another issue if reusing current IE tabular on LTM gNB-DUs List IE. LTM gNB-DU ID IE is a mandatory IE for intra-CU LTM but it is possible to be an optional IE for inter-CU LTM in case source gNB is an aggregated gNB.

Regarding the DU-CU TA INFORMATION TRANSFER message, the same issue is existing in current IE tabular, and the mandatory IE on the Source gNB-DU ID IE is possible to be an optional information for inter-CU LTM in case source gNB is an aggregated gNB.

Below IEs are from BLCR in [3] Early Sync Information Request; 0..1; YES; ignore. >Request for RACH Configuration; O; ENUMERATED (true, …); -. >LTM gNB-DUs List; 0..1; This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s).; YES; reject. >>LTM gNB-DUs Item IEs; 1..< maxnoofLTMg NBDUs>. >>>LTM gNB-DU ID; M; gNB-DU ID 9.3.1.9. >>>LTM gNB ID; O; Global gNB ID 9.3.1.305; -. #### 9.2.1.24 DU-CU TA INFORMATION TRANSFER This message is sent by the gNB-DU to inform the gNB-CU about TA information. Direction: gNB-DU gNB-CU IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.3.1.1; YES; ignore. Transaction ID; M; 9.3.1.23; YES; reject. DU to CU TA Information List; 0..1; YES; ignore. >DU to CU TA Information Item IEs; 1 .. <maxnoof TAList>; EACH; ignore. >>Candidate Cell ID; M; NR CGI 9.3.1.12; -. >>TA Value; M; INTEGER (0..4095); Indicates the TA value as defined in TS 38.213 [31].; -. >>Preamble Index; M; INTEGER (0..63); -. >>RA-RNTI; M; INTEGER (0..65535, ...); RA-RNTI as defined in TS 38.321 [16].; -. >>Source gNB-DU ID; M; gNB-DU ID 9.3.1.9; -. >>Tag ID Pointer; O; OCTET STRING; Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8].; -. >>Source gNB ID; O; Global gNB ID 9.3.1.305; -.

Therefore, RAN3 should re-think the F1/Xn IEs design on early TA RACH resource request & allocation & TA information transfer. One simple way is that define the related gNB-DU ID IE to an optional IE in the Rel-19 specification.

Proposal 1-3: RAN3 should define F1 gNB-DU ID IE to be an optional IE in the Rel-19 specification on the procedure of early TA RACH resource request & allocation & TA information transfer.

## Issue 2: FFS the detail in TA INFORMATION TRANSFER message

This issue is related to the early TA information transfer from candidate gNB-DU to source gNB-DU via candidate gNB-CU and source gNB-CU in step 20.

We already agreed to introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU.

- Turn the WA into agreement: Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU. Include the following information in the TA information transfer message: - Candidate Cell ID - TA value - Preamble index - RA-RNTI - FFS on the XnAP IDs.

Furthermore, the detail IEs are agreed to be included in F1AP DU-CU/CU-DU TA INFORMATION TRANSFER, including a candidate cell list that include related TA information and RACH resource information for each candidate cell. Additionally, if source gNB is a split gNB and early TA RACH resource is allocated based on gNB-DU as mentioned in the issue#1, the implicit gNB-DU ID should be indicated in the XnAP TA INFORMATION TRANSFER message to inform source gNB-CU to forward TA information to the indicated source gNB-DU.

Proposal 2-1: Propose to include an optional Early RACH Resources Requester ID information in XnAP TA INFORMATION TRANSFER message to inform source gNB-CU to forward TA information to the indicated source gNB-DU in case of split architecture for source gNB.

Regarding FFS on the XnAP IDs to be included in XnAP TA Information Transfer message, we have agreed that this procedure is a non-UE associated procedure, and related RACH resource information (Preamble index and RA-RNTI) for the indicated TA value of candidate cell is also carried to source gNB/DU to identify the UE to which this TA value is belonging. Therefore, it is following the R18 legacy mechanism. So we do not need to include the related XnAP IDs information to be used to identify UE.

Proposal 2-2: No need to include the XnAP IDs information in the XnAP TA Information Transfer message, since the RACH resource information for the indicated TA value of candidate cell is used for source gNB/DU to identify the UE to which this TA value is belonging.

## Issue 3: Stage 3 IE design on Data forwarding Info in LTM Configuration Update message

For supporting data forwarding in the subsequent inter-CU LTM scenarios, RAN3 have reached the following agreement in last meeting:

Reuse the LTM Configuration Update procedure to sync up configurations among candidate gNBs for subsequent LTM, including early sync configuration, configuration ID, and data forwarding addresses, etc. It is up to the network implementation when the sync up is performed: during the preparation step, or during the cell switch execution step, or after successful cell switch. Data Forwarding Information IE is a list including multiple data forwarding addresses from each candidate gNB(s).

From the agreement, we can see that Data Forwarding Info IE is a list including multiple data forwarding addresses from each candidate gNB(s) which is carried to other candidate gNBs via the LTM Configuration Update message for supporting early data forwarding in subsequent LTM. And after last RAN3 meeting, the merged XnAP BLCR in [2] is archived, we can see the Data Forwarding Info IE is included in the LTM Configuration Update message, and Editor’s note is indicated. The Data Forwarding Info IE is a list, the granularity is FFS. Therefore, we will continue to discuss this issue on FFS the details on data forwarding info IE.

#### 9.1.1.x3 LTM CONFIGURATION UPDATE This message is sent by the NG-RAN node1 to update LTM configuration data. Direction: NG-RAN node1  NG-RAN node2. Editor’s note: Details on IEs need to be continued. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.2.3.1; YES; reject. NG-RAN node 1 UE XnAP ID; M; NG-RAN node UE XnAP ID 9.2.3.16; Allocated at the NG-RAN node 1; YES; reject. NG-RAN node 2 UE XnAP ID; O; NG-RAN node UE XnAP ID 9.2.3.16; Allocated at the NG-RAN node 2; YES; reject. Target Cell Global ID (FFS); M; NR CGI 9.2.2.7; YES; reject. LTM Information Request; O; 9.2.1.xx1; YES; ignore. Early Sync Information Request; O; 9.2.1.xx3; YES; ignore. Early Sync Information; O; 9.2.1.xx6; YES; ignore. Early Sync Information for SUL; O; 9.2.1.xx6; YES; Ignore. Data Forwarding Info; FFS; YES; ignore. Key NG-RAN Star; O; BIT STRING (256); KNG-RAN* defined in TS 33.501 [28].; YES; ignore. Security Information for LTM (FFS on the name); O; 9.2.1.x12; YES; ignore. Editor’s note: The Data Forwarding Info IE is a list, the granularity is FFS.

Regarding FFS on the details on Data Forwarding Info IE, when reviewing the current the specification in XnAP TS 38.423, the basic principle is that one data forwarding address is allocated based on each SN in case of MR-DC, but different data forwarding address can be allocated based on each candidate PCell in case of SA. Therefore, we think that data forwarding address per-candidate cell is reasonable for inter-CU LTM.

Proposal 3-1: Regarding inter-CU LTM, Data forwarding address is based on candidate cell not candidate gNB.

To indicate the data forwarding address for each candidate cell, Data Forwarding Info from target NG-RAN node IE is generally used in the some previous examples. Since this IE is per-PDU session, we can design the Data Forwarding Info IE as below:

9.2.1.xx Data Forwarding Info This IE contains different data forwarding addresses information per-PDU session for different candidate LTM cells. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Data Forwarding Info List; 1; YES; ignore. >Data Forwarding Info Item; 1.. <maxnoof LTMCells>; –. >>LTM Cell ID; M; NR CGI 9.2.2.7; –. >>PDU Session Resources List; 1; –. >>>PDU Session Resources Item; 1. .<maxnoofPDU Sessions>; –. >>>>PDU Session ID; M; 9.2.3.18; –. >>>>Data Forwarding Info from target NG-RAN node; M; 9.2.1.16; –. Range bound; Explanation. maxnoofLTMCells; Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8.. Range bound; Explanation. maxnoofPDUSsessions; Maximum no. of PDU sessions. Value is 256.

Proposal 3-2: RAN3 to agree to use Data Forwarding Info from target NG-RAN node IE per-PDU session per-candidate cell as the included details in the Data Forwarding Info IE.

## Issue 4: FFS TA Information List IE in Cell Switch Notification message

Regarding Cell Switch Notification from source gNB-DU to candidate gNB-DU via source gNB-CU and candidate gNB-CU, we already agree to introduce a new XnAP UE-associated procedure, namely Cell Switch Notification message.

Following the current detail IEs in F1AP DU-CU CELL SWITCH NOTIFICATION in R18 intra-CU LTM, since TA values of candidate cells may be also forwarded from source gNB-DU to candidate gNB-DU for subsequent LTM use, we also propose to include TA Information List IE in the XnAP CELL SWITCH NOTIFICATION message for subsequent LTM use in new source gNB.

Proposal 4: TA Information List IE can be included in XnAP Cell Switch Notification message to forward the valid TA values of candidate cells for reuse in subsequent LTM.

## Issue 5: FFS on how to deliver the old target UE XnAP ID(s) to the new serving gNB

In RAN3#126 meeting, below agreement is reached to support establishing UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch.

To support subsequent LTM, the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch.

In plus, we need to consider how to identify the same UE when candidate gNB receives the LTM Configuration Update message from new source gNB. As below figure shows, the C-gNB2 (new source gNB2) initiates to establish UE association with C-gNB3, by sending LTM Configuration Update carrying one UE ID reference info for the candidate gNB to identify the same UE.

Through the discussion in last RAN3#127bis meeting, the below agreements are reached and old target UE XnAP ID can be included as the related UE ID reference info for the candidate gNB to identify the same UE.

For subsequent LTM, the candidate gNB uses the old target UE XnAP ID (the target UE AP ID it previously allocated between it and the old serving gNB) to identify the UE context when receiving the LTM Configuration Update message from the new serving gNB. FFS on how to deliver the old target UE XnAP ID(s) to the new serving gNB.

And next, the new issue is raised FFS on how to deliver the old target UE XnAP ID(s) to the new serving gNB.

Regarding this issue, it is possible to have two options:

Option#1: Reuse Cell Switch Notification message when LTM is triggered.

Option#2: Reuse the LTM Configuration Update message when the procedure of sync up configurations is performed for subsequent LTM.

It is up to the network implementation when the sync up is performed: during the preparation step, or during the cell switch execution step, or after successful cell switch.

From our view, even two options both can work, considering UE recovery in another LTM candidate gNB in case of LTM failure to target gNB, LTM Configuration Update message seems to be better option.

Proposal 5: Reuse the LTM Configuration Update message to transfer old target UE XnAP ID(s) to new serving gNB when the procedure of sync up configurations is performed for subsequent LTM.

## Issue 6: Enhancement for the LTM candidate cells cancel via the procedure of LTM Configuration Update/Acknowledge

Regarding the LTM candidate cells cancel or release, based on previous agreements, source gNB or candidate gNB can initiate LTM cells cancel/release by Handover Cancel message or LTM Cancel message.

The Handover Cancel message is reused by the source gNB to release the reserved resource for LTM candidate cells in the candidate gNBs. A candidate gNB can initiate cancellation of configured LTM candidate cell(s) of its own. Adopt Class-2 procedure for candidate gNB-initiated LTM cancellation. Confirm the message name as LTM Cancel for candidate gNB-initiated LTM cancellation.

However, after each inter-CU LTM Cell Switch, new source gNB may initiate to setup the UE associated Xn signalling connection with the candidate gNBs via LTM Configuration Update/Acknowledge. If some LTM candidate cells are released due to some reasons, this procedure cannot be initiated by source gNB or candidate gNB until the setup for the UE associated Xn signalling connection between new source gNB and candidate gNB. Therefore, if LTM candidate cells cancel/release information can be included in the LTM Configuration Update/Acknowledge message, the following Handover Cancel or LTM Cancel does not need be initiated so that can save signalling procedure.

In other words, it has a benefit to introduce LTM Candidate Cells To Be Cancelled List IE in the LTM Configuration Update/Acknowledge if there are LTM candidate cells to be released during the setup for UE associated Xn signalling connection between new source gNB and candidate gNB.

Proposal 6: Introduce LTM Candidate Cells To Be Cancelled List IE in the LTM Configuration Update/Acknowledge from new source gNB or other candidate gNB to trigger LTM candidate cells cancel.

## Issue 7: Co-existence between L3 SN change and inter-CU SCG LTM

Regarding to the R19 discussion on the co-existence between L3 HO and inter-CU LTM, below agreement is reached in the RAN2#127

L3 mobility (including both the network triggered L3 HO and CHO) can be configured to UE, while the inter-CU LTM is configured (w/o DC), and the following items can be considered (follow Rel-18 intra-CU LTM): - When performing the L3 mobility (HO or CHO), the UE does not autonomously release inter-CU LTM configurations, unless these are explicitly released by the network. - The RRCReconfiguration message to execute an L3 mobility (HO or CHO) procedure may reconfigure inter-CU LTM configurations. - For the execution order between CHO and LTM, Rel-18 principle is applied.

Let us review the R18 discussion from RAN3, the below agreements on LTM and L3 handover collision is achieved in the RAN3#121(-bis)

In case that L3 handover is triggered earlier than LTM (the gNB-DU receives the L3 handover command before LTM is triggered), L3 handover has high priority. In case that LTM is triggered earlier than L3 (the gNB-CU receives the LTM notify message from gNB-DU before L3 handover is triggered), LTM has high priority, In case that LTM and L3 handover are triggered almost simultaneously (cross signaling on F1). The (source) gNB-DU fails the L3 handover by responding with UE Context Modification Failure message with proper cause meaning LTM has high priority.

Looking through the above conclusions, one important enhancement is to introduce a cause indication of LTM command triggered in the UE Context Modification Failure message when LTM and L3 handover are triggered almost simultaneously (refer to below figure). And below procedure text is specified in TS 38.473.

“If the gNB-DU is not able to accept the UE CONTEXT MODIFICATION REQUEST message for mobility because an LTM command has been triggered to the UE, it shall reply with the UE CONTEXT MODIFICATION FAILURE message with an appropriate cause value.”

This enhancement is also applicable for the case of inter-CU LTM with SA and following scenarios on LTM with NR-DC

1. Inter-CU MCG LTM without SN release

2. Inter-CU MCG LTM with SN release

3. Inter-CU MCG LTM with SN addition

In above scenarios, when L3 HO is triggered, source gNB-CU also sends UE Context Modification Request message containing RRC Reconfiguration to source gNB-DU, in case if LTM is triggered by source gNB-DU at this moment, source gNB-DU responds with the UE Context Modification Failure message including a cause indication of LTM command triggered. It can avoid triggering simultaneous LTM and L3 HO from the view of UE.

However, regarding to R19 SN initiated inter-CU SCG LTM, LTM is triggered in source SN-DU in case S-SN is a split gNB. When L3 SN change is triggered, MN sends the RRC Reconfiguration message to UE after the completion of target PSCell resource preparation regardless SN change is initiated by MN or S-SN (refer to the message flows in below figures on SN change procedure-SN/MN initiated).

Due to different node finally sends L3 SN change command (RRC Reconfiguration message) and LTM cell switch command message to UE, it is possible for the UE to almost simultaneously receive LTM Cell Switch Command from source SN-DU and RRC Reconfiguration message from MN. In order to avoid inter-CU SCG LTM and L3 SN change collision, we propose to introduce one coordination mechanism to solve this issue.

Proposal 7: RAN3 to discuss one coordination mechanism to avoid inter-CU SCG LTM and L3 SN change triggered almost simultaneously.

## Issue 8: Reference Configuration

In last RAN3 meeting, there was no conclusion whether the source gNB can request the candidate gNB to generate reference configuration.

- Approach 1: Similar as F1AP, the source gNB can request a candidate-gNB to provide a reference configuration. [Nokia, Google, CATT, China Telecom, ZTE, Lenovo, Huawei, NTT Docomo, Samsung, CMCC]

- Approach 2: For inter-CU LTM, only the source gNB can provide the reference configuration. [Qualcomm, Ericsson]

- Approach 3: If the source CU does not provide the reference configuration, it is up to the candidate DU to provide the reference configuration to the source CU and the source DU can decide which one to adopt as a reference configuration. [LG Electronics]

Majority companies support Approach 1 while some companies think it should be only the source gNB which is in charge of generating the reference configuration. One company brought another solution to exchange the reference configuration, based on the assumption that both source gNB and candidate gNB is allowed to generate reference configuration.

As we showed our view in last meeting, we still support Approach 1 in terms of following same principle to intra-CU LTM in Rel-18.

We believe all the companies, no matter which Approach they support, have same understanding that it is source gNB who decides which gNB, source or candidate, is likely to have most common configurations, which becomes reference configuration.

When we follow Approach 1, the operation is simple and straightforward. When source think itself has the common configuration, it can simply provide the reference configuration to the candidate gNB. On the other side, if the source gNB has the thought that one of the candidate gNB has configuration in common, it will request to the corresponding gNB to generate the reference configuration.

When we go with Approach 2, when source gNB decides one of the candidate gNB might have configuration in common, there is nothing to do. Providing the reference configuration which is generated by the source gNB is meaningless in this case. When we bring extreme case, which there is no candidate cells under the source gNB, we cannot get any benefit from the reference configuration, as source gNB cannot generate the reference configuration without candidate cells with it.

With Approach 3, when source gNB does not provide reference configuration to the candidate gNB, it will always receive the reference configuration generated by each candidate gNB. To decide best one among them, the source gNB will always have to compare all the configurations. This might be helpful to achieve the better performance, but we think this cause another burden to the source gNB.

Therefore, we support Approach 1, which follows the basic principle mentioned above.

Proposal 8: RAN3 agree to allow the source gNB to request a candidate gNB to provide a reference configuration.

## Issue 9: Security Mechanism in inter-CU LTM

Regarding the Security Mechanism in inter-CU LTM, below agreement is reached in last RAN3#127bis meeting

- The new source gNB needs to provide the new KgNB*(s) to the corresponding candidate gNB via a LTM configuration update procedure. FFS on the procedure design either per cell or per gNB. - The new NCC value needs to be sent from gNB-CU to the gNB-DU via UE context modification procedure. Whether to define additional information from CU to DU to deliver NCC value.

After inter-CU LTM cell switch, the new Source gNB-CU will get new NCC value from CN via path switch request acknowledge message. Based on the agreement, new source gNB needs to generate a new KgNB* for corresponding candidate cell, and provide the new KgNB*(s) to the corresponding candidate gNB via a LTM configuration update procedure. Because it is possible that multiple LTM candidate cells exist in one candidate gNB, from the view of saving signaling, there seems to be no obstacle to triggering LTM Configuration Update procedure per-gNB.

Proposal 9-1: RAN3 to allow to use LTM Configuration Update procedure per gNB to transfer a key list (one KgNB* corresponding to one candidate cell) to corresponding candidate gNB from the view of saving signaling overhead.

Regarding the issue that whether to define additional information from CU to DU to deliver NCC value. RAN2 has achieved the below agreements:

- For security key update in inter-CU LTM, RAN2 agree to include actual NCC value in the LTM cell switch command MAC CE. - NCC is included in the LTM cell switch command MAC CE if the R19 set ID is different between the target cell and source cell. Conversely, if the R19 set ID is same for both cells, the NCC will not be included.

And furthermore, RAN2 has clarified the usage on the term “R19 set ID” from the view of UE.

The term “R19 set ID” refers to the “LTM no security change ID” configured to the UE for serving cell and candidate cells separately. RAN2 assumes that even for intra-CU LTM, the network is allowed to configure different R19 set IDs for different cells (including serving and/or intra-CU candidate cells) if the network wants to trigger security key update. From UE perspective, security key update is performed when the R19 set IDs are different between the serving cell and the target cell, whether it is intra-CU or inter-CU LTM can be transparent to the UE.

Therefore, from the view of network node, in case of split gNB, we can follow the conclusions from RAN2, the “R19 set ID” value of each LTM cell may be transferred from CU to DU, and let DU decide whether to carry NCC in the LTM cell switch command MAC CE comparing to the “R19 set ID” value between source cell and target cell.

Proposal 9-2: RAN3 to agree to transfer “R19 set ID” value of each LTM cell from CU to DU and let DU decide whether to carry NCC in the LTM cell switch command MAC CE comparing to the “R19 set ID” value between source cell and target cell.

In plus, regarding the inter-CU SCG LTM, from the UE side, RAN2 has archived below agreements on the “R19 set ID” configuration.

1. Rel-19 Set ID is configured for a candidate configuration. For DRB: - Inter-CU SCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell, > For MN terminated bearer without change of termination point, UE does not perform PDCP re-establishment. PDCP recovery is needed if it is split bearer. > For SN terminated bearer without change of termination point, UE performs PDCP re-establishment. > If there is change of termination point for a radio bearer (the keyToUse in the RadioBearerConfig is different from the keyToUse in the current UE configuration), UE performs PDCP re-establishment. 2. For SRBs in inter-CU SCG LTM, Rel-19 ID is used to determine whether PDCP re-establishment or PDCP SDU discard is performed for LTM execution for SRB3. 3. For SRB1/2 in inter-CU SCG LTM, PDCP re-established is not performed based on NW configuration (PDCP re-establishment flag and SDU discard).

- A list of sk-counters is linked to a Rel-19 set ID configured by the SN.

From the agreements, we can see that a list of sk-counters is linked to a Rel-19 set ID configured by the SN. Therefore, one SN is configured with a Rel-19 set ID, which is decided by Source SN or MN, RAN3 need FFS on this issue.

Due to the list of sk-counters is configuration by MN in the either MN-initiated or SN-initiated S-CPAC, following the similar mechanism, MN can decide the Rel-19 set ID configured for each SN.

But for the only SN-initiated inter-CU SCG LTM, another view may be existing source SN decides to configure the Rel-19 set ID for each SN. Therefore, RAN3 needs to discuss this issue.

Proposal 9-3: RAN3 to discuss the “R19 set ID“ for each SN is decided to be configured by Source SN or MN in scenario of inter-CU SCG LTM.

## Issue 10: FFS on IE including CSI-RS ReportConfig in LTM Configuration Update Acknowledge

Regarding CSI resource and CSI report configuration procedure, below agreements have been reached in RAN2/3.

CSI resource and report configuration: - The Rel-18 signaling structure for LTM CSI resource and report configuration is reused for inter-CU LTM, i.e. a common CSI resource configuration and cell-specific CSI report configuration. - The source CU is responsible to generate the common CSI resource configuration. - Current SSB information in Xn Setup and Configuration Update procedures can be reused for LTM preparation phase. - Follow F1AP, the source gNB-CU sends the CSI resource configuration of candidate cells to candidate gNB-CUs via Handover Request message for subsequent LTM, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message. - The source CU can request candidate CU to provide CSI-RS configuration in HANDOVER REQUEST message, and candidate CU signals the CSI-RS configuration in HANDOVER REQUEST ACKNOWLEDGEMENT message. - The source CU generates common CSI-RS Resource Configuration and sends it to candidate CU in LTM CONFIGURATION UPDATE message, the candidate CU signals the CSI-RS Report Configuration in LTM CONFIGURATION UPDATE ACKNOWLEDGEMENT message.

And in the current the BLCR in [2], below procedure is included on CSI resource and report configuration with highlight.

3. The source gNB requests inter-gNB LTM for one or more candidate cells belonging to one or more candidate gNB(s). The source gNB initiates a HANDOVER REQUEST message per candidate cell containing one candidate cell ID and the CSI resource configuration for subsequent LTM. The source gNB may request the candidate gNB(s) to provide the CSI-RS configuration. 4. Admission Control may be performed by the candidate gNB(s). 5. The candidate gNB(s) prepares the LTM configuration(s) and sends inter-gNB response (HO REQUEST ACKNOWLEDGE) to the source gNB including the generated RRC configurations for the accepted candidate cell. The candidate gNB(s) may also include the CSI-RS configuration upon request. 6. The source gNB sends an LTM CONFIGURATION UPDATE to the candidate gNB(s) to update the LTM configurations of candidate cell(s). The source gNB may include the common CSI-RS resource configuration. 7. The candidate gNB(s) sends the LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the source gNB. The candidate gNB(s) may also provide the CSI-RS report configuration.

Regarding the candidate gNB providing the CSI-RS report configuration via LTM Configuration Update Acknowledge in the step 7, since current the stage 3 BLCR in [2] LTM Configuration Update Acknowledge does not include corresponding IE,it is FFS on this IE for CSI-RS report configuration.

From our understanding, cell specific CSI-RS report configuration is included in the ltm-CSI-ReportConfigToAddModList IE in the CellGroupConfig IE, and in Xn interface, the inter-node RRC message HandoverCommand message includes this information. Therefore, CSI-RS report configuration information can be indicated by the HandoverCommand message in LTM Configuration Update Acknowledge.

In plus, LTM Configuration Update procedure is triggered per-cell or per-gNB, this issue needs also to be discussed and make a decision for RAN3. From the view of signaling overhead, the procedure can be designed per-gNB. In that case, a list including one or multiple cells’ CSI-RS report configuration is transferred by the LTM Configuration Update Acknowledge.

Proposal 10: RAN3 to add an IE on CSI-RS report configuration in LTM Configuration Update Acknowledge and this IE is a list which include one or more candidate cell’s CSI-RS report configuration, and each cell’s CSI-RS report configuration is included in the HandoverCommand message.

# 3 Conclusion

This contribution firstly analyses the whole message flows of inter-CU LTM and propose some enhancements for Xn interface on below aspects. Therefore, we invite RAN3 to agree below proposals for supporting the basic inter-CU LTM procedure.

## Issue 1: Early TA RACH resource allocation

Proposal 1-1: RAN3 support the early TA RACH resource allocation is based on gNB-DU in case of split gNB and/or based on gNB in case of aggregated gNB.

Proposal 1-2: RAN3 should discuss how source gNB-CU acquires the gNB-DU ID information to which inter-CU candidate cell belongs before initiation of LTM preparation.

Proposal 1-3: RAN3 should define F1 gNB-DU ID IE to be an optional IE in the Rel-19 specification on the procedure of early TA RACH resource request & allocation & TA information transfer.

## Issue 2: FFS the detail in TA INFORMATION TRANSFER message

Proposal 2-1: Propose to include an optional Early RACH Resources Requester ID information in XnAP TA INFORMATION TRANSFER message to inform source gNB-CU to forward TA information to the indicated source gNB-DU in case of split architecture for source gNB.

Proposal 2-2: No need to include the XnAP IDs information in the XnAP TA Information Transfer message, since the RACH resource information for the indicated TA value of candidate cell is used for source gNB/DU to identify the UE to which this TA value is belonging.

## Issue 3: Stage 3 IE design on Data forwarding Info in LTM Configuration Update message

Proposal 3-1: Regarding inter-CU LTM, Data forwarding address is based on candidate cell not candidate gNB.

Proposal 3-2: RAN3 to agree to use Data Forwarding Info from target NG-RAN node IE per-PDU session per-candidate cell as the included details in the Data Forwarding Info IE.

## Issue 4: FFS TA Information List IE in Cell Switch Notification message

Proposal 4: TA Information List IE can be included in XnAP Cell Switch Notification message to forward the valid TA values of candidate cells for reuse in subsequent LTM.

## Issue 5: FFS on how to deliver the old target UE XnAP ID(s) to the new serving gNB

Proposal 5: Reuse the LTM Configuration Update message to transfer old target UE XnAP ID(s) to new serving gNB when the procedure of sync up configurations is performed for subsequent LTM.

## Issue 6: Enhancement for the LTM candidate cells cancel via the procedure of LTM Configuration Update/Acknowledge

Proposal 6: Introduce LTM Candidate Cells To Be Cancelled List IE in the LTM Configuration Update/Acknowledge from new source gNB or other candidate gNB to trigger LTM candidate cells cancel.

## Issue 7: Co-existence between L3 SN change and inter-CU SCG LTM

Proposal 7: RAN3 to discuss one coordination mechanism to avoid inter-CU SCG LTM and L3 SN change triggered almost simultaneously.

## Issue 8: Reference Configuration

Proposal 8: RAN3 agree to allow the source gNB to request a candidate gNB to provide a reference configuration.

## Issue 9: Security Mechanism in inter-CU LTM

Proposal 9-1: RAN3 to allow to use LTM Configuration Update procedure per gNB to transfer a key list (one KgNB* corresponding to one candidate cell) to corresponding candidate gNB from the view of saving signaling overhead.

Proposal 9-2: RAN3 to agree to transfer “R19 set ID” value of each LTM cell from CU to DU and let DU decide whether to carry NCC in the LTM cell switch command MAC CE comparing to the “R19 set ID” value between source cell and target cell.

Proposal 9-3: RAN3 to discuss the “R19 set ID“ for each SN is decided to be configured by Source SN or MN in scenario of inter-CU SCG LTM.

## Issue 10: FFS on IE including CSI-RS ReportConfig in LTM Configuration Update Acknowledge

Proposal 10: RAN3 to add an IE on CSI-RS report configuration in LTM Configuration Update Acknowledge and this IE is a list which include one or more candidate cell’s CSI-RS report configuration, and each cell’s CSI-RS report configuration is included in the HandoverCommand message.

# 4 Reference

[1] R3-252517 (BL CR to 38.300) Support for Inter-CU LTM

[2] R3-252520	(BL CR to 38.423) Xn support for inter-CU LTM

[3] R3-252522	(BL CR to 38.473) Inter-CU LTM and intra-CU conditional LTM