3GPP TSG-RAN WG3 Meeting #128	R3-253657

Malta, MT, May 19th – 23rd, 2025

Agenda item:	9.2

Source: 	LG Electronics Inc., Huawei, ZTE

Title: 	Support for PDU session-level indirect data forwarding in split PDU session scenarios

Document for:	Discussion and decision

# Introduction

During HO/CHO with DC, the indirect data forwarding at T-MN split into CU-CP and CU-UP has been supported by introducing the Special Triggering Purpose IE onto E1AP [1] as follows:

- BRR CTXT SETUP REQ > PDU Session Resource To Setup List

- BRR CTXT SETUP REQ > PDU Session Resource To Setup List > DRB To Setup List

- BRR CTXT MOD REQ > PDU Session Resource To Setup List

- BRR CTXT MOD REQ > PDU Session Resource To Setup List > DRB To Setup List

- BRR CTXT MOD REQ > PDU Session Resource To Modify List > DRB To Setup List

In this contribution, we discuss why the same IE also needs to be introduced in the following (last) place:

- BRR CTXT MOD REQ > PDU Session Resource To Modify List

and propose to agree the corresponding TEI18 CR in [2].

# Discussion

The indirect data forwarding support during HO/CHO with DC required a special coordination between T-MN-CU-CP and T-MN-CU-UP. When T-MN decides to add a SN and host PDU session(s)/DRB(s) in the T-SN, for the purpose of relaying data to the T-SN having no direct path with the source side, RAN3 agreed to leverage the same PDU session or DRB into T-MN-CU-UP to support indirect data forwarding. The Special Triggering Purpose IE was introduced to help T-MN-CU-UP distinguish those PDU session(s)/DRB(s) are not real (virtual) and thus to avoid treating them as a regular PDU session or DRB requiring resource allocations.

The above summary effectively explains the rationale for introducing the new IE in the above placements (A) to (E). However, the story for (F) is fundamentally different.

To be more specific, the following illustrates the overall procedures and how this Special Triggering Purpose IE is used together with TNL request/provision between T-MN-CU-CP and T-MN-CU-UP:

Case (a) When T-MN decides to host some PDU sessions in its T-MN-CU-UP

Case (b) When T-MN decides all PDU sessions to be hosted in T-SN

Figure 1: Overall procedures for indirect data forwarding support during HO/CHO with DC

When T-MN receives HO request from the source, it first performs admission control on its own (step 2) and adds a target SN if needed (step 3). Once the target SN (T-SN) accepts the SN addition request, it provides the allocated data forwarding TNLs, if data forwarding has been requested from the source side and accepted. T-SN can also provide whether it has a direct path available with S-MN or S-SN. If no direct path is available, then T-MN can act as a relay for data forwarding toward T-SN and supply the forwarding TNLs allocated by T-SN to the T-MN-CU-UP, and also request to assign TNLs to receive forwarded data from the source side (step 4). The TNLs assigned by T-MN-CU-UP is delivered to the source side via HO REQ ACK (step 5). Then, indirect data forwarding works from the source side toward the T-SN via T-MN-CU-UP.

For this purpose, the Special Triggering Purpose IE was introduced in E1AP. This IE was introduced into the Bearer Context Setup procedure to support the case when T-MN-CU-CP decides all the PDU sessions to be hosted in T-SN. In this case, once SN Addition was successful, T-MN-CU-CP requests to establish a bearer context in the T-MN-CU-UP (Case (b) step 4a) and tells that PDU session(s) or DRB(s) requested to be established are not real (virtual) and only for the purpose of indirect data forwarding in the middle. The IE added onto (A) covers for a virtual PDU session where PDU-session level indirect data forwarding is needed toward the T-SN. The IE added onto (B) covers a DRB-level indirect data forwarding toward the T-SN. Once a bearer context is successfully established in T-MN-CU-UP, the subsequent Bearer Context Modification procedure supplies the destination TNLs allocated by T-SN, so that T-MN-CU-UP can map the reception TNLs assigned by itself for the virtually established PDU session(s) or DRB(s) with the corresponding destination TNLs received from T-MN-CU-CP to execute indirect data forwarding in the middle.

Note that TNLs cannot be provisioned through BRR CTXT SETUP REQ. The BRR CTXT SETUP REQ contains only the PDU Session Resource To Setup List IE, so it can only request CU-UP to assign TNLs for PDU session(s) or its underlying DRB(s) requested to be established. As a result, for case (b), two round-trips were inevitable to first establish a bearer context in CU-UP, then subsequently trigger BRR CTXT MOD REQ to provision the destination TNLs allocated by T-SN.

The Special Triggering Purpose IE was also introduced into the Bearer Context Modification procedure to support the case (a), since a bearer context has been already established in CU-UP before deciding to add a SN for the UE. The IE was added onto BRR CTXT MOD REQ > PDU Session Resource To Setup List (= 9.3.3.10 PDU Session Resource To Setup Modification List) in order to support indirect data forwarding of a PDU session entirely hosted in T-SN. This PDU session has nothing to do with T-MN, and thus the IE added onto (C) covers for a virtual PDU session where PDU-session level indirect data forwarding is needed toward the T-SN. The IE added onto (D) covers a DRB-level indirect data forwarding under that PDU session toward the T-SN. In this case, note that the same BRR CTXT MOD REQ message has to also include the PDU Session Resource To Modify List for those virtual PDU sessions requested to be established by PDU Session Resource To Setup List, in order to supply the corresponding destination TNLs allocated by T-SN.

At RAN3-124 (Fukuoka), the IE was also added onto BRR CTXT MOD REQ > PDU Session Resource To Modify List > DRB To Setup List [3]. This CR was to fix the ASN.1/tabular misalignment due to that same DRB To Setup List structure being shared for both BRR CTXT SETUP REQ > PDU Session Resource To Setup List and BRR CTXT MOD REQ > PDU Session Resource To Modify List. However, this fix was useful to cover a split PDU session scenario which had not been addressed in earlier discussions. Once T-MN decided to split a PDU session, some QoS flows within this PDU session is hosted in MN while the others hosted in SN. As in case (a), T-MN-CU-CP needs to first establish this PDU session in its T-MN-CU-UP via step 2. If T-SN accepts DRB-level data forwarding for this split PDU session (step 3), such DRB has to be virtually established in the T-MN-CU-UP for indirect data forwarding, and BRR CTXT MOD REQ > PDU Session Resource To Modify List > DRB To Setup List was the right place to flag that. The IE added onto (E) effectively covered this DRB-level indirect data forwarding of a split PDU session.

Observation 1: During HO/CHO with DC, the indirect data forwarding at T-MN split into CU-CP and CU-UP has been supported by introducing the Special Triggering Purpose IE onto E1AP as follows:

- BRR CTXT SETUP REQ > PDU Session Resource To Setup List

- BRR CTXT SETUP REQ > PDU Session Resource To Setup List > DRB To Setup List

- BRR CTXT MOD REQ > PDU Session Resource To Setup List

- BRR CTXT MOD REQ > PDU Session Resource To Setup List > DRB To Setup List

- BRR CTXT MOD REQ > PDU Session Resource To Modify List > DRB To Setup List

Observation 2: When T-MN decides to add a SN and host PDU session(s)/DRB(s) in the T-SN, for the purpose of relaying data to the T-SN having no direct path with the source side, RAN3 agreed to leverage the same PDU session or DRB into T-MN-CU-UP to support indirect data forwarding.

Observation 3: As a result, the new IE introduced in the above places was to help T-MN-CU-UP distinguish those PDU session(s)/DRB(s) are not real (virtual) and thus to avoid treating them as a regular PDU session or DRB requiring resource allocation:

- (A) and (B) apply when all PDU sessions are hosted in T-SN. Those PDU sessions (or their DRBs) requiring indirect data forwarding needs to be virtually established in T-MN-CU-UP.

- (C) and (D) apply when PDU sessions are hosted across T-MN and T-SN, and supporting PDU-session or DRB-level indirect direct forwarding of an entire T-SN hosted PDU session.  Such PDU session or DRB needs to be virtually established in T-MN-CU-UP.

- (E) enables DRB-level indirect forwarding of a split PDU session, where some QoS flows are hosted in T-MN and others in T-SN. Such T-SN terminated DRBs also needs to be virtually established in T-MN-CU-UP.

Observation 3: (E) was originally done to fix ASN.1/tabular misalignment (R3-243832), but it also proved usefulness for supporting DRB-level indirect data forwarding of a split PDU session, a scenario that had not been addressed in earlier discussions.

However, for the same split PDU session consideration, the same indication for PDU-session level indirect data forwarding was missing. The agreed R3-243832 [3] only addressed DRB-level indirect data forwarding by adding the IE only into DRB To Setup List.

One may argue that the IE into (F) is unnecessary as simply providing the destination TNL through BRR CTXT MOD REQ > PDU Session Resource To Modify List would work without any additional context.

However, looking into details, we believe that the IE is still required for (F) and the reason is as follows. For split PDU session and its PDU-session level indirect data forwarding, T-MN-CU-CP also needs to explicitly specify which QoS flows are subject to data forwarding:

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

#### 9.3.2.6	Data Forwarding Information

This IE provides the data forwarding information when performing handover or data offloading.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| UL Data Forwarding | O |  | UP Transport Layer Information 9.3.2.1 |  | - |  |
| DL Data Forwarding | O |  | UP Transport Layer Information 9.3.2.1 |  | - |  |
| Data Forwarding to NG-RAN QoS Flow Information List |  | 0..1 |  | Providing QoS flows accepted for data forwarding to the source gNB-CU-UP. | YES | ignore |
| >Data Forwarding to NG-RAN QoS Flow Information List Item |  | 1..<maxnoofQoSflows> |  |  | - |  |
| >>QoS Flow Identifier | M |  | 9.3.1.24 |  | - |  |
| PDU Set based Handling Indicator | O |  | ENUMERATED (supported, …) | Indicates the support of PDU Set based QoS handling. | YES | ignore |

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

From T-MN-CU-UP point of view, QoS flows (of a split PDU session) requested for forwarding are hosted in T-SN, not T-MN, thus never established or admitted in T-MN-CU-UP beforehand via step 2 (see case (a)). This split PDU session scenario is different to the scenarios (A) and (C) above where the entire PDU session is virtual. For (A) and (C), only QoS flows subject to indirect data forwarding can be established from the beginning in this virtual PDU session at T-MN-CU-UP. There is no need for T-MN-CU-CP to tell which QoS flows are subject to data forwarding (thus no need to bother to include Data Forwarding to NG-RAN QoS Flow Information List IE).

On the other hand, in the case of split PDU session, QoS flows subject to indirect data forwarding to T-SN are those terminated in T-SN and thus different to QoS flows hosted in T-MN. If T-MN-CU-CP doesn’t explicitly identify which QoS flows should be forwarded, it could lead T-MN-CU-UP to forward all QoS flows in this PDU session, incorrectly including those correctly hosted/admitted in T-MN-CU-UP.

However, telling which QoS flows to forward (i.e. T-SN hosted QoS flows among which T-SN accepted for PDU-session level data forwarding) is also ambiguous, because these QoS flows requested were never established or admitted in CU-UP. From CU-UP point of view, data forwarding request of QoS flows not hosted in CU-UP may be invalid, as those flows were never established or admitted in CU-UP.

Therefore, having the Special Triggering Purpose IE onto (F) can resolve the potential wrong CU-UP behavior or invalidated request. Though there is nothing virtual to be established, the IE in (F) can correctly specify T-MN-CU-UP that, in case of a split PDU session, an already established PDU session requires special handling, and requesting data forwarding of some QoS flows (not admitted within that PDU session) is for the purpose of indirect data forwarding, and thus should not be invalidated.

Observation 4: The same IE in the following placement (F) has a different story, as it supports PDU-session level indirect data forwarding for a split PDU session already established in T-MN-CU-UP. From CU-UP point of view, this PDU session is real, as it hosts some QoS flows. The IE is not to mark this PDU session as entirely virtual, but rather to indicate that special handling is required.

- BRR CTXT MOD REQ > PDU Session Resource To Modify List

Observation 5: In split PDU session cases, T-MN-CU-CP must explicitly identify which QoS flows (among those hosted in T-SN) are subject to data forwarding toward the T-SN. Failing to do so could lead T-MN-CU-UP to forward all QoS flows in this PDU session, incorrectly including those correctly hosted in T-MN-CU-UP.

Observation 6: Moreover, from CU-UP point of view, data forwarding request of QoS flows not hosted in CU-UP may be invalid, as those flows were never established or admitted in CU-UP. Without any additional context, there is a risk of potentially wrong CU-UP behavior or invalidated request.

Observation 7: Though there is nothing virtual to be established, having the Special Triggering Purpose IE onto (F) can prevent misbehaviour or invalidation. The IE can correctly specify T-MN-CU-UP that, in case of a split PDU session, an already established PDU session requires special handling, and requesting data forwarding of some QoS flows (not admitted within that PDU session) is for the purpose of indirect data forwarding, and thus should not be invalidated.

Based on the discussion in the present contribution and the observations above we propose:

Proposal: Add the Special Triggering Purpose IE into BRR CTXT MOD REQ > PDU Session Resource To Modify List IE, as provided in the CR [2].

# Conclusion

In the present contribution we make the following observations:

Observation 1: During HO/CHO with DC, the indirect data forwarding at T-MN split into CU-CP and CU-UP has been supported by introducing the Special Triggering Purpose IE onto E1AP as follows:

- BRR CTXT SETUP REQ > PDU Session Resource To Setup List

- BRR CTXT SETUP REQ > PDU Session Resource To Setup List > DRB To Setup List

- BRR CTXT MOD REQ > PDU Session Resource To Setup List

- BRR CTXT MOD REQ > PDU Session Resource To Setup List > DRB To Setup List

- BRR CTXT MOD REQ > PDU Session Resource To Modify List > DRB To Setup List

Observation 2: When T-MN decides to add a SN and host PDU session(s)/DRB(s) in the T-SN, for the purpose of relaying data to the T-SN having no direct path with the source side, RAN3 agreed to leverage the same PDU session or DRB into T-MN-CU-UP to support indirect data forwarding.

Observation 3: As a result, the new IE introduced in the above places was to help T-MN-CU-UP distinguish those PDU session(s)/DRB(s) are not real (virtual) and thus to avoid treating them as a regular PDU session or DRB requiring resource allocation:

- (A) and (B) apply when all PDU sessions are hosted in T-SN. Those PDU sessions (or their DRBs) requiring indirect data forwarding needs to be virtually established in T-MN-CU-UP.

- (C) and (D) apply when PDU sessions are hosted across T-MN and T-SN, and supporting PDU-session or DRB-level indirect direct forwarding of an entire T-SN hosted PDU session.  Such PDU session or DRB needs to be virtually established in T-MN-CU-UP.

- (E) enables DRB-level indirect forwarding of a split PDU session, where some QoS flows are hosted in T-MN and others in T-SN. Such T-SN terminated DRBs also needs to be virtually established in T-MN-CU-UP.

Observation 3: (E) was originally done to fix ASN.1/tabular misalignment (R3-243832), but it also proved usefulness for supporting DRB-level indirect data forwarding of a split PDU session, a scenario that had not been addressed in earlier discussions.

Observation 4: The same IE in the following placement (F) has a different story, as it supports PDU-session level indirect data forwarding for a split PDU session already established in T-MN-CU-UP. From CU-UP point of view, this PDU session is real, as it hosts some QoS flows. The IE is not to mark this PDU session as entirely virtual, but rather to indicate that special handling is required.

- BRR CTXT MOD REQ > PDU Session Resource To Modify List

Observation 5: In split PDU session cases, T-MN-CU-CP must explicitly identify which QoS flows (among those hosted in T-SN) are subject to data forwarding toward the T-SN. Failing to do so could lead T-MN-CU-UP to forward all QoS flows in this PDU session, incorrectly including those correctly hosted in T-MN-CU-UP.

Observation 6: Moreover, from CU-UP point of view, data forwarding request of QoS flows not hosted in CU-UP may be invalid, as those flows were never established or admitted in CU-UP. Without any additional context, there is a risk of potentially wrong CU-UP behavior or invalidated request.

Observation 7: Though there is nothing virtual to be established, having the Special Triggering Purpose IE onto (F) can prevent misbehaviour or invalidation. The IE can correctly specify T-MN-CU-UP that, in case of a split PDU session, an already established PDU session requires special handling, and requesting data forwarding of some QoS flows (not admitted within that PDU session) is for the purpose of indirect data forwarding, and thus should not be invalidated.

Based on the discussion in the present contribution and the observations above we propose:

Proposal: Add the Special Triggering Purpose IE into BRR CTXT MOD REQ > PDU Session Resource To Modify List IE, as provided in the CR [2].

# Reference

[1] TS 37.483, E1 Application Protocol (E1AP)

[2] R3-253658, “Correction on PDU session-level indirect data forwarding [Indirect Data Forwarding]”, LG Electronics Inc., Huawei, ZTE

[3] R3-243832, “Correction on indirect data forwarding [Indirect Data forwarding]”, Huawei, Nokia, Samsung, LG Electronics, ZTE, Ericsson